3GPP RAN WG2 Meeting #131                                                                                     R2-2506059

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.1.2.2

Source:	HONOR

Title:	Discussion on open issues of LCM for UE-side models (BM use case)

Document for:	Discussion and Decision

- Introduction

In RAN2#130 meeting, most of essential issues on LCM framework for UE-side model have been closed, and RAN2 made the following agreements related to RRC Open Issues 1-8.

Agreements (RRC1: Cause of inapplicability)

- Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified)

Agreements (RRC2: Content of otherConfig for enabling reports in UAI)

- Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled.  Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)

Agreements (RRC3: UE data collection request)

- The UE doesn’t need to measure the candidate data collection configuration(s).  This will be specified in RAN2 specs.

- For beam management, candidate data collection configuration includes at least:

-      CSI-ResourceConfigId of Set A

-      CSI-ResourceConfigId of Set B

-    One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements

FFS the details of how this is signalled (e.g. CSI-Report config or simplified signaling).

Agreements (RRC4: Activation of a periodic CSI report config upon change from inapplicable to applicable & RRC5: Reporting behaviour of inapplicable periodic beam prediction configuration)

- When UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

- The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable.

- The UE shall report when CSI-ReportConfig becomes not applicable.

Agreements (RRC6: Handling of AIML configurations upon release to IDLE/INACTIVE and during Reestablishment)

- On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations).  FFS Whether applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is supported (if it comes for free).

Agreement (RRC7: Applicability reporting for option B in RRCReconfigurationComplete)

- RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design.

Agreement (RRC8: Coexistence between Option A and B)

- RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

In this contribution, we will discuss the remaining issues on LCM framework for UE-side model for beam management (BM), including the data collection for UE-side model, applicability reporting and monitoring for AI-based BM.

- Discussion

  - Data collection for UE-side model

Following agreements were made for UE request of data collection measurement configuration in RAN2#129-bis meeting [1]:

RAN2#129-bis Agreement - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: - An indication on start/stop of data collection. - Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

UE can request to stop data collection based on its conditions (e.g., low power, full buffer, or lack of computing resource). It’s beneficial for gNB to know why the UE decides to stop, as this may affect the gNB’s subsequent behavior. For example, if the UE stops due to low power, the gNB may temporarily avoid assigning it further data collection tasks.  If UE lacks available CPU resources to handle the measured signals, the gNB may adjust the data collection configuration by extending the measurement period. Therefore, UE can send a request including the stop data collection indication along with a stopping cause to gNB for reference.

Proposal 1: UE can request to stop the data collection along with a stopping cause to gNB for reference.

From RAN2#129 [2], NW can stop UE from collecting data with or without UE’s request. Additional stopping conditions can be incorporated into the data collection configurations (e.g., DataCollectionPreference-r19) to ensure gNB directs UE to gather the desired data. For instance, if gNB intends for the UE to train a model for low-speed scenarios, it can configure UE to cease data collection when moving at high speeds. Upon fulfillment of the stopping conditions, the UE can transmit a UAI message (including a stop indication) to request the cessation of data collection.

RAN2#129 Agreement - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

Proposal 2: The gNB can control the UE on when to stop data collection (e.g., by configuring the stopping conditions of data collection included in DataCollectionPreference-r19) to ensure the acquisition of the desired data.

  - Applicability report

2.2.1 Configuration for applicability reporting

(RRC-40) Configuration for Option B for applicability reporting

Based on the email discussions following the RAN2#130 meeting [3], the majority of companies concur that RAN2 should confirm that, for Option B, one set or multiple sets of inference-related parameters are configured in OtherConfig.

Stemming from the agreements in RAN2#129-bis, if the list of inference-related parameters is fully contained within the existing CSI-ReportConfig, RAN2 may reconsider the use of CSI-ReportConfig for Option B. Though RAN1#121 [4] has assumed that the following parameters having been added to the CSI-ReportConfig in draft running CR can be reused for Option B of applicability check; however, it is still unclear if other parameters outside CSI-ReportConfig are needed for availability reporting. Thus, RAN2 can maintain this assumption.

Proposal 3: (RRC-40) RAN2 confirms that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for Option B.

RAN2#129-bis Agreement - RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

RAN1#121 Agreement - For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused: - For both BM-Case 1 and BM-Case 2: associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19 - For BM-Case 2: TimeGap-r19, nroftimeinstance-r19 Note: this doesn’t imply the associated ID is always present.

However, in the RAN1 LS (R1-2503242 LS on Rel-19 higher layers parameters list Post RAN1#121, with attachment R1-2503243), the RAN2 parent IE of all Rel-19 AI/ML-related higher-layer parameters is CSI-ReportConfig. This indicates that the list of inference-related parameters is fully contained within the existing CSI-ReportConfig and is common to both Option A and Option B. The subsequent question is how CSI-reportConfig is configured within OtherConfig. To address this, we propose several solutions to contain/signal the inference related parameters for Option B.

Solution 1: OtherConfig contains one or more sets of inference related parameters as a list of CSI-ReportConfigId, where one or more CSI-ReportConfig(s) indicating the inference related parameters are provided by the gNB within csi-ReportConfigToAddModList under CSI-MeasConfig. An example of ApplicabilityReportConfig-r19 is shown as below:

ApplicabilityReportConfig-r19 :: = SEQUENCE { -- Solution 1 inferenceRelatedParasList-r19 SEQUENCE (SIZE (1.. maxInferenceRelatedParas-r19)) OF CSI-ReportConfigId OPTIONAL, -- Need R }

Solution 2: OtherConfig contains one or more sets of inference related parameters as a new list of CSI-ReportConfig, i.e., creating a list of CSI-ReportConfig in OtherConfig, which is independent from CSI-ReportConfig under CSI-MeasConfig. An example of ApplicabilityReportConfig-r19 is shown as below:

ApplicabilityReportConfig-r19 :: = SEQUENCE { -- Solution 2 inferenceRelatedParasToAddModList SEQUENCE (SIZE (1.. maxInferenceRelatedParas-r19)) OF CSI-ReportConfig OPTIONAL, -- Need N }

The main benefit for Option B is to give some inference parameter suggestions for gNB reference, thereby reducing the possibility of gNB providing inapplicable full inference configurations (i.e., CSI-ReportConfig) in the RRCReconfiguration message. Ideally, the signaling overhead of step 3 for Option B is less than that of step 3 for Option A. However, solutions 1&2 still incorporate the full inference configuration into step 3 for Option B, resulting in the same signaling overhead as Option A. If so, how to distinguish Option A and Option B? Is there no need to propose Option B?

Solution 3: OtherConfig contains one or more sets of inference related parameters as a new list of CSI-ReportConfig-r19, which is different from CSI-ReportConfig under CSI-MeasConfig. Legacy CSI-ReportConfig may include mandatory parameters unnecessary for the applicability check, which can be optional in CSI-ReportConfig-r19. For example, CSI-ReportConfig-r19 only includes the parameters applied for option B of applicability check (e.g., as agreed in RAN1#121), and the remaining part of full inference configuration can be provided by gNB via the subsequent RRCReconfiguration message after initial applicability reporting. An example of ApplicabilityReportConfig-r19 is shown as below:

ApplicabilityReportConfig-r19 :: = SEQUENCE { -- Solution 3 inferenceRelatedParasToAddModList SEQUENCE (SIZE (1.. maxInferenceRelatedParas-r19)) OF CSI-ReportConfig-r19 OPTIONAL, -- Need N }

Solution 3 can reduce the signaling size of step 3 for Option B by providing the main part of full inference parameter only after UE determines the applicability status for each set of inference related parameters. However, this approach may deviate from the agreement established in RAN1#121. Alternatively, RAN2 can ask RAN1 whether the parameter used for applicability check can appear independently in OtherConfig.

Proposal 4: (RRC-40) If the assumption that one or multiple sets of inference related parameters for Option B are configured in OtherConfig is confirmed, RAN2 should discuss how OtherConfig contains CSI-ReportConfig based on the RAN1 LS R1-2503242.

Functionality identification

Recalling that the applicability/inapplicability reporting in Step 4 of Option B is for each set of inference related parameters. Thus, an index or ID (e.g., CSI-ReportConfigId) is needed to identify inference related parameters provided in Step 3.

Proposal 5: For applicability reporting under Option B, an index or ID (e.g., CSI-ReportConfigId) is introduced to distinguish each set of inference related parameters provided in Step 3.

2.2.2 Content of applicability report

Associated ID

(RRC-16) UE behaviour when the associated ID is not provided by the network

- Issue description: The UE behaviour when the associated ID is not provided by the network is not yet clarified. In the rapporteur’s view, if the associated ID is not provided by the network, the UE should still be able to provide the applicability status (applicable or inapplicable) of the configurations, which is anyway subject to UE implementation. - Proposed resolution: Adopt the proposal below. - Proposal 4 (RRC-16) If the network does not provide the associated ID, the UE the reports the applicability (applicable/inapplicable) based on the other provided information in the configuration.

Associated ID is introduced to keep the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 & BM-Case 2. If the associated ID is not provided by the network, the following alternatives for UE to judge the applicability status of the configurations were proposed by the companies in the email discussion [3]:

- Alt.1: UE optionally provides associated ID(s) to NW per configuration.

- Alt.2: UE assumes the previous/historical configured associated ID obtained in the current cell is supported (assuming that the associated ID is cell-specific).

- Alt.3: It’s up to the UE implementation to report applicability, e.g., the UE determines applicability based on other provided information.

- Alt.4: Associated ID is mandatory provided by the network, e.g., UE directly declares “inapplicability” when associated ID is missing.

In our view, Alt.1 is unfeasible since the UE may not have the capability to predict the applicable condition (i.e., associated IDs) for each configuration, but UE can provide the associated IDs of its locally stored models. Alt.2 is not applicable when the UE obtains the inference configuration without associated ID(s) in the current cell for the first time. For Alt.4, RAN1 has not yet achieved consensus on whether the associated ID is mandatory as of RAN1#121. Therefore, we prefer Alt.3 (i.e., rapporteur’s proposal) and Apple’s modification. Moreover, we kindly suggest discussing under what circumstance the associated ID is not provided by the network. We think during training, the model should always be linked to one or more associated IDs, indicating the conditions under which the training data was collected. If the model includes these IDs, the inference configuration should also include them. Otherwise, neither should contain them.

Proposal 6: (RRC-16) If the network does not provide the associated ID, it is up to UE implementation how the UE reports the applicability (applicable/inapplicable).

Inapplicability duration

(RRC-15) The time duration for an AI functionality to become available for inference when the UE reports applicability

- Issue description: The UE needs to synchronize the time duration for an AI functionality to become available for inference with the network when the UE reports applicability for either a full inference configuration or a set of inference parameters. Unlike conventional non-AI features, where the algorithm size is fixed, AI algorithms can vary significantly in size depending on their generalization performance and specific use cases. These sizes can range from several kilobytes to tens of megabytes. Additionally, UE implementations may use various types of memory to balance cost and performance, and these different memory types and constraints can affect the access speed of stored models. The UE should indicate the time duration for the AI functionality to become available when reporting applicability to the network. This time duration refers to how long it takes for the UE to load the AI models into RAM or another accessible memory for inference. By providing this information, the network can better understand the timeframe within which the configured radio resources and AI model can be utilized, enabling more efficient and effective resource management, especially for periodic CSI. Another option is to let RAN4 address this time duration in terms of requirements.

From RAN2#127 agreement [5], UE may decide its applicability based on NW-side additional condition (e.g., mapping relationship of Set A and Set B, the order of model input and output, etc), UE-side additional condition (e.g., power, buffer, etc) and model availability. Considering that UE reporting the unavailability cause related to the first two will have no impact on the gNB's behavior (e.g., if UE is in low power, gNB cannot predict when the UE will be fully charged), the “release configuration” flag introduced in RAN2#130 is believed to solely reflect the UE’s desire to release the configuration due to no available models.

RAN2#130 Agreement - Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified)

Open issue 16 considers the scenario where no model in RAM is directly available but one is still being loaded. In this case, the UE can decide whether to include the “release configuration” flag in the inapplicability report based on estimated loading time or simply report the estimated duration.

However, it may be challenging for UE to accurately estimate the loading time, as it is affected by factors such as the model size, RAM read and write speeds, the storage medium of the model and the system load. If R20 standardizes the model structure, including the model size, etc., the UE might be able to provide a more accurate estimate of the model loading time.

Proposal 7: (RRC-15) UE reporting inapplicability report with a non-availability duration is not supported in this release.

2.2.3 Consistency of Option A and Option B

For Option B, gNB will send the RRCReconfiguration message including the full inference configurations after UE reporting the applicability status of one or more sets of inference related parameters. Then, UE may be confused whether to report applicability/inapplicability (of full inference configurations) again. To avoid redundancy reports, a proposed solution is to add an indicator within RRCReconfiguration to assist UE to distinguish Option A or B the NW applies. However, we think it’s also feasible to continue the following procedures without this indicator. That is, UE takes the same behavior whatever Option A or Option B.

Our view is based on the following reasons. First, UE will inevitably send RRCReconfigurationComplete after receiving RRCReconfiguration. Thus, UE reporting applicability again will not cost significant signaling overhead. Second, considering that UE has provided some inference parameter suggestions (via applicability/non-applicability report) to gNB for reference, NW may configure fewer sets of full inference configuration(s) compared to Step 3 of Option A. Therefore, even if UE reports applicability/non-applicability again, the report content would be small (e.g., a small list of CSI-ReportConfigId for applicability/inapplicability). Moreover, though RAN1 has not decided which IEs are included in the set of inference related parameters, in our view, inference related parameters may only involve part of configuration information of Set A and B (e.g., subset of IEs in CSI-ReportConfig) or just a rough configuration reference. Thus, NW may still configure unsuitable full inference configurations even the applicability of inference related parameter is provided. Thus, it’s necessary for UE to report the applicability of CSI-ReportConfig from a functional perspective.

Proposal 8: For Option B, after receiving full inference configurations via CSI-ReportConfig in RRCReconfiguration message, UE sends RRCReconfigurationComplete message including applicability and/or inapplicability report of CSI-ReportConfig.

  - (RRC-12) Monitoring for AI based beam management

Inapplicable inference configuration and the linked monitoring behavior

RAN2#130 agreements have disclosed that when a periodic CSI-ReportConfig (inference configuration) is not applicable, gNB is expected to release the configuration. Naturally, when gNB releases an inapplicable inference configuration, the linked monitoring report configuration(s) should also be released.

RAN2#130 Agreement - When UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported). - The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable.

Proposal 9: (RRC-12) When UE indicates a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the inference configuration and the linked monitoring report configuration (if configured).

RAN1#119 agreement [6] mentioned that the monitoring report configuration reuses the legacy CSI framework and includes the ID of an inference report configuration (e.g., CSI-ReportConfigId) to indicate the correlation. From RAN1 120-bis [7], for UE-sided model monitoring Type 1 option 2, periodic inference report supports periodic, semi-persistent and aperiodic monitoring report types. If a periodic CSI-ReportConfig for monitoring is linked to a periodic CSI-ReportConfig for inference, when the inference configuration becomes inapplicable, UE cannot automatically release it until explicitly indicated by the gNB (as agreed in RAN2#130). Similarly, the linked periodic monitoring report will continue until the configuration is released.

RAN1#119 Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration. FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues. - UE measures the dedicated resource set(s) for monitoring. RAN1#120-bis Agreement For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

Proposal 10: (RRC-12) UE continues to perform the monitoring and reporting until the configuration is released.

It’s acknowledged that a monitoring report configuration can only be linked to one inference report; however, the gNB can configure multiple monitoring reports for a single inference report. When gNB instructs UE to release the CSI-ReportConfig(s) for inference configuration and linked monitoring configuration, only the CSI-ReportConfigId of the inference report configuration is included in the csi-ReportConfigToReleaseList.

Proposal 11: (RRC-12) When gNB instructs UE to release the CSI-ReportConfig for inference configuration and linked monitoring configuration, only the CSI-ReportConfigId of inference report configuration is included in the csi-ReportConfigToReleaseList.

- Conclusions

In this contribution, we discussed several remaining issues on LCM procedure of UE-sided model for AI/ML based beam management and made the following proposals:

Proposal 1: UE can request to stop the data collection along with a stopping cause to gNB for reference.

Proposal 2: The gNB can control the UE on when to stop data collection (e.g., by configuring the stopping conditions of data collection included in DataCollectionPreference-r19) to ensure the acquisition of the desired data.

Proposal 3: (RRC-40) RAN2 confirms that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for Option B.

Proposal 4: (RRC-40) If the assumption that one or multiple sets of inference related parameters for Option B are configured in OtherConfig is confirmed, RAN2 should discuss how OtherConfig contains CSI-ReportConfig based on the RAN1 LS R1-2503242.

Proposal 5: For applicability reporting under Option B, an index or ID (e.g., CSI-ReportConfigId) is introduced to distinguish each set of inference related parameters provided in Step 3.

Proposal 6: (RRC-16) If the network does not provide the associated ID, it is up to UE implementation how the UE reports the applicability (applicable/inapplicable).

Proposal 7: (RRC-15) UE reporting inapplicability report with a non-availability duration is not supported in this release.

Proposal 8: For Option B, after receiving full inference configurations via CSI-ReportConfig in RRCReconfiguration message, UE sends RRCReconfigurationComplete message including applicability and/or inapplicability report of CSI-ReportConfig.

Proposal 9: (RRC-12) When UE indicates a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration and the linked monitoring report configuration.

Proposal 10: (RRC-12) UE continues to perform the monitoring and reporting until the configuration is released.

Proposal 11: (RRC-12) When gNB instructs UE to release the CSI-ReportConfig for inference configuration and linked monitoring configuration, only the CSI-ReportConfigId of inference report configuration is included in the csi-ReportConfigToReleaseList.

- References

- RAN2#129-bis, Chair Notes.

- RAN2#129, Chair Notes.

- Email discussion - [POST130][026][AI PHY] 38.331 CR, Remaining RRC open issues in feature AIML PHY.

- RAN1#121, Chair Notes.

- RAN2#127, Chair Notes.

- RAN1#119, Chair Notes.

- RAN1#120-bis, Chair Notes.