3GPP RAN WG2 Meeting #131	R2-2506060

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.1.3

Source:	HONOR

Title:	Discussion on NW side data collection

Document for:	Discussion and Decision

1. Introduction

In Rel-19 AI/ML for NR air interface WID, the following objectives were achieved:

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models

In RAN2#130 meeting, the following agreements were made:

- Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”. - UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state - No additional signaling from the UE is required when the low power issue is resolved - No additional signaling from the UE is required when the buffer full issue is resolved

In this contribution, we discuss the remaining issues on NW side data collection.

2. Discussion

2.1 (issue RRC-21) Time related content of logged data

The issue is identified and described in the email discussion: it has not yet been clarified what information needs to be included with the logged data, to indicate a time gap between the logged data entries (i.e. a gap that is longer than the logging data periodicity). We understand that the issue is related to how to indicate the time information for logged data report with the consideration on signalling overhead.

In the previous meeting, the following agreements were achieved:

Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.

That is, if the sampling interval is larger than the configured logging periodicity, an indication of the “gap” is needed. To solve this problem, the next group of logged data should be generated. And the relative time with respect to the first group of logged data can be used to indicate this interval. It is required that the first group of logged data should be indicated in absolute time. To be precise, the timestamp indicates the absolute time or relative time of the first sample within this group. Within the logged data group, the time stamp for each sample is not needed.

Proposal 1: (RRC-21) An absolute timestamp is needed for the first logged data group, and a relative time stamp related to the first group is needed for the subsequent logged data group.

2.2 (issue RRC-29) Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

In the previous meeting, it was agreed:

- Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication

The previous agreements seem unclear. We understand the assistance information report will be triggered due to low power state, buffer state. In addition to providing the reasons for triggering the report, the availability indication of the data will also be provided in the assistance information. In the case, if the assistance information report is triggered due to low power state, but the amount of data has not reached the configured threshold value, the availability indication should be indicated to help the network fetches the buffered data.

Proposal 2：(RRC-29) The data availability indication should be sent when the UE has data below the threshold and low power state is sent.

2.3 (issue RRC-31) The release of logged AIML data in UE’s buffer

This issue is about under what conditions, the stored logged data will be released. In the email discussion, the following conditions are mentioned:

- Power off or deregistration

- 48 hrs after the release of logged measurement configuration

- Explicit indication from the serving gNB

We express our concern about the reasonability of the second condition. In the previous meeting, it was agreed that upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data. Therefore, we think it describes the condition applied to the connected state.

Generally speaking, it is a corner case that the RRC connection state is maintained for more than 48 hours. Even if the connected state is maintained for more than 48 hrs, the intention to release the logged data after such a long time is puzzling. Therefore, the time interval after the release of logged measurement configuration used to trigger the release of logged data needs further assessment.

Proposal 3: (RRC-31) The time interval after the release of logged measurement configuration used to trigger the release of logged data needs further assessment.

3. Conclusions

In this contribution, we discussed the issues related to NW side data collection. Based on the discussion, the following proposals are concluded:

Proposal 1: (RRC-21) An absolute timestamp is needed for the first logged data group, and a relative time stamp related to the first group is needed for the subsequent logged data group.

Proposal 2：(RRC-29) The data availability indication should be sent when the UE has data below the threshold and low power state is sent.

Proposal 3: (RRC-31) The time interval after the release of logged measurement configuration used to trigger the release of logged data needs further assessment.

4. References

- Remaining RRC open issues in feature AIML PHY