3GPP TSG-RAN WG2 Meeting #131	R2-2506067

Bengaluru, India, Aug. 25th – 29th, 2025

Agenda Item:	8.7.4

Source:	HONOR

Title: 	Discussion on DSR enhancements

Document for:	Discussion and Decision

1	Introduction

In the running CR discussion, critical issues were discussed and closed. Meanwhile, there are some left issues need to be discussed through on-line meeting. In this contribution, the left issues are discussed.

2	Discussion

(MAC-1)	Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

In the current running MAC specification, a PDCP SDU is associated with a DSR if its remaining time is lower than remainingTimeThreshold (triggering threshold). And the enhanced DSR can be cancelled when all the PDCP SDUs associated with the DSR have been discarded. Due to new reporting threshold which can be larger than remainingTimeThreshold is introduced, a MAC issue was raised for whether to cancel the DSR when there is left PDCP SDU whose remaining time is between remainingTimeThreshold and the largest configured reporting threshold. In this case, these data can be considered as non-urgent data based on the NW implementation. Meanwhile, the new reporting threshold is handled independently for reporting purposes and cannot alter DSR association or cancellation. Therefore, no further enhancement is needed.

Proposal 1: (MAC-1) Reusing existing DSR association and cancellation.

(RLC-13) Whether the delay-reporting or non-delay-reporting information is visible or not to RLC?

The RLC entity cannot distinguish whether the SDU is non-delay-reporting RLC SDU based on the PDCP indication. In the PDCP specification, the indication is as following [1]:

If dsr-ReportNonDelayCriticalData is configured, the transmitting PDCP entity shall further consider the following as delay-reporting PDCP data volume associated with the i:th dsr-ReportingThreshold: - the non-delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i; - the PDCP Data PDUs that contain the non-delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i. The transmitting PDCP entity provides a delay-reporting indication associated with the i:th dsr-ReportingThreshold for the PDCP Data PDU to lower layers when: - the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold; or - the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold.

If configured, the PDCP entity has already considered the non-delay-reporting PDCP SDU as delay-reporting PDCP data volume associated with the i:th dsr-ReportingThreshold. After receiving the above delay-reporting indication, the RLC entity can consider the non-delay-reporting PDCP SDU in the data volume calculation. Therefore, the RLC entity does not need to distinguish whether the SDU is a non-delayed reporting RLC SDU.

Proposal 2: (RLC-13) Non-delay-reporting information is not visible to RLC.

3	Conclusions

Proposal 1: (MAC-1) Reusing existing DSR association and cancellation.

Proposal 2: (RLC-13) Non-delay-reporting information is not visible to RLC.

4	References

[1] PDCP running CR for R19 XR.