3GPP TSG-RAN WG2 #131	R2-2506096

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.1.2.2

Source:	TCL

Title:	LCM for UE-side models for beam management

Document for:	Discussion and Decision

1	Introduction

At the RAN2#130 meeting, RAN2 achieved the following agreements regarding LCM for UE-sided model for the beam management use case [1].

Agreements - Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified) - Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified) - The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. 4 For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling) Agreements 1. When UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported). 2. The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable. 3. The UE shall report when CSI-ReportConfig becomes not applicable Agreements - (RRC 7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design. - RRC6) On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations). FFS Whether applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is supported (if it comes for free). - (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure

In this contribution, we further discuss the remaining open issues on LCM of UE-sided models for beam management use case.

2.1 Discussion

This section discusses the remaining RRC open issues from [2] related to agenda item 8.1.2.2, as identified in the rapporteur’s list following the email discussion [POST130][026][AI PHY] 38.331 Running CR (Ericsson).

RRC-11: How to configure RS configuration for UE sided data collection within CSI-ReportConfig

Issue description: According to RAN1 agreements, a CSI-ReportConfig without an actual CSI report can be configured for UE data collection purposes. Since the reportConfigType field is mandatory in CSI-ReportConfig, further discussion is needed on how to handle this situation. For example, instructing the UE to ignore the legacy reportConfigType could be considered. Proposed resolution: It is suggested to address this open issue directly in the RRC running CR, where companies can provide comments to the updated text provided by the rapporteur.

It was noted by RAN2 that the reportConfigType field in CSI-ReportConfig is mandatory for UE-side data collection, regardless of whether CSI reporting is required or not.

The following was agreed in RAN1#121 [3].

Agreement For data collection for UE-sided model, in CSI-report configuration, reportQuantity is set to “none-BM-r19”.

Regarding the RAN1 conclusion, it was agreed to introduce a new value "none-BM-r19" in the reportQuantity field of CSI-ReportConfig. We understand this value explicitly indicates that the CSI-ReportConfig is configured solely for UE-side model data collection purposes. Accordingly, when a UE receives a CSI-ReportConfig with reportQuantity = none-BM-r19, it shall perform local measurement or data collection only and not produce any actual CSI report.

Based on the above, two possible solutions are identified to handle the case where CSI-ReportConfig is configured but no actual CSI report is required:

- Option 1: Add a new value none-reporting under reportConfigType to explicitly indicate no actual CSI report for UE-side data collection.

- Option 2: Use the none-BM-r19 in reportQuantity to indicate no actual CSI report for UE-side data collection.

We slightly prefer Option 2 due to its simplicity and minimal impact on the current specification. This option only adds a value to the reportQuantity IE and does not require any structural changes to CSI-ReportConfig.

We suggest that RAN2 consider the RAN1-agreed none-BM-r19 in the CSI-ReportConfig to explicitly prevent an actual CSI report.

RRC-12: Monitoring for AI based beam management

Issue description: RAN1 has made progress related to the monitoring framework, i.e., Type 1 Option 2 UE assisted monitoring wherein UE will report the monitoring metric to gNB. The RRC impact can be analysed upon RAN1 conclusion. Proposed resolution: It is suggested to address this open issue directly in the RRC running CR, where the companies can provide comments to the updated text (i.e. inclusion of the RAN1 parameters for monitoring) provided by the rapporteur. If the companies identify conceptual/more general issues relates to monitoring, it is suggested that they discuss them in contributions.

For the Type 1 Option 2 of UE-side model monitoring, RAN1 has achieved the following agreements.

Agreement (RAN 1 #119) At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1signaling: Agreement (RAN 1 #120bis) For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 FFS: detailed rule or signalling

Based on the previous agreement, it is clear that RAN1 has discussed reusing the legacy CSI framework for performance monitoring configuration and reporting, as well as defining some detailed performance metrics.

From RAN2 point of view, we believe it is necessary to discuss the reporting type for performance metrics reporting. This is because performance metrics should be calculated by aggregating measurements across multiple time instances, rather than relying on a single measurement, to ensure the accuracy of the performance monitoring. Therefore, to reduce the signaling overhead, we propose that event-triggered reporting of monitoring reporting should be considered.

For performance monitoring, event-triggered reporting shall be supported

RRC-13: CSI prediction LCM framework

Issue description: For the CSI prediction use case, it is very likely the same framework (e.g., applicability report, inference configuration, data collection, monitoring) for AI based beam management can be used, but still it is upon RAN1/RAN2’s confirmation. From the rapporteur’s perspective it is overall straightforward to adopt the same LCM framework for the CSI prediction use case as for beam management. However, a few details need to be clarified for UE-side data collection, for the CSI prediction use case. From the rapporteur’s point of view, the procedures for UE data collection such as start/stop request, candidate configuration provision, request for preferred candidate configurations, etc. should be the same as for beam management. If this is acceptable, RAN2 needs to discuss what the content of the candidate configurations is, for the CSI prediction use case. Proposed resolution: Adopt the proposal below. Further proposal(s) can be made by the rapporteur after collecting input from the companies in the table below. Proposal 2 (RRC-13) The procedures for UE data collection for the CSI prediction use case are the same as for the beam management use case (start/stop request from UE, candidate configuration provision from NW, request for preferred candidate configurations from UE, etc.).

RAN1#121 agreed about the CSI prediction as follows.

Agreement For CSI prediction using UE-side model, for data collection for training, reportQuantity is set to ‘none-CSI-r19’.

RAN1#120bis agreed about the CSI prediction as follows.

Agreement For CSI prediction using UE-side model, at least for inference, introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction. - Note: terminology of “AI-CSI prediction” and “non-AI CSI prediction” is separate discussion - Detailed parameter name is upto RAN2

For the CSI prediction use case, reusing the legacy CSI framework for UE-side data collection is natural and reasonable. Therefore, general agreements on LCM aspects reached for the beam management use case are also applicable to the CSI prediction use case unless otherwise specified.

From our perspective, it is straightforward to introduce a new parameter (e.g., reportQuantity-CSI-r19) in CSI-ReportConfig representing the reporting quantity for CSI prediction. This would enable unambiguous differentiation from other reporting quantities. The detailed design of related parameters (e.g., csi-pai-r19, none-csi-r19) is left to RAN1.

Introduce a new parameter in reportQuantity-CSI-r19 CSI-ReportConfig to represent the reporting quantity for CSI prediction use case.

RRC-16: UE behaviour when the associated ID is not provided by the network

Issue description: The UE behaviour when the associated ID is not provided by the network is not yet clarified. In the rapporteur’s view, if the associated ID is not provided by the network, the UE should still be able to provide the applicability status (applicable or inapplicable) of the configurations, which is anyway subject to UE implementation. Proposed resolution: Adopt the proposal below. (RRC-16) If the network does not provide the associated ID, the UE the reports the applicability (applicable/inapplicable) based on the other provided information in the configuration.

In RAN1#119 meeting [4], RAN1 agreed on following for the associated ID in Step3

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

In the lasted RAN1#121 meeting, RAN1 further agreed on the following

Agreement For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused: - For both BM-Case 1 and BM-Case 2: - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19 - For BM-Case 2: - TimeGap-r19, nroftimeinstance-r19, - Note: this doesn’t imply the associated ID is always present

Regarding the recent RAN1 meeting progress, it was noted that whether the associated ID in Step 3 is mandatory or optional still remains FFS. Accordingly, RAN1 has not further discussed UE behavior with respect to the associated ID.

From our perspective, we prefer that the provision of the associated ID by the network be optional. First, it is widely recognized that the associated ID is a key factor in determining the applicability of a functionality. Second, in past RAN2 discussions, it took substantial time to reach agreement that both the UE and the network can determine the applicability, and no clear drawbacks to these approaches have been identified. Therefore, given that the mandatory/optional status of the associated ID is still unclear and to avoid spending additional time on this issue, we suggest that RAN2 assume the associated ID is optional in Step 3.

RAN2 to assume that associated ID is optional provided in Step 3.

If the associated ID is not provided by the network, the UE may determine the applicable functionality based on its internal conditions and other provided information from network. Then the UE shall to report the corresponding applicable functionality information in response to the Step 3 applicable functionality configuration.

The UE shall report the applicable functionality information in response to the Step 3 applicable functionality configuration.

3	Conclusion

Based on the discussion in the previous sections we propose the following:

- We suggest that RAN2 consider the RAN1-agreed none-BM-r19 in the CSI-ReportConfig to explicitly prevent an actual CSI report.

For performance monitoring, event-triggered reporting shall be supported

Introduce a new parameter in reportQuantity-CSI-r19 CSI-ReportConfig to represent the reporting quantity for CSI prediction use case.

RAN2 to assume that associated ID is optional provided in Step 3.

The UE shall report the applicable functionality information in response to the Step 3 applicable functionality configuration.

References

RAN2 130 Chair Notes.

R2-25xxxxx, “RRC open issues AIML PHY”, 3GPP TSG RAN WG2#131

RAN1#121 Chair Notes.