3GPP TSG-RAN WG2 #131	R2-2506109

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	8.1.3

Source:	Ericsson, Nokia, Huawei, HiSilicon, T-Mobile, BT Plc, Verizon, TIM, AT&T, Deutsche Telekom, LG Electronics

Title:	Discussion on NW-side data collection framework

Document for:	Discussion

# 1	Introduction

In RAN2#129bis it was discussed which framework to use for the radio configuration of NW-side data collection, with the following outcome:

Data collection configuration (Framework): Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.

At the RAN2#130 meeting two different options for the framework were discussed, namely based on L1 or L3. It was agreed that the data logging should, as a starting point, be captured in the RRC specification. A post meeting email discussion (“[POST130][031][AI PHY] NW side data collection”) was started to discuss the two different approaches for the data logging and configuration, with associated Text Proposals to RRC. Impacts to RAN1 and RAN3 were also part of the email discussion. This contribution discusses these two approaches.

The following agreements are relevant for this contribution:

- From RAN2#129: Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log - From RAN2#129-bis: The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc) - From RAN2#130: As a starting point, the data logging is captured in RRC specs.

# 2	Discussion on framework for NW-side data collection for beam management

In the email discussion [POST130][031][AI PHY] NW side data collection ([2]), two alternative approaches for introducing data logging and associated configuration for the beam management use case in RRC were discussed, including impact to RAN1 and RAN3:

- The logging configuration is introduced within the L1 CSI measurement framework, in a new list of configurations (CSI-LoggedMeasurementConfig-r19) within CSI-MeasConfig.

- The logging configuration is introduced in a new L3 measurement framework, in a new structure directly under the RRCReconfiguration message (LoggedDataCollectionConfig-19). The LoggedDataCollectionConfig-19 in turn includes a list of CSI measurement resources (BM-DataMeasResource-r19), a list of logging related configurations (BM-LoggingConfig-r19) and a list of configurations to link those two (LoggedDataCollectionLinkage-r19).

In the following sections we will go through the two approaches, and address comments provided in the email discussion [POST130][031][AI PHY] NW side data collection ([2]).

## 2.1	L1 based measurement configuration for data collection (approach 1)

An alternative solution for NW-side data collection configuration is to reuse the L1 CSI framework to configure the UE with data collection, approach (1) with associated Text Proposal (TP) in the email discussion “[POST130][031][AI PHY] NW side data collection”. It consists of that a new list of configurations (CSI-LoggedMeasurementConfig-r19) is included in CSI-MeasConfig. This new CSI-LoggedMeasurementConfig-r19 includes both the CSI resource configuration for the measurements and the logging configuration. As already described in the previous section, in order to configure the UE to perform the L1 measurements, there is then no need to include the related L1 measurement configuration into the L3 configuration. Even if the network wants to configure the L3 events for triggering the L1 data collection, the UE already knows the serving cell MO to use for the L3 measurements (that is included in the serving cell configuration together with the L1 CSI measurement configuration), and the gNB-DU is already aware of the serving cell MO because, according to the existing F1AP interface ([1]), that is already exchanged between the gNB-CU and gNB-DU within the cell group configuration (see CU to DU RRC Information IE, and DU to CU RRC Information IE, as defined in TS 38.473).

With approach (1) the current ASN.1 signalling architecture is maintained by letting L1 measurement configuration be included in CSI-MeasConfig (handled by the DU) and the L3 measurement configuration in the L3 framework (handled by the CU).

Based on legacy RRC and F1AP specifications, the UE is already aware of the serving cell MO to use for L3 measurements (in case the network wants to configure the UE with L3 event based L1 data collection), as well as the gNB-DU.

With approach (1) the current ASN.1 signalling architecture is maintained by letting L1 measurement configuration be included in CSI-MeasConfig (handled by the DU) and the L3 measurement configuration in the L3 framework (handled by the CU).

## 2.2	L3 based measurement configuration for data collection (approach 2)

The solution to provide the logging configuration in a new L3 measurement framework, approach (2) with associated Text Proposal (TP) in the email discussion “[POST130][031][AI PHY] NW side data collection”, consists of that the logging configuration is introduced in a new structure in the RRCReconfiguration message (LoggedDataCollectionConfig-19). This LoggedDataCollectionConfig-19 in turn includes a list of CSI measurement resources (BM-DataMeasResource-r19), a list of logging related configurations (BM-LoggingConfig-r19) and a list of configurations to link those two (LoggedDataCollectionLinkage-r19).

In the following, we highlight our concerns related to the inclusion of the L1 measurement configuration into the L3 measurement configuration for the purpose of NW-side data collection configuration:

- Change to current handling of the L1 measurement configurations at the gNB-CU: The L3 measurements are configured by the gNB-CU, whereas L1 measurements are configured by the gNB-DU. In legacy, the gNB-CU is not supposed to open and/or modify the L1 measurement configuration. In order to include the L1 configuration into the L3 configuration (i.e. the new BM-DataMeasResource), the gNB-CU would be required to read the configuration sent by the gNB-DU and to incorporate it into the L3 configuration. Additionally, since the L1 CSI measurement configuration may also be used for legacy BM configurations, the gNB-CU would also need to discriminate the L1 CSI measurement configuration and determine whether a received L1 measurement configuration coming from the gNB-DU contains a data collection configuration or not.

- Impact on the signalling framework: The motivation of having a L1/L3 unified framework for the AI/ML logging configuration is not consistent with the current ASN.1 signalling architecture, in which the L1 and L3 measurement configurations are natively split.
Introducing the proposed L3 structure in approach (2) adds redundant signalling and introduces many links to IEs within the CSI measurement framework, which adds complexity in both specifications and implementations.

- Supporting of dynamic activation/deactivation of NW-side data collection configurations: Even if it is FFS whether the dynamic activation/deactivation is supported, we believe that keeping the data collection configuration into the L1 CSI measurement configuration would facilitate the introduction of such a feature, because MAC already supports the dynamic activation/deactivation of L1 measurements and related reports, whereas such a feature does not exist for the L3 measurements. We note that this feature can be beneficial to reduce the overhead of continuous reconfiguration of periodic configurations, and also to minimize the collection of redundant data, because the gNB would be able to dynamically decide based on existing measurement reports whether some data collection is needed or not.

Given the above analysis, we observe and propose the following:

Including the L1 CSI resource configuration into an L3 configuration for the NW side data collection raises the following concerns:

  - Unclear benefits, given the current mechanisms for the gNB-DU and gNB-CU to configure the L1 CSI measurements, and the L3 measurements respectively, for a serving cell

  - Added complexity to the gNB-CU, to change the handling of L1 measurements configurations received from the gNB-DU

  - Added complexity to the signalling framework, given the many links between the new L3 structure and the existing L1 CSI measurement framework.

  - Added complexity if dynamic activation/deactivation of NW-side data collection configurations is supported

Some companies commented in the email discussion “[POST130][031][AI PHY] NW side data collection” that approach (2) would be more future proof. In the TP for approach (2) the following new IEs are introduced:

- BM-DataLoggingConfig

- BM-DataLoggingConfigId

- BM-DataMeasResource

- BM-DataMeasResourceId

These are all use case specific IEs (for beam management) which cannot be reused for other use cases. We would then have to introduce new such IEs for AI/ML for Mobility in Rel-20.

One could perhaps argue that the LoggedDataCollectionConfig can be reused, but it includes the above use case specific resource and logging configurations, which cannot be reused. The only thing within LoggedDataCollectionConfig that can be reused is the “linkage configuration” (LoggedDataCollectionLinkage). This is however something that is not needed for configuration of network data collection. It is argued that it will reduce the RRC overhead, by associating multiple measurement resource configurations with one logging configuration. With the additional configuration needed to support such mapping, it will however cause more overhead in most configurations. It is not needed at all with approach (1).

It can also be noted that for features where network data logging is to be introduced in future releases, the new L3 structure proposed in approach (2) may not be the best option. It may e.g. be more suitable to include it within the MeasConfig in case they are for L3 measurements (as for AI/ML for Mobility).

We do thus not consider approach (2) to be more future proof than approach (1).

There is very limited possibility to reuse any of the new IEs in the approach (2) TP for future features.

It is not clear that the new structure in the approach (2) TP is suitable for future features, e.g. AI/ML for Mobility.

Approach (2) is not more future proof than approach (1).

## 2.3	Comparison of RAN1 impacts for the two approaches

In the email discussion [POST130][031][AI PHY] NW side data collection it was commented by a few companies that there would be more RAN1 impact for the solution in approach (1) than for the solution in approach (2). The argument used is that since the new logging configuration in approach (1) is within CSI-MeasConfig, RAN1 could think that they should specify the logging in their specifications. RAN1 would however, independent of the agreed approach, be informed about the RAN2 agreement to capture the logging in RRC. It can also be noted that for the L1 event triggered reporting that is introduced for Rel-19 LTM, the event configuration is within CSI-MeasConfig but the associated event evaluation is specified in the MAC specification (38.321).

According to one comment in the email discussion, the calculation of occupied CSI Processing Unit (CPU) for the UE would also need to be discussed by RAN1 for approach (1) whereas it is claimed that approach (2) doesn’t occupy CPU due to that the UE can handle it “similar to L3 RRM”. However, independent of the approach, the UE will perform L1 measurements and the UE behavior for performing those does not depend on where the configuration is included, i.e. it is the same for both approaches. The logged data will also in both cases be reported to the network via an RRC message (UEInformationResponse). The impacts to RAN1 should thus not differ between the two approaches and they should anyway be minor.

To include the logging configuration within CSI-MeasConfig does not mean that there is any RAN1 impact for specification of logging procedures

The UE behavior for performing L1 measurements does not depend on where we include the configuration in RRC

The RAN1 impact should be minor for both approaches.

## 2.4	Comparison of RAN3 impacts for the two approaches

Two companies claimed in the email discussion that there would be more RAN3 impact for approach (1) than for approach (2). This was based on a comparison of 4 aspects:

- CU-DU interaction for configuring the event (for event triggered logging)

- CU-DU interaction for configuring the measurement resources

- CU-DU interaction for retrieving logged data

- CU-DU interaction for de-configuring logging configurations upon low power state indication

It is commented that approach (1) would have RAN3 impact for a), c) and d) whereas approach (2) only has RAN3 impact for b). We here analyze these impacts.

For a) “CU-DU interaction to configure the event (for event triggered logging)”, it was commented that there is, for approach (1), a need to send the L3 MO parameters (MeasObjectNR) of the ServingCellMO as well as parameters for configuration of the event from the CU to the DU (threshold and timeToTrigger).

It is not true that there is any impact for transmission of the measurement object for the serving cell, since it is already known by the gNB-DU. The MeasConfig is present in the UE Context Setup Request message that is sent from the CU to the DU. It can be noted that the measurement object for the serving cell would anyway not really be needed by the DU to set the event configuration.

There is also no need to associate the L1 CSI measurement configuration related to a certain serving cell with a measurement object (i.e. MeasObjectNR), because the measurement object associated to such cell is already known by the UE from the serving cell configuration.

Assuming that the CU is in control of the configuration of events for start/stop of the data logging (i.e. threshold, timeToTrigger and possibly hysteresis), these would need to be conveyed to the DU. This would then typically be part of the CU-DU signalling to trigger the logging configuration, which is needed for both approaches.

For b) “CU-DU interaction for configuring the measurement resources”, for approach (2) the CU needs to include the CSI resource configuration that the UE is to perform the L1 measurements on in the new L3 structure. There is then an impact due to that the DU needs to provide this configuration of CSI resources to the CU. For approach (1) the corresponding configuration is included in the configuration by the DU and does thus not need to be visible to the CU.

For c) “CU-DU interaction for retrieving logged data”, it was commented that (for approach (1)) the DU needs to send the CSI-MeasConfig to the CU so that the CU would be able to understand the data that is reported by the UE in the UEInformationResponse message. This is however not true since the CU can understand the data that is received from the UE and it does not need to understand the related configuration.

The commented impacts for d) “CU-DU interaction for de-configuring logging configurations upon low power state indication” was that in case the UE has a low power state there would be a need to send an indication to request the DU to de-configure the data collection configuration and release CSI resource. The CSI resources are handled by the DU, which then knows if/when it is possible to stop transmissions on any of them due to that a UE would not perform data collection. This is then common for both approach (1) and approach (2) that the CU would need to inform the DU if data collection configuration should be de-configured for a UE.

All in all, it can be observed that, based on the 4 aspects, approach (1) thus has RAN3 impact for aspects a) and d) whereas approach (2) has RAN3 impact for aspects b) and d). For both approaches there would be an interaction to trigger the logging configuration. Any event configuration from CU to DU (for aspect a) for approach (1)) would then typically be included in that signalling to trigger the logging configuration. Based on this, it should be at least as much RAN3 impact for approach (2) as for approach (1).

There is at least as much RAN3 impact for approach (2) as for approach (1)

## 2.5	Summary

Unlike approach (2), the approach (1) adheres to the existing signalling framework with a clear split between L1 and L3 configurations. As has been discussed above, RAN1 impact should be the same for both approaches and the RAN3 impact for approach (2) should be at least the same as for approach (1). It has also been discussed above that approach (2) is not more future proof than approach (1). Finally, approach (2) has much bigger impact on specifications and increases implementation complexity while approach (1) achieves the same goal with significantly less specifications/implementation impacts.

Based on this we propose to include the L1 CSI measurement configurations related to NW-side data collection within the CSI-MeasConfig, based on the TP for approach (1).

The L1 CSI measurement configurations related to NW-side data collection are included within the CSI-MeasConfig, based on the TP for approach (1) in the “[POST130][031][AI PHY] NW side data collection” email discussion.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Based on legacy RRC and F1AP specifications, the UE is already aware of the serving cell MO to use for L3 measurements (in case the network wants to configure the UE with L3 event based L1 data collection), as well as the gNB-DU.

Observation 2	With approach (1) the current ASN.1 signalling architecture is maintained by letting L1 measurement configuration be included in CSI-MeasConfig (handled by the DU) and the L3 measurement configuration in the L3 framework (handled by the CU).

Observation 3	Including the L1 CSI resource configuration into an L3 configuration for the NW side data collection raises the following concerns:

a.	Unclear benefits, given the current mechanisms for the gNB-DU and gNB-CU to configure the L1 CSI measurements, and the L3 measurements respectively, for a serving cell

b.	Added complexity to the gNB-CU, to change the handling of L1 measurements configurations received from the gNB-DU

c.	Added complexity to the signalling framework, given the many links between the new L3 structure and the existing L1 CSI measurement framework.

d.	Added complexity if dynamic activation/deactivation of NW-side data collection configurations is supported

Observation 4	There is very limited possibility to reuse any of the new IEs in the approach (2) TP for future features.

Observation 5	It is not clear that the new structure in the approach (2) TP is suitable for future features, e.g. AI/ML for Mobility.

Observation 6	Approach (2) is not more future proof than approach (1).

Observation 7	To include the logging configuration within CSI-MeasConfig does not mean that there is any RAN1 impact for specification of logging procedures

Observation 8	The UE behavior for performing L1 measurements does not depend on where we include the configuration in RRC

Observation 9	The RAN1 impact should be minor for both approaches.

Observation 10	There is at least as much RAN3 impact for approach (2) as for approach (1)

Based on the discussion in the previous sections we propose the following:

Proposal 1	The L1 CSI measurement configurations related to NW-side data collection are included within the CSI-MeasConfig, based on the TP for approach (1) in the “[POST130][031][AI PHY] NW side data collection” email discussion.

# 4	References

- 3GPP TS 38.473 F1 application protocol (F1AP), v18.5.0

- R2-2505859, Report of email discussion [POST130][031][AI PHY] NW side data collection, Ericsson, ZTE Corporation, 3GPP RAN2#131, Bengaluru, India, 25th – 29th August 2025