3GPP TSG-RAN WG2 Meeting #131	R2-2506117

Bangalore, India  Aug 25th – 29th , 2025

Agenda item:	8.1.2.2

Title:	Further Discussion on LCM for AIML based beam management

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#130, following agreements were reached[1]

Agreements - Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified) - Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified) - The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. 4 For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)

Agreements

When UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable.

The UE shall report when CSI-ReportConfig becomes not applicable

Agreements

- (RRC 7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design.

- (RRC6) On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations).  FFS Whether applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is supported (if it comes for free).

- (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

Further consideration based on previous agreements and post email discussion is provided in this contribution.

Discussion

RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability

As the Rapp indicated these sizes can range from several kilobytes to tens of megabytes. Additionally, UE implementations may use various types of memory to balance cost and performance, and these different memory types and constraints can affect the access speed of stored models.

It seems reasonable for the UE to indicate the time duration for an AI functionality to become available for inference when the UE reports applicability. However, we think it is reasonable to have RAN4 to evaluate the possible range of the time duration requirement before RAN2 take any actions.

Proposal 1: (RRC-15) The possible range of the time duration for an inference configuration to become available for inference should be first evaluated by RAN4 before RAN2 take further actions.

RRC-16: UE behaviour when the associated ID is not provided by the network

This issue is related to the UE behaviour when the associated ID is not provided by the network in inference configuration. The argument point is whether UE need to report all applicable associated ID or UE determines the applicability when associated ID is not provided by the network.

First of all, we needs to clarify that the UE side model is trained based on the data collection from the serving cell with data collection configuration which is configured with Associated ID. Thus the UE links the model to associated ID of the collected data set. In other words, it is the network configured the associate ID for UE to perform data collection. So why network will not provide associated ID in the inference configuration? We can not understand why network provide it in inference configuration but provide it in data collection configuration.

Observation 1: (RRC-16) It is the network configures the associate ID for UE side model data collection. The network has no reason not to provide Associated ID in inference configuration but provide it in data collection configuration.

Some companies may argue that the network may want to gather all UE supported associated ID related to the inference configuration. However we think it is totally unnecessary for network to do do. Remember that the NW can configure a list of inference configuration with associated ID or a list of inference related parameter sets with associated ID to UE. In this way, the NW will know for what associated ID the UE has a valid model. This is more straightforward and already supported.

Observation 2: (RRC-16) Network already can configure multiple inference related parameter sets with Associated ID to the UE to request for applicability reporting. There is no need to have another method to gather applicability reporting.

On the other hand, if network does not provide the associated ID in inference configuration, UE can just report inapplicable to the network. Even UE reports applicable status to network, the performance monitoring mechanism will work.

Proposal 2: (RRC-16) No enhancement is pursued for the case that associated ID is not provided by the network in either inference configuration or inference related parameter set. UE determine the applicability according to its implementation if associated ID is not provided by network.

RRC-42: Whether applicability reporting comes for free in RRCReestablishmentComplete and RRCResumeComplete

2 (RRC6) On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations). FFS Whether applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is supported (if it comes for free).

In legacy behavior, NW is able to indicate the UE to restore MCG SCell and SCG and reconfigure them in RRCResume. Therefore, the NW can configure inference configuration in RRCResume for both MCG and SCG serving cells for free because inference configuration is included in servingCellConfig IE. We think there is no reason not to support it.

As to the applicability reporting in RRCResumeComplete, we have similar understanding to the rapporteur. RRCResumeComplete already support applicability reporting for SCG serving cells. For MCG serving cell, the same text procedure that is captured in RRCReconfigurationComplete can be reused in RRCResumeComplete to minimize the specification effort.

Observation 3: (RRC-42) NW is already able to reconfigure inference configuration for MCG/SCG serving cells for free because inference configuration is included in servingCellConfig. The text procedure captured for RRCReconfigurationComplete can be reused in RRCResumeComplete to support applicability status reporting of the inference configuration configured for MCG serving cell.

Proposal 3: (RRC-42) NW can reconfigure inference configuration for both MCG and SCG serving cells. Applicability status of inference configuration is added in RRCResumeComplete for both.

As to the activation of the inference configuration with periodic CSI reporting that is configured in RRCResume, the same handling as in RRCReconfiguration can be reused. I.e. the UE activate the applicable inference configuration upon reporting its applicability status via RRCResumeComplete.

Proposal 4: (RRC-42) If inference configuration with periodic CSI reporting is configured in RRCResume, the UE autonomously activate the applicable inference configuration upon reporting applicability status via RRCResumeComplete.

Another related issue is the activation status of the restored inference configuration with periodic CSI reporting. Considering the applicability status may be changed compared to UE is transited to RRC_INACTIVE, there are two approach to handle the activation status.

- Option1: To follow the activation status of the inference configuration as the UE is transited to RRC_INACTIVE. Thus even applicability status changes compared to the UE is transited to RRC_INACTIVE, the UE will NOT activate/deactivate it autonomously. Instead, the UE report the applicability status to NW via RRCResumeComplete. This is same as the handle for the applicability change when UE is kept in RRC_CONNECTED.

- Option 2: To activate the inference configuration according to the applicability status upon RRC resumption. I.e. UE activate autonomously inference configuration with periodic CSI reporting if UE thinks it is applicable and report applicability status via RRCResumeComplete. This is similar to RRC configuration procedure.

Option 1 is more straight forward and aligned with already agree UE behavior upon change of the applicability status. However, it requires the NW to remember applicability status and activation status of each inference configuration.

Option 2 is also acceptable because it is same as the configuration via RRCReconfiguration. With this option the NW is not required to remember the activation status of inference configuration. But it is unclear whether there is issue if the inference configuration is not activated and there is no legacy CSI reporting is configured.

Based on above, we propose RAN2 to discuss and decide the UE behavior regarding the restored inference configuration with periodic CSI reporting.

Proposal 5: (RRC-42) RAN2 to discuss and decide one of following options regarding activation of the restored inference configuration with periodic CSI reporting:

Option 1: To follow the activation status of the inference configuration as the UE is transited to RRC_INACTIVE. UE activates autonomously the inference configuration with periodic CSI reporting if it is activated before the UE is transited to RRC_INACTIVE.

Option 2: To activate the inference configuration according to the applicability status upon RRC resumption. The UE autonomously activates the applicable inference configuration with periodic CSI reporting upon reporting applicability status via RRCResumeComplete.

RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations

This option issue is about what kind of applicability report can be included in RRCReconfigurationComplete message. The related RAN2#2 agreement was reached in RAN2#129 as following.

3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

However, it seems there is no consensus on whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configuration. We think it is also unclear whether RRCReconfigurationComplete contains the applicability/inapplicability reporting for inference related parameter set (option B).

In our understanding, there are following options of interpretation:

- Option 1: Applicability status of all configured inference configuration are included in RRCReconfigurationComplete. Update applicability status can also be included in UAI.

- Option 2: Only initial applicability status of new configured inference configuration are included in RRCReconfigurationComplete. Update report are only included in UAI.

- Option 3: Both initial and update applicability status of inference configuration are included in RRCReconfigurationComplete. Update applicability status can also be included in UAI.

Note that options means for each time UE sends RRCReconfigurationComplete, no matter the corresponding RRCReconfiguration include configuration related to inference configuration, UE will include applicability status of all configured inference configuration. This is unnecessary waste of signaling overhead.

Observation 4: (RRC-44)UE has to include applicability status of all configured inference configuration in EVERY RRCReconfigurationComplete, no matter whether the RRCReconfiguration contains inference configuration related configuration. This is waste of signaling overhead.

We think it is necessary that RRCReconfigurationComplete can include updated applicability status of the inference configuration that is received in previous RRCReconfiguration.

This is useful when UE decide a previous applicable inference configuration become inapplicable when UE receives new configured inference configuration. This may occur if the CPU/memory becomes insufficient and UE decide to sacrifice the previous applicable inference configuration.

In this case, it is beneficial for speeding up the applicability reporting if UE can include updated applicability status in RRCReconfigurationComplete. And to include updated applicability status in RRCReconfigurationComplete is almost for free. So we see no reason not to support it.

Observation 5:(RRC-44) It is beneficial to include updated applicability status in RRCReconfigurationComplete to speed up the applicability reporting especially for the case that previous configured inference configuration becomes inapplicable due to the activation of new configured inference configuration.

Based on above discussion, we propose that:

Proposal 6: (RRC-44) RAN2 to agree that RRCReconfigurationComplete contains both initial applicability status of new configured inference configuration received in the corresponding RRCReconfiguration, and the updated applicability status of the inference configuration received in earlier RRCReconfiguration.

RRC-46: Whether the associated ID is cell specific or multi-cell specific

According to previous RAN1 agreement, associated ID is assumed as cell specific, i.e. UE assumes that NW-side additional conditions with the same associate ID are consistent at least within a cell. The RAN2 discussion was carried based on this assumption. For example, the applicability reporting is performed on per cell basis.

Some companies think it is not good enough to have the associated as cell specific and there is need for a multiple-cell specific associated ID. The multiple-cell specific associated ID may be unique with an area or PLMN (per area per PLMN specific associated ID).

However, to have a multiple-cell specific associated ID will result in exposure of network configuration which is not acceptable by us. Remember that the associated ID is related to the network configuration. UE regards the same associated ID is associated to same configuration, which including antenna angle, beam configuration, code book, etc. Although the associated ID itself may not encode such information, one can deduce whether two cell have same configuration by comparing the configured associated ID.

Because of this, it is also difficult to coordinate the allocation of associated ID between different network device vendors. This kind of coordination means the exposure of network configuration and find which one can be regarded as a unique associated ID.

Observation 6: (RRC-46) Multiple-cell specific associated ID will result in exposure of network configuration. One may deduce the network configuration be comparing the associated ID configured for a serving cell. Thus it is difficult to coordinate the allocation of Associated ID between different network device vendors.

If associated ID is defined as multiple-cell specific, the value range the associated ID will be huge. For single cell specific Associated ID, the beam configuration is limited. While if it become PLMN specific, we needs to consider all possible combination of beam configuration, antenna configuration and code book of all kind of cell configurations which may be different from cell to cell based on the cell deployment.

Observation 7: (RRC-46) The value range of multiple-cell specific Associated ID may be huge when considering the combination of all possible cell configuration.

Further, to support multiple-cell specific associated ID may have impact on the handling of AIML functions. For example, during previous handover impact on handover impact, RAN2 assume the associated ID is cell specific. If we change this assumption, we may need to re-discuss the procedures. However, it it too late to raise this issue in this stage.

Observation 8: (RRC-46) AIML discussion is based on the single cell Associated ID assumption. If we change it to be multiple-cell specific, we may need to re-discuss may functions, e.g. impact on applicability reporting during handover. Thus it seems too late to have such change in this stage.

Proposal 7: (RRC-46) Multiple-cell specific associated ID is not supported in Rel-19.

RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided

According to previous agreed procedure of applicability reporting option B, the UE is firstly configured with inference related parameter set. If UE reports applicability of the inference related parameter set, NW may configure a full inference configuration corresponding to the previous inference related parameters.

If the inference related parameter set is not released, both the inference related parameter set and corresponding inference configuration are maintained in UE side. According to current applicability reporting procedure, UE the will report the updated applicability status for both of these two configuration, which is highly inefficient.

One solution as pointed out is to allow the UE to either provide the applicability of inference related parameter set or inference configuration once the inference configuration is provided based on the applicability of inference related parameter set.

However, the question of the solution is that how UE is able to associate an inference configuration to an inference parameter set. One method is to compare all parameters configured in inference related parameter set against the inference configuration, including resource set, associated ID for set A/B, and all other report related parameters. We think this is not a good method to let UE guess whether an inference configuration and an inference related parameter set is associated.

Another method is to associated the inference related parameter set to an inference configuration explicitly, e.g. by adding the inference related parameter set config ID when configuring inference configuration.

Observation: (RRC-48) Currently there is no association between inference related parameter set and inference configuration. UE can not know whether an inference configuration is configured corresponding to a inference related parameter set. To associate an inference related parameter set and an inference configuration requires additional standard effort.

However, we think another simple solution is to let network release the inference related parameter set when corresponding inference configuration is configured. Considering the corresponding inference configuration is already configured, there is no need to keep the inference related parameter set any more.

Observation 9: (RRC-48) If NW does not need applicability report for an configured inference related parameter set after NW provides corresponding inference configuration to UE, the NW can simply release the inference related parameter set. No specification impact is foreseen.

Based on above discussion and observation, we have following proposal.

Proposal 8: (RRC-48) RAN2 to agree no optimization is pursued for applicability reporting for inference related parameter set after the corresponding full inference configuration is provided. It is up to NW decision  to release the inference related parameter set after corresponding inference configuration is provided to UE.

Conclusion

Observation 1: (RRC-16) It is the network configures the associate ID for UE side model data collection. The network has no reason not to provide Associated ID in inference configuration but provide it in data collection configuration.

Observation 2: (RRC-16) Network already can configure multiple inference related parameter sets with Associated ID to the UE to request for applicability reporting. There is no need to have another method to gather applicability reporting.

Observation 3: (RRC-42) NW is already able to reconfigure inference configuration for MCG/SCG serving cells for free because inference configuration is included in servingCellConfig. The text procedure captured for RRCReconfigurationComplete can be reused in RRCResumeComplete to support applicability status reporting of the inference configuration configured for MCG serving cell.

Observation 4: (RRC-44)UE has to include applicability status of all configured inference configuration in EVERY RRCReconfigurationComplete, no matter whether the RRCReconfiguration contains inference configuration related configuration. This is waste of signaling overhead.

Observation 5:(RRC-44) It is beneficial to include updated applicability status in RRCReconfigurationComplete to speed up the applicability reporting especially for the case that previous configured inference configuration becomes inapplicable due to the activation of new configured inference configuration.

Observation 6: (RRC-46) Multiple-cell specific associated ID will result in exposure of network configuration. One may deduce the network configuration be comparing the associated ID configured for a serving cell. Thus it is difficult to coordinate the allocation of Associated ID between different network device vendors.

Observation 7: (RRC-46) The value range of multiple-cell specific Associated ID may be huge when considering the combination of all possible cell configuration.

Observation 8: (RRC-46) AIML discussion is based on the single cell Associated ID assumption. If we change it to be multiple-cell specific, we may need to re-discuss may functions, e.g. impact on applicability reporting during handover. Thus it seems too late to have such change in this stage.

Observation 9: (RRC-48) If NW does not need applicability report for an configured inference related parameter set after NW provides corresponding inference configuration to UE, the NW can simply release the inference related parameter set. No specification impact is foreseen.

Proposal 1: (RRC-15) The possible range of the time duration for an inference configuration to become available for inference should be first evaluated by RAN4 before RAN2 taking further actions.

Proposal 2: (RRC-16) No enhancement is pursued for the case that associated ID is not provided by the network in either inference configuration or inference related parameter set. UE determines the applicability according to its implementation if associated ID is not provided by network.

Proposal 3: (RRC-42) NW can reconfigure inference configuration for both MCG and SCG serving cells. Applicability status of inference configuration is added in RRCResumeComplete for both.

Proposal 4: (RRC-42) If inference configuration with periodic CSI reporting is configured in RRCResume, the UE autonomously activate the applicable inference configuration upon reporting applicability status via RRCResumeComplete.

Proposal 5: (RRC-42) RAN2 to discuss and decide one of following options regarding activation of the restored inference configuration with periodic CSI reporting:

Option 1: To follow the activation status of the inference configuration as the UE is transited to RRC_INACTIVE. UE activates autonomously the inference configuration with periodic CSI reporting if it is activated before the UE is transited to RRC_INACTIVE.

Option 2: To activate the inference configuration according to the applicability status upon RRC resumption. The UE autonomously activates the applicable inference configuration with periodic CSI reporting upon reporting applicability status via RRCResumeComplete.

Proposal 6: (RRC-44) RAN2 to agree that RRCReconfigurationComplete contains both initial applicability status of new configured inference configuration received in the corresponding RRCReconfiguration, and the updated applicability status of the inference configuration received in earlier RRCReconfiguration.

Proposal 7: (RRC-46) Multiple-cell specific associated ID is not supported in Rel-19.

Proposal 8: (RRC-48) RAN2 to agree no optimization is pursued for applicability reporting for inference related parameter set after the corresponding full inference configuration is provided. It is up to NW decision to release the inference related parameter set after corresponding inference configuration is provided to UE.

References

- RAN2#130 chairman notes

- R2-25xxxxx Remaining RRC open issues in feature AIML PHY