3GPP TSG-RAN2 Meeting #131	R2-2506129

Bangalore, India  Aug 25th – 29th , 2025

Source: 			ZTE Corporation

Title: 	Discussion on AI mobility for network side model

Agenda item:	8.3.3

Document for: 	Discussion and Decision

- Introduction

In the previous study, we have studied the following use case:

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes , all sub cases | Yes |
| Frequency domain | Yes , all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

In this contribution, we’d like to discuss their specification impact.

- Discussion

2.1 Support of RRM sub use case 1&3

During the AI-mob discussion, three RRM sub use case are considered, as follows:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated; - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s); - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

While, in the last meeting, some companies have concern on the necessity of supporting RRM sub use case 1&3 for NW-sided model, in this sub-section, we’d like to share our view.

The inference input of sub use case 1 and 3 is L1 filtered beam results. If reporting L1 beam level measurement results is supported, the network can predict beam results based on the received L1 beam results, which helps the target cell to select RACH resource. Besides, with received L1 filtered beam results, the NW side model can perform spatial domain prediction, and the network can only transmit a sub set of RSs, which reduces network energy consumption. From UE’s perspective, the UE only needs to perform measurement on a sub set of RSs, which reduces UE’s power consumption in measurement.

Observation 1: There are many advantages if reporting L1 beam level results (RRM sub use case 1&3) is supported for NW side model:

- Facilitate RACH resource selection in target cell during handover;

- Enables NW side spatial domain prediction, so that the network can only transmit a subset of RSs, which also reduces UE’s power consumption in measurement.

The existing RRM measurement framework already supports the UE to report L1 filtered beam results by setting L3 beam level filter co-efficient to zero, as follows:

2> filter the measured result, before using for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, by the following formula: Fn = (1 – a)*Fn-1 + a*Mn where Mn is the latest received measurement result from the physical layer; Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE; Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

Observation 2: The existing RRM measurement framework already supports the UE to report the L1 filtered beam results by setting L3 beam-level filter co-efficient to zero.

For training data collection, we have agreed that the UE can be configured to log L3 cell level measurements and L3 beam level measurements, as follows:

- UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed)

If the network sets L3 beam-level co-efficient to zero, the UE can perform measurement and log L1 filtered beam results. And since the L1 filtered beam results and L3 cell level results can be obtained simultaneously, the logging periodicity for L1 filtered beam results is the same as for L3 cell level results.

Observation 3: For training data collection, the UE can measure and log L1 filtered beam results if the network sets L3 beam-level filter co-efficient to zero. The logging periodicity is same as for L3 cell level beam results.

Based on the analysis above, we think for both training data collection and inference input of NW side model, UE reporting L1 filtered beam level results is supported.

Proposal 1: 	UE reporting L1 filtered beam level results is supported for NW side model, this applies to both training data collection and inference input.

2.2 Support of Spatial domain prediction

The definition of spatial domain prediction is as follows:

Intra-frequency intra-cell spatial domain prediction is evaluated for the 1st study goal by measuring a sub set of configured SSB as input to the model to derive L3 filtered cell-level measurements for every time instance of the same cell. It is only evaluated for FR2 intra-frequency scenario and is applicable for RRM sub-use case 1 and 3. MRRS(s) should be aligned among companies without defining detailed pattern.

The NW side spatial domain prediction benefits both network and UE. Specifically: the network can send only a sub set of RSs, which saves network energy; and from UE’s perspective, the UE can also only measure a sub set of RSs, which also reduces UE measurement efforts.

Observation 4: By supporting NW side spatial domain prediction, the network can switch-off some RS for energy saving, the UE can also be benefit from measuring less RSs.

The existing RRM measurement configuration already supports configuring UE to measure a sub set of SSB via SSB-ToMeasure.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent the UE measures on all SS-blocks

So, as long as the UE can report L1 beam results to the network (i.e. P1), there is no additional spec impact to support spatial domain prediction for NW side model.

Observation 5: The existing RRM configuration already supports configuring UE to measure a subset of SSBs, i.e. via SSB-ToMeasure.

Observation 6: As long as L1 beam results can be reported to the network (P1), there is no additional spec impact foreseen in supporting spatial domain prediction for NW side model.

So, we think spatial domain prediction for NW side model shall be supported.

Proposal 2: 	Spatial domain prediction is supported for NW side model, no spec impact is foreseen.

2.3 Support of Temporal domain prediction case B

In the last RAN2 meeting, some companies argued about the necessity of temporal domain case B for NW side. In this clause, we’d like to share our view on it.

Observation 7: In last RAN2 meeting, some companies argued about the necessity of supporting Temporal domain case B for NW side model.

In the last meeting, RAN2 has agreed that skipping pattern can be provided for temporal domain case B, as follows:

- The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction)

This enables the network to only send RSs at some of time instances, which saves network energy.

Observation 8: RAN2 agreed that skipping pattern can be configured for temporal domain case B, this enables the network to reduce RS transmission occasions for energy saving.

Similar to training data collection in the UE side model, for both temporal domain prediction case A and case B, the L3 measurement results with the same measurement interval are required. After reporting these training data to the network, it is up to network implementation to determine how to train the AI model, e.g. to train the AI model for case A or for case B, the prediction window length, the skipping pattern...

Observation 9: There is no difference in data collection for temporal domain case A and case B.

For temporal domain case B, in order to only send RSs at part of time instance, the UE needs to know when to perform measurement and when the UE does not need to perform measurement. The network needs to configure skipping pattern to the UE, same as in temporal domain case A for UE side model. For configuring skipping pattern, we think it can be configured explicitly (e.g. the number of continuous time instances for actual measurement, the number of time instances for prediction…), or implicitly by updating the SMTC configuration. This issue is the same as temporal domain case B for UE side model and can be further discussed in WI phase.

Observation 10: For temporal domain Case B, the skipping pattern can be configured explicitly or implicitly by updating the SMTC configuration. This is same as for UE-side model and can be discussed together in WI phase.

Based on the analysis, we think the spec impact of temporal domain case B for network side model is limited, and it shall be supported. For how to configure the skipping pattern to the UE, this can be discussed together with UE side model in WI.

Proposal 3: 	Temporal domain prediction Case B is supported for NW side model, FFS how to configure the skipping pattern to the UE, can be discussed together with UE side model in WI.

  - Potential spec impact for NW side model

- 2.4.1 Training data collection

As analysed in the clause 2.1, the UE can measure and log L1 filtered beam results if the network sets L3 beam level filter co-efficient to zero, and the logging periodicity for L1 filtered beam results is the same as for L3 cell level results. So, we think in addition to L3 cell level results, logging L1 filtered beam level results is also supported.

Proposal 4: 	In addition to L3 cell level results, logging L1 filtered beam level results is supported, the same logging periodicity is applied for both L3 cell level results and L1 filtered beam level results.

In the AI-phy BM, the UE needs to perform measurement with indicated periodicity. However, for L3 measurement, when to perform measurement is up to UE implementation based on the current specification, as long as satisfying the measurement periodicity requirements provided in the TS 38.133.

Observation 11: Different from AI-Phy BM, for L3 measurements, the UE may adjust the measurement period in real time, so, it is unclear whether the UE obtains measurement results at equal intervals.

In order to obtain better prediction accuracy of temporal domain case A and case B, the measurement interval between measurement results is expected to be the same. So, it is important for the network to know the time info of each measurement result. So, in order to inform the network time info, either the UE needs to log the measurement results at a fixed time interval (e.g. the interval can be configured by network), or the UE needs to send time info together with the measurement results.

Observation 12: For temporal domain prediction case A and case B, it is important for network to know the time info of each measurement result, because this will impact the AI prediction accuracy.

Proposal 5: 	Either the UE logs measurement results at a fixed time interval (e.g. the interval can be configured by network), or UE sends time info together with the measurement results.

In order to train the AI model with frequency domain prediction, the measurement results at two different frequencies are needed. And considering the UE cannot perform measurement on two different frequencies at the same time, there is a time difference between the time instances that the UE performs measurement at two frequencies. For better prediction accuracy, it is expected that the training data (one pair of measurement results at two frequencies) can have small time difference.

For training data collection of frequency domain prediction, the UE is expected to minimize the time difference of time instance that obtained the measurement results to be logged at two frequencies. Specifically, the network indicates the measurement results of which two frequencies can be used for model training. And upon receiving this information, the UE performs measurement at two frequencies and obtained the measurement results with a minimized time difference.

Proposal 6: 	To facilitate frequency domain prediction, the network can indicate a pair of frequencies, the UE is expected to measure the two frequencies as close as it can in the time domain.

- 2.4.2 Inference input

In order to report inference input, the UE needs to report the measurement results to the network. To reduce signaling overhead, we think the network can configure the UE to package multiple measurement results in time domain and send them within one measurement report to the network. For example, the network indicates the UE the number of logged/reported measurement results (i.e. N), the UE logs N historical measurement results and reports them within one measurement report to the network.

Figure 1: an example for temporal domain prediction case B

Proposal 7: 	To avoid signalling overhead, the network can configure the UE to package multiple measurement results in the time domain and send one measurement report to the network.

- Conclusion and proposals

Support of RRM sub use case 1&3

Observation 1: There are many advantages if reporting L1 beam level results (RRM sub use case 1&3) is supported for NW side model:

- Facilitate RACH resource selection in target cell during handover;

- Enables NW side spatial domain prediction, so that the network can only transmit a subset of RSs, which also reduces UE’s power consumption in measurement.

Observation 2: The existing RRM measurement framework already supports the UE to report the L1 filtered beam results by setting L3 beam-level filter co-efficient to zero.

Observation 3: For training data collection, the UE can measure and log L1 filtered beam results if the network sets L3 beam-level filter co-efficient to zero. The logging periodicity is same as for L3 cell level beam results.

Support of spatial domain prediction

Observation 4: By supporting NW side spatial domain prediction, the network can switch-off some RS for energy saving, the UE can also be benefit from measuring less RSs.

Observation 5: The existing RRM configuration already supports configuring UE to measure a subset of SSBs, i.e. via SSB-ToMeasure.

Observation 6: As long as L1 beam results can be reported to the network (P1), there is no additional spec impact foreseen in supporting spatial domain prediction for NW side model.

Support of temporal domain prediction case B

Observation 7: In last RAN2 meeting, some companies argued about the necessity of supporting Temporal domain case B for NW side model.

Observation 8: RAN2 agreed that skipping pattern can be configured for temporal domain case B, this enables the network to reduce RS transmission occasions for energy saving.

Observation 9: There is no difference in data collection for temporal domain case A and case B.

Observation 10: For temporal domain Case B, the skipping pattern can be configured explicitly or implicitly by updating the SMTC configuration. This is same as for UE-side model and can be discussed together in WI phase.

Potential spec impact for NW side model

Observation 11: Different from AI-Phy BM, for L3 measurements, the UE may adjust the measurement period in real time, so, it is unclear whether the UE obtains measurement results at equal intervals.

Observation 12: For temporal domain prediction case A and case B, it is important for network to know the time info of each measurement result, because this will impact the AI prediction accuracy.

Proposal 1: 	UE reporting L1 filtered beam level results is supported for NW side model, this applies to both training data collection and inference input.

Proposal 2: 	Spatial domain prediction is supported for NW side model, no spec impact is foreseen

Proposal 3: 	Temporal domain prediction Case B is supported for NW side model, FFS how to configure the skipping pattern to the UE, can be discussed together with UE side model in WI.

Proposal 4: 	In addition to L3 cell level results, logging L1 filtered beam level results is supported, the same logging periodicity is applied for both L3 cell level results and L1 filtered beam level results.

Proposal 5: 	Either the UE logs measurement results at a fixed time interval (e.g. the interval can be configured by network), or UE sends time info together with the measurement results.

Proposal 6: 	To facilitate frequency domain prediction, the network can indicate a pair of frequencies, the UE is expected to measure the two frequencies as close as it can in the time domain.

Proposal 7: 	To avoid signalling overhead, the network can configure the UE to package multiple measurement results in the time domain and send one measurement report to the network.

- Reference

- Report of RAN2 130 meeting.