3GPP TSG-RAN WG2 Meeting #131	R2-2506132

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.6.4

Source:	Nokia

Title:	Considerations on Conditional LTM

WID/SID:	NR_Mob_Ph4-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#129bis the following was agreed for Conditional LTM:

Agreements on C-LTM - Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and keep the early TA timer running. - C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running and starts the PTAG using the remaining time from the C-LTM timer. - RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE. - For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

In RAN2#130 the following was agreed for Conditional LTM:

Agreements on C-LTM - (MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed. - For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant. - Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed. - During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM. - In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS. - RAN2 will not do anything for the separate TCI mode optimization.

In this document, we present our views on the remaining aspects for conditional LTM, including early synchronization procedures, cell switch and co-existence with other features.

# 2	Discussion

## 2.1	Early synchronization

### 2.1.1	RACH-based early TA validation

After a TA value has been obtained, it must be ensured that at the time of the conditional cell switch, the TA value is valid. RAN2 has agreed that the UE shall maintain TA value based on a candidate time alignment timer (TAT). A potential drawback of this approach is that it does not consider the actual channel condition between the UE and the candidate cell while the TAT is running. For example, while the TAT is running, the UE may move such that the TA value maintained by the running timer is no longer valid.

Observation 1: TAT-based validation does not consider the actual channel condition between the UE and the candidate cell.

To resolve this issue, TA validity verification based on L1/L3 measurements has been proposed. L1/L3-measurement-based TA validation and candidate TAT-based TA validation are not mutually exclusive. In fact, an approach similar to CG-SDT can be followed for more reliable TA validation, where the UE is configured to perform both TAT- and measurement-based TA validation.

Proposal 1: When CLTM is configured, for the maintenance of the TA value a maintenance method similar to CG-SDT, i.e., based both on a TAT and measurement-based validation, can be followed.

### 2.1.2	Handling TAs of collocated CLTM candidates

From the current form of the running CRs for RRC and MAC specification, it is evident that the UE may have to store and maintain multiple TAs for supporting RACH-less CLTM. In intra-CU setups, it is likely that LTM candidates are collocated, thus it would be redundant for the UE to separately store a TA and maintain it with an associated TAT for such candidates.

Observation 2: For collocated LTM candidates, it is redundant for the UE to separately store a TA and maintain it with an associated TAT.

It is possible that early TA acquisition is performed for a candidate, but a conditional cell switch is triggered to another candidate that is collocated with the candidate for which early TA acquisition was performed. In addition, a conditional cell switch may be triggered to a candidate that is collocated with the current serving cell. It should be noted that this issue cannot be circumvented by the use the RACH-less HO IE in the candidates’ RRC configuration, as this would solve the issue for the initial cell switch but not for subsequent ones. In such scenarios, the UE will not use the available TA for accessing the target and will unnecessarily trigger a RACH-based CLTM cell switch.

Observation 3: The UE may unnecessarily trigger a RACH-based CLTM cell switch to a candidate, although a TA has been obtained for a collocated candidate or current serving cell.

To reduce TA maintenance effort on the UE side and avoid unnecessary RACH-based CLTM cell switches, it would be useful to provide an indication to the UE to allow it to reuse a TA among collocated candidates

Proposal 2: To reduce TA maintenance effort on the UE side and avoid unnecessary RACH-based CLTM cell switches, provide an indication to the UE to allow it to reuse a TA among collocated candidates.

## 2.2	Conditional LTM cell switch

### 2.2.1 CLTM execution

#### 2.2.1.1 TA handling when the serving cell is a candidate cell

As stated in the Introduction section, regarding the handling of the time alignment timers (TATs) of CLTM candidate cells, the following agreements were made in RAN2#129bis:

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and keep the early TA timer running. - C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running and starts the PTAG using the remaining time from the C-LTM timer.

These agreements define the UE handling of TATs of candidate cells (target and other candidate cells) and the PTAG TAT of the new serving cell, but they do not address if and how the TA and corresponding TAT of the previous serving cell should be used. Therefore, with the current agreements, the TA of the previous serving cell and the status of the corresponding TAT are not maintained after the cell switch. In case the previous serving cell is configured as a CLTM candidate and a subsequent conditional cell switch to that cell is triggered, the UE will have to obtain the TA again, performing an unnecessary random-access procedure either during RACH-based conditional cell switch or during early TA acquisition. We also note that such UE behavior is inconsistent with the one agreed for all other candidate cells, where the available TAs and corresponding TATs are maintained upon cell switch.

Observation 4: Upon a conditional cell switch, the TA and the corresponding TAT of the previous serving cell are not maintained, potentially leading to an unnecessary random-access procedure, in case that cell is a CLTM candidate.

The issue is depicted in Figure 1, where an example scenario with two CLTM candidates, with one of them corresponding to the current serving, is considered.

Figure 1: Issue with handling the TA of the previous serving cell.

A possible way to resolve this issue is that, if the previous serving cell is configured as a CLTM candidate, upon cell switch, the UE stores the TA of the previous serving cell as the TA of the corresponding candidate configuration and starts the corresponding candidate TAT with the remaining time of the TAT of the PTAG of previous serving cell. An alternative solution is that the UE always maintains the TA and corresponding TAT of the candidate configuration corresponding to the serving cell by duplicating the TA and the status of the corresponding TAT of the serving cell. For example, when the UE receives a TAC MAC CE for the PTAG of the serving cell, in addition to the actions related to the serving cell TA and TAT, it should also update the TA and restart the TAT of the corresponding candidate configuration.

From the two alternatives considered, Alt. 1 is preferred, as it has a lower implementation complexity, since the UE does not need to maintain the candidate TAT corresponding to the serving cell while being served by it. A TP showing the required changes in the specification (in the branch treating the case when the CLTM target is not configured with two TAGs) is shown below:

5.2 Maintenance of Uplink Time Alignment … 1> when a conditional LTM cell switch procedure is triggered for a CLTM candidate cell or indicated by upper layer as specified in clause 5.y.3: 2> if the CLTM candidate cell is not configured with two TAGs and the ltm-Candidate-TimeAlignmentTimer associated with the CLTM candidate cell is running as specified in clause 5.2x: 3> if the (previous) serving cell is configured as an LTM candidate: 4> store the TA value of the PTAG for the LTM candidate corresponding to the (previous) serving cell 4> start or restart the ltm-Candidate-TimeAlignmentTimer associated with the LTM candidate corresponding to the serving cell with the length of the remaining time of the TimeAlignmentTimer of the PTAG 3> apply the stored TA value associated with the CLTM candidate cell for the PTAG as specified in clause 6.1.3.4x; 3> start or restart the timeAlignmentTimer associated with the PTAG with the length of the remaining time of the ltm-Candidate-TimeAlignmentTimer.

Proposal 3: To allow the UE to use the available TA for a subsequent conditional cell switch, upon cell switch, the UE stores the TA of the previous serving cell as the TA of the corresponding candidate configuration and starts the corresponding candidate TAT with the remaining time of the TAT of the PTAG of previous serving cell.

A TP showing the required changes in the specification (in the branch treating the case when the CLTM target is not configured with two TAGs) is shown below:

5.2 Maintenance of Uplink Time Alignment … 1> when a conditional LTM cell switch procedure is triggered for a CLTM candidate cell or indicated by upper layer as specified in clause 5.y.3: 2> if the CLTM candidate cell is not configured with two TAGs and the ltm-Candidate-TimeAlignmentTimer associated with the CLTM candidate cell is running as specified in clause 5.2x: 3> if the (previous) serving cell is configured as an LTM candidate: 4> store the TA value of the PTAG for the LTM candidate corresponding to the (previous) serving cell 4> start or restart the ltm-Candidate-TimeAlignmentTimer associated with the LTM candidate corresponding to the serving cell with the length of the remaining time of the TimeAlignmentTimer of the PTAG 3> apply the stored TA value associated with the CLTM candidate cell for the PTAG as specified in clause 6.1.3.4x; 3> start or restart the timeAlignmentTimer associated with the PTAG with the length of the remaining time of the ltm-Candidate-TimeAlignmentTimer.

#### 2.2.1.2 Coordination on the timing of RACH-less CLTM execution

Based on the current specification, it is not possible for NW to detect that UE has just left the current serving cell and has started RACH-less CLTM cell switch. One of the problems related to this fact is such that candidate target cells must be ready for admission of the UE all the time i.e., starting from evaluation phase. It leads to huge waste of PUSCH resources since CG needs to be kept reserved for a long time. Moreover, in single CLTM cell switch, UE will use only one CG resource related to one beam of one candidate target cell.

From our perspective, source cell could be able to provide some information about the switch to target cells if only it can efficiently detect that UE has started the CLTM cell switch. Once CLTM cell switch is detected by the source cell, it can inform candidate target cell(s) about this fact providing all the information it currently has. In other words, source cell could mimic the behavior specified for non-conditional LTM cell switch by sending Cell Switch Notification. Note that even without any information about initial UL transmission parameters, CLTM Cell Switch Notification will be useful for candidate target cells since it will indicate the moment of RACH-less CLTM cell switch. In a result, candidate target cells can delay actual reservation of CG resources.

Observation 5: Proper detection of CLTM cell switch at the source cell will, at least, allow network to minimize the waste of CG resources by delaying the actual reservation of the resources

When UE decides to switch to candidate target cell, it immediately, and silently detaches from the source cell. It means that UL transmission between UE and the source cell stops including all the periodic transmissions over PUCCH and PUSCH. It means that the source cell can detect it based on existing messaging (e.g., CSIs, PHRs) which always disappear after detaching. However, using existing messaging for that purpose will delay the detection because periodicity of such messaging is set consistently with how frequently the NW needs to be informed by the UE about e.g. PHR, PMI, RI, CQI, etc. The purpose of this messaging is not to detect any kind of RLF.

Observation 6: Usage of existing periodical UL messaging to detect CLTM cell switch can be suboptimal

Proposal 4: Dedicated, periodical UL messaging (PULSE message) shall be specified to enable the source to detect CLTM cell switch. It could be realized:

- either using PUCCH (e.g., new type of CSI or SRI with dedicated resources and special handling without triggering a grant)

- or over PUSCH (e.g., new type of UL MAC CE)

Please note that PUSCH-based PULSE message could allow UE to pass some up-to-date information to the source cell. However, the PUSCH-based PULSE approach can also be problematic since it requires dynamic granting. In addition, it will affect UE performance, if UE is expected to provide some additional information to the NW. That is because the periodicity of such PULSE message shall be short enough to allow NW for swift detection of CLTM cell switch. That is why, we would prefer PUCCH-based PULSE messaging to lower the impact on UE’s performance.

## 2.3 Co-existence aspects

### 2.3.1 Co-existence with network-triggered LTM

In RAN2#127-bis the following agreements were made

- RACH-less Conditional intra-CU LTM is supported. - PDCCH ordered early TA acquisition is supported for conditional LTM

From the agreements above, the early synchronization behavior for conditional LTM is like that of conventional LTM with exception in terms of delivery of the TA information to the UE by the serving DU. In case of conventional LTM, the TA value acquired during the early synchronization phase is stored in the serving DU and shared with the UE along with the MAC CE cell switch command during the execution phase based on UE measurements. Moreover, in RAN2#128 the following agreements were made

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. - Candidate cell TA is maintained by a new timer

Observation 7: The network may configure UE for early synchronization based on PDCCH order for the selected CLTM candidate cell(s) and deliver the early TA to the UE via TAC MAC CE.

While CLTM execution conditions are being evaluated at the UE, the network may trigger cell switch command towards a CLTM prepared candidate cell for which TA has been already acquired during early synch.

Observation 8: If the TA in the CSC MAC CE is set as FFF, the UE may use the TA it acquired via TAC MAC CE if it is still valid (CLTM TimeAlignmentTimer running) to perform RACH-less network triggered LTM cell switch execution.

Proposal 5: A valid TA obtained over TAC MAC CE for a selected CLTM candidate cell, can also be used by the UE to perform a RACH-less network triggered LTM cell switch execution towards the same selected candidate, if TA in CSC MAC CE is set as FFF.

### 2.3.2 Co-existence with CHO

In Rel-18 and Rel-19 it has been decided that CHO can co-exist with regular LTM. It means that LTM candidate target cell can be also configured as CHO candidate target cell. We cannot see any reason to block this possibility in the context of CLTM.

Proposal 6: Follow similar principle as Rel. 18 LTM, CLTM and CHO can be configured independently for different candidate cells.

On the other hand, configuring CLTM candidate target cell to be also CHO candidate target cell for the same UE requires further discussion, since both features aim to provide robustness gains.

Observation 9: configuring CLTM candidate target cell to be also CHO candidate target cell for the same UE requires further discussion, since both features aim to provide robustness gains.

Proposal 7: RAN2 to discuss if co-existence of CHO and CLTM for the same candidate target cell is supported.

# 3	Conclusion

This document has made the following observations:

Observation 1: TAT-based validation does not consider the actual channel condition between the UE and the candidate cell.

Observation 2: For collocated LTM candidates, it is redundant for the UE to separately store a TA and maintain it with an associated TAT.

Observation 3: The UE may unnecessarily trigger a RACH-based CLTM cell switch to a candidate, although a TA has been obtained for a collocated candidate or current serving cell.

Observation 4: Upon a conditional cell switch, the TA and the corresponding TAT of the previous serving cell are not maintained, potentially leading to an unnecessary random-access procedure, in case that cell is a CLTM candidate.

Observation 5: Proper detection of CLTM cell switch at the source cell will, at least, allow network to minimize the waste of CG resources by delaying the actual reservation of the resources

Observation 6: Usage of existing periodical UL messaging to detect CLTM cell switch can be suboptimal

Observation 7: The network may configure UE for early synchronization based on PDCCH order for the selected CLTM candidate cell(s) and deliver the early TA to the UE via TAC MAC CE.

Observation 8: If the TA in the CSC MAC CE is set as FFF, the UE may use the TA it acquired via TAC MAC CE if it is still valid (CLTM TimeAlignmentTimer running) to perform RACH-less network triggered LTM cell switch execution.

Observation 9: configuring CLTM candidate target cell to be also CHO candidate target cell for the same UE requires further discussion, since both features aim to provide robustness gains.

And proposed the following:

Proposal 1: When CLTM is configured, for the maintenance of the TA value a maintenance method similar to CG-SDT, i.e., based both on a TAT and measurement-based validation, can be followed.

Proposal 2: To reduce TA maintenance effort on the UE side and avoid unnecessary RACH-based CLTM cell switches, provide an indication to the UE to allow it to reuse a TA among collocated candidates.

Proposal 3: To allow the UE to use the available TA for a subsequent conditional cell switch, upon cell switch, the UE stores the TA of the previous serving cell as the TA of the corresponding candidate configuration and starts the corresponding candidate TAT with the remaining time of the TAT of the PTAG of previous serving cell.

Proposal 4: Dedicated, periodical UL messaging (PULSE message) shall be specified to enable the source to detect CLTM cell switch. It could be realized:

- either using PUCCH (e.g., new type of CSI or SRI with dedicated resources and special handling without triggering a grant)

- or over PUSCH (e.g., new type of UL MAC CE)

Proposal 5: RAN2 to agree that a valid TA obtained over TAC MAC CE for a selected CLTM candidate cell, can also be used by the UE to perform a RACH-less network triggered LTM cell switch execution towards the same selected candidate, if TA in CSC MAC CE is set as FFF.

Proposal 6: Follow similar principle as Rel. 18 LTM, CLTM and CHO can be configured independently for different candidate cells.

Proposal 7: RAN2 to discuss if co-existence of CHO and CLTM for the same candidate target cell is supported.