3GPP TSG-RAN WG2 Meeting #131	R2-2506135

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.3.2.1

Source:	Nokia

Title:	Functionality management for RRM measurement prediction

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# 1	Introduction

It was agreed in RAN2#129bis that:

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

The following agreements were made in RAN2#130:

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

In the contribution we present our views on the time domain measurement prediction. We highlight how the different use-cases identified in the study can be merged into the same procedure and provide a discussion on the parameters that become relevant from configuration viewpoint. We also discuss the monitoring aspects relevant to measurement predictions and the associated KPIs.

# 2	Discussion

## 2.1	Time-Domain RRM Measurement Prediction Functionality Management

The measurement prediction can be understood as a predictive filter applied to the 5G measurement framework (TS 38.300). We will look at how the prediction can be included in the measurement framework in different sub-cases.

Observation 1: RRM measurement prediction serves as a predictive filter added in the 5G measurement framework.

Figure 2.1-1 Temporal Domain Case A Predictive Filter Sub-Case 2

Let’s first look at the sub-case 2, where both the input and the output of the filter are cell-level L3-filtered measurements. It is shown in Figure 2.1-1. For each measurement period, the cell-level L3 measurement Ct is given as input into the temporal domain case A predictive filter, which may additionally buffer the OW measurements, and predicts future measurements CPWp in the PW.

As agreed in RAN2#129bis, the UE may be configured by the NW to report measurements and/or predictions:

Agreements

- For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

- For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

- Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

The reporting configuration for both may be same or different. The predicted measurements are reported as configured in the reporting configuration and the reporting may be periodic or event-based, as in legacy.

Figure 2.1-2 Temporal Domain Case A Predictive Filter Sub-Case 1

Sub-case 1, where both the input and the output of the filter are L1-filtered beam measurements is shown in Figure 2.1-2. The filter predicts measurements for each beam in the PW, and then the legacy beam consolidation and L3 filtering steps are applied to the predicted L1 measurements to derive the predicted cell-level L3 measurements.

Figure 2.1-3 Temporal Domain Case A Predictive Filter Sub-Case 3

Finally, sub-case 3, where the input to the predictive filter are beam-level L1-filtered measurements and the output are cell-level L3-filtered measurements is shown in Figure 2.1-3.

Observation 2: In sub-case 3, it should be clarified during the WI phase, whether beam consolidation and L3 filter coefficient can be configured for the predicted measurements as in legacy.

Temporal domain case B RRM measurement prediction/reduction can also be understood, alternatively to skipping measurements, as predictive up-sampling. Like the temporal domain case A prediction, case B takes a time series of past measurements as an input and predicts a series of future measurements. The only difference to Case A is that in Case B measurements in-between the measurement periods are predicted. Measurements are reduced by making the measurement period longer (i.e., reducing the measurement frequency), but compensating for this by the UE reporting the predicted, up-sampled measurements in between, as shown in Figure 2.1-4.

Figure 2.1-4 Examples of OW, PW and PP configurations resulting in different scenarios

Cases A and B can be parametrized with Prediction Window (PW) defined as number of future measurements predicted in a time series and with Prediction Period (PP), which is the length of each subsequent period in the predicted time series as a ratio of the Measurement Period (MP) or sample period. In temporal domain Case A, future measurement periods may be predicted, in which case the PP can be same as the measurement or sample period or a multiple of it. In temporal domain case B, however, for predictive up-sampling, the PP can be a fraction/division of the measurement or sample period. It is also possible combine temporal domain case A and B with a combination of a PW longer than the MP while PP a fraction of the measurement or sample period. Examples of these scenarios are illustrated in Figure 2.1-1, which shows also the correlation between the up-sampling ratio and the MRRT.

Observation 3: Temporal domain case B may be understood as predictively up sampling the measurement rate in-between the configured measurement periods.

Observation 4: The actual measurements can be processed and reported exactly as legacy in both Case A and B.

Proposal 1: Both case A and B RRM measurement prediction may be parametrized with Prediction Window defined as number of future measurements predicted in a time series and with Prediction Period, which is the length of each subsequent period in the predicted time series as a ratio of the measurement or sample period.

Observation 5: The only difference in intra-cell temporal domain case A and B RRM measurement prediction configuration may be in configuring a different prediction period.

Observation 6: A combination of temporal domain case A and B can also be configured with a combination of PW and PP.

Figure 2.1-5 Temporal Domain Case B Predictive Filter Sub-Case 1 with unfiltered L1 measurements as input

Figure 2.1-5 shows the measurement framework with unfiltered L1 measurements as input in sub-case 1. This scenario is different compared to the Sub-Case 1 using L1-filtered beam measurements as input, since the skipping happens on individual SSB burst samples and not in measurement periods. Furthermore, the L1 filter will already aggregate measurements and predictions together, and after this the measurement framework can be processed as legacy. Since the L1 filter is not standardized, this option may have limited or not spec impact.

Observation 7: In sub-case 1, if using unfiltered L1 measurements as input, there may be limited or no specification impact.

## Time-Domain RRM Measurement Prediction Monitoring

How UE-side RRM measurement predictions may be used by the UE or the network, depends on the accuracy and reliability of the predictions. For example, the accuracy determines whether the predictions can be utilized by the NW in optimizing handover execution decisions, or only in less critical decisions. At the same time, the accuracy of the predictions may depend on the scenario and environment where the predictions are made. Therefore, online monitoring of UE RRM measurement prediction accuracy by UE and/or NW may be required in some scenarios.

For ML accuracy monitoring in RRM measurement prediction, the following may need to be specified:

- How the ground truth is established

- Which ML monitoring metrics should be computed

- What metrics should be reported and when

We may focus on scenario, where UE is monitoring of UE-side ML and sharing the metrics to the NW. Here the following has been document in the TR 38.744 draft:

For UE-sided monitoring, UE reports performance result i.e. RSRP difference to network based on ground-truth measurement result and inference output. It is up to network’s implementation to make management decision based on received performance result.

Let’s address the first question, how the ground truth can be established in time-domain intra-cell RRM measurement prediction.

In temporal domain case A, if the predictions are for future measurement periods (or aligned with them), the ground truth can be established if and when the predicted measurement period is measured. To compute error, the UE will need to buffer the previously performed predictions. If the UE does not measure the predicted measurement period, e.g., if it executes a handover before measuring the predicted measurement period, it is not possible to establish the ground truth.

Observation 8: In temporal domain case A measurement prediction, the ground truth can often be established for monitoring the prediction error when the predicted measurement periods is measured.

Figure 2.2-1 Measurement Skipping Pattern (MSP) with non-sliding L1 filter and L3 filter option 3 with monitoring

In temporal domain case B, the skipped/up-sampled measurement steps are by default not measured (to enable measurement reduction), so establishing the ground truth is more difficult to establish than in case A. As proposed in R2-2504284, the UE may measure a certain percentage of skipped measurement steps/periods for monitoring purposes, as shown in Figure 2.2-1. This may require coordination with the network, e.g., to have the required reference signals and measurement gaps configured.

Figure 2.2-2 Ground truth establishment for time domain measurement prediction

Depending on the prediction periodicity and prediction window, predictions overlapping with actual measurements, where ground truth is established, may be used in monitoring. Alternatively, the error may be computed and approximated using the predictions closest in time to actual measurements.

Observation 9: In temporal domain case B, additional actions or measurements are needed to establish the ground truth for computing error metrics for monitoring.

Proposal 2: RAN2 to consider temporal domain case B metrics, where predictions overlapping with actual measurements, where ground truth is established, may be used in monitoring. Alternatively, the error may be computed and approximated using the predictions closest in time to actual measurements.

TR 38.744 documents that:

For UE-sided monitoring, UE reports performance result i.e. RSRP based on ground-truth measurement result and inference output.

Figure 2.2-3 Error computation for one time domain measurement prediction sample

Figure 2.2-3 shows the different RSRP difference metrics used in the study item evaluation. Since each inference may predict a time series of one or more predicted future time steps, for each prediction the error may be computed as:

- The Mean Absolute Error (MAE) of all predictions in the PW compared to the ground truth (e)

- Error of a specific step, typically of the last step (eN)

- Error for each predicted time step ([e1, e2, …, eN])

  - Note that this is an array instead of a scalar, in case a array is predicted

Proposal 3: RAN2 to consider the same error metrics in inference time monitoring of time domain RRM measurement prediction as were used in the evaluation:

- The Mean Absolute Error (MAE) of all predictions in the PW compared to the ground truth

- Error of a specific step, typically of the last step

- Error for each predicted time step (an array)

MAE may not be an ideal metric in all mobility scenarios, where it is important if the error individual predictions is significant enough to lead to potentially wrong mobility decisions. The acceptable error would typically depend on the mobility related configurations, such as the offset and hysteresis parameters configured in the predicted A3 measurement event. Therefore, in RAN4 measurement accuracy requirements (3GPP TS 38.133 Section 10), the required accuracy is given as percentage of reported measurements, which have to be within a specified maximum error threshold of the ground truth. Similar metric could be used also in inference time monitoring of RRM measurement prediction accuracy.

The sample accuracy ea metric can be defined as:

fe= 1, e≤thr0, e>thr

ea=1Ni=1Nf(ei)

Where thr is the configured threshold for maximum error.

Observation 10: In RAN4, measurement accuracy performance requirements are defined as fixed values (e.g., ±X dB), with the expectation that error of 99.9 % of reported measurements fall within the specified maximum error threshold.

Proposal 4: In addition to the previously studied performance metric for monitoring of measurement prediction, a similar accuracy metric to that used for measurement accuracy requirements in RAN4 can be considered (e.g., a maximum of ±X dB threshold, with the expectation that error of Y % of predicted measurements fall within the specified threshold).

# 3	Conclusion

This document has made the following observations:

Observation 1: RRM measurement prediction serves as a predictive filter added in the 5G measurement framework.

Observation 2: In sub-case 3, it should be clarified during the WI phase, whether beam consolidation and L3 filter coefficient can be configured for the predicted measurements as in legacy.

Observation 3: Temporal domain case B may be understood as predictively up sampling the measurement rate in-between the configured measurement periods.

Observation 4: The actual measurements can be processed and reported exactly as legacy in both Case A and B.

Observation 5: The only difference in intra-cell temporal domain case A and B RRM measurement prediction configuration may be in configuring a different prediction period.

Observation 6: A combination of temporal domain case A and B can also be configured with a combination of PW and PP.

Observation 7: In sub-case 1, if using unfiltered L1 measurements as input, there may be limited or no specification impact.

Observation 8: In temporal domain case A measurement prediction, the ground truth can often be established for monitoring the prediction error when the predicted measurement periods is measured.

Observation 9: In temporal domain case B, additional actions or measurements are needed to establish the ground truth for computing error metrics for monitoring.

Observation 10: In RAN4, measurement accuracy performance requirements are defined as fixed values (e.g., ±X dB), with the expectation that error of 99.9 % of reported measurements fall within the specified maximum error threshold.

And proposed the following:

Proposal 1: Both case A and B RRM measurement prediction may be parametrized with Prediction Window defined as number of future measurements predicted in a time series and with Prediction Period, which is the length of each subsequent period in the predicted time series as a ratio of the measurement or sample period.

Proposal 2: RAN2 to consider temporal domain case B metrics, where predictions overlapping with actual measurements, where ground truth is established, may be used in monitoring. Alternatively, the error may be computed and approximated using the predictions closest in time to actual measurements.

Proposal 3: RAN2 to consider the same error metrics in inference time monitoring of time domain RRM measurement prediction as were used in the evaluation:

- The Mean Absolute Error (MAE) of all predictions in the PW compared to the ground truth

- Error of a specific step, typically of the last step

- Error for each predicted time step (an array)

Proposal 4: In addition to the previously studied performance metric for monitoring of measurement prediction, a similar accuracy metric to that used for measurement accuracy requirements in RAN4 can be considered (e.g., a maximum of ±X dB threshold, with the expectation that error of Y % of predicted measurements fall within the specified threshold).