3GPP TSG-RAN WG2 Meeting #131	R2-2506153

# Bengaluru, India, August 25–29, 2025

Agenda Item:	8.8.2

Source:	Ericsson

Title:	DL coverage enhancements

Document for:	Discussion/Decision

# Introduction

During RAN2#130, RAN2 made good progress on the necessary changes to the SMTC framework to accommodate the newly introduced extended initial SSB periodicity and link-level beam hopping. The corresponding agreements are cited in the Annex.

In this contribution, we address the remaining FFS for RRC_IDLE/INACTIVE state (see Annex), and continue our analysis for RRC_CONNECTED state.

# Discussion

## 2.1	SMTC selection while in RRC_IDLE/INACTIVE

At the last meeting, RAN2 agreed to introduce a location-based SMTC selection procedure, where each SMTC can be associated with a reference location (related to the neighbor cells that need to be measured by the UE). Moreover, RAN2 captured an FFS on whether to also support an SSB-index based SMTC selection mechanism (see Annex), which we will discuss hereafter.

The discussion on the FFS on additionally supporting SSB-index-based SMTC selection should focus on the question whether there are relevant scenarios where location-based SMTC selection (the baseline) does not work, or does not work well. If the answer to the question is no, the introduction of an additional solution to the same problem is not necessary and may in fact only lead to market fragmentation and increased 3GPP specification and system complexity.

- The discussion on additionally supporting SSB-index-based SMTC selection should focus on whether there are relevant scenarios where location-based SMTC selection (the baseline) does not work.

It is understood in RAN2 that SSB-index-based SMTC selection is only applicable to cases where there are multiple SSB beams per cell (serving cell). Even though the 3GPP specifications allows for a multi-beam Uu interface in NTN, the general assumption so far has been a single SSB beam per cell. Location-based SMTC selection, on the other hand, is independent of SSB beams and works equally well with either a single SSB beam or multiple SSB beams.

- SSB-index-based SMTC selection is only applicable to the multiple SSB beam scenario, which can already be addressed by location-based SMTC selection.

While RAN2 has not identified a scenario where location-based SMTC selection does not work, the advocates of SSB-index-based SMTC selection have suggested some drawbacks of location-based SMTC selection.

One suggested drawback is the need for the UE to acquire its GNSS position in RRC Idle/Inactive mode and a resulting increase in UE power consumption. In our understanding, the UE anyway maintains an approximate GNSS position estimate. Since Rel-17, 3GPP has introduced several features that require the UE to know its GNSS position in RRC Idle/Inactive mode, e.g., neighbour-cell measurement relaxation. Moreover, the process of determining the GNSS position and (re-)selecting SMTCs is up to UE implementation. We assume that it is sufficient for the UE to perform this process in the order of once per service time of the satellite (typically about 5 minutes for LEO and Earth-fixed cells) or even less. Hence, we expect no considerable increase in UE energy consumption.

- Location-based SMTC selection is not expected to considerably increase the UE energy consumption in RRC Idle/Inactive mode.

Another suggested drawback is the signalling overhead for indicating the reference locations. It is true that the signalling overhead is not insignificant. However, introducing SSB-index-based SMTC selection is not an effective way to address any potential concerns, since SSB-index-based SMTC selection only applies to a limited scenario. We are open to discuss signalling optimizations for location-based SMTC selection – a broadly applicable solution – should companies in RAN2 conclude signalling overhead is an issue worth addressing. One promising solution has been proposed in [4].

- Due to the limited applicability, SSB-index-based SMTC selection does not effectively address any potential concerns about the signalling overhead of location-based SMTC selection.

We conclude that there is no sufficient motivation to introduce SSB-index-based SMTC selection as an additional method for SMTC selection.

SSB-index-based SMTC selection is not pursued in Rel-19.

## 2.2	SMTC selection upon entering RRC_CONNECTED

During RAN2#130, RAN2 has captured as a working assumption (WA) to introduce a mechanism to assist the network to configure SMTCs in RRC_CONNECTED mode. We now analyze alternative solutions for performing SMTC selection for a UE in RRC_CONNECTED, namely Alt. 1 and Alt. 2 (see Annex) as well as an alternative way that is based on legacy means.

### Based on legacy means (no enhancement)

It was proposed that the network could rely on legacy D1/D2 measurement events to understand which SMTCs are relevant for the UE, hence which SMTCs to configure. In practice, the network would need to configure one event per neighboring cell (e.g., six events) for this purpose. Upon triggering of these events, the network would (re-)configure the UE with the appropriate SMTC(s), i.e., the SMTC(s) of the associated neighbor cell(s).

Figure 1 visualizes this concept assuming D1 measurement events in a simplified manner. Reference locations and distance thresholds of different events are shown in different colors, i.e., grey-purple, grey-yellow, etc. The color grey is always associated with the serving cell, while the other colors are associated with the neighbor cells.

Figure 1: Visualization of an approach using the legacy D1 measurement event to identify the relevant SMTC(s) in an NTN deployment.

The advantage of this approach is that no enhancement is needed. Unfortunately, it has some serious drawbacks too. We consider three:

First, if the UE is within the distance threshold of reference location 1, the events would not trigger. In this case the network would have to blindly configure the UE. Hence the distance threshold of reference location 1 should be as small as possible. However, it cannot be smaller than 50 m as per current RRC specification.

Second, the areas where the conditions of the different events are fulfilled may largely overlap, as visualized in Figure 1. In this case, multiple events would trigger. Considering as an example the UE location indicated in Figure 1: Both the event illustrated in purple and the event illustrated in yellow would trigger. This may pose a problem for a UE that does not support parallelSMTC-r17 (see the table below), because for such UE, the network can only configure one additional SMTC (in addition to the serving cell’s SMTC). If the SMTCs of the neighbor cells differ, which is likely, the network must again blindly configure the UE. Back to the example in Figure 1: The network must blindly choose between the SMTCs associated with the purple event and the yellow event (assuming they differ), since it cannot know to which of the two reference locations the UE is closer.

- For SMTC selection using D1 measurement events: If multiple events trigger but the UE does not support parallelSMTC-r17, the network cannot know which associated SMTC is most relevant and must blindly choose an SMTC.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| parallelSMTC-r17 Indicates whether the UE supports NTN SSB based RRM measurements on target cells belonging to 4 SMTC-s on a single frequency carrier. If a UE does not include this field but includes nonTerrestrialNetwork-r17, the UE supports NTN SSB based RRM measurements on target cells belonging to 2 SMTC-s on a single frequency carrier. | UE | No | FDD only | FR1 only |

Third, the approach requires configuring up to six additional D1/D2 measurement events, which creates a significant signaling overhead, only for the sake of finding out which SMTC(s) to configure. We calculated a signaling overhead of 180 bits per event, which amounts to 1080 bits in case of six events. Considering one event, the overhead is mainly driven by the two reference locations (2 x 48 bits), the two distance thresholds (2 x 16 bits), and the location-based hysteresis parameter (16 bits), which accounts for 80% of the overhead. For six events this means 12 reference locations, 12 distance thresholds and 6 location-based hysteresis parameters.

After the network has identified the relevant SMTC(s), it must send an additional, second RRC reconfiguration message (in addition to the usual, first RRC reconfiguration message) to perform the reconfiguration, causing yet additional signaling overhead due to the need for two RRC reconfiguration messages.

- For SMTC selection using D1 measurement events, the signalling overhead for indicating six events amounts to 1080 bits, and further signalling overhead is caused by the subsequent measurement reporting and necessary additional RRC reconfiguration.

- An approach relying on legacy means (e.g., D1/D2 measurement events) to select SMTC(s) has significant drawbacks including the need for blind configuration in certain cases as well as significant a signalling overhead.

Thus, we understand that introducing a solution brings substantial benefits in terms of energy saving and signaling overhead. We therefore propose to confirm the WA (to introduce a mechanism that assists the network to configure SMTCs in RRC_CONNECTED).

RAN2 to confirm the WA (to introduce a mechanism that assists the network to configure SMTCs in RRC_CONNECTED).

### Based on Alternative 1 (Alt. 1)

Now considering Alt. 1: In this solution, as described in [3], the network configures the UE with a list of reference locations in OtherConfig, and further, configures the UE to report the N closest reference locations. Once the UE reports this information (via UE assistance information), the network uses the information to select the relevant SMTC(s). Subsequently, the network configures the selected SMTC(s). A flowchart of this solution is shown in Figure 2.

Figure 2: Flowchart of Alt. 1 (from RAN2#130) for selecting SMTC(s) in RRC_CONNECTED requiring two RRC reconfigurations.

This solution overcomes the first and second drawback (potential need for blind configuration), and partly the third drawback (signaling overhead) with respect to the above approach, which relies on legacy D1/D2 measurement events. Hence, it offers significant advantages over a “no enhancement” outcome. Comparing the signaling overhead, for example, this solution requires signaling of no more than five reference locations, since the serving-cell SMTC is always measured and does thus not need to be associated with a reference location. The signaling overhead for those five reference locations amounts to 5 x 48 bit = 240 bit (approximately 22% of the 1080 bits calculated above). One remaining drawback is that the solution still requires an additional, second RRC reconfiguration, as well as a UEAssistanceInformation message, as described in [4] and visualized in Figure 2.

- Alt. 1 overcomes many of the drawbacks of an approach relying on legacy means to select SMTC(s), except for the requirement for an additional RRC reconfiguration for final configuration of the selected SMTC(s).

In both the case of “no enhancement” and Alt. 1: Due to delta signaling, the additional RRC reconfiguration message may not be very large. Nevertheless, the requirement to send two RRC reconfiguration messages (instead of the usual one RRC reconfiguration message) upon each and every UE transition to RRC_CONNECTED is a huge burden for the network. This approach does not scale well, especially considering the large cells in NTN compared to TN and the potentially large number of served UEs as a consequence thereof. In our view this is a serious issue, also since radio resources in NTN are very scarce and valuable.

- The requirement to send two RRC reconfiguration messages for each UE entering RRC_CONNECTED poses a scalability issue, especially given the large number of UEs and the scarceness of radio resources in NTN cells.

#### Reducing signalling overhead with Alternative 1b (Alt. 1b)

To overcome this issue, instead of relying on a list of reference locations provided via dedicated RRC signalling, the UE could reuse the list of reference locations already indicated via system information broadcast (e.g., in SIB2). In that case, the UE already knows which reference locations are relevant, e.g., which reference locations are the N closest, from RRC_IDLE/INACTIVE mode operation. Hence, the UE can report the relevant (e.g., N closest) reference locations already upon entering RRC_CONNECTED mode, e.g., via RRCSetupComplete or RRCResumeComplete message, when coming from RRC_IDLE or RRC_INACTIVE mode, respectively. One simple way of indicating this is using a 5-bit bitmap, where each bit in the bitmap corresponds to a reference location. If the UE supports less than N+1 parallel SMTCs (“+1” since the serving-cell SMTC must be measured), the UE may report less reference locations according to its UE capabilities.

- Alt. 1b is a variant of Alt. 1 in which the UE reports the relevant (e.g., N closest) reference location as identified in RRC_IDLE/INACTIVE during RRC connection establishment/resume.

The key advantage of this solution (Alt. 1b) is that the network can configure the associated SMTC(s) already in the subsequent, usual RRC reconfiguration message. As a result, there is no need for an additional RRC reconfiguration message, and also no need for a UEAssistanceInformation message, for each UE entering RRC_CONNECTED mode. Furthermore, this solution (Alt. 1b) eliminates the signalling overhead for indicating the reference locations in dedicated RRC signalling for each UE. In conclusion, this approach reduces the signalling overhead for SMTC selection in RRC_CONNECTED mode to a minimum and helps harmonize the feature for both RRC_IDLE/INACTIVE and RRC_CONNECTED modes.

- Alt. 1b effectively eliminates the need for additional signalling messages, minimizes the signalling overhead, and harmonizes the features for RRC_IDLE/INACTIVE and RRC_CONNECTED modes.

### Based on Alternative 2 (Alt. 2)

Finally considering Alt. 2: This solution is very similar to Alt. 1, except one aspect: The SMTC selection happens at the UE side. Like in Alt. 1, the UE is configured with a list of reference locations (likewise at most five reference locations). But in contrast to Alt. 1, the UE is also configured with SMTCs associated to those reference locations. This allows the UE to already select and apply the relevant SMTC(s). Similar to Alt. 1, the UE may be configured to select a certain number of SMTCs, e.g., K, among the configured SMTCs (excluding the SMTC of the serving cell). Subsequently, the UE reports the selected SMTC(s) to the network, e.g., via the RRCReconfigurationComplete message. The indication of the selected SMTC(s) does not result in the transmission of additional signalling messages. The network can consider the UE’s SMTC selection in its scheduling decisions, e.g., by refraining from scheduling during SMTC windows. A flowchart of this solution is shown in Figure 3.

Figure 3: Flowchart of Alt. 2 (from RAN2#130) for selecting SMTC(s) in RRC_CONNECTED requiring only one (the usual) RRC reconfiguration.

This solution has the advantage that it does not necessitate additional RRC reconfigurations for configuring SMTC(s). Besides, this solution most closely resembles the RAN2-agreed solution for SMTC selection in RRC_IDLE/INACTIVE (where SMTC selection is also at the UE). SMTC selection at the UE is beneficial since the UE has more information than the network, e.g., it knows the actual distance to the reference locations instead of just knowing which N of the reference locations are the closest ones.

In contrast to “no enhancement” or Alt. 1, this solution does not require reporting of location information to the network and does not expose the UE’s location like the above two alternatives. It is instead a more general solution and does therefore not depend on regulatory restrictions as discussed in our previous contribution [5].

- Alt. 2 solves the issue of SMTC selection in RRC_CONNECTED as good as Alt. 1 but with less signalling overhead in terms of number of RRCReconfiguration messages, thus solving the otherwise arising scalability challenge.

A UE selects the most relevant SMTC(s) based on location information included in the measurement configurations and reports its selection to the network.

In RRC_CONNECTED, the network can configure the UE to select a certain number of SMTCs, e.g., K, among the configured SMTCs.

## 2.3	Stage 3 impact of 6 SMTCs

During the last running RRC offline discussion, it became apparent that companies have a different understanding of the present RAN2 agreements. The addition of the new list allows to provide a new SMTC periodicity for the 3 entries of smtc4list. Besides, it allows to configure 2 more SMTCs for the Rel-19 UEs that support the feature.

If RAN2 introduces multiple periodicities per frequency layer, take the text proposal in section 3.1 as a baseline.

Provided RAN4 concerns, multiple periodicities may not be finally agreed to be introduced in Rel-19. In this case, we see little benefit of the previous meeting RAN2 agreement to introduce a new SMTC list instead of extending legacy smtc4list. Provided it would be a simple extension of the present list to include 2 more SMTCs, i.e., there is no new field description, we propose to re-consider the previous agreement and extend the smtc4list.

If RAN2 decides not to introduce multiple SMTC periodicities per frequency layer, take the text proposal in section 3.2 as a baseline.

# 3	Text Proposals

## 3.1	6 STMC offsets with multiple periodicities

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=                            SEQUENCE {

cellReselectionInfoCommon           SEQUENCE {

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)          OPTIONAL,       -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                     OPTIONAL,       -- Need S

rangeToBestCell                     RangeToBestCell                                 OPTIONAL,       -- Need R

q-Hyst                              ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars           SEQUENCE {

mobilityStateParameters             MobilityStateParameters,

q-HystSF                        SEQUENCE {

sf-Medium                       ENUMERATED {dB-6, dB-4, dB-2, dB0},

sf-High                         ENUMERATED {dB-6, dB-4, dB-2, dB0}

}

}                                                                                   OPTIONAL,       -- Need R

...

},

cellReselectionServingFreqInfo      SEQUENCE {

s-NonIntraSearchP                   ReselectionThreshold                            OPTIONAL,       -- Need S

s-NonIntraSearchQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need S

threshServingLowP                   ReselectionThreshold,

threshServingLowQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need R

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                      OPTIONAL,       -- Need R

...

},

intraFreqCellReselectionInfo        SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                      OPTIONAL,       -- Need R

q-QualMin                           Q-QualMin                                       OPTIONAL,       -- Need S

s-IntraSearchP                      ReselectionThreshold,

s-IntraSearchQ                      ReselectionThresholdQ                           OPTIONAL,       -- Need S

t-ReselectionNR                     T-Reselection,

frequencyBandList                   MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need S

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need R

p-Max                               P-Max                                           OPTIONAL,       -- Need S

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S

ss-RSSI-Measurement                 SS-RSSI-Measurement                             OPTIONAL,       -- Need R

ssb-ToMeasure                       SSB-ToMeasure                                   OPTIONAL,       -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

...,

[[

t-ReselectionNR-SF                  SpeedStateScaleFactors                          OPTIONAL        -- Need R

]],

[[

smtc2-LP-r16                        SSB-MTC2-LP-r16                                 OPTIONAL,        -- Need R

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                    OPTIONAL         -- Cond SharedSpectrum

]],

[[

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                    OPTIONAL         -- Cond SharedSpectrum2

]],

[[

smtc4list-r17                       SSB-MTC4List-r17                                OPTIONAL         -- Need R

]],

[[

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760              OPTIONAL,       -- Need R

frequencyBandListSUL-v1760          MultiFrequencyBandListNR-SIB-v1760              OPTIONAL        -- Need R

]],

[[

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18         OPTIONAL        -- Need S

]],

[[

smtc5list-r19                       SSB-MTC5List-r19                                OPTIONAL,        -- Need R

referenceLocationList-r19           ReferenceLocationList-r19                       OPTIONAL         -- Need R

]]

},

...,

[[

relaxedMeasurement-r16              SEQUENCE {

lowMobilityEvaluation-r16           SEQUENCE {

s-SearchDeltaP-r16                  ENUMERATED {

dB3, dB6, dB9, dB12, dB15,

spare3, spare2, spare1},

t-SearchDeltaP-r16                  ENUMERATED {

s5, s10, s20, s30, s60, s120, s180,

s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                   OPTIONAL,       -- Need R

cellEdgeEvaluation-r16              SEQUENCE {

s-SearchThresholdP-r16              ReselectionThreshold,

s-SearchThresholdQ-r16              ReselectionThresholdQ                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition-r16     ENUMERATED {true}                               OPTIONAL,       -- Need R

highPriorityMeasRelax-r16           ENUMERATED {true}                               OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]],

[[

cellEquivalentSize-r17                  INTEGER(2..16)                                  OPTIONAL,       -- Cond HSDN

relaxedMeasurement-r17                  SEQUENCE {

stationaryMobilityEvaluation-r17        SEQUENCE {

s-SearchDeltaP-Stationary-r17           ENUMERATED {dB2, dB3, dB6, dB9, dB12, dB15, spare2, spare1},

t-SearchDeltaP-Stationary-r17           ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

},

cellEdgeEvaluationWhileStationary-r17   SEQUENCE {

s-SearchThresholdP2-r17                 ReselectionThreshold,

s-SearchThresholdQ2-r17                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition2-r17        ENUMERATED {true}                           OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]]

}

RangeToBestCell    ::= Q-OffsetRange

ReferenceLocationList ::= SEQUENCE (SIZE(1..5)) OF ReferenceLocation

-- TAG-SIB2-STOP

-- ASN1STOP

SIB2 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

cellEdgeEvaluation Indicates the criteria for a UE to detect that it is not at cell edge, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.2).

cellEdgeEvaluationWhileStationary Indicates the criteria for a UE to detect that it is not at cell edge while stationary, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.4).

cellEquivalentSize The number of cell count used for mobility state estimation for this cell as specified in TS 38.304 [20].

cellReselectionInfoCommon Cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection.

cellReselectionServingFreqInfo Information common for non-intra-frequency cell re-selection i.e. cell re-selection to inter-frequency and inter-RAT cells.

combineRelaxedMeasCondition When both lowMobilityEvaluation and cellEdgeEvaluation criteria are present in SIB2, this parameter configures the UE to fulfil both criteria in order to relax measurement requirements for cell reselection. If the field is absent, the UE is allowed to relax measurement requirements for cell reselection when either or both of the criteria are met. (See TS 38.304 [20], clause 5.2.4.9.0)

combineRelaxedMeasCondition2 When both stationaryMobilityEvaluation and cellEdgeEvaluationWhileStationary criteria are present in SIB2, this parameter configures the UE to fulfil both criteria in order to relax measurement requirements for cell reselection. If the field is absent, the UE is allowed to relax measurement requirements for cell reselection when only the stationary criteria is met. (See TS 38.304 [20], clause 5.2.4.9.0)

deriveSSB-IndexFromCell This field indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the serving frequency as specified in TS 38.133 [14].

frequencyBandList Indicates the list of frequency bands for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.3.

frequencyBandListAerial Indicates the list of frequency bands for aerial operation for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.3.

highPriorityMeasRelax Indicates whether measurements can be relaxed on high priority frequencies. If the field is absent, the UE shall not relax measurements on high priority frequencies beyond "Thigher_priority_search" unless both low mobility and not at cell edge criteria are fulfilled (see TS 38.133 [14], clauses 4.2.2.7, 4.2.2.10 and 4.2.2.11).

intraFreqCellReselectionInfo Cell re-selection information common for intra-frequency cells.

lowMobilityEvaluation Indicates the criteria for a UE to detect low mobility, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.1).

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation. If the field is absent the UE uses the measurement quantity as specified in TS 38.304 [20].

p-Max Value in dBm applicable for the intra-frequency neighbouring NR cells. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emissions requirements as specified in TS 38.106 [79].

q-Hyst Parameter "Qhyst" in TS 38.304 [20], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on.

q-HystSF Parameter "Speed dependent ScalingFactor for Qhyst" in TS 38.304 [20]. The sf-Medium and sf-High concern the additional hysteresis to be applied, in Medium and High Mobility state respectively, to Qhyst as defined in TS 38.304 [20]. In dB. Value dB-6 corresponds to -6dB, dB-4 corresponds to -4dB and so on.

q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells.

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells.

rangeToBestCell Parameter "rangeToBestCell" in TS 38.304 [20]. The network configures only non-negative (in dB) values.

referenceLocationList Indicates a reference location associated to an SMTC configuration in smtc4list and smtc5list. If present, it includes the same number of entries as the total of smtc4list and smtc5list. If smtc4list is present, the first entry in referenceLocationList corresponds to the first entry in smtc4list and so on until the last entry of smtc4list. Then, the next entry corresponds to the first entry of smtc5list. If smtc4list is not present, the first entry in the list corresponds to the first entry in smtc5list and so on.

relaxedMeasurement Configuration to allow relaxation of RRM measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9). In NTN, this field is only applicable for GSO neighbour cells.

s-IntraSearchP Parameter "SIntraSearchP" in TS 38.304 [20].

s-IntraSearchQ Parameter "SIntraSearchQ" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SIntraSearchQ.

s-NonIntraSearchP Parameter "SnonIntraSearchP" in TS 38.304 [20]. If this field is absent, the UE applies the (default) value of infinity for SnonIntraSearchP.

s-NonIntraSearchQ Parameter "SnonIntraSearchQ" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SnonIntraSearchQ.

s-SearchDeltaP Parameter "SSearchDeltaP" in TS 38.304 [20]. Value dB3 corresponds to 3 dB, dB6 corresponds to 6 dB and so on.

s-SearchDeltaP-Stationary Parameter "SSearchDeltaP-Stationary" in TS 38.304 [20]. Value dB2 corresponds to 2 dB, dB3 corresponds to 3 dB and so on.

s-SearchThresholdP, s-SearchThresholdP2 Parameters "SSearchThresholdP" and "SSearchThresholdP2" in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP.

s-SearchThresholdQ, s-SearchThresholdQ2 Parameters "SSearchThresholdQ" and "SSearchThresholdQ2" in TS 38.304 [20]. The network configures s-SearchThresholdQ and s-SearchThresholdQ2 to be less than or equal to s-IntraSearchQ and s-NonIntraSearchQ.

smtc Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequency cells. If the field is broadcast by an NTN cell, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference.

smtc2-LP Measurement timing configuration for intra-frequency neighbour cells with a Long Periodicity (LP) indicated by periodicity in smtc2-LP. The timing offset and duration are equal to the offset and duration indicated in smtc in intraFreqCellReselectionInfo. The periodicity in smtc2-LP can only be set to a value strictly larger than the periodicity in smtc in intraFreqCellReselectionInfo (e.g. if smtc indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if smtc indicates sf160, smtc2-LP cannot be configured). The pci-List, if present, includes the physical cell identities of the intra-frequency neighbour cells with Long Periodicity. If smtc2-LP is absent, the UE assumes that there are no intra-frequency neighbour cells with a Long Periodicity. This field is not configured together with smtc4list.

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. If smtct5list is present and only periodicity is present for an SSB-MTC5, the PCI list and offset corresponds to the same entry in smtc4list. The network can configure a maximum of 5 SMTC offsets between smtc4list and smtc5list.

ssb-PositionQCL-Common Indicates the QCL relation between SS/PBCH blocks for intra-frequency neighbor cells as specified in TS 38.213 [13], clause 4.1.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent the UE measures on all SS-blocks.

stationaryMobilityEvaluation Indicates the criteria for a UE to detect stationary mobility, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.0).

t-ReselectionNR Parameter "TreselectionNR" in TS 38.304 [20].

t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 38.304 [20]. If the field is absent, the UE behaviour is specified in TS 38.304 [20].

threshServingLowP Parameter "ThreshServing, LowP" in TS 38.304 [20].

threshServingLowQ Parameter "ThreshServing, LowQ" in TS 38.304 [20].

t-SearchDeltaP Parameter "TSearchDeltaP" in TS 38.304 [20]. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on.

t-SearchDeltaP-Stationary Parameter "TSearchDeltaP-Stationary" in TS 38.304 [20]. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on.

#### –	SSB-MTC

The IE SSB-MTC is used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

SSB-MTC2 ::=                        SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need M

periodicity                         ENUMERATED {sf5, sf10, sf20, sf40, sf80, spare3, spare2, spare1}

}

SSB-MTC2-LP-r16 ::=                 SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need R

periodicity                         ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

SSB-MTC3-r16 ::=                    SEQUENCE {

periodicityAndOffset-r16            CHOICE {

sf5-r16                                     INTEGER (0..4),

sf10-r16                                    INTEGER (0..9),

sf20-r16                                    INTEGER (0..19),

sf40-r16                                    INTEGER (0..39),

sf80-r16                                    INTEGER (0..79),

sf160-r16                                   INTEGER (0..159),

sf320-r16                                   INTEGER (0..319),

sf640-r16                                   INTEGER (0..639),

sf1280-r16                                  INTEGER (0..1279)

},

duration-r16                        ENUMERATED {sf1, sf2, sf3, sf4, sf5},

pci-List-r16                        SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,  -- Need M

ssb-ToMeasure-r16                   SetupRelease { SSB-ToMeasure }                                          OPTIONAL   -- Need M

}

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

SSB-MTC5-r19 ::=             SEQUENCE {

pci-List-r19                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need R

periodicity-r19              ENUMERATED {sf10, sf20, sf40, sf80, sf160}                                     OPTIONAL,  -- Need R

offset-r19                   INTEGER (0..159)                                                               OPTIONAL   -- Need R

}

SSB-MTC-AdditionalPCI-r17 ::=       SEQUENCE {

additionalPCIIndex-r17              AdditionalPCIIndex-r17,

additionalPCI-r17                   PhysCellId,

periodicity-r17                     ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 },

ssb-PositionsInBurst-r17            CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

},

ss-PBCH-BlockPower-r17              INTEGER (-60..50)

}

-- TAG-SSB-MTC-STOP

-- ASN1STOP

## 3.2	6 SMTC offsets without multiple periodicities

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=                            SEQUENCE {

cellReselectionInfoCommon           SEQUENCE {

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)          OPTIONAL,       -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                     OPTIONAL,       -- Need S

rangeToBestCell                     RangeToBestCell                                 OPTIONAL,       -- Need R

q-Hyst                              ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars           SEQUENCE {

mobilityStateParameters             MobilityStateParameters,

q-HystSF                        SEQUENCE {

sf-Medium                       ENUMERATED {dB-6, dB-4, dB-2, dB0},

sf-High                         ENUMERATED {dB-6, dB-4, dB-2, dB0}

}

}                                                                                   OPTIONAL,       -- Need R

...

},

cellReselectionServingFreqInfo      SEQUENCE {

s-NonIntraSearchP                   ReselectionThreshold                            OPTIONAL,       -- Need S

s-NonIntraSearchQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need S

threshServingLowP                   ReselectionThreshold,

threshServingLowQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need R

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                      OPTIONAL,       -- Need R

...

},

intraFreqCellReselectionInfo        SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                      OPTIONAL,       -- Need R

q-QualMin                           Q-QualMin                                       OPTIONAL,       -- Need S

s-IntraSearchP                      ReselectionThreshold,

s-IntraSearchQ                      ReselectionThresholdQ                           OPTIONAL,       -- Need S

t-ReselectionNR                     T-Reselection,

frequencyBandList                   MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need S

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need R

p-Max                               P-Max                                           OPTIONAL,       -- Need S

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S

ss-RSSI-Measurement                 SS-RSSI-Measurement                             OPTIONAL,       -- Need R

ssb-ToMeasure                       SSB-ToMeasure                                   OPTIONAL,       -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

...,

[[

t-ReselectionNR-SF                  SpeedStateScaleFactors                          OPTIONAL        -- Need R

]],

[[

smtc2-LP-r16                        SSB-MTC2-LP-r16                                 OPTIONAL,        -- Need R

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                    OPTIONAL         -- Cond SharedSpectrum

]],

[[

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                    OPTIONAL         -- Cond SharedSpectrum2

]],

[[

smtc4list-r17                       SSB-MTC4List-r17                                OPTIONAL         -- Need R

]],

[[

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760              OPTIONAL,       -- Need R

frequencyBandListSUL-v1760          MultiFrequencyBandListNR-SIB-v1760              OPTIONAL        -- Need R

]],

[[

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18         OPTIONAL        -- Need S

]],

[[

smtc4list-v19xy                     SSB-MTC4List-v19xy                              OPTIONAL,        -- Need R

referenceLocationList-r19           ReferenceLocationList-r19                       OPTIONAL         -- Need R

]]

},

...,

[[

relaxedMeasurement-r16              SEQUENCE {

lowMobilityEvaluation-r16           SEQUENCE {

s-SearchDeltaP-r16                  ENUMERATED {

dB3, dB6, dB9, dB12, dB15,

spare3, spare2, spare1},

t-SearchDeltaP-r16                  ENUMERATED {

s5, s10, s20, s30, s60, s120, s180,

s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                   OPTIONAL,       -- Need R

cellEdgeEvaluation-r16              SEQUENCE {

s-SearchThresholdP-r16              ReselectionThreshold,

s-SearchThresholdQ-r16              ReselectionThresholdQ                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition-r16     ENUMERATED {true}                               OPTIONAL,       -- Need R

highPriorityMeasRelax-r16           ENUMERATED {true}                               OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]],

[[

cellEquivalentSize-r17                  INTEGER(2..16)                                  OPTIONAL,       -- Cond HSDN

relaxedMeasurement-r17                  SEQUENCE {

stationaryMobilityEvaluation-r17        SEQUENCE {

s-SearchDeltaP-Stationary-r17           ENUMERATED {dB2, dB3, dB6, dB9, dB12, dB15, spare2, spare1},

t-SearchDeltaP-Stationary-r17           ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

},

cellEdgeEvaluationWhileStationary-r17   SEQUENCE {

s-SearchThresholdP2-r17                 ReselectionThreshold,

s-SearchThresholdQ2-r17                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition2-r17        ENUMERATED {true}                           OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]]

}

RangeToBestCell    ::= Q-OffsetRange

ReferenceLocationList ::= SEQUENCE (SIZE(1..5)) OF ReferenceLocation

-- TAG-SIB2-STOP

-- ASN1STOP

SIB2 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

cellEdgeEvaluation Indicates the criteria for a UE to detect that it is not at cell edge, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.2).

cellEdgeEvaluationWhileStationary Indicates the criteria for a UE to detect that it is not at cell edge while stationary, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.4).

cellEquivalentSize The number of cell count used for mobility state estimation for this cell as specified in TS 38.304 [20].

cellReselectionInfoCommon Cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection.

cellReselectionServingFreqInfo Information common for non-intra-frequency cell re-selection i.e. cell re-selection to inter-frequency and inter-RAT cells.

combineRelaxedMeasCondition When both lowMobilityEvaluation and cellEdgeEvaluation criteria are present in SIB2, this parameter configures the UE to fulfil both criteria in order to relax measurement requirements for cell reselection. If the field is absent, the UE is allowed to relax measurement requirements for cell reselection when either or both of the criteria are met. (See TS 38.304 [20], clause 5.2.4.9.0)

combineRelaxedMeasCondition2 When both stationaryMobilityEvaluation and cellEdgeEvaluationWhileStationary criteria are present in SIB2, this parameter configures the UE to fulfil both criteria in order to relax measurement requirements for cell reselection. If the field is absent, the UE is allowed to relax measurement requirements for cell reselection when only the stationary criteria is met. (See TS 38.304 [20], clause 5.2.4.9.0)

deriveSSB-IndexFromCell This field indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the serving frequency as specified in TS 38.133 [14].

frequencyBandList Indicates the list of frequency bands for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.3.

frequencyBandListAerial Indicates the list of frequency bands for aerial operation for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.3.

highPriorityMeasRelax Indicates whether measurements can be relaxed on high priority frequencies. If the field is absent, the UE shall not relax measurements on high priority frequencies beyond "Thigher_priority_search" unless both low mobility and not at cell edge criteria are fulfilled (see TS 38.133 [14], clauses 4.2.2.7, 4.2.2.10 and 4.2.2.11).

intraFreqCellReselectionInfo Cell re-selection information common for intra-frequency cells.

lowMobilityEvaluation Indicates the criteria for a UE to detect low mobility, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.1).

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation. If the field is absent the UE uses the measurement quantity as specified in TS 38.304 [20].

p-Max Value in dBm applicable for the intra-frequency neighbouring NR cells. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emissions requirements as specified in TS 38.106 [79].

q-Hyst Parameter "Qhyst" in TS 38.304 [20], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on.

q-HystSF Parameter "Speed dependent ScalingFactor for Qhyst" in TS 38.304 [20]. The sf-Medium and sf-High concern the additional hysteresis to be applied, in Medium and High Mobility state respectively, to Qhyst as defined in TS 38.304 [20]. In dB. Value dB-6 corresponds to -6dB, dB-4 corresponds to -4dB and so on.

q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells.

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells.

rangeToBestCell Parameter "rangeToBestCell" in TS 38.304 [20]. The network configures only non-negative (in dB) values.

referenceLocationList Indicates a reference location associated to an SMTC configuration in smtc4list and smtc4list -v19xy. If present, it includes the same number of entries as the total of smtc4list and smtc4list-v19xy. If smtc4list is present, the first entry in referenceLocationList corresponds to the first entry in smtc4list and so on until the last entry of smtc4list. Then, the next entry corresponds to the first entry of smtc4list-v19xy.

relaxedMeasurement Configuration to allow relaxation of RRM measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9). In NTN, this field is only applicable for GSO neighbour cells.

s-IntraSearchP Parameter "SIntraSearchP" in TS 38.304 [20].

s-IntraSearchQ Parameter "SIntraSearchQ" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SIntraSearchQ.

s-NonIntraSearchP Parameter "SnonIntraSearchP" in TS 38.304 [20]. If this field is absent, the UE applies the (default) value of infinity for SnonIntraSearchP.

s-NonIntraSearchQ Parameter "SnonIntraSearchQ" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SnonIntraSearchQ.

s-SearchDeltaP Parameter "SSearchDeltaP" in TS 38.304 [20]. Value dB3 corresponds to 3 dB, dB6 corresponds to 6 dB and so on.

s-SearchDeltaP-Stationary Parameter "SSearchDeltaP-Stationary" in TS 38.304 [20]. Value dB2 corresponds to 2 dB, dB3 corresponds to 3 dB and so on.

s-SearchThresholdP, s-SearchThresholdP2 Parameters "SSearchThresholdP" and "SSearchThresholdP2" in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP.

s-SearchThresholdQ, s-SearchThresholdQ2 Parameters "SSearchThresholdQ" and "SSearchThresholdQ2" in TS 38.304 [20]. The network configures s-SearchThresholdQ and s-SearchThresholdQ2 to be less than or equal to s-IntraSearchQ and s-NonIntraSearchQ.

smtc Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequency cells. If the field is broadcast by an NTN cell, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference.

smtc2-LP Measurement timing configuration for intra-frequency neighbour cells with a Long Periodicity (LP) indicated by periodicity in smtc2-LP. The timing offset and duration are equal to the offset and duration indicated in smtc in intraFreqCellReselectionInfo. The periodicity in smtc2-LP can only be set to a value strictly larger than the periodicity in smtc in intraFreqCellReselectionInfo (e.g. if smtc indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if smtc indicates sf160, smtc2-LP cannot be configured). The pci-List, if present, includes the physical cell identities of the intra-frequency neighbour cells with Long Periodicity. If smtc2-LP is absent, the UE assumes that there are no intra-frequency neighbour cells with a Long Periodicity. This field is not configured together with smtc4list.

smtc4list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc4list-v19xy, it is up to the UE to select which SMTCs to consider.

ssb-PositionQCL-Common Indicates the QCL relation between SS/PBCH blocks for intra-frequency neighbor cells as specified in TS 38.213 [13], clause 4.1.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent the UE measures on all SS-blocks.

stationaryMobilityEvaluation Indicates the criteria for a UE to detect stationary mobility, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.0).

t-ReselectionNR Parameter "TreselectionNR" in TS 38.304 [20].

t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 38.304 [20]. If the field is absent, the UE behaviour is specified in TS 38.304 [20].

threshServingLowP Parameter "ThreshServing, LowP" in TS 38.304 [20].

threshServingLowQ Parameter "ThreshServing, LowQ" in TS 38.304 [20].

t-SearchDeltaP Parameter "TSearchDeltaP" in TS 38.304 [20]. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on.

t-SearchDeltaP-Stationary Parameter "TSearchDeltaP-Stationary" in TS 38.304 [20]. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on.

#### –	SSB-MTC

The IE SSB-MTC is used to configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

SSB-MTC2 ::=                        SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need M

periodicity                         ENUMERATED {sf5, sf10, sf20, sf40, sf80, spare3, spare2, spare1}

}

SSB-MTC2-LP-r16 ::=                 SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need R

periodicity                         ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

SSB-MTC3-r16 ::=                    SEQUENCE {

periodicityAndOffset-r16            CHOICE {

sf5-r16                                     INTEGER (0..4),

sf10-r16                                    INTEGER (0..9),

sf20-r16                                    INTEGER (0..19),

sf40-r16                                    INTEGER (0..39),

sf80-r16                                    INTEGER (0..79),

sf160-r16                                   INTEGER (0..159),

sf320-r16                                   INTEGER (0..319),

sf640-r16                                   INTEGER (0..639),

sf1280-r16                                  INTEGER (0..1279)

},

duration-r16                        ENUMERATED {sf1, sf2, sf3, sf4, sf5},

pci-List-r16                        SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,  -- Need M

ssb-ToMeasure-r16                   SetupRelease { SSB-ToMeasure }                                          OPTIONAL   -- Need M

}

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

SSB-MTC4-v19xy ::=           SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need R

offset-r17                   INTEGER (0..159)

}

SSB-MTC-AdditionalPCI-r17 ::=       SEQUENCE {

additionalPCIIndex-r17              AdditionalPCIIndex-r17,

additionalPCI-r17                   PhysCellId,

periodicity-r17                     ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 },

ssb-PositionsInBurst-r17            CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

},

ss-PBCH-BlockPower-r17              INTEGER (-60..50)

}

-- TAG-SSB-MTC-STOP

-- ASN1STOP

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	The discussion on additionally supporting SSB-index-based SMTC selection should focus on whether there are relevant scenarios where location-based SMTC selection (the baseline) does not work.

Observation 2	SSB-index-based SMTC selection is only applicable to the multiple SSB beam scenario, which can already be addressed by location-based SMTC selection.

Observation 3	Location-based SMTC selection is not expected to considerably increase the UE energy consumption in RRC Idle/Inactive mode.

Observation 4	Due to the limited applicability, SSB-index-based SMTC selection does not effectively address any potential concerns about the signalling overhead of location-based SMTC selection.

Observation 5	For SMTC selection using D1 measurement events: If multiple events trigger but the UE does not support parallelSMTC-r17, the network cannot know which associated SMTC is most relevant and must blindly choose an SMTC.

Observation 6	For SMTC selection using D1 measurement events, the signalling overhead for indicating six events amounts to 1080 bits, and further signalling overhead is caused by the subsequent measurement reporting and necessary additional RRC reconfiguration.

Observation 7	An approach relying on legacy means (e.g., D1/D2 measurement events) to select SMTC(s) has significant drawbacks including the need for blind configuration in certain cases as well as significant a signalling overhead.

Observation 8	Alt. 1 overcomes many of the drawbacks of an approach relying on legacy means to select SMTC(s), except for the requirement for an additional RRC reconfiguration for final configuration of the selected SMTC(s).

Observation 9	The requirement to send two RRC reconfiguration messages for each UE entering RRC_CONNECTED poses a scalability issue, especially given the large number of UEs and the scarceness of radio resources in NTN cells.

Observation 10	Alt. 1b is a variant of Alt. 1 in which the UE reports the relevant (e.g., N closest) reference location as identified in RRC_IDLE/INACTIVE during RRC connection establishment/resume.

Observation 11	Alt. 1b effectively eliminates the need for additional signalling messages, minimizes the signalling overhead, and harmonizes the features for RRC_IDLE/INACTIVE and RRC_CONNECTED modes.

Observation 12	Alt. 2 solves the issue of SMTC selection in RRC_CONNECTED as good as Alt. 1 but with less signalling overhead in terms of number of RRCReconfiguration messages, thus solving the otherwise arising scalability challenge.

Based on the discussion in the previous sections we propose the following:

Proposal 1	SSB-index-based SMTC selection is not pursued in Rel-19.

Proposal 2	RAN2 to confirm the WA (to introduce a mechanism that assists the network to configure SMTCs in RRC_CONNECTED).

Proposal 3	A UE selects the most relevant SMTC(s) based on location information included in the measurement configurations and reports its selection to the network.

Proposal 4	In RRC_CONNECTED, the network can configure the UE to select a certain number of SMTCs, e.g., K, among the configured SMTCs.

Proposal 5	If RAN2 introduces multiple periodicities per frequency layer, take the text proposal in section 3.1 as a baseline.

Proposal 6	If RAN2 decides not to introduce multiple SMTC periodicities per frequency layer, take the text proposal in section 3.2 as a baseline.

# References

R2-2501415, Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements, Huawei, RAN2#129, February 2025.

RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN#106, Dec. 2024.

R2-2504571, Discussion on Downlink Coverage Enhancements, CSCN, Huawei, HiSilicon, ZTE corporation, Sanechips, CATT, RAN2#130, May 2025.

R2-2504207, Open issues on Downlink Coverage Enhancement, Samsung, RAN2#130, May 2025.

R2-2504653, DL coverage enhancements, Ericsson, RAN2#130, May 2025

# Annex

During RAN2#130, the following agreements related to DL coverage enhancements were captured:

WA: We introduce a mechanism to assist the NW to configure the SMTCs in connected mode

RAN2 to agree that the maximum configured SMTCs per frequency for idle/inactive UEs is 6.

To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

- Alt 1: UE provide the closest N reference locations/neighbor cells to network.

- Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)