3GPP TSG RAN WG2 #131	R2-2506165

Bangalore, India, 25-29 August 2025

Agenda Item:	8.13.2
Source:	Toyota ITC
Title:	Relay (re)selection in multi-hop Relay

Document for:	Discussion and Decision

# Introduction

The Rel-19 Work Item on NR sidelink multi-hop relay was approved in [1] and then further revised in [2]. It includes the following objectives, from [2]:

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

This document discusses some remaining issues for this Work Item.

2.1	Relay (re)selection and latency

At RAN2-130, the LS R2-2504809 [3] to SA2/CT1 “LS on relay reselection notification agreements” that gathered some RAN2 agreements on this overall topic included the following:

“(…) the notification message is used to inform L2 or L3 U2N Remote UE(s) when the L2 or L3 U2N Relay UE performs handover, detects Uu RLF, or its Uu RRC connection establishment/resume fails. Upon reception of the notification message, it is up to U2N Remote UE implementation whether to release or keep the unicast PC5 link. If U2N Remote UE decides to release the unicast PC5 link, it triggers the PC5 release procedure and may perform cell or relay reselection”

This includes one of the cases where a Relay performs relay reselection.

In addition; at RAN2-130, fast forwarding was discussed and the following was agreed:

RAN2-130 agreement:

Do not pursue fast forwarding.

Continue during this meeting to discuss including the RRC state in relay discovery, including:

- 	Spec impact to capture it in the RRC container

-	Implications for (re)selection procedure

Continue discussion of whether/how to extend timers.

One consequence of not having fast forwarding is the impact on latency. When this was discussed at RAN2-130, some companies mentioned that one solution is that during relay selection, the remote UE could select a smaller hop count.

Observation 1: Not having fast forwarding may have some impact on latency.

Observation 2: Minimizing the number of hop counts may have positive impacts on latency and reliability.

Proposal 1: During relay (re)selection, the remote UE may take into account the total number of hop counts to select the relay UE.

2.2	Forwarding of notification message from intermediate UE

So far, as from the current RAN2 decisions, if the child UE in RRC_IDLE/INACTIVE receives the notification, it is up to UE implementation of the child UE to keep or release the PC5 link. Depending on the exact issue, the UE may decide one or another. Therefore, it appears useful that the cause value is forwarded by the intermediate UE in the notification message.

Observation 3: The use of the cause value in the notification message may be useful for a UE in RRC_IDLE/INACTIVE to decide to keep or release the PC5 link.

Proposal 2: The cause value is forwarded in the notification message towards the child UE.

# Conclusion

This contribution discusses relay (re)selection and proposes the following:

Observation 1: Not having fast forwarding may have some impact on latency.

Observation 2: Minimizing the number of hop counts may have positive impacts on latency and reliability.

Proposal 1: During relay (re)selection, the remote UE takes into account the total number of hop counts to select the relay UE.

Observation 2: The use of the cause value in the notification message may be useful for a UE in RRC_IDLE/INACTIVE to decide to keep or release the PC5 link.

Proposal 2: The cause value is forwarded in the notification message towards the child UE.

# References

[1] RP-241609, RAN-104. Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.

[2] RP-242349, RAN-105. Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.

[3] R2-2504809, LS on LS on relay reselection notification agreements.