3GPP TSG-RAN WG2 Meeting #131	R2-2506176

Bengaluru, IN, Aug 25-29, 2025

Agenda Item:	8.17

Source:	THALES

Title:	Remaining issues for IoT-NTN TDD mode

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss open issues and discussions listed in [2-7] :

- Neighbour cell list for measurements and cell reselection

- DRX cycle aligned to 90 ms periodicity

- Subframes between preamble transmission and RA Response Window

- Support of OCC

- Support of CB-Msg3-EDT

- Support of PWS

# 2	Discussion

Issue#1 : neighbour cell list for measurements and cell reselection

For measurements and cell reselection purposes, a proposal was made [8] to introduce a new list of neighbour cells operating in IoT NTN TDD. For measurement, SIB4-NB is used for intra-frequency neighbour cells information and SIB5-NB for inter-frequency neighbour cells information. IoT NTN TDD mode can re-use the existing neighbour cell lists as it is done for TN.

There is no problem of prioritization since the TDD mode is in this release, only supported in band n249. So, a UE supporting IoT NTN TDD mode can decide to prioritize or de-prioritize cell selection on n249 without more information provided in a new list of neighbour cell. The statement may be reconfigured in future release if TDD mode is supported for NB-IoT NTN.

The introduction of a new neighbour list for cells operating in TDD may provide timing offset information on when to measure and re-select the neighbour cells. It is an optimization for the UE since the UE may know the timing associated to a frequency, so reduce the measurement time and the delay to re-select cell. However, regarding the time left to the Rel-19, we think this optimization should be de-prioritize.

Proposal 1: No need to introduce a new list of neighbour cells operating in TDD mode in Rel-19

Issue#2 : DRX cycle aligned to 90 ms and paging

It was proposed in RAN2#129-bis and RAN2#130 to introduce a new drx-Cycle value aligned with the 90 ms TDD frame periodicity [9]. The DRX cycle is mainly used by the UE to get paging. UE paging is possible only in certain frames and subframes. The following agreements were made in RAN2#129bis and RAN2#130:

Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe. The remaining paging repetitions falling on the invalid DL SFNs are postponed to the next valid DL SFNs. It is up to network to configure the gap between two POs (i.e., parameter NB) to be sufficiently long such that it includes enough number of valid DL subframes for NumRepetitionPaging-r13 (no spec impact)

So, the paging occasion (PO) in a paging frame (PF) falling in a non-D subframes is postponed to the next D subframes and the PO subframe value is not changed (e.g. Ns=1, i_s=0, so the PO on SF#9, see 3GPP TS 36.304 clause 7.2). The alignment of the drx-Cycle on 90 ms can help to avoid postponement of paging occasions for some UEs, however, it will not increase the flexibility or the paging capacity of the network. Even with a drx-Cycle aligned on 90 ms may lead to a postponement because of the way PF selection works to select SFN. Therefore, there is no clear benefit to introduce 90 ms aligned drx-Cycle configurations, the postponement mechanisms are sufficient with the appropriate configurations by the network operator for the UE to monitor paging occasions.

Proposal 2: No need to introduce a drx-Cycle value that is aligned with the 90 ms periodicity.

Issue#3 : Subframes between preamble transmission and RA Response Window

In the email discussion [5], it was discussed which preamble format and number of NPRACH repetitions are supported for IoT NTN TDD in Table 5.1.4-1 in TS 36.321. The IoT NTN TDD mode is based on FDD mode for procedures [1]. In the same time, RAN1 concluded that format 2 is not supported [10]. RAN1 does not limit the number of repetitions in their study, so it is proposed to support less and more than 64 NPRACH repetitions, up to implementation according to the link budget, the collision avoidance, the system capacity, etc.

Proposal 3: Preamble format 0 and 1 and NPRACH repetitions < 64 and >= 64 are supported for IoT NTN TDD mode, based on FDD procedures. Consider the table 5.1.4-1 modifications for TS 36.321 below:

Table 5.1.4-1: Subframes between preamble transmission and RA Response Window in NB-IoT

| TDD/FDD/IoT NTN TDD mode | Preamble format | Number of NPRACH repetitions | X |
| --- | --- | --- | --- |
| FDD/IoT NTN TDD | 0 or 1 | >= 64 | 41 |
| FDD/IoT NTN TDD | 0 or 1 | < 64 | 4 |
| FDD | 2 | >= 16 | 41 |
| FDD | 2 | < 16 | 4 |
| TDD | Any | Any | 4 |

Issue#4 : Support of IoT NTN Rel-19 features

At RAN2#130, the agreements below stated that features specified in Rel-19 for IoT NTN (WI IoT_NTN_Ph3-Core) that could work without enhancements or minor specific enhancements can be supported for IoT NTN TDD :

Proposal 6: RAN2 to agree on one of the two options below for IoT-NTN TDD mode: Option 1: UE radio access capability parameter signalled and corresponding to a field in RRC (specified in clause 4 of 36.306). Option 2: Optional feature without UE radio access capability parameters (specified in clause 6 of 36.306). - ESA thinks we should not preclude the possibility in the next meeting to suggest minor changes as part of the TDD Mode WI to ensure that the features discussed in the R19 IoT NTN WI are also applicable to TDD mode In IoT_NTN_Ph3-Core, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode. RAN2 understands that, as part of the IoT_NTN_TDD WI, we can discuss on a case by case basis whether minor specific enhancements – not affecting other WGs - can be supported to ensure that (some of) the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode

Especially, the objective “capacity enhancements for uplink” [12] is interesting in regard to the limited capacity of the IoT NTN TDD (only 8.9% of the available uplink subframe compared to IoT NTN):

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

For the OCC support, based on RRC CR discussion [13] and RAN1 agreements, there is nothing that preclude the usage for IoT NTN TDD. NPUSCH transmissions overlapping with non-U subframes are postponed to the next U subframes and OCC is based on slot, thus, orthogonality of OCC code can be maintained.

Proposal 4: OCC defined for IoT NTN Rel-19 can be supported for IoT NTN TDD mode (No spec impact)

For the CB-Msg3, procedure is discussed in [11] and configurations in [13]. The procedures do not preclude the usage of IoT NTN TDD mode. For the configurations, the definitions are still under discussion [13]:

#### CB-Msg3-ConfigSIB-NB The IE CB-Msg3-ConfigSIB-NB is used to specify the CB-Msg3 EDT configuration. CB-Msg3-ConfigSIB-NB information element -- ASN1START CB-Msg3-ConfigSIB-NB-r19 ::= SEQUENCE { cb-Msg3-MinRSRP-Threshold-NB-r19 NRSRP-Range-NB-r14 OPTIONAL, --Need OR cb-Msg3-RSRP-CE-Level-NB-r19 CB-Msg3-RSRP-CE-Level-NB-r19 OPTIONAL, --Need OR cb-Msg3-ConfigList-NB-r19 CB-Msg3-ConfigList-NB-r19 } CB-Msg3-ConfigList-NB-r19 ::= SEQUENCE (SIZE (1.. maxCE-Level-NB-r19)) OF CB-Msg3-Config-NB-r19 CB-Msg3-Config-NB-r19 ::= SEQUENCE { cb-Msg3-TBS-NB-r19 ENUMERATED {b328, b408, b504, b584, b680, b808, b936, b1000}, cb-Msg3-NumOfReplicas-NB-r19 INTEGER (1..4), cb-Msg3-TimeResource-NB-r19 SEQUENCE { npusch-Periodicity-r19 ENUMERATED {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560}, npusch-StartSFN-r19 INTEGER (0..1023), npusch-StartSubframe-r19 INTEGER (0..9) }, cb-Msg3-PhysicalConfig-r19 ::= SEQUENCE { npusch-NumRUsIndex-r19 INTEGER (0..7), npusch-NumRepetitionsIndex-r19 INTEGER (0..7), npusch-SubCarrierSetIndex-r19 CHOICE { khz15 INTEGER (0..18), khz3dot75 INTEGER (0..47) }, npusch-MCS-r19 CHOICE { singleTone INTEGER (0..10), multiTone INTEGER (0..13) }, ack-NumRepetitions-NB-r19 ACK-NACK-NumRepetitions-NB-r13, p0-UE-NPUSCH-r19 INTEGER (-8..7), alpha-NB-r19 ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}, npdcch-CarrierIndex-r19 INTEGER (1..maxNonAnchorCarriers-NB-r14) OPTIONAL, -- Need OP npdcch-NumRepetitions-r19 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1}, npdcch-StartSF-CSS-r19 ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}, npdcch-Offset-CSS-r19 ENUMERATED {zero, oneEighth, oneFourth, threeEighth} }, cb-Msg3-TxWindow-NB-r19 SEQUENCE { windowStartSFN-NB-r19 INTEGER (0..1023), windowStartSubframe-NB-r19 INTEGER (0..9), windowSize-NB-r19 ENUMERATED {FFS}, windowPeriodicity-NB-r19 ENUMERATED {FFS} } OPTIONAL, --Need OP cb-Msg3-ResponseWindow-NB-r19 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}, cb-Msg3-MaxAttemptNum-NB-r19 ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1}, ... } CB-Msg3-RSRP-CE-Level-NB-r19 ::= SEQUENCE (SIZE(1..2)) OF RSRP-Range -- ASN1STOP

If the number of replicas is equal to one:

- npusch-Periodicity-r19 : npusch opportunity for CB-Msg3 transmissions. The configured periodicity are not aligned with the IoT NTN TDD periodicity of 90 ms

- npusch-StartSFN-r19 : The start SFN can be configured on a valid D frame

- npusch-StartSubframe-r19 : The start subframe can be configured on a valid D subframe

If the number of replicas two, three or four:

- windowStartSubframe-NB-r19 : By configuration, the network can configure the window start subframe on a D subframes

- windowSize-NB-r19 : The size is to be done, and corresponds to the transmission window size for the replicas, the number of replicas is configured by cb-Msg3-NumOfReplicas-r19 (up to 4).

- windowPeriodicity-NB-r19 : The periodicity of the transmission window

The periodicity and window length are not well aligned on IoT NTN TDD mode. 2 options can be adopted:

- Add configurations that are aligned with the IoT NTN TDD mode periodicity of 90 ms for npusch-Periodicity and windowPeriodicity-NB (approached adopted for RAR in RAN1 : “nprach-Parameters-r14 and NPRACH-Parameters-NB-r13 = {ms90, ms180, ms160, ms240, ms320, ms640, ms1280, s2560}”)

- Support postponement of CB-Msg3-EDT occasion to the next valid D frame

Proposal 5: Support the CB-Msg3 and CB-Msg4 for IoT NTN TDD mode, with one of the adaptations below:

Option a) Add configurations that are aligned with the 90 ms periodicity for npusch-Periodicity and windowPeriodicity-NB

Option b) Support postponement of CB-Msg3-EDT occasions to the next valid D frame within the transmission window when the number of replicas > 1

Finally, the PWS [12] for IoT NTN can be supported in IoT NTN TDD mode without further adaptations since the SIB10-NB, SIB11-NB and SIB12-12 for PWS follow the scheduling rules already discussed in RAN2 for other SIBs. DRX Paging and H-SFN were already discussed also for IoT NTN TDD.

Proposal 6: IoT NTN PWS Rel-19 can be supported in IoT NTN TDD (no spec impact)

# 3	Conclusion

In this contribution, the following proposals was provided:

Proposal 1: No need to introduce a new list of neighbour cells operating in TDD mode in Rel-19

- Proposal 2: No need to introduce a drx-Cycle value that is aligned with the 90 ms periodicity.

Proposal 3: Preamble format 0 and 1 and NPRACH repetitions < 64 and >= 64 are supported for IoT NTN TDD mode, based on FDD procedures. Consider the table 5.1.4-1 modifications for TS 36.321 below:

Table 5.1.4-1: Subframes between preamble transmission and RA Response Window in NB-IoT

| TDD/FDD/IoT NTN TDD mode | Preamble format | Number of NPRACH repetitions | X |
| --- | --- | --- | --- |
| FDD/IoT NTN TDD | 0 or 1 | >= 64 | 41 |
| FDD/IoT NTN TDD | 0 or 1 | < 64 | 4 |
| FDD | 2 | >= 16 | 41 |
| FDD | 2 | < 16 | 4 |
| TDD | Any | Any | 4 |

Proposal 4: OCC defined for IoT NTN Rel-19 can be supported for IoT NTN TDD mode (No spec impact)

Proposal 5: Support the CB-Msg3 and CB-Msg4 for IoT NTN TDD mode, with one of the adaptations below:

Option a) Add configurations that are aligned with the 90 ms periodicity for npusch-Periodicity and windowPeriodicity-NB

Option b) Support postponement of CB-Msg3-EDT occasions to the next valid D frame within the transmission window when the number of replicas > 1

Proposal 6: IoT NTN PWS Rel-19 can be supported in IoT NTN TDD (no spec impact)

# References

RP-243293, Revised WID on introduction of IoT-NTN TDD mode, RAN#106, 2024-12, Madrid, ES.

Report RAN2#130

Email discussion [Post130][310][IoT NTN TDD] Stage2 CR (Iridium)

Email discussion [Post130][311][IoT NTN TDD] RRC CR (Huawei)

Email discussion [POST130][312][IoT NTN TDD] MAC CR (Toyota)

Email discussion [Post130][313][IoT NTN TDD] 36.304 CR (Xiaomi)

Email discussion [Post130][314][IoT NTN TDD] capability CR (Samsung)

R2-2504320	Discussion on new NB-IoT NTN TDD mode	Qualcomm Incorporated, RAN2#130, May 2025.

R2-2504510	Further discussion on support of TDD mode for IoT-NTN	Nokia, Nokia Shanghai Bell, RAN2#120, May 2025.

Report RAN1#120, Feb. 2025, Athens, GR.

Email discussion [Post130][307][R19 IoT NTN] MAC CR (Mediatek)

RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#106, 2024-12, Madrid, ES.

Email discussion [Post130][306][R19 IoT NTN] RRC CR (Huawei)