3GPP TSG-RAN WG2 Meeting #131                                    	R2-2506178

Bengaluru, India, 25th – 29th August 2025

Agenda item:	8.19.1

Source:	Aalyria

Title:	Introduction of RRC Release with Redirection for NB-IoT NTN to NR-NTN

Document for:	Discussion and Decision

- Introduction

In this document, we provide the rationale for introduction of inter-RAT redirection for NB-IoT.

- Discussion

Background and Motivation

NB-IoT NTN is deployed today and supported by real products, including Smartphones, for emergency SOS, messaging.  Those devices already support wideband 3GPP technologies today (e.g. LTE, NR)

Devices today supporting NB-IoT NTN will likely keep supporting it, when NR NTN is added).

Several NTN operators will deploy both NR NTN and NB-IoT NTN in the same system both for single-RAT devices, as well as Multi-RAT devices

NB-IoT offers good deep penetration coverage at lower Power Spectral Density (PSD), owing to the small BW. Thus, can be a good fallback technology either:

  - When a device loses coverage of the NR NTN carrier

  - When an Operator chooses to switch off coverage of the NR NTN carrier to save satellite power or prioritize other service areas

Problem Statement:

Today, NB-IoT (not just NTN) does not support inter-RAT redirection (absence of inter-RAT RedirectedInfo IE in RRC Release and EDT Complete messages).

When a device falls back or selects from NR NTN to NB-IoT NTN, there is no way currently for an Operator to send the device back up to NR NTN

- Ex. Scenario 1 – Loss of NR NTN coverage

  - A device loses coverage to NR NTN carrier and finds NB-IoT NTN carrier.  When the device goes back into coverage of NR NTN, the Operator has no ability to request the device to redirect to the specific NR NTN carrier, the UE might start searching, take a long time to find and run out of battery.

- Ex. Scenario 2 – NR NTN switch-off

  - Operator opts to switch-off an active NR NTN cell which has no users, to address an urgent coverage need elsewhere or conserve power/energy on the satellite, but leaves NB-IoT cell layer active.  New users become active and find the NB-IoT layer.  Operator decides to re-activate the NR carrier, but has no way of directing users towards it.

  - Operator opts to switch-off an active NR NTN cell which has some inactive or low-activity users, to address an urgent coverage need elsewhere or conserve power/energy on the satellite, but leaves NB-IoT cell layer active.  Operator wants to direct users to NB-IoT carrier first, but has no way of doing it and, even once the carrier is back up, the operator has no way of directing users from NB-IoT NTN back to NR NTN.

Proposals:

Introduce support for Inter-RAT RedirectedInfo IE part of RRC Release and EDT Data Complete in NB-IoT (at least from NB-IoT NTN to NR NTN)

Introduce support for inter-RAT redirection from NR-NTN to NB-IoT NTN

- Conclusion

Following proposals are made:

- Introduce support for Inter-RAT RedirectedInfo IE part of RRC Release and EDT Data Complete in NB-IoT (at least from NB-IoT NTN to NR NTN)

Introduce support for inter-RAT redirection from NR-NTN to NB-IoT NTN