3GPP TSG-RAN WG2 #131	R2-2506185

# Bangalore, India, 25-29 August 2025

Agenda Item:	8.9.1

Source:	Ericsson

Title:	IoT NTN phase 3 open issues in Stage 2

Document for: Discussion

# 1	Introduction

Here we collect open issues related to the stage 2 for IoT NTN phase 3 in Release-19.

This document is the outcome of [Post130][305][R19 IoT NTN] Stage2 CR (Ericsson).

# 2	[Post130][305][R19 IoT NTN] Stage2 CR (Ericsson)

[Post130][305][R19 IoT NTN] Stage2 CR (Ericsson)

Scope: discuss the running Stage 2 CR

Intended outcome: Endorsed CR

Deadline: long

# 3	Issues identified

## 3.1	Issue 1: HARQ modelling

For CB-Msg3 for User Plane CIoT EPS optimisation, how does HARQ ACK work if NW configures HARQ enabled/disabled before suspending the UE?

Obviously, UE must ACK the CB-Msg4 if CB-Msg4 includes HARQ ACK resources regardless of if HARQ is disabled or not (as NW would not include HARQ resources if it did not want an ACK).

Shall the UE retain a HARQ enable/disable config (received in RRC_CONNECTED) after suspend/resume and follow that after it has received a CB-Msg4 with a CRNTI?

If the CB-Msg4 is successfully decoded and it contains a HARQ ACK resource allocation for a UE, the UE shall always send HARQ ACK.

However, when the CB-Msg4 contains a matching UE contention resolution identity and a C-RNTI and if it later is scheduled again, the DCI will contain resources for sending HARQ ACK but UE may have a configuration of disabled HARQ feedback.

If the CB-Msg4 is successfully decoded and it contains a matching UE contention resolution identity and a C-RNTI, if UE is later scheduled it follows the configured HARQ feedback enabled/disabled.

## 3.2	Issue 2: stage 2 modelling of CB-Msg3-EDT

QC “It seems cleaner to introduce 7.3x for CB-EDT and do same as MT-EDT to capture the difference.”

Rapporteurs proposes:

RAN2 to confirm the current modelling in stage 2 where CB-öMsg3-EDT is a variant of existing EDT procedures.

## 3.3	Issue 3: PWS modelling

vivo “Maybe we have to clarify that NB-IoT NTN also supports PWS in a general way.”

QC “May be we should have a section for NTN support for warning system. We do not change to E-UTRAN for NB-IoT.”

Rapporteur proposal:

RAN2 to discuss whether there is a need for further description of PWS in stage 2.

# 4	Summary

We recommend discussing above open issues at the RAN2#131 meeting.