3GPP TSG RAN WG2#131	R2-2505015

Bengaluru, India, 25th - 29thAugust 2025

3GPP TSG-RAN WG1 Meeting #121	R1-2504858

St Julian's, Malta, 19 - 23 May, 2025

Title:	Reply LS on simultaneous configuration of SBFD and DC

Response to:	R2-2503191

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN1

To:	RAN2

Cc:	RAN3, RAN4

Contact person:			Yanping Xing

xingyanping@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN1 would like to thank RAN2 for the LS on simultaneous configuration of SBFD and DC in R2-2503191.

For the question on whether SBFD and DC can be configured simultaneously, and whether there is any issue with such configuration for an SBFD-aware UE, RAN1 made the following agreement.

Agreement

- From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier.

- Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly.

### 7.6.2 NR-DC The UE procedures described in this clause are not applicable if the UE is provided scg-State [12, TS 38.331]. If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in clauses 7.1 through 7.5. If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by nrdc-PCmode-FR1 for FR1 and/or by nrdc-PCmode-FR2 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, or semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 or FR2 as defined in [8-3, TS 38.101-3]. If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2, the UE determines a transmission power for the MCG or for the SCG as described in clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively. If a UE is provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2 - if the UE is not provided tdd-UL-DL-ConfigurationCommon for the MCG or SCG, the UE determines a transmission power for the MCG or for the SCG as described in clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively - if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, or is indicated as SBFD symbol to a UE by tdd-UL-DL-ConfigurationCommon, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power - otherwise, the UE determines a power for the transmission on SCG or the MCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in clauses 7.1 through 7.5 without considering PSCG or PMCG respectively <unchanged texts omitted>

- It is up to RAN2 to decide whether or not to support simultaneous configuration of SBFD and DC.

- CC: RAN3, RAN4.

# 2	Actions

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above conclusion into account.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN1 Meeting #122	25 - 29 Aug 2025	Bangalore, India

TSG RAN1 Meeting #122bis	13 - 17 Oct 2025	Prague, CZ