3GPP TSG RAN WG2#131	R2-2505034

Bengaluru, India, 25th - 29thAugust 2025

3GPP TSG-RAN WG3 Meeting #128	R3-253816

St Julian’s, Malta, 19 – 23 May, 2025

Title:	Reply LS on per-UE UE performance metrics

Response to:	R3-253023- S5-252056

Release:	Release 18

Work Item:	NR_AIML_NGRAN-Core

Source:	RAN3

To:	SA5

Cc:	RAN2

Contact Person:

#### Name:	Anna Pantelidou

E-mail Address:	anna.pantelidou@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA5 for their reply LS on per-UE UE performance metrics. RAN3 would further like to ask for clarifications about the granularity of Average Packet Delay UL/DL and Average Packet Loss UL/DL. RAN3 assumes that UE level performance is derived from DRB level measurements of the concerned UE.

Average packet delay measurements:

RAN3 has noticed that Average Packet Delay UL/DL measurements comprise packet delay measurement components listed in TS 38.314 clause 4.2.1.2.1 which are measured with a per-DRB per-UE granularity and referenced to corresponding measurements’ definitions in TS 28.558 clause 6.3.1.1. RAN3 assumes that the UL and DL Average Packet Delay per-UE measurements are derived from per-DRB measurements across all configured DRBs for the concerned UE. RAN3 would like to ask SA5 whether the measurements are aggregated per-DRB per-UE to reflect corresponding per-UE metrics. If such per UE aggregation is not supported and if such UE aggregation is feasible, RAN3 would like to ask SA5 to specify these measurements.

Average packet loss measurements:

In RAN3#127-bis, RAN3 sent an LS to SA5 (R3-252436) to ask SA5 if it is feasible to measure Packet Loss Rate in UL and DL at PDCP level for both split and non-split architecture and, if feasible, RAN3 asked SA5 to specify these measurements for both split and non-split architecture. In the LS from SA5 R3-253824, SA5 has confirmed that the following metrics can be used for UE Packet Loss:

- UL PDCP SDU Loss Rate – TS 28.552 clause 5.1.3.1.1

- DL PDCP SDU Drop Rate in gNB-CU-UP – TS 28.552 clause 5.1.3.2.1

RAN3 would like to further ask whether it is feasible to specify the above Packet Loss Rate metrics with a per-DRB per-UE granularity in the UL and DL at PDCP layer, which may also be aggregated to reflect a per-UE Packet Loss Rate metric.

If the above is feasible then RAN3 would like to ask SA5 to carry out specification work accordingly.

2. Actions:

To SA5 : 	RAN3 would like to ask SA5 to take the above into account and provide feedback to RAN3.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #129	25 - 29 August 2025	Bengaluru, India

TSG-RAN WG3 Meeting #129-bis	13 - 17 October 2025	Prague, CZ