3GPP TSG RAN WG2#131	R2-2505050

Bengaluru, India, 25th - 29thAugust 2025

# 3GPP TSG-RAN WG4 Meeting #115	R4-2508433

Malta, MT, 19th – 23rd May, 2025

Title:	Reply LS on SMTC enhancements

Response to:	R4-2505313/R2-2503068

Release:	Rel-19

Work Item:	Non-Terrestrial Networks (NTN) for NR Phase 3 (NR_NTN_Ph3-Core)

Source:	RAN4

To:	RAN2

Cc:	-

Contact person:	Ziquan HU

huziquan@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN4 thanks RAN2 for the information in LS R2-2503068. RAN4 discussed the RRM requirements impact of the SMTC enhancement, and would like to provide following conclusions achieved in RAN4#115:

- For the SMTC enhancement on configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements:

From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but that requires some clarifications/modifications to the existing RRM requirements.

- RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer.

Note: RAN4 has not estimated the full impact during RAN4#115 meeting. There is a concern on the workload on to support those agreements in Rel-19 timeframe from RAN4 requirements perspective.

- For the SMTC enhancement on configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs:

From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. However, the current requirements in TS 38.133 for the scenarios when the number of configured SMTCs exceed the parallelSMTC-r17 for idle/inactive UEs are captured below for reference:

For Measurements of intra-frequency NR cells with NTN carrier: The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected. For Measurements of inter-frequency NR cells with NTN carrier: The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the value indicated by parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in table 4.2C.2.4-1 and table 4.2C.2.4-2 is expected.

# 2	Actions

To RAN2

ACTION: RAN4 kindly requests RAN2 to take the above information into consideration for future work.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #116	25 – 29 August 2025	Bengaluru, IN

TSG-RAN WG4 Meeting #116-bis	13 – 17 October 2025	Prague , CZ