3GPP TSG-RAN WG2 Meeting #131 	 R2-2505082

Bangalore, India, August 25th – 29th, 2025

Agenda item:	8.9.3

Source:	vivo

Title:	Remaining Issues on CB-msg3-EDT Mechanism

Document for:	Discussion and Decision

1 Introduction

In this contribution, we will further discuss the remaining issues on CB-msg3-EDT mechanism used for IoT-NTN.

2 Discussion

  - 2.1 Questions from RAN1 LS R1-2504959

In the RAN1 Reply LS [1], there are several questions listed below that require further confirmation from RAN2,

- RAN1 wonders whether RAN2 intends to support multi-PRB allocation or sub-PRB allocation or both

- To be confirmed by RAN2 whether to support both singleTone and multitone, or singleTone only for high leyer parameter npusch-MCS-r16.

For question 1), considering that both the multi-PRB allocation and sub-PRB allocation mechanisms already exist and can be applied to CB-Msg3 without impacting RAN1/2 functions, it is logical to support both mechanisms to enhance the flexibility of network configuration. For instance, if the expected TBS is relatively large and the system load is low, the network can adopt multi-PRB allocation to achieve a higher UE peak data rate. Conversely, when there are multiple UEs pending data transmission, the network may signal sub-PRB allocation, which allows more resources to be randomly selected, thereby helping to avoid collisions.

For question 2), we understand that RAN2 intends to reuse the PUR mechanism in NB-IoT. Specifically, CB-Msg3 PUSCH can operate in either single-tone or multi-tone mode, and it is expected that the parameter npusch-MCS-r16 will be reused for CB-Msg3.

Proposal 1: RAN2 confirms that both multi-PRB allocation and sub-PRB allocation are supported for CB-Msg3.

Proposal 2: RAN2 confirms that both single-tone and multi-tone are supported for CB-Msg3, and intends to reuse the parameter npusch-MCS-r16 for CB-Msg3.

In addition, RAN1 also indicates that there is no consensus in RAN1 regarding the need to define a set of narrowbands for MPDCCH configuration. That implies that the network will respond to all the UEs with CB-Msg4 on a single MPDCCH narrowband. This is consistent with legacy EDT and PUR mechanisms. To avoid impacts to RAN1, RAN2 should confirm that there is only one narrowband configured for UE MPDCCH monitoring.

Proposal 3: RAN2 confirms only one narrowband is configured for MPDCCH monitoring, as the running CR implementation.

Besides, if Proposal 1 and 2 are agreeable, then we think RAN2 should send an LS to RAN1 to inform the agreement.

Proposal 4: RAN2 sends an LS to RAN1 to inform them of Proposal 1 and Proposal 2.

  - 2.2 MAC-2 CB-RNTI

In the discussion of [Post130][307][R19 IoT NTN], a remaining issue remains regarding whether the network can simply configure RNTI = X. In our understanding, there is no compelling reason to adopt this approach, considering that the formulation method is needed anyway.

Specifically, a single RNTI is only effective under extremely low-load conditions. In such scenarios, when a single RNTI is employed, Msg4 generally does not multiplex excessive UE responses (thus, false alarms are not a significant issue). However, as the load increases, the network may configure substantial CB-Msg3 resources. Utilizing a single RNTI in this context could lead to an excessively large Msg4 size due to a number of responses multiplexed, and UEs that fail in contention resolution would suffer from a false alarm by any UE having a corresponding response within the same selected CB-Msg3 Tx window.

Based on the foregoing analysis, we propose that a unified solution is sufficient, as it can accommodate all load scenarios effectively.

Proposal 5 (MAC-2): RAN2 confirms that the network configuration of RNTI = X is not supported.

Further, it is suggested that RAN2 should further discuss the following points regarding CB-RNTI,

- Whether to define Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity).

- Whether to define Y = 16.

- Whether starting value X is optionally configurable.

The first bullet concerns the definition of the starting point for the CB-Msg3 transmission window. In the current running CR, a logical CB-Msg3 Tx window index is utilized (i.e., Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19). From the UE's perspective, using the SFN-id would be more intuitive and straightforward, as the UE with DL synchronization can clearly know the SFN_id of the starting point of the CB-Msg3 transmission window. With this approach, the UE would not need to calculate the virtual CB-Msg3 Tx window index, which helps simplify UE implementation.

The second bullet concerns the value of Y, defined as Y=ceil (Msg4_WS/Msg3_WP). A concern has been raised that Y might become excessively large, potentially causing the calculated CB-RNTI value to exceed its allowable range. In the current running CR, the maximum value of Msg4_WS is sf10240, while the minimum value of Msg3_WP is still FSS (note that the minimum value of CB-Msg3 PUSCH periodicity is sf2). Take an example, if we set the minimum value of CB-Msg3 transmission window to n5, Y would equal 1024. The potential calculated CB-RNTI would not exceed 65535. No issue is anticipated in this scenario. Thus, we think there is no need to limit Y to 16. And RAN2 can carefully design the value range of Msg3_WP.

For the final bullet point, given that network configuration of RNTI = X is not supported, we see no strong motivation to make X a configurable parameter in the CB-RNTI formula. The network can allocate the remaining unused RNTIs to other UEs instead.

Proposal 6 (MAC-2): For CB-RNTI, define Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity).

Proposal 7 (MAC-2): For CB-RNTI, RAN2 confirms that there is no need to define Y=16. The configurable value of Msg3_WP is designed taking the valid range of CB-RNTI into account.

Proposal 8 (MAC-2): For CB-RNTI, RAN2 confirms that X is a hard-coded value.

  - 2.3 MAC-9 Processing time

In the discussion of [Post130][307][R19 IoT NTN], RAN2 is asked to choose between 3ms or 0ms processing time when determining the Msg4 monitoring starts.

In our understanding, 0ms is okay based on the assumption that both CB-Msg3 transmission window length and periodicity are indicated in multiples of the CB-msg3-PUSCH occasion periodicity, similar to how the configuredGrantTimer is specified in NR. From Figure 1, it can be observed that this approach enables the UE to maintain a certain gap between the ending point of transmitted CB-Msg3 and the starting point of the CB-Msg4 window. The legacy 3ms processing time is not needed for CB-Msg3, as DSA is always mandatory.

Figure 1: CB-Msg3 transmission window and CB-Msg4 window

Proposal 9 (RRC-1.4): CB-Msg3 transmission window length and periodicity are configured in multiples of the CB-Msg3 PUSCH periodicity (e.g., value ranging from 1..32).

Proposal 10 (MAC-9): 0ms processing time is used for the start of CB-Msg4 monitoring window.

  - 2.4 RRC Aspects

In our understanding, there are 2 remaining issues regarding RRC aspects,

- The RRC behavior when receiving the CB-Msg3 EDT failure indication from MAC.

- The RRC behavior when receiving a CB-Msg4 without RRC message (but with contention resolution identity) in the CB-Msg3 in CP solution.

For these 2 points, we think the legacy principles listed below can be reused.

TS 36.331 5.3.3.3d -----------------------------the unrelated part is omitted----------------------------------- For CP transmission using PUR, upon indication from lower layers that transmission using PUR is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received. Upon reception of PUR fallback or PUR failure indication from lower layers, the procedure ends. NOTE: For transmission using PUR, further UE actions upon reception of PUR fallback or PUR failure indication from lower layers (see TS 36.321 [6]) is left up to implementation.

That is, further UE action upon reception of CB-Msg3 failure indication from MAC layers is left up to implementation, e.g., it is up to UE implementation to initiate legacy connection establishment, EDT, or PUR.

For CP CB-Msg3, upon indication from MAC layer that transmission using CB-Msg3 is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received. And the  procedure ends.

Proposal 11 (MAC-15): RAN2 agrees that further UE action upon reception of CB-Msg3 failure indication is left up to implementation, e.g., it is up to UE implementation to initiate legacy connection establishment, EDT, or PUR.

Proposal 12 (RRC-1.2): For CP CB-Msg3, upon indication from MAC layer that transmission using CB-Msg3 is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received. And the procedure ends.

3 Conclusion

In this contribution, we have provided our understanding of the remaining issues on the CB-Msg3 EDT mechanism. And we made the following proposals:

Questions from RAN1 LS R1-2504959:

Proposal 1: RAN2 confirms that both multi-PRB allocation and sub-PRB allocation are supported for CB-Msg3.

Proposal 2: RAN2 confirms that both single-tone and multi-tone are supported for CB-Msg3, and intends to reuse the parameter npusch-MCS-r16 for CB-Msg3.

Proposal 3: RAN2 confirms only one narrowband is configured for MPDCCH monitoring, as the running CR implementation.

Proposal 4: RAN2 sends an LS to RAN1 to inform them of Proposal 1 and Proposal 2.

MAC-2 CB-RNTI:

Proposal 5 (MAC-2): RAN2 confirms that the network configuration of RNTI = X is not supported.

Proposal 6 (MAC-2): For CB-RNTI, define Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity).

Proposal 7 (MAC-2): For CB-RNTI, RAN2 confirms that there is no need to define Y=16. The configurable value of Msg3_WP is designed taking the valid range of CB-RNTI into account.

Proposal 8 (MAC-2): For CB-RNTI, RAN2 confirms that X is a hard-coded value.

MAC-9 Processing time:

Proposal 9 (RRC-1.4): CB-Msg3 transmission window length and periodicity are configured in multiples of the CB-Msg3 PUSCH periodicity (e.g., value ranging from 1..32).

Proposal 10 (MAC-9): 0ms processing time is used for the start of CB-Msg4 monitoring window.

RRC Aspects:

Proposal 11 (MAC-15): RAN2 agrees that further UE action upon reception of CB-Msg3 failure indication is left up to implementation, e.g., it is up to UE implementation to initiate legacy connection establishment, EDT, or PUR.

Proposal 12 (RRC-1.2): For CP CB-Msg3, upon indication from MAC layer that transmission using CB-Msg3 is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received. And the procedure ends.

4 Reference

[1] R1-2504959,. Reply LS on CB Msg3 EDT for IoT NTN Ph3, MediaTek Inc.