3GPP TSG-RAN WG2 Meeting #131		R2-2505085

Bangalore, India, 25th – 29th August 2025

Source:	vivo

Title:	(RRC-5/11/12) Notification message handling for intermediate relay UE

Agenda Item:		8.13.2

Document for:	Discussion and Decision

- Introduction

In RAN2 #130 meeting, the discussion in agenda ‘Relay discovery and (re)selection’ mainly fell into the notification message and corresponding procedure at intermediate relay UE. A bunch of agreements were reached; however, it is still not crystally clear for all the UE behaviours and how the whole procedure works in different scenarios, e.g., when intermediate relay UE in different RRC states.

This contribution will try to summarize the agreements so far and try to propose way forward for how to define UE behaviours and related cause value in notification message, helping to complete the WI.

- Discussion

  - RRC-5/12: NotificationMessageSidelink message handling and cause value

In open issue list brought by rapporteur[1], RRC-5 was raised to discuss the NotificationMessageSidelink message handling.

The related RAN2 agreements so far are summarized as follows[2][3][4]:

RAN2#129 meeting agreements - Upon intermediate relay UE performing relay (re)selection, it can notify its child node. If the cause of (re)selection already causes a notification, there is no double-trigger of the notification. - FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger) - FFS if the notification is required in all cases - FFS the content in the notification - FFS the child node behaviour after receiving this notification - FFS other condition/timing that the intermediate relay UE notifies its child node. RAN2#129bis meeting agreements - When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE). FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment). This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message. - When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE. - FFS detailed information in the indication and child UE handling. - FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count. - FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery). RAN2#130 meeting agreements - The notification message is used for the L2 child UE in case of upstream relay (re)selection triggered by AS layer failure cases (Uu/PC5 link failure/release, using legacy cause value) where the upper layer does not trigger link release towards the child. For reselection triggered by upper layers we rely on the upper-layer notification/release mechanism already defined. - Remote UE handling of the notification is up to upper layer implementation. - Leaving remote UE handling of the notification to implementation applies to the upper-layer notification for remote UEs in RRC_IDLE/RRC_INACTIVE. UEs in RRC_CONNECTED follow the later agreement to trigger RRC reestablishment. No case is expected where a UE in RRC_CONNECTED receives the upper-layer notification. - Notification is always sent for AS failure cases at the intermediate relay UE (e.g., upstream RLF that prevents the intermediate relay from having a path to the network), using the legacy mechanism. FFS new cause values. - In principle, RAN2 consider that no AS-layer notification is sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE when the AS-visible parameters that could affect relay reselection are not changed. FFS which parameters are related. This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified. - FFS if notification behaviour can be optimized for intermediate relay UEs in RRC_CONNECTED. - Upon receiving the notification message or upon PC5 link release, remote UE in RRC_CONNECTED needs to initiate RRC Reestablishment as legacy. - When the intermediate Relay UE in RRC_CONNECTED receives the notification message from its parent intermediate Relay UE/last Relay UE or when the PC5 link is released, it initiates RRC Reestablishment.

According to the agreements so far, the UE behaviours are illustrated in the following mind map:

Figure 1. Mind map for summary of agreements of NotificationMessageSidelink message procedure

As discussed in [1], when an intermediate relay UE receives a NotificationMessageSidelink message from the parent relay, it has two options:

- Option 1: Forward a copy of the original notification (with the same cause) to its child UEs.

- Option 2: Generate a new notification message and send it to the child UEs, with an updated cause that reflects its own response, such as relay reselection or re-establishment.

Till now, the cause values are defined as follows[5]:

NotificationMessageSidelink-r17-IEs ::=   SEQUENCE {

indicationType-r17                        ENUMERATED {

relayUE-Uu-RLF, relayUE-HO, relayUE-CellReselection,

relayUE-Uu-RRC-Failure

}                                     OPTIONAL,  -- Need N

lateNonCriticalExtension                  OCTET STRING                          OPTIONAL,

nonCriticalExtension                      NotificationMessageSidelink-v1800-IEs OPTIONAL

}

NotificationMessageSidelink-v1800-IEs ::= SEQUENCE {

sl-IndicationType-r18                     ENUMERATED {relayUE-PC5-RLF, spare1}  OPTIONAL,  -- Need N

sl-DestinationIdentityRemoteUE-r18        SL-DestinationIdentity-r16            OPTIONAL,  -- Need N

nonCriticalExtension                      SEQUENCE {}                           OPTIONAL

}

NotificationMessageSidelink-v19xy-IEs ::= SEQUENCE {

indicationType-r19                     ENUMERATED {relayUE-RelayReselection, spare1}  OPTIONAL,  -- Need N

nonCriticalExtension                      SEQUENCE {}                           OPTIONAL

}

In our understanding, option-1 would be straightforward and we can reuse the legacy procedure as much as possible. Option-1 does not mean that there will be double-trigger of the notification message, because it is all about cause value, the exact timing to send notification message can be the same for both option-1 and option-2.

Observation 1: Neither option-1 (copy received cause value) nor option-2 (regenerate cause value) will lead to double trigger of the notification message, because they are just related to how to set cause value and the exact timing to send notification message can be the same (e.g., after reselection happens).

Then, the next question is that, what would be the real benefit for option-2. In our understanding, the option-2 would introduce new cause values, e.g., relayUE-RelayReselection as proposed in running CR, or even new relayUE-RRCreestablishment, to reflect the CONNECTED intermediate relay UE behaviour.

Observation 2: Option-2 (regenerate cause value) will introduce one or two new cause value (e.g. relayUE-RelayReselection and relayUE-RRCreestablishment) to reflect the CONNECTED/ IDLE/ INACTIVE intermediate relay UE behaviour.

Therefore, in our understanding, it is ok to just go for option-1. However, the following two new cases should be discussed first[1]:

Additionally, there were following cases mentioned the open issue list discussion which will need further discussion during the next meeting - Regarding the relay UE in idle/inactive state, the relay UE will perform relay reselection and/or cell selection if the relay UE receives notification message/PC5-S release message from its parent relay UE, or detects PC5 RLF since the intermediate relay UE may be in the cell coverage. The ‘cell selection’ was not discussed before. Therefore, we need to further discuss ‘cell selection’ in the intermediate relay UE side. And the corresponding cause value proposed in this document also should be updated. - We may miss one case that the intermediate relay UE in idle/inactive/connected sate initiates notification message procedure due to the detection of PC5 RLF. The intermediate relay UE in idle/inactive state will perform relay reselection and/or cell selection upon detection of PC5 RLF. The intermediate relay UE in connected state will perform re-establishment upon detection of PC5 RLF. Therefore, the proposal for cause value of ‘relay reselection’ and ‘re-establishment’ cannot work in this case. The suitable cause value could be ‘PC5 RLF’ in this case. Companies are encouraged to discuss the above two cases further along with text proposal for the enhancements at the next meeting

In our opinion, for cell (re)selection, it may happen on the IDLE/INACTIVE intermediate relay UE side after it receives notification message, because the cell (re)selection and relay reselection run independently. But if we go for option-1, then the cause value setting will not be affected no matter the intermediate relay UE do relay reselection or cell (re)selection, because the intermediate Relay UE will just send the copied cause value which actually reflects where the link failure really happens, e.g., relayUE-Uu-RLF, relayUE-HO, relayUE-CellReselection, relayUE-Uu-RRC-Failure, which all mean that the link between last relay and the gNB fails.

Observation 3: It is possible that IDLE/INACTIVE intermediate relay UE performs cell (re)selection after it receives notification message.

Observation 4: If we go option-1, the cause value setting will not be affected no matter whether the intermediate relay UE do relay reselection or cell (re)selection or RRC re-establishment, because the intermediate Relay UE will just send the copied cause value.

For PC5 RLF case, the relayUE-PC5-RLF was defined for L2 U2U in Rel-18. Now we have two cases for multi-hop U2N relay:

Case 1: Intermediate relay UE detects PC5 RLF;

In this case, we understand it should set the cause value to ‘relayUE-PC5-RLF’ to its child UE.

Case 2: First relay UE receive the notification message, including cause value ‘relayUE-PC5-RLF’.

In this case, if we go option-1, the first relay UE can just copy the cause value and send it to remote UE.

Based on the above analysis, we propose:

Proposal 1: (RRC-5/12) When an intermediate relay UE receives a NotificationMessageSidelink message from the parent relay, it forward a copy of the original notification (with the same cause) to its child UEs.

Proposal 2: (RRC-5/12) When intermediate relay UE detects PC5 RLF, it should set the cause value to ‘relayUE-PC5-RLF’ to its child UE.

With the above proposal 1 and proposal 2, the specification impact would be minimized. The following TP can be considered.

Proposal 3: (RRC-5/12) If Proposal 1 and Proposal 2 are agreed, adopt the following TP.

##### 5.8.9.10.2 Initiation The Relay UE may initiate the procedure when one of the following conditions is met: 1> if the UE is acting as U2N Relay UE: 2> upon Uu RLF as specified in 5.3.10; 2> upon reception of an RRCReconfiguration including the reconfigurationWithSync; 2> upon cell reselection; 2> upon L2 U2N Relay UE's RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5; 2> upon detection of PC5 RLF for the hop between itself and its parent relay UE as specified in 5.8.9.3; 2> upon initiating RRC connection re-establishment procedure or PC5-RRC connection release due to reception of NotificationMessageSidelink message as defined in 5.8.9.10.4; 1> if the UE is acting as L2 U2U Relay UE: 2> upon detection of PC5 RLF for the hop between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.3; 2> upon PC5-RRC connection release for the per-hop link between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.5; Note 1: The Notification Message may not be sent by an intermediate relay UE in RRC_IDLE or RRC_INACTIVE to its child UEs if there are no changes to AS-visible parameters (e.g., the hop count provided by upper layers or the serving cell of the last U2N relay UE) that could impact relay reselection. Editor’s Note: FFS which parameters are related. This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified. ##### 5.8.9.10.3 Actions related to transmission of NotificationMessageSidelink message The Relay UE shall set the indication type as follows: 1> if the UE is acting as U2N Relay UE: 2> if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RLF: 3> set the indicationType as relayUE-Uu-RLF; 2> else if the UE initiates transmission of the NotificationMessageSidelink message due to reconfiguration with sync: 3> set the indicationType as relayUE-HO; 2> else if the UE initiates transmission of the NotificationMessageSidelink message due to cell reselection: 3> set the indicationType as relayUE-CellReselection; 2> if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RRC connection establishment/Resume failure: 3> set the indicationType as relayUE-Uu-RRC-Failure; 2> else if the UE initiates transmission of the NotificationMessageSidelink message due to detection of PC5 RLF for the hop between itself and its parent relay UE: 3> set the indicationType as relayUE-PC5-RLF; 2> else if the UE initiates transmission of the NotificationMessageSidelink message due to PC5-RRC connection release because of reception of NotificationMessageSidelink message: 3> set the indicationType to the same value as the indicationType received in NotificationMessageSidelink; 2> submit the NotificationMessageSidelink message to lower layers for transmission.

Besides what have been discussed, there is another case that when intermediate relay UE receives Notification message, but does not release PC5-RRC connection[5]:

4> else (i.e., maintain the PC5 RRC connection): 5> if the UE is L2 U2N Remote UE or L2 U2N Child Relay UE and the indicationType is relayUE-HO or relayUE-CellReselection: 6> consider cell re-selection occurs;

This case is not covered by above TP. We think it can be further discussed after we agreed above proposals and TPs.

Then if we agree above proposals, the intended intermediate relay UE behaviour is as follows:

1/ When IDLE/INACTIVE intermediate relay UE receives notification message with cause value:

- Cause value: relayUE-Uu-RLF, relayUE-HO, relayUE-CellReselection, relayUE-Uu-RRC-Failure, relayUE-PC5-RLF (when it is first relay UE)

- Intermediate relay UE behaviour:

  - Determine whether to release or keep the PC5 unicast link based on implementation (follow legacy, as in running CR)

    - If release, indicate upper layer and trigger relay reselection (legacy, as in running CR), or cell (re)selection (follow legacy)

      - if relay reselection, and AS-visible parameter changed

        - Copy same cause value to forward to its child UE

      - if relay reselection, and AS-visible parameter not changed

        - No notification message forwarding

      - If cell (re)selection

        - Copy same cause value to forward to its child UE

    - If maintain, and indicationType is relayUE-HO or relayUE-CellReselection

      - Consider cell re-selection occurs (legacy, as in running CR)

        - Copy same cause value to forward to its child UE (new, no agreements)

2/ When CONNECTED intermediate relay UE receives notification message with cause value:

- Cause value: relayUE-Uu-RLF, relayUE-HO, relayUE-CellReselection, relayUE-Uu-RRC-Failure, relayUE-PC5-RLF (when it is first relay UE)

- Intermediate relay UE behaviour:

  - initiate the RRC connection re-establishment procedure

    - Always send the notification message, copy same cause value to forward to its child UE

3/ When intermediate relay UE detect PC5 RLF:

- Release PC5 unicast link and trigger relay reselection (legacy, as in running CR), or cell (re)selection (follow legacy)

  - if relay reselection, and AS-visible parameter changed

    - Copy same cause value to forward to its child UE

  - if relay reselection, and AS-visible parameter not changed

    - No notification message forwarding

  - If cell (re)selection

    - Copy same cause value to forward to its child UE

  - RRC-11: AS-Layer Notification Behavior of Intermediate Relay UEs in RRC_IDLE/RRC_INACTIVE

Another RRC issue that is related to notification message is[1]:

Open Issue RRC-11: AS-Layer Notification Behavior of Intermediate Relay UEs in RRC_IDLE/RRC_INACTIVE During RAN 2 #130 RAN2 made an agreement that in principle, RAN2 considers that no AS-layer notification is required to be sent by an intermediate relay UE operating in RRC_IDLE or RRC_INACTIVE, when there is no change in AS-visible parameters (such as the hop count provided by upper layers or the serving cell of the last U2N relay UE) that could influence relay reselection behavior by the Remote UE. Additionally, this issue can be revisited if it is found that there are parameters not visible to the AS layer that could nevertheless impact relay reselection and require the Remote UE to be notified. Proposal 5 – RAN 2 should discuss if there are any parameter not visible to AS layer that could change and would affect reselection and force the remote UE to be notified.

In our understanding, the former agreement as follows was reached because companies what to have some recovery mechanism at intermediate relay UE so no notification message is sent under some cases:

- In principle, RAN2 consider that no AS-layer notification is sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE when the AS-visible parameters that could affect relay reselection are not changed.  FFS which parameters are related.  This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified.

In our opinion, there is no such parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified. It means that when the hop count provided by upper layers or the serving cell of the last U2N relay UE does not change, the notification message may not be sent, as captured in current running CR NOTE1[5]:

Note 1: The Notification Message may not be sent by an intermediate relay UE in RRC_IDLE or RRC_INACTIVE to its child UEs if there are no changes to AS-visible parameters (e.g., the hop count provided by upper layers or the serving cell of the last U2N relay UE) that could impact relay reselection.

Therefore, we propose:

Proposal 4: (RRC-11) There are no parameter not visible to AS layer that could change and would affect reselection and force the remote UE to be notified by intermediate relay UE in AS-layer notification message. I.e., keep the current NOTE-1 in 5.8.9.10.2 in RRC running CR.

Another issue, although not in any open issue list, is an FFS left in last meeting:

- FFS if notification behaviour can be optimized for intermediate relay UEs in RRC_CONNECTED.

In our understanding, the intermediate relay UE behaviour for sending notification messages should be the same for different RRC states, because if we go for option-1, anyway the cause value is copied from previous notification message. It is also better to have unified design for different RRC states. Moreover, there is no more spec impact to optimize the UE behaviour of intermediate relay UEs in RRC_CONNECTED, we can just delete ‘IDLE/INACTIVE’ in above NOTE-1, that is all.

Observation 5: If we go for option-1, the intermediate relay UE behaviour for sending notification messages should be the same for different RRC states, as anyway the cause value is copied from previous notification message.

Proposal 5: Notification behaviour will be same for intermediate relay UEs in RRC_CONNECTED/ IDLE/ INACTIVE, and the spec impact is to delete ‘in RRC_IDLE or RRC_INACTIVE’ in NOTE-1 in 5.8.9.10.2.

- Conclusion

In this contribution, we discussed NotificationMessageSidelink message related procedure and cause value.

We have the following Observations and Proposals:

RRC-5/12

Observation 1: Neither option-1 (copy received cause value) nor option-2 (regenerate cause value) will lead to double trigger of the notification message, because they are just related to how to set cause value and the exact timing to send notification message can be the same (e.g., after reselection happens).

Observation 2: Option-2 (regenerate cause value) will introduce one or two new cause value (e.g. relayUE-RelayReselection and relayUE-RRCreestablishment) to reflect the CONNECTED/ IDLE/ INACTIVE intermediate relay UE behaviour.

Observation 3: It is possible that IDLE/INACTIVE intermediate relay UE performs cell (re)selection after it receives notification message.Observation 4: If we go option-1, the cause value setting will not be affected no matter whether the intermediate relay UE do relay reselection or cell (re)selection or RRC re-establishment, because the intermediate Relay UE will just send the copied cause value.

Proposal 1: (RRC-5/12) When an intermediate relay UE receives a NotificationMessageSidelink message from the parent relay, it forward a copy of the original notification (with the same cause) to its child UEs.

Proposal 2: (RRC-5/12) When intermediate relay UE detects PC5 RLF, it should set the cause value to ‘relayUE-PC5-RLF’ to its child UE.

Proposal 3: (RRC-5/12) If Proposal 1 and Proposal 2 are agreed, adopt the following TP.

RRC-11

Proposal 4: (RRC-11) There are no parameter not visible to AS layer that could change and would affect reselection and force the remote UE to be notified by intermediate relay UE in AS-layer notification message. I.e., keep the current NOTE-1 in 5.8.9.10.2 in RRC running CR.

FFS in RAN2#130

Observation 5: If we go for option-1, the intermediate relay UE behaviour for sending notification messages should be the same for different RRC states, as anyway the cause value is copied from previous notification message.

Proposal 5: Notification behaviour will be same for intermediate relay UEs in RRC_CONNECTED/ IDLE/ INACTIVE, and the spec impact is to delete ‘in RRC_IDLE or RRC_INACTIVE’ in NOTE-1 in 5.8.9.10.2.

- References

- R2-250XXXX Open issues for Multi hop Sidelink Relay in TS 38.331_V03_Rapp;

- RAN2 #129 meeting chair notes;

- RAN2 #129bis meeting chair notes;

- RAN2 #130 meeting chair notes;

- R2-250XXXX Merged RRC CR for R19 Multihop_SL_Relay_v01_Rapp;