3GPP TSG-RAN WG2 Meeting #131		R2-2505087

Bangalore, India, 25th – 29th August 2025

Source:	vivo

Title:	(RRC-14) Reporting of Target Relay UEs for scenario C and D

Agenda Item:		8.13.4

Document for:	Discussion and Decision

- Introduction

In RAN2 #130 meeting, most issues related to agenda ‘8.13.4 Service continuity’ have been resolved and a bunch of agreements were reached on scenario C and D[1].

Meanwhile, in the status report for multi-hop relay in RAN #108, no open issues were identified for service continuity[2]:

#### 2.2.2 Remaining Open issues - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. - Relay discovery and (re)selection - Further details of notifications - Impact on SRAP and QoS handling for multi-hop - Whether to introduce reflective bearer mapping as an optional feature. - Control plane procedures - Whether to confirm the working assumption on indication of whether it is RRC_CONNECTED in the discovery message RRC container for RRC connection setup

In this contribution, we would like to discuss the remaining open issue proposed by rapporteur which is RRC-14:

- Open Issue RRC-14: Clarification on Reporting of Target Relay UEs in RRC_CONNECTED State for Scenarios C and D

- Discussion

In RAN2 #130 meeting[1], it was agreed:

- For scenario C, the legacy path-switching command and procedure are reused for the Remote UE and the target first Relay UE. Upon receiving the path-switching command, the Remote UE performs the legacy UE behavior toward the target first Relay UE. The (first)intermediate/last Relay UE belonging to the target path should be in the RRC-connected state. - For scenario D, the legacy path-switching command and procedure are reused for the Remote UE and the first Relay UE in the target path. Upon receiving the path-switching command, the Remote UE performs legacy UE behavior toward the first Relay UE on the target path. The (first)intermediate/last Relay UE belonging to the target path should be in the RRC-connected state.

This is aligned with WID[3]:

- Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

Then the following open issue is raised by RRC rapporteur[4]:

Open Issue RRC-14: Clarification on Reporting of Target Relay UEs in RRC_CONNECTED State for Scenarios C and D Clarification is needed to explicitly state that Remote UEs shall only report target relay UEs that are in RRC_CONNECTED state when operating in Scenario C and Scenario D. This ensures consistent behavior across implementations and avoids ambiguity regarding the state of candidate relay UEs during reporting procedures. The Relay UE includes an indication of its RRC_CONNECTED status in the discovery message RRC container. The Remote UE can utilize this indication to determine the connection state of each candidate Relay UE and subsequently report only those candidate First /Intermediate Relay UEs that are in the RRC_CONNECTED state. Such clarification can be included in the Stage 2 specification, specifically in the description of the signalling flows for Scenario C and Scenario D or in RRC Specification. Proposal 8 – RAN2 should clarify in TS 38.300 or TS 38.331 that, for Scenario C and Scenario D, the Remote UE shall report only Relay UEs in RRC_CONNECTED state for path switching, based on the Relay State indication received in the discovery message RRC container.

Till now, the agreements related to measurements for scenario C and D are as follows[1]:

- For intra-gNB direct to multi-hop indirect path switch (scenario C), remote UE evaluates both relay link (for the link between remote UE and candidate first relay UE) and Uu link, where the sidelink relay measurement report shall include at least candidate first relay UE's source L2 ID, serving cell ID, and sidelink measurement quantity result.

- For intra-gNB single-hop indirect to multi-hop indirect path switch (scenario D), the remote UE reports serving single-hop relay UE and candidate first relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

Observation 1: For scenario C and D, the remote UE will report the source L2 ID and serving cell ID for candidate first relay UE(s).

In our understating, as we only support path switch in the same cell, when the gNB find the candidate first relay UE(s) is not under the same serving cell based on its serving cell ID, it will not trigger path switch to it.

Observation 2: If the gNB find the candidate first relay UE(s) is not under the same serving cell, it will not trigger path switch to it.

On the other hand, it the gNB find the candidate first relay UE(s) is under the same serving cell by reporting from remote UE A, then it means the candidate first relay UE(s) is either a working remote UE or a working first relay UE for another remote UE-B. In both cases, we understand the gNB knows if it is CONNECTED for the candidate first relay UE(s), because the gNB can identify it by the UE’s L2 ID as it will be reported by UE in SidelinkUEInformationNR message. So, there is no need to optimize the UE behaviour to only report target relay UEs that are in RRC_CONNECTED, because the path switch is anyway handled by gNB implementation.

Observation 3: If the gNB find the candidate first relay UE(s) is under the same serving cell, it knows the RRC state information for the candidate first relay UE(s), and path switch can be triggered by gNB implementation to ensure to switch to RRC_CONNECTED (first)intermediate/last Relay UE.

Therefore, we propose that no such optimization is needed, and further spec impact can be avoided.

Proposal 1: (RRC-14) No need to report only Relay UEs in RRC_CONNECTED state for path switching. It is up to network implementation to switch the UE to the target path where the (first)intermediate/last Relay UE are in the RRC-connected state.

- Conclusion

In this contribution, we discussed the only left open issue RRC-14 service continuity.

We have the following Observations and Proposals:

Observation 1: For scenario C and D, the remote UE will report the source L2 ID and serving cell ID for candidate first relay UE(s).

Observation 2: If the gNB find the candidate first relay UE(s) is not under the same serving cell, it will not trigger path switch to it.

Observation 3: If the gNB find the candidate first relay UE(s) is under the same serving cell, it knows the RRC state information for the candidate first relay UE(s), and path switch can be triggered by gNB implementation to ensure to switch to RRC_CONNECTED (first)intermediate/last Relay UE.

Proposal 1: (RRC-14) No need to report only Relay UEs in RRC_CONNECTED state for path switching. It is up to network implementation to switch the UE to the target path where the (first)intermediate/last Relay UE are in the RRC-connected state.

- References

- RAN2 #130 meeting chair notes;

- RP-250969, Status Report for NR_SL_relay_multihop;

- RP-250188, Revised WID on NR sidelink multi-hop relay;

- R2-250XXXX Merged RRC CR for R19 Multihop_SL_Relay_v01_Rapp;