3GPP TSG-RAN WG2 Meeting #131	R2-2505091

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.2.2

Source: 	CATT

Title: 	Discussion on Paging for Ambient IoT

Document for: 	Discussion and Decision

1	Introduction

Based on the progress of RAN2#130 [1] and the guideline of open issue list [2], this contribution further discusses the open issues of the AIoT paging mechanisms.

2	Discussion

2.1		Issue 1-3: Paging ID length field

On Paging ID length field, there is an open issue in the open issue list for further discussion.

| Issue 1-3: Paging ID length field | The field to indicate the paging ID length, e.g., value range, how many bits, format design, taking into account of CT4 and SA2 reply LS in C4-252466 and S2-2505793. - Relevant agreements: - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - RAN2 sent LS to CT4 and SA2 in R2-2503197 asking for their feedback on the above agreement, for RAN2 to determine the field for paging ID length. CT4 replied with LS and CR in C4-252466 (LS on paging ID). SA2 reply LS is in S2-2505793. - Note: SA2 already agreed the filtering information and captured it in clause 5.8 in 23.369, this may enable some extent of RAN2 discussion before their feedback. - Status in running CR: the field name is captured in 6.2.1.1 without the detailed format. | Companies are invited to input views for Q#3 |
| --- | --- | --- |

As discussed under Q#3 in [2], companies reach consensus that the paging ID length field can directly indicate the length of the paging ID. And an 8-bit paging ID length is proposed based on the filter definition provided by CT4 [3].

In general, we agree with the majority that the paging ID length field should directly indicate the length of the paging ID. However, the size of the paging ID length field may need to await SA3’s further progress. SA2’s reply LS is excerpted hereafter:

# 1 Overall description SA2 thanks RAN2 for the LS on paging ID length (S2-2504515/R2-2503197). SA2 has discussed the issue and would like to provide the following feedback. In TS 23.369 clause 6.2.2, it is specified that the AIOTF sends the AIoT Identification Information to be included in the paging message to the NG-RAN node, and the AIoT Identification Information can include Filtering Information or a single AIoT Device Identifier. For the Filtering Information, SA2 has agreed the attached pCR (S2-2505850) and asks CT4 to continue the stage 3 work of the Filter Information. Security aspect of the Filter Information will be decided by SA3. For the single AIoT Device Identifier, what identifier(s) to be used is under discussion in SA3. SA2 kindly asks CT4 to provide the maximum length of the Filtering Information, and kindly asks SA3 to provide feedback on security aspect of the Filtering Information and on the single AIoT Device Identifier. SA2 kindly asks RAN2 to take the above information into account. # 2 Actions To CT4 ACTION: SA2 kindly asks CT4 to provide the maximum length of the Filtering Information. To SA3 ACTION: SA2 kindly asks SA3 to provide feedback on security aspect of the Filtering Information and on the single AIoT Device Identifier. To RAN2 ACTION: SA2 kindly asks RAN2 to take the above information into account.

Reference to the yellow highlight part, the security aspect of the filter information and AIoT Device Identifier will be decided by SA3. As a consequence, the paging ID field in the R2D paging message may not carry the filter information/permanent identifier in its original format; instead, it could be an encrypted value whose length differs from the original. Hence, RAN2 could assume an 8-bit length field to indicate the paging ID length for now, with the understanding that this can be updated based on SA3 further progress, if needed.

Proposal 1: (1-3) The length of the paging ID is directly indicated in an 8-bit paging ID length field. The size of paging ID length field can be updated based on SA3 further progress, if needed.

2.2		Issue 1-7: Security parameter in Paging message

Based on SA3’s LS [4], Q#8 was discussed in [2].

| (New)Issue 1-7: Security parameter | How to include the security parameters in Paging message. - SA3 sent a LS after May meeting in S3-252392, indicating “SA3 is assuming the ability to include an additional parameter, a 128bit random number in the paging request message.” The potential RAN2 impact is to add another field in paging message to contain this security parameter. - Status in running CR: not captured yet | Companies are invited to input views for Q#8 |
| --- | --- | --- |

In the post email discussion, companies reached consensus that the 128-bit random number can be contained in the paging message, resolving the yellow highlight part in SA3’s LS excerpted below. SA3’s LS also ask the feasibility of including additional security parameters and their maximum size, shown as the cyan highlight part.

1. Overall Description: SA3 is currently working on the authentication procedure. For this procedure, SA3 is assuming the ability to include an additional parameter, a 128bit random number in the paging request message. There may be other security parameters required. Therefore, SA3 would like to request RAN2 to confirm the feasibility of including additional security parameters and their maximum size. 2. Actions: To RAN2 ACTION: RAN2 is kindly requested to take the above information into account and provide feedback.

The maximum size for additional security parameters depends on the maximum R2D TBS and the current paging message design. Based on the RAN1 agreement below, the D2R TBS is up to 128 bytes, we therefore assume 128 bytes is the maximum TB size for R2D transmission as well.

Agreement Confirm the working assumption with following updates Working assumption - For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size except for Msg1 and Msg3 transmission in CBRA and 1st D2R Message in CFRA: - 7 bits for byte-level D2R payload size indication - Regarding the TBS of Msg3 in CBRA, and 1st D2R Message in CFRA - From RAN1 perspective, it is up to other WGs to decide the actual payload value set and how device knows the actual payload size

Taking the 128 bytes as assumption of maximum R2D TBS, the actual TB size of R2D depends on the channel condition and coverage requirement. From RAN2’s perspective, we prefer to support the requirement on the additional security parameters since we respect the requirement from SA3. So a field (limited-bits) indicating additional security parameters information is feasible in the A-IoT paging message, regardless of whether there is paging ID is included or not. However, the number of bits required for this field should be as small as possible. We assume the required bits is much smaller than the 128bit random number, otherwise SA3 would specify the exact bits to RAN2.

Considering the existing MAC PDU of paging message, up to 256bits for paging ID and the 128bit random number, it seems the maximum size of existing A-IOT paging message is 432bits. According to the discussion in SA3, the bits of additional security parameters are just few bits.

It seems up to 16bits for SA3 additional security parameters is feasible within total 448bits-length of A-IOT paging message.

On the other hand, it is no clear yet that whether these SA3 parameters including a 128bit random number and additional security parameters are mandatory or not. SA3 should clarify this requirement for the MAC PDU design of A-IoT paging message. If these two parameters are not always present, 2-bits for the indication of these two fields are required.

Proposal 2: (1-7) Send a reply LS to SA3 and CC SA2, indicating:

- It is feasible to include the 128bit random number in the paging message, regardless of whether there is a paging ID is included or not. RAN2 will include it in the A-IoT paging message.

- It is feasible to include additional security parameters within a limited-length in the A-IoT paging message, regardless of whether there is a paging ID is included or not.

- The number of bits for this additional security parameters field should be as small as possible, e.g. the maximum size is 16bits. Furthermore, the length of the field should be a fixed-length without a length field introduced. RAN2 will include this fixed-length field (e.g. 16 bits) for the additional security parameters in the A-IoT paging message if the maximum size as 16bits is accepted by SA3.

- RAN2 would like to ask SA3 that whether the 128bit random number and additional security parameters are always present in the paging request message.

2.3		Issue 1-6: Paging ID visibility

In the open issue list, there is an open issue on visibility of the Paging ID, which is marked as not critical.

| Issue 1-6: Paging ID visibility | Whether Paging ID is invisible or visible to MAC. - Relevant agreements: - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer. - From the previous discussion, there are some motivations to make paging ID visible to MAC: - 1. Reader can operate on the paging ID for further sub-grouping. The Rapp understands this can be considered as an enhancement from reader side for better system efficiency. From device side, since there is an explicit indication for CBRA and CFRA, the device (even in multi-device CFRA) can determine how to perform random access instead of paging ID/group ID. In this case, such visibility is not an essential function. And according to guidance from chairlady, such enhancement can be considered with lower priority. - 2. Reader can associate the paging ID/device ID and AS ID for a given device within a service request. The Rapp understands according to RAN3 LS R3-252481, reader will allocate NGAP device ID for each device and maintain the per-session per-device context, via which the reader can associate the command receiving from the NG interface with the AS ID assigned for a device, i.e. such device management/association does not rely on the paging ID/device ID. - 3. The Temp ID may have impact on this visibility discussion. The Rapp understands SA3 has not concluded on the solution of Temp ID. But majority seems think this Temp ID is maintained/managed between CN and device, since they already concluded there is no AS security in A-IoT. Therefore, no RAN2 discussion is needed before SA3 further inputs. | Not critical |
| --- | --- | --- |

In past meetings, discussion on the paging ID’s visibility have focused on whether there is a need for visibility at the MAC layer. However, the visibility of the paging ID to the MAC layer actually depends on how the AIOTF sends the paging ID to reader, i.e., design of SA2/RAN3 signalling, rather than on whether it is useful at the AIoT interface. After checking the specification of SA2 [5], we think there is a need to revisit the assumption achieved in RAN#129.

Reference to 23.369 [5], we observe that the assumption achieved by RAN2 is not aligned with the SA2 understanding. Take the step 7 in “Figure 6.2.2-1: Inventory Procedure” as an example, it can be seen that the inventory request carries no AIoT NAS data. That is, the AIOTF sends the inventory request with AIoT identification information as clear-text IE. As a consequence, the reader only can contain the paging ID as clear-text field in the R2D paging message.

7. The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, and assistance information to the selected NG-RAN as specified in TS 38.413 [10]. […] 10. NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s) as specified in TS 38.413 [10]. The NG-RAN may aggregate multiple Inventory Report messages based on the assistance information before reporting the response to the AIOTF as described in clause 5.9. The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

Observation 1: (1-6) In TS 23.369, the AIOTF send the inventory request message with AIoT Identification Information as cleartext IE.

We therefore understand that the paging ID is visible to the AIoT MAC layer. Note that this clarification serves only to align our understanding with SA2’s design; it does not imply that the AIoT MAC layer will use the paging ID for any further enhancement. We can send an LS to SA2 and RAN3 to confirm this understanding.

Proposal 3: (1-6) The paging ID is visible to the AIoT MAC layer.

3	Conclusion

In this contribution, paging functionality for A-IoT is discussed with giving corresponding observations and proposals:

Proposal 1: (1-3) The length of the paging ID is directly indicated in an 8-bit paging ID length field. The size of paging ID length field can be updated based on SA3 further progress, if needed.

Proposal 2: (1-7) Send a reply LS to SA3 and CC SA2, indicating:

- It is feasible to include the 128bit random number in the paging message, regardless of whether there is a paging ID is included or not. RAN2 will include it in the A-IoT paging message.

- It is feasible to include additional security parameters within a limited-length in the A-IoT paging message, regardless of whether there is a paging ID is included or not.

- The number of bits for this additional security parameters field should be as small as possible, e.g. the maximum size is 16bits. Furthermore, the length of the field should be a fixed-length without a length field introduced. RAN2 will include this fixed-length field (e.g. 16 bits) for the additional security parameters in the A-IoT paging message if the maximum size as 16bits is accepted by SA3.

- RAN2 would like to ask SA3 that whether the 128bit random number and additional security parameters are always present in the paging request message.

Observation 1: (1-6) In TS 23.369, the AIOTF send the inventory request message with AIoT Identification Information as cleartext IE.

Proposal 3: (1-6) The paging ID is visible to the AIoT MAC layer.

4	Reference

- RAN2#130 Chair’s notes.

- Summary of A-IoT MAC open issues

- C4-252466	Reply LS on paging ID length

- S3-252392	LS on security parameter in paging request message

- 3GPP TR 23.369	Architecture support for Ambient power-enabled Internet of Things.