3GPP TSG-RAN WG2 Meeting #131		R2-2505128

Bengaluru, India, August 25th – 29th, 2025

Source:	vivo

Title:	Discussion on functionality management for RRM measurement prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

# Introduction

At the RAN2#130 meeting, the following agreements were reached regarding applicability reporting, inference configuration/reporting and performance monitoring for UE-sided model:

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology). Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them. Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

In this contribution, we will further discuss issues related to these aspects.

# Discussion

## 2.1 Applicability reporting

For AI/ML-based beam management use case, RAN2 further reached the following agreements about applicability reporting:

Agreements - Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified) - Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified) Agreements 1. When UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported). 2. The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable. 3. The UE shall report when CSI-ReportConfig becomes not applicable Agreements - (RRC 7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design. - (RRC6) On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations). FFS Whether applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is supported (if it comes for free). - (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

We think that all the above ageements about applicability reporting for beam management use case can be applicable for AI mobility with some minor adaptations. RAN2 can further confirm every agreement and reach the conclusions.

- Confirm that the following agreements on applicability reporting in AI-based beam management are applicable for AI mobility:

- UE may include “release configuration” flag in applicability reporting to indicate UEs preference to release a non-applicable configuration.

- Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled.

- When UE indicates that an inference configuration is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

- The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE-reported predicted values after UE indicates that an inference configuration is not applicable.

- The UE shall report when an inference configuration becomes non-applicable.

- RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A).

- On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration.

- Option A and Option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

AI-based BM takes CSI framework as a baseline, which only involves prediction related to the RS of the serving cell. The inference configuration of AI-based BM is relatively simple, making the applicability determination fairly clear. For AI-based mobility, the current framework is based on RRM measurement configuration/reporting. RRM measurement configuration can include different granularities, such as frequency, cell, and beam. In such cases, for a single inference configuration, only part of the configuration may be applicable. Under these circumstances, the UE should be able to report which part of the inference configuration is applicable.

- For AI/ML-based mobility, UE could indicate partial applicability in the applicability reporting, e.g., only part of the cells in the inference configuration are predictable.

Additionally, for inter-frequency prediction, simulation results indicate that the performance of the AI model meets requirements only under specific frequencies as input and output. Therefore, for a specific frequency as output, the UE should indicate the required measurements of the specific frequency as input to the network, meaning the network needs to configure the corresponding measurement object. The UE can indicate this information via applicability reporting, indicating the associated conditions under which the inference configuration is applicable, such as the need for the network to configure the corresponding measurement object to acquire input.

- UE could indicate the associated conditions under which the inference configuration is applicable, e.g., the required measurements of specific frequency as input for inter-frequency prediction.

## 2.2 Inference configuration/reporting

In the current AI/ML for mobility study, RAN2 agreed the following two goals:

- Goal 1: measurement reduction

- Goal 2: handover performance improvement

In the whole RRM measurement procedure, there are three phases:

- Phase 1: the UE performs the measurement to obtain the measurement results.

- Phase 2: the UE determines whether to trigger the measurement report based on measurement results.

- Phase 3: if triggered, the UE sets the content of the measurement report and sends the measurement report to the network.

Achievement of Goal 1 depends on measurement reduction during Phase 1. For Goal 1, we think that RAN2 should discuss how to perform the prediction (e.g., prediction granularity) to achieve the measurement reduction goal.

The main purpose of measurement reduction goal is to save the UE power and achieve no/less measurement gap. There are two main steps when the UE performs the measurement of a frequency in Phase 1:

- Step 1: the UE receives the signal of the frequency via RF receiver.

- Step 2: the UE processes the signal to obtain the measurement result of all the cells in the frequency via baseband.

According to the industry materials, the proportion of power consumption in Step 1 is about 80% and the one in Step 2 is about 20%. In NR, the measurement object is configured per frequency for RRM measurement. It means that the UE needs to measure all the cells for a measured frequency. The difference in power consumption between measuring all the cells of a frequency and measuring one cell of the frequency may be quite small since the main power consumption is receiving the signal of the frequency by the RF receiver. Moreover, if there is any one cell of the measured frequency that needs to be measured with RF retuning, the measurement gap needs to be configured.

Observation 1: The most power consumption of UE measurement is the RF receiver, not the baseband processing. If there is any one cell of a specific frequency that needs to be measured, no gain of measurement gap reduction can be achieved and the gain of power saving is quite limited.

Therefore, we think that for frequency domain prediction, RAN2 should consider the per-frequency granularity prediction (i.e., predict all the cells of a frequency) to achieve the goal of measurement reduction. Similarly, for temporal domain prediction, all the cells of the measured frequency should be predicted in the predicted time instances to save the signal receiving of the RF receiver in the predicted time instances to achieve the measurement reduction effect.

Goal 2 is related to Phase 2 and Phase 3. To reduce the power consumption of inference, gNB may provide a list of cells that need to be predicted. Therefore, we propose:

- Support different granularity in the inference configuration:

- For measurement reduction, the inference configuration should be per-frequency granularity, i.e., all the cells in a frequency layer should be predicted.

- For HO performance improvement, the inference configuration can be per-frequency or per-cell granularity.

In addition, at the RAN2#129bis meeting, RAN2 agreed that the UE applies inference configuration for the applicable RRM measurement prediction upon receiving configuration via RRC and it is FFS when the UE can perform inference. From our understanding, for different goals, the time of performing inference should be different. In general:

- For measurement reduction, the simulation results of SLS is based on low UE mobility (30km/h). Therefore, similar to relaxed measurement, the UE should perform the inference in the center of the serving cell, or with low mobility.

- For mobility performance improvement, to reduce power consumption of inference, the UE should perform the inference in the edge of serving cell or with high mobility.

- Support event-triggered inference for RRM measurement prediction:

- For measurement reduction, similar to relaxed measurement, the inference can be triggered when UE is not in the cell edge, or when UE is with low mobility.

- For HO performance improvement, to reduce power consumption, the inference can be triggered when UE is in the cell edge or with high mobility.

## 2.3 Performance monitoring

As to NW-decision for UE-sided monitoring, to avoid unnecessary reporting when the model works well, event-triggered monitoring reporting can be supported. That is, the network configures the triggering condition for monitoring reporting. When the condition is fulfilled, the UE sends the monitoring reporting to the network, and the network makes the management decision based on the triggered monitoring reporting from the UE.

- As to NW-decision for UE-sided monitoring, support event-triggered monitoring reporting to reduce signaling overhead of monitoring report, e.g., when the monitoring result is above the threshold configured by NW.

For UE-sided monitoring of UE-sided model, it was agreed that UE-decision can be further considered: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases.

For the activation of functionality, UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. If UE can make management decision on deactivation based on UE-sided monitoring results, the subsequent UE behavior should be clarified, e.g., releasing the inference configuration by itself. If so, it’s not aligned with the UE behavior of applicability reporting, i.e., UE shall maintain all the full inference configuration(s) no matter the configuration is applicable or inapplicable until the network releases it explicitly.

In addition, in AI-based BM, when the functionality changes from applicable to non-applicable, the UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable. We think the principle is also valid for AI-based mobility.

- Do not support UE-decision for UE-sided monitoring. When the monitoring result cannot fulfill the performance requirement:

- The NW is expected to release the inference configuration.

- UE continues to perform the inference and reporting until the configuration is released.

- It is up to network implementation what to do with UE reported RRM measurement prediction after UE reports the monitoring result.

For NW-sided monitoring, when the network receives a measurement output or monitoring result in a monitoring report, the network needs to know which inference output it associates with, since the inference output may be received earlier than the monitoring report, e.g., for temporal domain case A.

- For NW-sided monitoring, network should be able to associate the measurement based on monitoring configuration and inference output. Details is left up to the WI phase.

# Conclusion

In the contribution, we have the following observations and proposals:

Applicability reporting

- Confirm that the following agreements on applicability reporting in AI-based beam management are applicable for AI mobility:

- UE may include “release configuration” flag in applicability reporting to indicate UEs preference to release a non-applicable configuration.

- Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled.

- When UE indicates that an inference configuration is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

- The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported predicted values after UE indicates that an inference configuration is not applicable.

- The UE shall report when an inference configuration becomes non-applicable.

- RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A).

- On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration.

- Option A and Option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

- For AI/ML-based mobility, UE could indicate partial applicability in the applicability reporting, e.g., only part of the cells in the inference configuration are predictable.

- UE could indicate the associated conditions under which the inference configuration is applicable, e.g., the required measurements of specific frequency as input for inter-frequency prediction.

Inference configuration/reporting

Observation 1: The most power consumption of UE measurement is the RF receiver, not the baseband processing. If there is any one cell of a specific frequency that needs to be measured, no gain of measurement gap reduction can be achieved and the gain of power saving is quite limited.

- Support different granularity in the inference configuration:

- For measurement reduction, the inference configuration should be per-frequency granularity, i.e., all the cells in a frequency layer should be predicted.

- For HO performance improvement, the inference configuration can be per-frequency or per-cell granularity.

- Proposal 5.	Support event-triggered inference for RRM measurement prediction:

- For measurement reduction, similar to relaxed measurement, the inference can be triggered when UE is not in the cell edge, or when UE is with low mobility.

- For HO performance improvement, to reduce power consumption, the inference can be triggered when UE is in the cell edge or with high mobility.

Performance monitoring

- As to NW-decision for UE-sided monitoring, support event-triggered monitoring reporting to reduce signaling overhead of monitoring report, e.g., when the monitoring result is above the threshold configured by NW.

- Do not support UE-decision for UE-sided monitoring. When the monitoring result cannot fulfill the performance requirement:

- The NW is expected to release the inference configuration.

- UE continues to perform the inference and reporting until the configuration is released.

- It is up to network implementation what to do with UE reported RRM measurement prediction after UE reports the monitoring result.

- For NW-sided monitoring, network should be able to associate the measurement based on monitoring configuration and inference output. Details is left up to the WI phase.

# Reference

- R2-2501822	Text proposal of TR 38.744 V0.0.7.