3GPP TSG-RAN WG2 Meeting #131	R2-2505129

Bengaluru, India, Aug. 25th – 29th, 2025

Source:	vivo, Qualcomm Incorporated, Huawei, HiSilicon, CATT, ZTE, Ericsson, CMCC, NTT DOCOMO, Interdigital

Title:	Recommendation on specification for L3 beam-level prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

- Introduction

At the RAN2#129-bis meeting, based on the consensus on the feasibility and necessity of L3 beam-level prediction in [1], it was agreed that the impact of the L3 Beam-level prediction specification will be studied in the SI phase.

However, whether L3 beam-level prediction is included in the WI is still pending in the baseline WID [2] endorsed at the RANP#108. The primary concerns are regarding the progress during the SI phase and the workload in the normative phase for RAN2 and RAN4.

- Specify support for RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction case A and case B, and Frequency domain prediction for collocated cases. Cell level predication is supported Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI. Note 2: Check in RAN#109 whether L3 beam level prediction is also supported, based on the final outcome of the SI.

In this contribution, we will discuss the general potential specification impact and workload of L3 beam-level prediction from RAN2 and RAN4 perspectives, and then propose a recommendation on the specification of L3 beam-level prediction in WI.

- Discussion

## Necessity and importance of L3 beam-level prediction

In the existing non-AI handover procedure, L3 beam-level measurement results are exchanged between the source and target nodes to configure RACH resources, thereby reducing handover latency and ensuring robustness. For AI-based mobility, the absence of L3 beam-level prediction may reduce the performance gains of AI/ML-based approach over legacy mobility. For instance:

- For temporal domain Case A prediction, if the PW length is large, the UE can predict measurement events that will be fulfilled in advance and report them early. In this case, the time gap between the handover command and the measurement report becomes significant. If the target gNB configures the RACH resources based on beam-level measurements at the reporting time, the CFRA procedure during handover may fail due to wrong beam selection. Alternatively, CBRA would have to be used, but this could increase handover delay due to potential preamble collisions and raise the likelihood of handover failure.

- For frequency domain prediction, if L3 beam-level prediction is unavailable, only CBRA can be employed. As a result, an AI/ML-based approach may have poorer performance than legacy HO.

Consequently, it becomes critical to include L3 beam-level prediction in the measurement prediction report. Otherwise, the network may be unable to make an informed mobility decision, e.g., allocating RACH resources on the target cell for the upcoming UE. Therefore, relying solely on cell-level prediction is not sufficient and may lead to suboptimal or even failed mobility procedures.

Observation 1: L3 beam-level prediction is necessary for AI/ML-based mobility. Relying solely on cell-level prediction may reduce the performance gains of AI/ML-based approach over legacy non-AI approach.

## Potential R2 impact for L3 beam-level prediction

In the previous two meetings, RAN2 discussed some general LCM procedures and the corresponding potential specification impacts. This section will analyze the conclusions of each LCM component (e.g., inference, training, monitoring) to identify their applicability to L3 beam-level prediction.

### Inference configuration for UE-sided model

For the existing RRM measurement, the network can request the UE to report L3 beam-level measurement results in the RRM measurement report, i.e., via configuring the reportQuantityRS-Indexes , maxNrofRS-IndexesToReport and includeBeamMeasurements. Similarly, the network can also send L3 beam-level prediction requests and corresponding inference configurations when configuring L3 cell-level prediction.

The current agreements on inference configuration for UE-sided model are as follows.

- The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference - For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details - For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details - Baseline is that this applies to all A1-6 events, unless technical problems are identified. Baseline quantity is RSRP. - The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

In our view, the above agreements can be applicable for L3 cell-level and beam-level prediction. Specifically:

- The inference configuration and reporting for L3 cell-level/beam-level prediction are based on the RRM measurement framework;

- The UE can be configured with periodic or event-triggered reporting of L3 cell-level/beam-level prediction;

- The potential “inference parameters” for inference configuration include: PW length (for temporal case A), Skipping pattern (for temporal case B), Measured and predicted beam pattern (for spatial domain prediction), Measured and predicted frequency (For inter-frequency domain prediction);

- Model-related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length) can be up to UE implementation.

### 2.1.2	Inference input for NW-sided model

As to the inference input, the current conclusions have already taken L3 beam-level prediction into account, e.g., one measurement report may include L3 cell-level/beam-level measurement results at multiple time instances to support L3 cell-level/beam-level prediction.

Furthermore, L1-filtered beam-level measurement results can be reported with the existing RRM configuration by setting FilterCoefficient to 0.

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero. FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction. - To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

### 2.1.3	Data collection for model training

Similarly, data collection for model training has also considered L3 beam-level prediction. Specifically, the collected data content includes L3 beam-level measurements and L1-filtered beam-level measurements.

For the collection of data for the training of a network-sided model for RRM measurement prediction: - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed) - The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS. - The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity. - The UE sends availability indication of collected logged data via UAI or RRCReconfigurationComplete message for HO case. - The availability indication can be triggered due to: - full buffer being reached (if configured), (FFS if buffer is per use case) - buffer threshold being reached (if configured), or (FFS if buffer is per use case) - low power (if configured) - Upon sending the availability indication, UE indicates: - Data is available - Reason for the triggering of the availability indication (full buffer. Threshold) - Low power indication - The UE sends the collected data upon explicit/on-demand request from the network (using UEInformationRequest/Response signaling) - The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO). - The UE releases the collected data upon transitioning to IDLE/INACTIVE - For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial. It can be up to WI phase if this is done depending on whether a simple solution can be found For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

### 2.1.4	Performance monitoring

For performance monitoring, the current agreement mainly covers an overall monitoring framework, including NW-sided monitoring and UE-sided monitoring.

- RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case - For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement - FFS on UE awareness and preference for NW sided model - For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. - For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

In our view, this framework applies to both cell-level and beam-level prediction. Specifically:

- For performance monitoring of NW-side model, UE can be configured to provide actual L3 cell-level/beam-level measurement results with the existing RRM framework;

- For NW-sided monitoring of UE-sided model, UE can be configured to report L3 cell-level/beam-level measurements and predictions.

- For UE-side monitoring of UE-sided model, UE can generate monitoring results for L3 cell-level/beam-level predictions.

### 2.1.5	Summary of potential R2 impact

From the previous agreements, it can be seen that the RRM measurement framework is reused across various LCM procedures, which already supports both cell-level and beam-level configuration/reporting. Furthermore, it can be observed that the current agreements regarding the specification impact of RRM measurement prediction either apply simultaneously to both cell-level and beam-level predictions, or have already considered both cell-level and beam-level predictions concurrently. Furthermore, to make it clear, it’s beneficial to capture in the TR that the concluded potential specification impact of RRM measurement prediction is applicable to both L3 cell-level and beam-level predictions, unless otherwise explicitly described.

Observation 2: The existing RRM framework, which is reused as a baseline across various LCM procedures of RRM measurement prediction, already allows for configuration and reporting of both L3 cell-level and beam-level measurements.

Observation 3: The current agreements on the specification impact (e.g., inference, data collection, monitoring) of RRM measurement prediction either apply simultaneously to both L3 cell-level and beam-level predictions, or have already explicitly considered both L3 cell-level and beam-level predictions.

Proposal 1: Capture in the TR that the concluded potential specification impact of RRM measurement prediction is applicable for both L3 cell-level and beam-level predictions unless otherwise explicitly described.

## Potential R4 workload for L3 beam-level prediction

During the last RAN4 meetings, regarding RRM prediction, RAN4 primarily discussed and analyzed the potential RRM impacts. This mainly included candidate KPIs and their definitions, factors that may affect prediction performance, as well as the impact of measurement errors.

First, concerning the definition of KPIs, RAN4 has already defined the absolute and relative accuracy of predicted L3-RSRP as the candidate metrics for RRM requirements in RRM measurement prediction.

The absolute accuracy of predicted L3-RSRP is defined as: - Absolute accuracy of predicted L3-RSRP = reported predicted L3-RSRP – ground truth of L3-RSRP. The relative accuracy of predicted L3-RSRP for intra-frequency prediction is defined as: - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported RSRP of cell 2 can be measured or predicted. - Editor Note: The relative RSRP accuracy for beam level measurements may be further discussed during WI phase depending upon RAN2 progress. The relative accuracy of predicted L3-RSRP for inter-frequency prediction is defined as: - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported RSRP of cell 2 can be measured or predicted. Editor Note: RAN4 will further discuss whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy.

Regarding the applicability of the definition to L3 beam-level prediction, the current situation is:

- For the absolute RSRP accuracy definition, as shown below, this definition is common to both cell-level and beam-level prediction.

The absolute accuracy of predicted L3-RSRP is defined as: - Absolute accuracy of predicted L3-RSRP = reported predicted L3-RSRP – ground truth of L3-RSRP.

- For the relative RSRP accuracy definition, beam-level prediction was also involved in the detailed discussions on the definition in previous RAN4 meetings, and the following definition was agreed upon during the RAN4#114bis meeting, as highlighted in the section below.

Agreement in RAN4#114bis Issue 2-1-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed.

It can be observed that the above RAN4 agreements already refer explicitly to beam-level prediction, but are dependent on whether beam-level prediction is supported from signalling perspective (which is pending RAN2 conclusion). If RAN2 agrees to support L3 beam-level prediction in Rel-20, it is straightforward that the definitions agreed upon during the study item phase can be used as the baseline.

Observation 4: Some of the RAN4 agreements already refer explicitly to beam-level prediction, but are dependent on whether beam-level prediction is supported from signalling perspective (which is pending RAN2 conclusion).

Secondly, the potential factors currently identified by RAN4 that affect prediction performance should be applicable to both cell-level and beam-level prediction. For example, for intra-frequency temporal domain prediction, RAN4 has considered factors such as UE speed, MRRT, OW/PW length, and measurement error. The difference lies in the fact that the extent of their impact on prediction performance may vary between cell-level and beam-level prediction, and thus may require separate quantitative evaluations. However, since the platform, simulation assumptions, and dataset for cell-level prediction can be largely reused for beam-level prediction, the extra workload for beam-level prediction can be acceptable.

Additionally, regarding the performance monitoring mechanism not covered by RAN4 during the SI phase, in the Rel-19 WI "AI/ML for NR Air Interface," a basic version of this mechanism has already been designed for the BM use case, introducing the report quantity of accurate reference signal prediction instance(s) that meets the condition. Specifically, this condition refers to a certain prediction accuracy ratio of the top beam(s), i.e., whether the Top 1 or 2 beam(s) is among the top-K predicted beam(s).

Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling Agreement For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N) - Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring - condition: - for the transmission occasion of monitoring resources, it has a linked inference report - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance. - Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig - the size of CSI field associated with the RS-PAI is log2(N+1)

Currently, for beam-level measurement, if the network configures reportQuantityRS-Indexes and maxNrofRS-IndexesToReport, the UE will include beam measurement information in the triggered measurement report, which at least includes the index of the best beam(s). If the network further sets includeBeamMeasurements to true, the UE will additionally report beam-level measurement results. This is because the beam-level measurement results are mainly used to allocate RACH resources associated with the best beam(s). Therefore, similar to AI-based beam management, L3 beam-level prediction should also place great importance on the accuracy of best beam prediction, which can be considered as a starting point for discussing the performance metric for monitoring L3 beam-level prediction.

From the above analysis, it can be seen that from the RAN4 perspective, the support of L3 beam-level prediction can refer as much as possible to the outcomes discussed in L3 cell-level prediction and/or AI-based beam management. The main workload actually lies in performance evaluation and requirement definition. However, we understand that RAN2 has already conducted much constructive preliminary work on performance evaluation during the SI phase. Based on this, RAN4 can further proceed with the specific requirement definition.

Observation 5: With L3 cell-level prediction and/or AI-based BM as reference, the workload of RAN4 on L3 beam-level prediction is expected to be acceptable.

## Recommendation on the specification of L3 beam-level prediction

Based on the above analysis of the specification impact of L3 beam-level prediction, it can be observed that RAN2 has made sufficient progress on L3 beam-level prediction during SI phase, which is comparable to L3 cell-level prediction. In addition, the extra workload of RAN4 on L3 beam-level prediction is expected to be acceptable. In conclusion, we propose:

Proposal 2: From RAN2's perspective, for RRM measurement prediction of both UE- and network-sided models, support of L3 beam-level prediction is recommended for normative work.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Observation 1: L3 beam-level prediction is necessary for AI/ML-based mobility. Relying solely on cell-level prediction may reduce the performance gains of AI/ML-based approach over legacy non-AI approach.

Observation 2: The existing RRM framework, which is reused as a baseline across various LCM procedures of RRM measurement prediction, already allows for configuration and reporting of both L3 cell-level and beam-level measurements.

Observation 3: The current agreements on the specification impact (e.g., inference, data collection, monitoring) of RRM measurement prediction either apply simultaneously to both L3 cell-level and beam-level predictions, or have already explicitly considered both L3 cell-level and beam-level predictions.

Observation 4: Some of the RAN4 agreements already refer explicitly to beam-level prediction, but are dependent on whether beam-level prediction is supported from signalling perspective (which is pending RAN2 conclusion).

Observation 5: With L3 cell-level prediction and/or AI-based BM as reference, the workload of RAN4 on L3 beam-level prediction is expected to be acceptable.

Proposal 1: Capture in the TR that the concluded potential specification impact of RRM measurement prediction is applicable for both L3 cell-level and beam-level predictions unless otherwise explicitly described.

Proposal 2: From RAN2's perspective, for RRM measurement prediction of both UE- and network-sided models, support of L3 beam-level prediction is recommended for normative work.

- Reference

- R2-2501789	Way forward on study of beam-level prediction	vivo, Ericsson, ZTE, Huawei, HiSilicon, CATT, NTT DOCOMO, CMCC, Samsung, Interdigital

- RP-251864	New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR