3GPP TSG-RAN WG2 Meeting#131	R2-2505132

Bengaluru, India, 25th – 29th August, 2025

Source:	vivo

Title:	Discussion on NW-sided model in AI enhanced mobility

Agenda Item:	8.3.3

Document for:	Discussion and Decision

- Introduction

RAN2#130 meeting [1] has achieved several agreements regarding data collection for inference and model training of NW-sided model of AI enhanced mobility.

Agreements

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used.   For L1-filtered beam-level RSRP reporting can be configured by setting coefficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

- To support cell/beam level prediction, one enhancement is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

Agreements For the collection of data for the training of a network-sided model for RRM measurement prediction: - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed) - The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS. - The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity. - The UE sends availability indication of collected logged data via UAI or RRCReconfigurationComplete message for HO case. - The availability indication can be triggered due to: - full buffer being reached (if configured), (FFS if buffer is per use case) - buffer threshold being reached (if configured), or (FFS if buffer is per use case) - low power (if configured) - Upon sending the availability indication, UE indicates: - Data is available - Reason for the triggering of the availability indication (full buffer. Threshold) - Low power indication - The UE sends the collected data upon explicit/on-demand request from the network (using UEInformationRequest/Response signaling) - The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO). - The UE releases the collected data upon transitioning to IDLE/INACTIVE - For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial. It can be up to WI phase if this is done depending on whether a simple solution can be found

In this contribution, we keep on discussion about possible potential specification impacts of NW-sided model, Based on which the recommendation of sub use cases is provided as well.

- Discussion

## Measurements for inference or performance monitoring

At RAN2#130 meeting, it was agreed that the legacy RRM measurement configuration/reporting can be reused for network-side RRM measurement prediction.

As for the reporting scheme, the current RRM measurement framework can support both periodic and event-triggered reporting. When it comes to the inference input or monitoring of NW-sided model, NW may still intend to have the flexibility on the timing to obtain the measurement predictions and/or corresponding measurements for monitoring. For example, when NW would like to obtain the prediction periodically, it can configure UE with periodic reporting. Or when NW only wants the prediction at specific event occurrence, the event-triggered reporting can be employed. Therefore, both reporting types are useful in certain scenarios and need to be supported.

- To acquire measurements for inference or monitoring for NW-sided model, both periodic and event-triggered reporting schemes are supported.

According to the understanding of model input for different use cases, we further illustrate the potential inputs for different sub-use cases:

Table 2.1-1: Potential inference input of sub-use cases

| Use case | Sub-use case | Input of Temporal domain prediction | Input of Frequency domain prediction | Input of Spatial domain prediction |
| --- | --- | --- | --- | --- |
| Cell-level prediction | 1 | Multiple instances of L1-filtered beam-level RSRP | L1-filtered beam-level RSRP of Frequency 1 | Set B of L1-filtered beam-level RSRP |
|  | 2 | Multiple instances of L3 Cell-level RSRP | L3 Cell-level RSRP of Frequency 1 | - |
|  | 3 | Multiple instances of L1-filtered beam-level RSRP | L1-filtered beam-level RSRP of Frequency 1 | Set B of L1-filtered beam-level RSRP |
| Beam-level prediction | 4 | Multiple instances of L1-filtered beam-level RSRP | L1-filtered beam-level RSRP of Frequency 1 | Set B of L1-filtered beam-level RSRP |
|  | 5 | Multiple instances of L3 beam-level RSRP | L3 beam-level RSRP of Frequency 1 | - |
|  | 6 | Multiple instances of L1-filtered beam-level RSRP | L1-filtered beam-level RSRP of Frequency 1 | Set B of L1-filtered beam-level RSRP |

It can be observed that for sub-use cases 1, 3, 4, 6, if supported, L1 filtered beam-level RSRP is required to be reported in measurement report. And it is already agreed that L1 filtered beam-level can be achieved by NW implementation with setting filter coefficient k to 0.

Further, for some sub-use cases (e.g., spatial domain predication), the network needs to acquire the measurements of specific sets of beams (i.e., similar to set B beams in AI BM) as model inputs. Since the current RRM measurement configuration does not indicate specific beams to measure, a step forward is that, in terms of specific beams as input, does this require additional specification impact? From our understanding, an implementation-based solution can be employed to fulfil the requirement. Specifically, the network can configure UE with all the beams to be reported as MO and not apply any reported filtering parameter for RSRP of the reported beams, i.e., does not configure absThreshSS-BlocksConsolidation/ absThreshCSI-RS-Consolidation. In this way, the UE can report all configured set B beams’ RSRP fully with the current method.

- For NW-sided model, the existing spec can support NW to acquire measurement results of specific beams by setting no threshold for the consolidation of beam-level measurement results, i.e., no additional specification impact is foreseen to support sub-use cases 1, 3, 4 or 6 of NW-sided model.

Since RAN2 has agreed that the measurement report can be enhanced to include multiple instances in time, here we mainly focus on the details about how to configure UE to report these instances.

For both periodical and event-triggered reports, it is straightforward that the network needs to instruct UE about the amount of time instances and the interval between every adjacent time instances.

- For multiple time instances in one measurement report for NW-sided model, the network can configure the amount and the interval of reported time instances.

Furthermore, for event-triggered reporting, it needs to clarify the logging behaviour is performed before or after the measurement event is satisfied. On the one hand, it is easy to understand that UE receives the configuration with indication about multiple time instances within one measurement report, then it performs logging consistently until the event triggered, it would report the latest set of instances. On the other hand, UE cannot estimate the exact timing of event triggering. If UE keeps logging and storing all the time, it would cause power consumption. Besides, since NW-side data collection has agreed on measurement-event triggered data logging, we consider it suits AI mobility as well. In this sense, we would like to start this discussion about the exact timing of logging the reported time instance for event-trigged reporting.

- RAN2 to discuss that for event-triggered measurement report, the timing of the multiple instances is before or after that the event is triggered.

## Data collection for model training

For the event-triggered data logging, the following A1/A2-like events were introduced in the AI/ML-based beam management.

Support the use of L3 measurement event triggered (i.e., L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.

The mechanism is utilized to collect the data of specific areas (e.g., cell centre or cell edge) within the serving cell. The event-triggered logging could be useful to collect a complete dataset, avoid redundant data collection, save air interface resources, and save UE power. The intention is also valid for AI/ML-based mobility. Furthermore, the events can be further extended to include the events related to neighbour cell. As measurement event A1-A6 were agreed to be supported in AI/ML-based mobility and the measurement event prediction can up to NW implementation, all the events like A1-A6 should be supported for event-triggered data logging to improve the efficiency of data collection.

- In the NW-side data collection for AI/ML-based mobility, Events A1-A6 should be supported for event-triggered data logging.

## Recommendation of sub-use cases

As discussed in Section 2.1, from our understanding, there is no additional specification impact foreseen to support sub-use cases 1, 3, 4 or 6 of NW-sided model. Additionally, the sub-use case type of NW-sided model can be transparent to the UE, meaning the network can use the same measurement results as input to perform predictions in the temporal domain, frequency domain, or spatial domain. Therefore, we see no strong reason to exclude any studied use case for NW-sided model.

However, to support NW-sided model of some sub-use cases, extra signalling overhead may be introduced, e.g., temporal domain case B. Since case B is mainly for UE measurement reduction and power saving, if UE needs to report multiple instances as NW-sided model’s input, the target motivation may not be achieved by supporting the required inputs. Therefore, we think it’s a critical principle that we should not introduce specific enhancements for measurement reduction use cases of NW-sided model that may introduce additional signaling overhead (e.g., smaller periodicity for periodic reporting) in WI phase.

- All sub-use cases of NW-sided model are recommended for normative work, with the sub-use case type of NW-sided model is transparent to the UE.

- During the WI phase, do not introduce specification impacts specific for measurement reduction use cases of NW-side model which may introduce additional signaling overhead.

- Conclusion

Based on the above analysis, we come to the following proposals:

Measurements for inference or performance monitoring

- To acquire measurements for inference or monitoring for NW-sided model, both periodic and event-triggered reporting schemes are supported.

- For NW-sided model, the existing spec can support NW to acquire measurement results of specific beams by setting no threshold for the consolidation of beam-level measurement results, i.e., no additional specification impact is foreseen to support sub-use cases 1, 3, 4 or 6 of NW-sided model.

- For multiple time instances in one measurement report for NW-sided model, the network can configure the amount and the interval of reported time instances.

- RAN2 to discuss that for event-triggered measurement report, the timing of the multiple instances is before or after that the event is triggered.

Data collection for model training

- In the NW-side data collection for AI/ML-based mobility, Events A1-A6 should be supported for event-triggered data logging.

Recommendation for normative work

- All sub-use cases of NW-sided model are recommended for normative work, with the sub-use case type of NW-sided model is transparent to the UE.

- During the WI phase, do not introduce specification impacts specific for measurement reduction use cases of NW-side model which may introduce additional signaling overhead.

- Reference

- Report of 3GPP TSG RAN WG2 meeting#130