3GPP TSG-RAN WG2 Meeting #131	R2-2505150

Bengaluru, India, 25 – 29, Aug, 2025

Agenda Item:	8.1.3

Source:	Xiaomi

Title:	Discussion on NW-side data collection

Document for:	Discussion

Introduction

For NW side model, data collection is essential to collect data to train AI model. In this contribution, we discuss open issues of NW-side data collection.

Discussion

Issue 21

This issue is about how to indicate the logged data is consecutive or not. NW shall be able to identify the gap in the logged data for model training. If there is gap during data logging due to logging radio event, UE shall indicate the gap to NW. Two options are observed to support the gap indication,

Option 1: UE explicit indicate the gap along with logged data after logging gap is detected.

Option 2: UE creates another list when logging gap is detected. The logged data within the same list is consecutive.

Theoretically, both options can work. However, we understand the maximum number of lists supported by UE may be a fixed number. In option 2, if gap occurs frequently, the lists may be exhausted, but each list only contains small number of entries with logged data. Option 2 is less efficient compared with option 1.

In option 1, it’s not clear whether absolute time or 1bit indication is used to indicate the gap. We understand UE can indicate the absolute time along with the first entry with logged data after gap. Timing information is already supported in MDT and is more useful for NW to decide how to train the AI model, compared with 1bit indication. Furthermore, it’s agreed the logging periodicity is configurable by NW, which is separate from the CSI resource periodicity. Therefore, UE shall also indicate the corresponding logging periodicity along with the logged data. The logging periodicity can be indicated by explicit value or by configuration id implicitly.

Proposal 1: (RRC-21) UE explicit indicates the absolute time along with first logged data after logging gap is detected. Additionally, for consecutive logged data, UE also indicates the corresponding CSI resource measurement logging periodicity along with the logged data (explicitly or implicitly).

Issue 25

This issue is about whether multiple configurations can be provided to UE and whether dynamic activation/deactivation is needed.

We understand the data would be use cases specific, e.g. BM and CSI, or even sub use case specific. The required CSI resource pattern or periodicity may be different for different use cases. For example, one data sample of spatial BM prediction includes measurement results of set A and B beam at the same time with the periodicity of 40 ms. While, another data sample of temporal BM prediction includes historic measurement results of set B beam and one measurement result of set A beam with the periodicity of 60 ms. It’s difficult to merge the required CSI resource into one data collection configuration. Furthermore, the logging event may not be the same for different use cases. For example, spatial BM prediction model may require data at cell edge, while the temporal BM prediction may require data at cell centre. If only one data collection configuration is allowed, NW can’t configure appropriate logging event. UE has to always collect data, which results in unnecessary power consumption.

Furthermore, more AI use cases are expected to be supported in future, e.g. AI mobility. It’s straightforward for NW to configure multiple data collection configuration for BM and mobility.

Proposal 2: (RRC-25) Multiple NW-side training data collection configurations can be configured to UE.

A subsequent question is whether dynamic activation/deactivation of data collection is supported. It’s always possible for NW to setup/release the data collection configuration by RRC. According to LS from RAN1 [2], it is observed that the latency requirement of training data collection is rather relaxed, which means the data collection configuration(s) will be valid for a very long time from network point of view. There is no need to introduce dynamic control.

Proposal 3: (RRC-25) L1/L2 signalling based dynamic activation/deactivation of NW-side training data collection is not supported.

Issue 28

This issue is about how to handle the logging and UAI configuration upon IDLE/INACTIVE/RLF. In general, we understand the logging and UAI configuration should be handled in the same way either both kept or released. If logging configuration is kept and UAI configuration is released, UE would log data but has no way to indicate the data availability. If logging configuration is released, there is no need to keep the UAI configuration, since no data would be logged any more.

Observation 1: Logging and UAI configuration should be handled in the same way upon IDLE/INACTIVE/RLF

Upon IDLE, we understand the configuration should be released. We don’t see the motivation to keep the configuration, since new serving cell is not aware of the stored configuration.

Upon INACTIVE, the configuration can be kept since the new serving cell can retrieve the configuration from anchor cell.

Upon RLF, in legacy, the UAI configuration is released. It’s not decided yet where to put the logged measurement configuration. The potential IE can be CSI-MeasConfig. However, CSI-MeasConfig would not be released upon RLF. Aligned handling should be ensure upon RLF. We prefer to release the configuration. NW can reconfigure UE after reestablishment if needed.

Proposal 4: (RRC-28) Logging and UAI configuration are released upon IDLE and RLF.

Proposal 5: (RRC-28) Logging and UAI configuration are kept upon INACTIVE.

Issue 29

This issue is about the relationship between data availability report and low power state. NW is expected to release the data collection configuration upon reception of low power indication from UE. The logged data would also be released if data collection configuration is released. NW may want to retrieve the available logged data before releasing the data collection configuration to avoid data lost. Therefore, UE shall report the data availability if there is any logged data when low power state is detected. The cause value can be set to low power.

Proposal 6: (RRC-29) UE reports data availability if there is any logged data when low power state is detected. The cause value of data availability report can be set to low power.

Issue 30

It’s not clear whether semi-persistent CSI resource can be supported for data collection. Semi-persistent CSI resource is supported for UE side data collection. We believe the current NW data collection framework can also support semi-persistent CSI resource without any enhancement.

Proposal 7: (RRC-30) Semi-persistent CSI resource is supported for data collection without enhancement.

Issue 31

This issue is about how to release the logged data. Following options are provided by rapporteur,

- Power off or deregistration

- 48 hrs after the release of logged measurement configuration

- Explicit indication from the serving gNB.

We agree to release the logged data upon power off or deregistration.

In MDT, the data would be released after 48 hours. Because UE may be in IDLE state and can’t report the logged MDT data. However, UE only supports AI data collection in CONNECTED state. NW shall be able to retrieve the data before releasing the logged measurement configuration. Therefore, logged data should also be released if logged measurement configuration is released. No need to wait for 48 hours. NW shall retrieve the logged data before release the logged measurement configuration. On the other hand, we see problems, if UE still kept data after logged measurement configuration is released. After measurement configuration release, NW may configure UE to collect data by different configuration less than 48 hours. If UE still keeps the old data, UE would report both newly collected data and old data. NW may not be able to differentiate the data. Therefore, UE shall release the logged data upon logged measurement configuration release.

We are unclear about the motivation for NW to indicate UE to release the logged data while keeping the logged measurement configuration. Such behaviour would result in data and UE power waste. Therefore, the explicit indication for data release is not needed.

Proposal 8: (RRC-31) UE releases logged data in following cases

- Power off or deregistration

- Release of logged measurement configuration

Issue 33

NW side data collection would consume UE storage and power. Similar as MDT, user consent shall be considered when NW decides to configure UE to perform NW side data collection. Currently, user consent is only about whether MDT is allowed as in SA5 spec [1]. It’s not clear whether user consent regrading NW side data collection can reuse the same user consent indication of MDT or separate user consent is introduced. We understand user may have different consents regarding MDT and NW side data collection. Because MDT and NW side data collection are two separate features. Furthermore, compared with MDT, NW side AI may have benefit on UE side from power saving point of view. We propose to introduce separate user consent regarding NW side data collection.

Proposal 9: (RRC-33) User consent regarding NW side data collection is separate from legacy MDT user consent.

Even if the user consent is separate, the framework of existing user consent regarding MDT can still be reused. But SA5 needs to define new user consent bit for NW data collection. Therefore, RAN2 should send LS to ask SA5 to introduce NW side data collection specific user consent.

Proposal 10: (RRC-33) Sent LS to ask SA5 to introduce NW side data collection specific user consent.

Issue 34

This issue is about whether the enhancements for NW-side data collection are per use case or common for all use cases. For R19 AI air, logged data only supports CSI measurement results. We don’t’ see the need to further differentiate the use cases. However, in future, other data may be logged, e.g. RRM measurement results for AI mobility. Maybe the data availability can be per use case. But we can discuss the need in AI mobility

Proposal 11: (RRC-34) Enhancements for NW-side data collection are common for all use cases in R19 except Pos.

Issue 36

This issue is about how data is forwarded to OAM or source gNB after HO. We understand it’s up to NW implementation whether the data should be forwarded to OAM or source gNB. UE doesn’t need to involved in the data forwarding. The spec impact is mainly on RAN3. We can send LS to RAN3 to trigger the discussion if needed.

Proposal 12: (RRC-36) There’s no RAN2 impact on how the data is forwarded to OAM or source gNB after handover.

Issue 39

This issue is about whether the logged data request/report should be use case specific. In R19, the variable only stores CSI measurement results. We don’t see the motivation to further differentiate the use case, e.g. temporal/spatial prediction. However, in future, more AI use cases would be supported, e.g. AI mobility. NW may configure UE to log data for BM and mobility at the same time. However, NW may only request UE to report logged data of specific use case. For example, NW may request UE to only report the BM logged data before handover, since AI model for BM may be cell specific. But UE can still retain mobility logged data during handover, since AI model for mobility may be applied among multiple cells. Therefore, the logged data request/report should be use case specific, i.e. BM or mobility, if other AI use cases are supported. We can discuss the enhancement in future.

Proposal 13: (RRC-39) Logged data request/report should common across BM and CSI prediction in R19.

Issue 49

This issue is about the content of logged data. RAN2 has agreed L1-RSRP and beam-ID should be collected. RAN1 hasn’t discussed the data content yet. We understand it’s efficient to support the model training. We can assume L1-RSRP and beam-ID as baseline. If RAN1 comes with new content, we can update accordingly. We don’t see the need to send LS since RAN1 would not be able to conclude soon.

Proposal 14: (RRC-49) It is sufficient to collect the L1-RSRP and/or beam ID as agreed by RAN2.

Issue 50

This issue is about suitability of data collection configuration. We don’t understand the intended scenario where the data collection configuration is not suitable. If ‘data collection configuration is not suitable’ means UE can’t comply the configuration, reconfiguration failure would occur. On the other hand, we understand NW should configure the data collection according to UE capability. The reconfiguration failure should not be a common case. Therefore, we don’t see the motivation to introduce an indication to handle the corner case.

Proposal 15: (RRC-50) Do not introduce an indication from the UE to NW about unsuitable data collection configurations in Rel-19

Issue 51

This issue is about how to handle the logged data upon inter-RAT handover. We understand the logged data should be discarded upon inter-RAT handover, since E-UTRA is not capable of retrieving the logged data. On the other hand, inter-RAT handover is not a common case. There is no strong need for UE to keep logged data and report the data after going back to NR again.

Proposal 16: (RRC-51) UE discards the logged data upon inter-RAT handover.

Issue 52

This issue is about how to handle logged data during CHO/LTM. We understand current solution can also apply to CHO/LTM.

Upon CHO execution, UE would apply the conRRCReconfig which contains RRCReconfiguration message during CHO execution.

condRRCReconfig-r16 OCTET STRING (CONTAINING RRCReconfiguration

Upon LTM execution, UE would apply the ltm-CandidateConfig which contains RRCReconfiguration message.

ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration)

Target cell can include the data release/retain indication in advance. UE would know whether to release/retain the data upon CHO/LTM execution.

Proposal 17: (RRC-52) Reuse data release/retain indication during CHO/LTM. No need for further enhancement.

Open issue UECap-1

During the capability email discussion, it’s agreed 64KB is mandatory supported as minimum memory for data logging. However, it’s still open whether UE can support other memory size and indicate via capability. We understand it’s possible that different UE chipset/vendor has different implementation. The supported storage size may be different. As indicated in [1], different types of training data may require different storage sizes. In the future, more AI use cases would be supported, e.g. AI mobility. To support data collection for more use cases, larger memory would be required. UE can report the supported data logging storage size to NW. NW can decide which data is to be logged by UE and the appropriate configuration, e.g. report period. The reported logging storage size can be used across all use cases. The potential buffer size can be 128KB.

Proposal 18: (UECap-1) UE can support different data logging AS buffer- size across all use cases, which can be indicated by UE capability. The potential AS buffer size can be 128KB.

Issue 43

This issue is about the buffer threshold value. The intention of buffer threshold trigger report is to indicate NW before buffer is full. It’s agreed the minimum AS buffer size is 64KB. We don’t see the motivation to define the small value as threshold value, e.g. 1KB, 2KB…52KB. The small value would result in frequent report and AS buffer waste.

Observation 2: Small buffer threshold value would result in frequent report and AS buffer waste.

Also, how to determine the buffer threshold reached is left to UE implementation. We understand one or two large values are enough. If UE supports 64KB buffer size, 60KB can be used as threshold value. Furthermore, UE may support larger buffer size, e.g. 128KB. 120KB can be used as threshold value in this case.

Proposal 19: (RRC-43) 60KB as buffer threshold value is considered when UE has a minimum 64kB AS buffer size. Other threshold values depending on the AS buffer size capability supported by UE.

Open issue UECap-2

It’s still FFS whether FFS whether the following components are part of logged data collection capability or separate capabilities:

- periodic logging

- event-based logging

- measurement quantities and resources (Set A/B, maximum resources of Set A/B, etc)

- provide assistance information (e.g., data is available, reason for trigger, low power indication) for NW-side data collection

For the periodic logging, it’s the basic capability to log data. Data logging event is the same as measurement report event. We don’t see much complexity to support event-based logging. Therefore, we believe periodic logging and event-based logging are essential capability to support logged data and can be part of the logged data collection capability.

For the measurement quantities and resource, we understand the motivation to define separate capability is to differentiate the UE supporting BM and other use cases, e.g. AI mobility. We agree the UE may only support data collection for BM or mobility or both. Since the logged data collection capability is common to all use cases, it’s necessary to define separate capability to indicate the measurement quantities UE can log.

For the assistance information, we understand the assistance information is essential to support the data collection other than the buffer threshold trigger.

Proposal 20: (UECap-2) Periodic and event-based logging are part of basic logged data collection capability.

Proposal 21: (UECap-2) Define separate capability to indicate the supported measurement quantities for logged data collection.

Proposal 22: (UECap-2) Assistance information for NW side data collection is part of basic logged data collection capability.

Logging configuration framework

After email discussion, rapporteur propose to discuss and decide on alternative to introduce logging configuration for network data collection:

a.	logging configuration is introduced as a new list of configurations under CSI-MeasConfig,

b.	logging configuration is introduced in a new L3 measurement framework, at the same level as MeasConfig and CellGroupConfig

We understand both options are technically feasible. But we have following concerns on option a, i.e. under CSI-MeasConfig,

- RAN1 may need to change the UE behaviour regarding the CSI-MeasConfig, which requires additional RAN1 work. RAN1 may not be able to finish the work considering the limited time.

- Not forward compatible for AI mobility data collection, since AI mobility data is based on L3 measurement results

The impact to RAN3 is similar for both options due to CU-DU split. However, RAN3 has not considered CU-DU split yet.

Therefore, we would prefer option b, i.e. logging configuration is introduced in a new L3 measurement framework.

Proposal 23: Logging configuration is introduced in a new L3 measurement framework.

Conclusion

Based on the discussion in section 2, we have following proposals:

Proposal 1: (RRC-21) UE explicit indicates the absolute time along with first logged data after logging gap is detected. Additionally, for consecutive logged data, UE also indicates the corresponding CSI resource measurement logging periodicity along with the logged data (explicitly or implicitly).

Proposal 2: (RRC-25) Multiple NW-side training data collection configurations can be configured to UE.

Proposal 3: (RRC-25) L1/L2 signalling based dynamic activation/deactivation of NW-side training data collection is not supported.

Observation 1: Logging and UAI configuration should be handled in the same way upon IDLE/INACTIVE/RLF

Proposal 4: (RRC-28) Logging and UAI configuration are released upon IDLE and RLF.

Proposal 5: (RRC-28) Logging and UAI configuration are kept upon INACTIVE.

Proposal 6: (RRC-29) UE reports data availability if there is any logged data when low power state is detected. The cause value of data availability report can be set to low power.

Proposal 7: (RRC-30) Semi-persistent CSI resource is supported for data collection without enhancement.

Proposal 8: (RRC-31) UE releases logged data in following cases

- Power off or deregistration

- Release of logged measurement configuration

Proposal 9: (RRC-33) User consent regarding NW side data collection is separate from legacy MDT user consent.

Proposal 10: (RRC-33) Sent LS to ask SA5 to introduce NW side data collection specific user consent.

Proposal 11: (RRC-34) Enhancements for NW-side data collection are common for all use cases in R19 except Pos.

Proposal 12: (RRC-36) There’s no RAN2 impact on how the data is forwarded to OAM or source gNB after handover.

Proposal 13: (RRC-39) Logged data request/report should common across BM and CSI prediction in R19.

Proposal 14: (RRC-49) It is sufficient to collect the L1-RSRP and/or beam ID as agreed by RAN2.

Proposal 15: (RRC-50) Do not introduce an indication from the UE to NW about unsuitable data collection configurations in Rel-19

Proposal 16: (RRC-51) UE discards the logged data upon inter-RAT handover.

Proposal 17: (RRC-52) Reuse data release/retain indication during CHO/LTM. No need for further enhancement.

Proposal 18: (UECap-1) UE can support different data logging AS buffer- size across all use cases, which can be indicated by UE capability. The potential AS buffer size can be 128KB.

Observation 2: Small buffer threshold value would result in frequent report and AS buffer waste.

Proposal 19: (RRC-43) 60KB as buffer threshold value is considered when UE has a minimum 64kB AS buffer size. Other threshold values depending on the AS buffer size capability supported by UE.

Proposal 20: (UECap-2) Periodic and event-based logging are part of basic logged data collection capability.

Proposal 21: (UECap-2) Define separate capability to indicate the supported measurement quantities for logged data collection.

Proposal 22: (UECap-2) Assistance information for NW side data collection is part of basic logged data collection capability.

Proposal 23: Logging configuration is introduced in a new L3 measurement framework.

Reference

[1] R2-2311720	Reply LS on Data Collection Requirements and Assumptions