3GPP TSG-RAN WG2 Meeting #131	                                          R2-2505151

Bengaluru, India, 25 – 29, Aug, 2025

Source: 	Xiaomi

Title:  	Discussion on functionality management for RRM prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

Introduction

In AI mobility, AI can be used to predict the RRM measurement result without actual measurement. In this discussion, we discuss inference solution and functionality management for UE sided model RRM prediction.

Discussion

Inference solution for UE side model

First, we would like to discuss the general assumption of the AI mobility solution, i.e. how to use the inference results in mobility procedure. The model input and output may be different for use cases. We will discuss the spec impact per use case.

In last meeting, following scope is agreed for further study,

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes , all sub cases | Yes |
| Frequency domain | Yes , all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

Temporal domain case B

UE can obtain the L3 cell level measurement result by AI prediction on skipped instances. The predicted L3 cell level measurement result can replace the skipped L3 cell level measurement results in all legacy procedures, e.g. measurement event evaluation, S criteria evaluation. If measurement report is triggered, UE is required to report the latest measurement results. When measurement report is triggered, the latest measurement result can either be predicted or measured results. Meanwhile, UE will indicate whether the reported measurement result is predicted or measured to assist NW decision.

Proposal 1: In temporal domain case B, UE can use predicted L3 cell level measurement result on skipped instances instead of measured results in legacy procedures (e.g. measurement event evaluation, S criteria evaluation, measurement report, etc).

Temporal domain case A

In UE sided model, UE can also perform inference to predict future L3 cell level measurement result on temporal domain. We understand the inference result can be used in the following ways,

- UE reports the predicted results to NW. It’s up to NW to decide how to use the predicted results. The report shall indicate the predicted measurement results, cell ID and timing.

- UE can evaluate the measurement event based on the predicted results. This is the indirect event prediction. We will discuss this in next clause.

Proposal 2: UE can report the predicted L3 cell level measurement results, which includes predicted measurement results, cell ID and timing. It’s up to NW to decide how to use the predicted results in Temporal domain case A.

During the simulation, multiple outputs were provided, including

- predicted measurement result on each timing instance

- average measurement results over PW

- measurement result on last timing instance

These outputs can also be considered as prediction result and reported to NW. However, NW may only be interested in the predicted measurement results which fulfils certain condition. For example, when the measurement result is predicted to be higher or lower than a threshold. It is more efficient for UE to only report the measurement result which fufills condition(s) configured by NW. Therefore, RAN2 can further discuss which inference result is reported to NW in temporal domain case A

Proposal 3: RAN2 discuss which inference result is reported to NW within PW in temporal domain case A:

- predicted measurement result on each timing instance

- average measurement results over PW

- measurement result on last timing instance

- measurement result fulfilling certain condition, e.g. higher/lower than certain threshold, and corresponding timing instance

Frequency domain prediction

In UE sided model, UE can perform inference to predict L3 cell level measurement result on frequency domain. We understand the inference result can be used in following ways,

- UE reports the predicted results to NW. It’s up to NW to decide how to use the predicted results. The report shall indicate the predicted measurement results, cell ID and frequency.

- UE can evaluate the measurement event based on the predicted results without measurement gap. This event prediction is done in frequency domain and not in temporal domain. This prediction can be used to predict inter-frequency measurement event, e.g. A5. In following fig, UE’s serving cell is on F1 and target cell is on F2. UE is able to evaluate the target cell fulfilling A5 event without actually measuring on F2. UE can trigger the measurement report based on predicted event.

Proposal 4: In frequency domain prediction, UE can report following predicted results to NW. It’s up to NW to decide how to use the predicted results.

- the predicted L3 cell level measurement results, which includes predicted measurement results, cell ID and frequency

- the fulfilled event which is evaluated based on predicted L3 cell level measurement results on frequency domain

Spatial domain prediction

During the simulation phase, spatial domain prediction is not high priority and only limited simulation results are provided. RAN2 didn’t reach any consensus on the feasibility and prediction accuracy of spatial domain prediction. Furthermore, in AI air study, RAN1 only studied intra-cell Beam prediction. The feasibility of inter-cell spatial domain prediction is even more doubtful. Therefore, we suggest consider spatial domain prediction in low priority. We can first discuss the spec impact on other scenarios.

Observation 1: Only intra-cell spatial domain prediction feasibility has been evaluated by RAN1 for AI air BM. There’s no consensus on whether inter-cell spatial domain prediction is feasible without simulation evaluation.

Proposal 5: Spatial domain prediction is considered as a low priority use case.

L3 beam prediction

RAN2 has agreed L3 beam level prediction specification impact will be studied.

For beam-level measurement prediction, the following agreements have been reached in RAN2 #127:

RAN2#127: Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

For the L3 beam level measurement prediction, two open issues can be discussed:

Issue 1: Whether to consider all sub use cases for UE sided model.

In the cell level measurement prediction, for UE sided mode, all sub cases are supported. Similar to cell level prediction, the UE can acquire both L1 and L3 beam level measurement results based on measurements. Hence all sub cases can be considered for L3 beam level measurement prediction.

Proposal 6: For L3 beam level measurement prediction, all the following sub cases can be considered,

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result(s) is generated

- Sub-use case 2: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

- Sub-use case 3: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Issue 2: Whether to consider temporal domain, frequency domain and spatial domain L3 beam level measurement prediction.

Temporal domain

As discussed in the above section, the predicted L3 beam result(s) can be used in measurement report if configured by the network.

For cell level measurement prediction, both intra-frequency temporal domain case A and case B are supported.

For case A, to improve handover performance, the predicted future L3 beam result(s) can also be included in the MR, if configured by the network, as shown in Figure 1.

Figure 1

Recalling that the goal of case B is to reduce measurement efforts in temporal domain by using predicted measurements, for case B, the L3 beam-level measurement prediction can also be used to predicted the latest L3 beam result(s) for MR when the MR is triggered in the PW of case B, as shown in Figure 2. On the other hand, if the MR is triggered in the OW of case B, the L3 beam-level measurement prediction is not needed.

Figure 2

Based on the above analysis, the temporal domain L3 beam-level measurement prediction can be considered, where future or latest L3 beam-level measurement result(s) can be predicted.

Proposal 7: The temporal domain L3 beam-level measurement prediction can be considered, where future or latest L3 beam-level measurement result(s) can be predicted.

Frequency domain

For frequency domain L3 beam level measurement prediction, it is inter-frequency inter-cell inter-beam prediction. So far, RAN2 has not done any simulation results for the case, and we may not have enough time to do simulation for this case in Rel-19. Hence, we suggest the frequency domain L3 beam-level measurement prediction is not considered in Rel-19 SI.

Proposal 8: The frequency domain L3 beam-level measurement prediction is not considered in Rel-19 SI.

Spatial domain

As discussed above, L3 beam level measurement prediction should also be deprioritized, following spatial domain cell level measurement prediction.

If RAN2 wants to study the spatial domain L3 beam-level measurement prediction, we can focus on intra-frequency intra-cell spatial domain prediction.

Proposal 9: The spatial domain L3 beam-level measurement prediction is low priority.

Functionality management

Applicability management

In AI air, associated ID is introduced to reach consistency between training and inference. Cell configuration can be mapped to associated ID. In last meeting, it’s agreed associated ID can be configured in AI mobility. In AI mobility, AI functionality can be used to predict measurement result of serving cell and neighbour cell. Therefore, NW shall provide neighbour cell’s associated ID, if the prediction input/output involves neighbour cell. The neighbour cell’s associated ID can be provided to UE via serving cell, which may require NW inter-node coordination. We can further discuss how to provide neighbour cell’s associated ID in WI.

Proposal 10: Neighbour cell’s associated ID can be provided to UE.

However, large signalling may be required to provide all neighbour cells’ associated ID, especially in inter-frequency case. Note the cell configuration considered for generalization in AI mobility is Uma or Umi. The cell configuration may be common applicable per frequency. The cell on the same frequency may be expected to be deployed with the same cell configuration. It may be more efficient to provide the associated ID per frequency. The signalling overhead can be significantly reduced.

Proposal 11: The associated ID can be applicable and provided per frequency.

Inference control

It’s FFS when the UE can perform inference in AI mobility. Unlike BM, for case A/event prediction, the inference is expected to enhance handover, which occurs at cell edge. The inference result may be useless in cell centre and consumes UE power. However, NW may provide the inference configuration before UE moves to cell edge. Therefore, UE shall not be required to perform inference immediately after receiving the inference configuration. Alternatively, the UE can perform inference only when it’s about to perform handover at cell edge. UE can determine whether it’s at cell edge based on event (A2, A3, A5 alike event), which can be configured by NW.

Proposal 12: The UE can perform the inference of applicable functionality if event (A2, A3, A5 alike event) fulfilled, which is configured by NW

Conclusions

According to the analysis given above, we have the following observations and proposals:

Temporal domain case B

Proposal 1: In temporal domain case B, UE can use predicted L3 cell level measurement result on skipped instances instead of measured results in legacy procedures (e.g. measurement event evaluation, S criteria evaluation, measurement report, etc).

Temporal domain case A

Proposal 2: UE can report the predicted L3 cell level measurement results, which includes predicted measurement results, cell ID and timing. It’s up to NW to decide how to use the predicted results in Temporal domain case A.

Proposal 3: RAN2 discuss which inference result is reported to NW within PW in temporal domain case A:

- predicted measurement result on each timing instance

- average measurement results over PW

- measurement result on last timing instance

- measurement result fulfilling certain condition, e.g. higher/lower than certain threshold, and corresponding timing instance

Frequency domain prediction

Proposal 4: In frequency domain prediction, UE can report following predicted results to NW. It’s up to NW to decide how to use the predicted results.

- the predicted L3 cell level measurement results, which includes predicted measurement results, cell ID and frequency

- the fulfilled event which is evaluated based on predicted L3 cell level measurement results on frequency domain

Spatial domain prediction

Observation 1: Only intra-cell spatial domain prediction feasibility has been evaluated by RAN1 for AI air BM. There’s no consensus on whether inter-cell spatial domain prediction is feasible without simulation evaluation.

Proposal 5: Spatial domain prediction is considered as a low priority use case.

L3 beam prediction

Proposal 6: For L3 beam level measurement prediction, all the following sub cases can be considered,

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result(s) is generated

- Sub-use case 2: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

- Sub-use case 3: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Proposal 7: The temporal domain L3 beam-level measurement prediction can be considered, where future or latest L3 beam-level measurement result(s) can be predicted.

Proposal 8: The frequency domain L3 beam-level measurement prediction is not considered in Rel-19 SI.

Proposal 9: The spatial domain L3 beam-level measurement prediction is low priority.

Functionality applicability management

Proposal 10: Neighbour cell’s associated ID can be provided to UE.

Proposal 11: The associated ID can be applicable and provided per frequency.

Inference control

Proposal 12: The UE can perform the inference of applicable functionality if event (A2, A3, A5 alike event) fulfilled, which is configured by NW

Reference

[1] TR 38.843

[2] RP-242393 SID of AI/ML mobility