3GPP TSG-RAN WG2 Meeting #131	                                                                                   R2-2505167

Bangalore, India, 25th – 30th August, 2025

Source:	CATT

Title:	Discussion on Conditional Intra-CU LTM

Agenda Item:    8.6.4

Document for:  Discussion and Decision

Introduction

This paper further discusses the remaining issues on the conditional intra-CU LTM [1]. In detail, the following issues are covered by our discussion,

- NW triggered LTM to the CLTM candidate with early TA

- Power ramping for CLTM candidate with two-TA

- SN gap due to CLTM fast recovery

The detailed discussion is provided in Section 2, and followed by Section 3 that summarizes the contribution of this paper.

Discussion

NW triggered LTM to the CLTM candidate with early TA

The following issue was discussed in last meeting but currently it still remains to be open,

| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received. [Rapp]: New added based on Anil’s comments. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

The intention is to further clarify whether UE performs RACH-based LTM or RACH-less LTM to the target cell if the target cell is a CLTM candidate associated with running TAT.

As captured in the current MAC running CR, the TA field of the (enhanced) LTM cell switch command MAC CE is mandatory present with a valid TA value or FFF to indicate TA field is not valid. From this perspective, it can be the NW implementation to include the TA value in the LTM command MAC CE to trigger the RACH-less access to the target cell.

If the UE is allowed to further check the running of the target cell TAT to determine the RACH-less access to the target cell even if the LTM command includes the FFF value, it will cause many other potential clarifications by this enhancement. For example, how to achieve the NW intention to trigger the CBRA RACH-based LTM to the target cell by not including the valid TA in the LTM command but the UE indeed has a running TAT associated with that target cell. In this situation, the UE behaviour is not aligned with the NW intention.

In summary, we suggest not making enhancement on this issue.

Proposal 1 (MAC-14): Upon receiving (Enhanced) LTM Cell switch Command MAC CE, UE performs RACH-less LTM cell switch only if the MAC CE includes a valid TA value or UE has performed UE based TA measurement to the target cell, regardless of whether CLTM candidate TAT timer is running for the target cell.

Power ramping for CLTM candidate with two-TA

As per the current MAC running CR, the rapporteur captures the following Editor’ Note,

| MAC-26 | How to consider the TAG ID when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1 for an (C)LTM candidate cell which is configured with two TAGs in RACH-less (C)LTM_ Editor’s NOTE: In RACH-less (C)LTM, for an (C)LTM candidate cell which is configured with two TAGs, TAG ID is not considered when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1. Rapporteur realized the same issue exists for Rel-18 LTM, which was missing. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

The intention is to clarify how to perform power ramping upon early RACH to the candidate cell for CLTM.

In Rel-18, according to the current MAC spec,

### 5.1.1 Random Access procedure initialization When the Random Access procedure is initiated on a Serving Cell or for an LTM candidate cell, the MAC entity shall: 1> flush the Msg3 buffer; 1> flush the MSGA buffer; 1> set the PREAMBLE_TRANSMISSION_COUNTER to 1; 1> if the Random Access procedure is initiated on a Serving Cell; or 1> if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell and the PDCCH order indicates preamble initial transmission; or 1> if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell, which is different from the cell to which the UE performed the last Random Access Preamble transmission, and the PDCCH order indicates preamble re-transmission: 2> set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

When the UE is indicated to perform preamble re-transmission to the same candidate cell consecutively, it will perform power ramping without setting the preamble counter to 1. Note that the power ramping does not consider the same TAG if the candidate cell is configured with two-TA. This is because in Rel-18, the NW maintains the early TA for a candidate cell. Then it’s up to NW implementation to trigger the preamble re-transmission to the same TRP to acquire the early TA for that TRP.

When considering Rel-19 CLTM, UE could maintain the validation for early TA, and the NW can configure the separated TATs for UE to monitor the early TA associated with the corresponding TRP. That means, the current spec does not take 2-TA into consideration to prevent the NW to do early TA to two TRPs if a candidate cell is configured with two-TA.

The problem exists if the NW triggers PDCCH order to one TRP without successfully acquiring the valid TA, and then triggers another PDCCH order to another TRP. But the UE couldn’t distinguish the association of the SSB and the TAG ID, so the UE couldn’t distinguish which TAG the PDCCH order is intended to. Considering the limited time, we don’t think it is a good idea to introduce the association of TAG and SSB or to introduce TAG indication in the PDCCH order, which may involve RAN3 or RAN1. Besides, only one TA is needed when the UE perform CLTM execution, we think it is not a valid case to acquire two early TA in the same time. So it could up to network to solve the problem, i.e., if the network triggers PDCCH order to acquire early TA for one TRP, and if it wants to trigger PDCCH order for early TA for another TRP, it should indicate preamble initial transmission.

Observation 1: UE should set the PREAMBLE_POWER_RAMPING_COUNTER to 1 when initiating Random Access procedure for early TA acquisition on another TRP of the same candidate cell while the early TA acquisition of one TRP is ongoing. This can be ensured through NW configuration.

Proposal 2 (MAC-26): When network triggers PDCCH order to another TRP of the same candidate cell while the early TA acquisition of one TRP is ongoing, it should indicate preamble initial transmission in the PDCCH order.

SN gap due to CLTM fast recovery

In Rel-18, fast recovery may happen for the NW triggered LTM, and it’s up to NW implementation to handle the SN gap to stop the PDCP re-ordering timer. This avoids the fast recovery failure. The NW can be aware of the fast recovery because if the UE accesses to the candidate cell, e.g. cell_A, due to the LTM cell switch command, the source cell will inform the candidate cell_A. Then the PDCP entity of the NW could stop the re-ordering timer to provide the RRCReconfigurationComplete message to RRC.

A NOTE is captured in 38.300 as follows,

NOTE: PDCP SN gap for SRB may exist upon LTM attempt toward the selected cell after LTM fails. It is up to network implementation to avoid the latency caused by the PDCP SN gap.

PDCP SN gap issue exists in CLTM fast recovery as well. The question is whether it is feasible to up to network implementation to avoid the latency caused by the PDCP SN gap during the CLTM fast recovery. We need to clarify how the network can determine a CLTM fast recovery, given that the CLTM execution and CLTM fast recovery are initiated by UE. For example, UE intends to access to cell_A due to satisfying the corresponding execution condition. Then UE suffers handover failure and performs fast recovery to cell_B. In this case, the key point is whether the NW can be aware that the UE accessing to cell_B is for CLTM fast recovery. From our view, this can be done from the NW side. Upon receiving the C-RNTI MAC CE during CBRA, the NW knows this UE is configured with this cell as LTM candidate cell. And the first signaling transmitted on SRB1 should be RRCReconfigurationComplete. Hence when the PDCP entity for SRB1 receives the first PDU for one UE which configured this cell as candidate CLTM cell, if SN gap exists, it can know the cell switch is due to fast recovery, and so the PDCP entity of the NW can stop the re-ordering timer and then submit the RRCReconfigurationComplete message to RRC. So from our view, there is no further optimization for this scenario.

Observation 2: NW could perform contention resolution based on C-RNTI MAC CE in Msg3. When PDCP SN gap happens, NW can determine a CLTM fast recovery according to the C-RNTI MAC CE in msg3 and that the target cell is configured as a CLTM candidate. Then the NW can stop the re-ordering timer to avoid the latency.

Proposal 3: It’s up to NW implementation to avoid the latency caused by the PDCP SN gap due to CLTM fast recovery.

Conclusion

Based on the analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: UE should set the PREAMBLE_POWER_RAMPING_COUNTER to 1 when initiating Random Access procedure for early TA acquisition on another TRP of the same candidate cell while the early TA acquisition of one TRP is ongoing. This can be ensured through NW configuration.

Observation 2: NW could perform contention resolution based on C-RNTI MAC CE in Msg3. When PDCP SN gap happens, NW can determine a CLTM fast recovery according to the C-RNTI MAC CE in msg3 and that the target cell is configured as a CLTM candidate. Then the NW can stop the re-ordering timer to avoid the latency.

NW triggered LTM to the CLTM candidate with early TA

Proposal 1 (MAC-14): Upon receiving (Enhanced) LTM Cell switch Command MAC CE, UE performs RACH-less LTM cell switch only if the MAC CE includes a valid TA value or UE has performed UE based TA measurement to the target cell, regardless of whether CLTM candidate TAT timer is running for the target cell.

Power ramping for CLTM candidate with two-TA

Proposal 2 (MAC-26): When network triggers PDCCH order to another TRP of the same candidate cell while the early TA acquisition of one TRP is ongoing, it should indicate preamble initial transmission in the PDCCH order.

SN gap due to CLTM fast recovery

Proposal 3: It’s up to NW implementation to avoid the latency caused by the PDCP SN gap due to CLTM fast recovery.

Reference

[1] Draft_RAN2_130_Meeting_Report_v3