3GPP TSG-RAN WG2 Meeting #131	R2-2505189

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.3.3

Source:	OPPO

Title:	Discussion on LCM procedure of network sided model

Document for:	Discussion, Decision

Introduction

At RAN2#129bis meeting, RAN2 agreed following study scope on spec impact:

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes , all sub cases | Yes |
| Frequency domain | Yes , all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

Table 1

Then in RAN2#130 meeting RAN2 agreed that:

Agreements

1	The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts.     Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s).

2	The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

There is FFS whether sub-case 1 and sub-case 3 should be supported during the discussion in RAN2#130 meeting. In this paper we will discuss the scope of network sided model again. In addition, data collection, inference input and configuration and performance monitoring are also discussed.

Discussion

Scope of network sided model

For network sided model, here is basic flow chart:

Figure 2.1-1

First gNB need configure UE with a measurement task. UE then reports measurement results to gNB after performing measurement, based which gNB will perform inference. Regarding signaling over air, there are several combinations between RRM sub-cases and different scenarios as illustrated in Table 2.1-1:

| Inference input to model | Temporal domain case A | Temporal domain case B | Inter-frequency prediction | Spatial domain prediction |
| --- | --- | --- | --- | --- |
| RRM sub-case 1/3 | L1 beam level results of cell(s) without skipping | L1 beam level results of cell(s) with skipping in temporal domain | L1 beam level results of cell(s) without skipping of measured frequency | Partial L1 beam level results of cell(s) without skipping |
| RRM sub-case 2 | L3 cell level results of cell(s) without skipping | L3 cell level results of cell(s) with skipping in temporal domain | L3 cell level results of cell(s) without skipping of measured frequency | N/A |

Table 2.1-1: Combinations of sub-cases and prediction in different domain for cell level L3 RSRP

From Table 2.1-1, it is clear what UE should report to gNB as inference input is either L1 beam level or L3 cell level measurement result periodically. The essential difference between L1 beam level and L3 cell level measurement result in terms of signaling overhead is that there are more than one (up to 96) beams per cell could be reported in theory.

Observation 1: Reporting L1 beam level measurement result will increase signaling overhead significantly compared to L3 cell level measurement result significantly.

For temporal domain case B, if UE can already support it there is no additional benefit for network to support it at all. Some company argue that UE may not support it. However, it should be noted that periodical measurement report is necessary to support inference operation in network side, which will demand more transmission power. And thus, obviously defeat the very purpose of temporal domain case B. This problem of scenario B is more serious for sub-case 1 and 3 based on observation 1. Network can do it by implementation. For example, if network wants measurement results by configuration SMTC period1, it can configure a SMTC period 2 where SMTC period 2 is larger than SMTC period 1. And gNB can have more measurement results by measurement prediction. Having said that, we think it is not necessary to introduce any spec impact for temporal domain case B.

Proposal 1: No spec impact is introduced for supporting temporal domain case B

For inter-frequency prediction there is no observation is concluded during the study that sub case 1/3 is better than sub case 2. If sub-case 2 is supported for inter-frequency prediction it is not clear why sub case 1/3 are still required. As UE vendor our concern on sub-case 1/3 is signaling overhead and additional spec impact in UE.

In RAN2#130 meeting, RAN2 achieved following agreements:

Agreements

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used.   For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

- To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

Table 2.1-2

RAN2 confirmed that the report of L1 filtered beam-level measurement can be supported by legacy measurementReport message. One comprising way is that RAN2 confirm that sub-case 1 and 3 are supported but without spec impact for inference configuration and report, performance monitoring etc. However, to enable offline training of the model, they can be considered in data collection. It means the only potential spec impact is that data collection for sub case 1 and 3. The situation is similar for temporal domain case A and spatial domain prediction.

Proposal 2: No spec impact on inference configuration/report and performance monitoring is introduced for supporting sub-case 1/3.

Proposal 3: Data collection for sub case 1/3 can be considered.

Proposal 4: Proposal 2 and 3 are valid for spatial domain prediction, inter-frequency prediction and temporal domain case A

For L3 beam level prediction, there is similar concern on signaling overhead and utility in network side. And we think the same approach can be agreeable. Hence, we propose:

Proposal 5: No spec impact on inference configuration/report and performance monitoring is introduced for L3 beam level prediction for all sub-cases (4,5,6)

Proposal 6: Data collection for L3 beam level prediction can be considered.

Inference input configuration and report

There maybe spec impact of sub-case2 for inter-frequency prediction and temporal domain case A. For sub case 2, only L3 cell level measurement results need be reported. So, for inter-frequency domain prediction, the prediction mainly relies on frequency correlation between two frequency carrier. The number of measurement results in temporal domain doesn’t really matter. But for temporal domain case A, more measurement result as inference input could help improve the performance. That’s why we suggest that the agreement achieved at RAN2#130 meeting need updated as following:

Proposal 7: To support sub-case 2 of temporal domain case A, one enhancement is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model

Proposal 8: To support sub-case 2 of inter-frequency prediction, existing measurementReport message is used i.e. no spec impact is introduced for input of inference.

Data collection

In RAN2#130 meeting, RAN2 made big progress on data collection issues. There are some left open issues, which either depends on other aspect e.g. the support of sub-case 1 and sub-case 3, or similar issues are being discussed in AIML_over_air item.

Some open issues being discussed in [POST130][026][AI PHY] 38.331 CR (Ericsson). For AI mobility item we should wait for further progress since they can be either easily borrowed or stage 3 detail.

One conclusion about UE assistant information can be digested. Following text is cited from [POST130][026][AI PHY] 38.331 CR (Ericsson):

- Open issue RRC-20: Further procedures for UE assistance information related to logging

Issue description: It has not yet been discussed whether further procedures for UE reporting assistance information related to logging is need. Such further procedures may be, e.g. prohibit timers, indication that battery state is not low any longer, indication that the memory is not full any longer, etc.

The issue is captured as an editor’s note in the running CR, in clause 5.7.4.2 and 5.7.4.3.

Proposed resolution: This issue was resolved by the RAN2#130 agreements:

3 UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state 4 No additional signaling from the UE is required when the low power issue is resolved 5 No additional signaling from the UE is required when the buffer full issue is resolved

We believe those 3 agreements can be adopted also for AI mobility.

Proposal 9: For AI mobility, the following agreements also applies:

UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state

No additional signaling from the UE is required when the low power issue is resolved

No additional signaling from the UE is required when the buffer full issue is resolved

In simulation, we assume that the measurement results of two different frequencies can be obtained simultaneously. However, measuring two different frequencies introduces delays due to the need to switch between frequencies. Currently, it is up to UE to determine the exact timing for measure within a measurement period. If the time gap between the logged measurements of two different frequencies is large. The correlation between the two measurement results will decrease. This will impair the performance of AI/ML. To make sure the correlation between measured results of different frequencies does not degrade much, it is proposed that

Proposal 10: To ensure spatial and temporal correlation between samples, a maximum allowable time interval between measurements of two correlated frequencies is configured for inter-frequency prediction data collection.

Performance monitoring

Following agreement is achieved on performance monitoring in RAN2#130 meeting:

- RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case

- For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement

- FFS on UE awareness and preference for NW sided model

Some company raise user consent issue for data collection in AIML_over_air [1]. Our view is that RAN2 doesn’t touch the yellow open issue till there is some progress is made in AIML_over_air.

Proposal 11: No use consent issue is discussed in AI mobility study still some progress is made in AIML_over_air.

Conclusion

Observation 1: Reporting L1 beam level measurement result will increase signaling overhead significantly compared to L3 cell level measurement result significantly.

Proposal 1: No spec impact is introduced for supporting temporal domain case B

Proposal 2: No spec impact on inference configuration/report and performance monitoring is introduced for supporting sub-case 1/3.

Proposal 3: Data collection for sub case 1/3 can be considered.

Proposal 4: Proposal 2 and 3 are valid for spatial domain prediction, inter-frequency prediction and temporal domain case A

Proposal 5: No spec impact on inference configuration/report and performance monitoring is introduced for L3 beam level prediction for all sub-cases (4,5,6)

Proposal 6: Data collection for L3 beam level prediction can be considered.

Proposal 7: To support sub-case 2 of temporal domain case A, one enhancement is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model

Proposal 8: To support sub-case 2 of inter-frequency prediction, existing measurementReport message is used i.e. no spec impact is introduced for input of inference.

Proposal 9: For AI mobility, the following agreements also applies:

UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state

No additional signaling from the UE is required when the low power issue is resolved

No additional signaling from the UE is required when the buffer full issue is resolved

Proposal 10: To ensure spatial and temporal correlation between samples, a maximum allowable time interval between measurements of two correlated frequencies is configured for inter-frequency prediction data collection.

Proposal 11: No use consent issue is discussed in AI mobility study still some progress is made in AIML_over_air.

Reference

[1] R2-2503526 User consent for NW-side data collection, NTT DOCOMO, Apple, vivo, Xiaomi