3GPP TSG-RAN WG2 Meeting #131	R2-2505190

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.3.3

Source:	KT Corporation

Title:	Discussion on AI/ML performance monitoring for network-sided model

WID/SID:	FS_NR_AIML_Mob (Release-19)

Document for:	Discussion and Decision

Introduction

In the RAN WG2 #130 meeting [1], it was agreed that UE would not be involved in network-sided AI/ML model management decisions, however UE’s provision of actual measurement data for performance monitoring is required. The issue of whether UE can be aware of and express preferences for network-sided model operations is FFS.

Agreements on performance monitoring 1 RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case 2 For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement 3 FFS on UE awareness and preference for NW sided model

This paper discusses UE awareness and preference for performance monitoring on the network-sided model.

Discussion

2.1	Motivations

In network-sided AI/ML architectures, the training and inference of models are executed entirely at the network side. The UE typically behaves transparently, unaware of whether AI/ML is being used, and merely performs measurement and reporting as instructed by the network.

However, to ensure the trustworthiness and efficiency of AI/ML-driven network functions, such as beam management and mobility optimization [2-3], the network should continuously monitor the performance of the deployed models. This requires measurement data from UEs for both inference and monitoring of the NW-sided model, as shown in Figure 1.

In addition, since various types of UEs such as IoT devices, robots, drones, NTN-enabled devices other than smartphones will be served, it can sometimes be difficult for them to properly handle the data collection for performance monitoring.

While current specifications allow the reuse of legacy measurement configuration procedures for data collection, they do not support any mechanism for the UE to recognize the intent of the configuration or provide feedback regarding its willingness or capability to comply. This may lead to inefficient or even failed data collection under certain UE-side constraints.

Figure 1. Performance monitoring for network-sided model

2.2	Issues

AI/ML-related measurement or data collection configurations have so far been designed primarily for model training or inference purposes. However, when the same configurations are reused for model performance monitoring (i.e., label data collection), the following issues arise:

- The UE is not informed whether the configuration is intended for performance monitoring or model inference.

- The UE may experience resource constraints (e.g., battery drain, processor overload, UE with limited capability), unfavorable radio conditions (e.g., weak signal, high interference), or its data privacy policy.

- The UE has no means to express preference or objection, potentially leading to degraded device behavior or poor data quality.

Observation 1: Performance monitoring can be impacted from the UE’s resource constraints (e.g., battery drain, processor overload, UE with limited capability), unfavorable radio conditions (e.g., weak signal, high interference), or its data privacy policy.

Observation 2: In the absence of mechanisms for UE to express resource constraints, preferences, or feedback, the network may apply suboptimal or burdensome configurations.

Observation 3: Differentiating configurations by intent (i.e., inference vs. monitoring) and enabling UE-level selective participation can improve both UE performance and data privacy.

2.3	Indication of Performance Monitoring Purpose

The network should be able to explicitly indicate whether a given measurement or data collection configuration is intended for performance monitoring (e.g., label data reporting) rather than for model inference or other general purposes. Furthermore, the intensity of performance monitoring can be indicated to tell the UE how relevant that data collection is to performance monitoring. This allows the UE to distinguish the purpose and respond accordingly.

Proposal 1: New IEs in RRCReconfiguration or MAC-CE messages should be introduced to explicitly indicate that a configuration is intended for AI/ML performance monitoring or/and the intensity of performance monitoring.

2.4	UE’s Awareness and Reporting of Preference and Impact for Monitoring

The UE should be able to recognize the intent of a received configuration and internally evaluate its ability or willingness to support it. Such evaluation may take into account UE’s resource conditions (e.g., battery level, processing load, UE type), current radio environment (e.g., signal quality, mobility), or data privacy. This allows UE to participate selectively in performance monitoring where feasible.

Proposal 2: New IEs in UEAssistanceInformation message should be introduced to explicitly indicate preference applicability or priority for AI/ML performance monitoring.

The UE can also provide feedback to the network indicating its perceived burden or impact, and optionally the reason for rejection or reduced capability. This enables the network to assess UE burden and decide accordingly.

Proposal 3: New IEs in UEAssistanceInformation message should be introduced to explicitly indicate the impact due to performance monitoring and reason for rejection or reduced capability.

2.5	Network’s Adaptive Performance Monitoring

The network can adapt its behavior dynamically based on the feedback received from the UE. This may include maintaining or removing a configuration, adjusting its timing or reporting frequency, lowering its execution priority, or reassigning the task to other UEs where applicable. Such flexibility would enable more efficient use of network and UE resources while maintaining the quality of AI/ML model validation.

Proposal 4: Based on feedback from the UE, the network may maintain or remove the configuration, delay activation, lower configuration priority, apply relaxed parameters, or redistribute monitoring load across multiple UEs.

Conclusion

In this contribution, we have the following proposals for UE awareness and preference on the network-sided model:

Proposal 1: New IEs in RRCReconfiguration or MAC-CE messages should be introduced to explicitly indicate that a configuration is intended for AI/ML performance monitoring or/and the intensity of performance monitoring.

Proposal 2: New IEs in UEAssistanceInformation message should be introduced to explicitly indicate preference applicability or priority for AI/ML performance monitoring.

Proposal 3: New IEs in UEAssistanceInformation message should be introduced to explicitly indicate the impact due to performance monitoring and reason for rejection or reduced capability.

Proposal 4: Based on feedback from the UE, the network may maintain or remove the configuration, delay activation, lower configuration priority, apply relaxed parameters, or redistribute monitoring load across multiple UEs.

References

[1] Draft Report of 3GPP TSG RAN WG2 meeting #130, Malta

[2] 3GPP TR 38.843 V18.0.0 (2023-12): “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”.

[3]	3GPP TR 38.744 V1.0.0 (2025-6): “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Release 19)”