3GPP TSG-RAN WG2 Meeting#131	       	R2-2505195

Bengaluru, India 25th – 29th Aug, 2025

Source:	vivo

Title:	Remaining issues on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

# Introduction

The post RAN2#130 meeting email discussion [1] captures the remaining issues on NW-side data collection.

In this contribution, we will provide our analysis of those identified open issues and summarize our view with proposals.

# Discussion

## Open issue RRC-21: Time related content of logged data

Issue description: It has not yet been clarified what information needs to be included with the logged data, to indicate a time gap between the logged data entries (i.e. a gap that is longer than the logging data periodicity).

This issue refers to the RAN2#129bis agreement:

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

RAN2#130 agreement:

Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.

For the indication of the “gap”, the timestamp of each logged measurement entry would be necessary. As the network would provide the UE with an absolute timestamp at the time of configuring the NW-side data collection configuration, along with the measurement results, the UE can report this absolute timestamp. Additionally, for each group the relative time stamp can also be logged.

Therefore,

- [RRC-21] When reporting measurement results, for each measurement group the UE can include the absolute timestamp provided at the time of data collection configuration and a relative timestamp at the first sample of each measurement group.

## Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

Issue description: RAN2#127 made the following agreement regarding NW-side data collection:

1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

The second FFS on dynamic activation/deactivation has not yet been addressed, as also pointed out in the stage-2 running CR draft.

As agreed at the RAN2#130 meeting, data collection can configure measurements for multiple sets of resources and use cases (e.g., BM, Mobility, etc.), in other words, the configuration may include a data collection configuration containing L1 and L3 measurement related reconfigurations. At least for the L3 measurement configuration, we do not see that there is any strong motivation for dynamic activation/deactivation. We think RRC configuration is enough, and as soon as the UE is configured, the UE can apply the configurations for data collection.

Therefore,

- [RRC-25] Dynamic activation/deactivation is NOT supported for data collection measurement configurations.

## Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

Issue description: Whether data availability indication should be sent when the UE has data and low power state is sent and what cause should be included then. The UE may have some data when indicating low power state. Even though this data volume may be lower than full buffer/threshold, it is still useful to let the network know about that so that the data can be fetched.

For the UE in a low power state, we can consider the case as follows:

- When the UE receives data collection configuration, if the UE is already in a low power state, the UE should not start data logging and can instantly report its low power state to the network. As the UE has not started data logging, there is no available logged data to indicate to the network.

- When the UE receives data collection configuration, if the UE is not in a low power state, the UE can start data logging. In the meantime, if the UE experiences a low power state, the UE may have already stored some data. If the network has configured a data threshold, and if the stored data has not reached the configured threshold, the UE can just report the low power state to the network, and the UE is not required to report the available data. Because those data may not be useful to the network, subsequent data reporting would just deepen the UE low power state unnecessarily.

- [RRC-29] When the UE is reporting a low power state, the UE just reports the low power state, and the UE is not required to report available data if the data has not reached the configured buffer threshold.

## Open issue RRC-30: Semi-persistent resources for data collection

Issue description: RAN2 excluded usage of aperiodic CSI resource for data collection, but it is still unclear whether semi-persistent resources are needed for this.

Semi-persistence resource configuration would additionally require resource activation, e.g., introduction of MAC CE. But, there is no strong motivation to configure semi-persistent resource for data collection. RRC configuration of resources for data collection enough.

- [RRC-30] No need for semi-persistent resources configuration for data collection.

## Open issue RRC-31: The release of logged AIML data in UE’s buffer

Issue description: RAN2 needs to discuss and capture in the spec when the logged data for AIML in UE buffer will be released. In addition to the retain/release of logged data during HO covered in Open issue RRC-7, the following can be also considered:

- Power off or deregistration

- 48 hrs after the release of logged measurement configuration

- Explicit indication from the serving gNB.

The UE releasing the data in the buffer at power-off or upon deregistration seems like quite normal behaviour. Because when the UE would power on or register again in guarantee. The data in the buffer may not be of any usefulness when the power is on or registered again.

UE releasing the data in the buffer after the release of the logged measurement configuration is similar to UE behaviour with regard to legacy logged MDT. This should also be supported by logged data for AIML in the UE buffer.

An explicit indication, from the serving cell, to release the data in the UE buffer may be useful. But, we do not think it is necessary. The reasons are as follows:

- When NW retrieves the data, the UE buffer can be emptied, and there is no need for further indication from the NW for the UE to release the data.

- If UE NW does not retrieve the data, on power-off, deregistration, or after 48 hrs, UE can still release the data in the buffer.

Therefore,

- [RRC-31] The logged data for AIML in the UE buffer will be released on the following conditions:

- Power-off or deregistration

- 48 hrs after the release of logged measurement configuration

- No explicit indication is needed from the serving gNB.

## Open issue RRC-34: Whether enhancements for NW-side data collection are per use case or common for all AI-related use cases

Issue description: It can be discussed whether the enhancements for NW-side data collection are per use case or common for all AI-related use cases, e.g., AS layer memory, logged data availability, low-power state indication.

We think that the enhancements for NW-side data collection may be different for different use cases. It should be discussed case by case. Though some enhancements can be common for all cases. For example, for AS layer memory, low power should be common for all cases.

Therefore,

- [RRC-34] A common approach should be considered for enhancements for NW-side data collection, and the following enhancements are common for all use cases:

- AS layer memory;

- logged data availability;

- low-power state indication.

## Open issue RRC-36: How data is forwarded to OAM or source gNB after HO

Issue description: RAN2 agreed that target gNB can fetch data collected in the source gNB after HO. We need to decide how this data is forwarded to OAM or source gNB, e.g. via inter-node RRC message or in some other way. RAN3 involvement may be needed.

How data are forwarded from the source gNB to the target gNB, the following two ways can be considered:

- Via inter-node RRC message

- Via Xn message, similar to RETRIEVE UE CONTEXT procedure from target gNB to source gNB

Consequently, on how the target gNB can fetch data collected in the source gNB after HO, a similar approach to RETRIEVE UE CONTEXT can be adopted. The specification details can be left to RAN3.

On how data is forwarded from the target gNB to OAM is out of RAN2’s understanding and scope. This should be discussed in SA5.

Therefore.

- [RRC-36] On how data is forwarded to OAM or the source gNB, the following is proposed:

- How data is forwarded to OAM is out of RAN2 scope and should be referred to SA5;

- Xn signaling procedure can be adopted on how data is forwarded to source gNB, and details should be discussed in RAN3;

- RAN2 to LS to RAN3/SA5 to let them consider how data is forwarded to OAM or source gNB.

# Conclusion

In this contribution, some remaining issues on NW-side data collection for model training for beam management and we have the following proposals:

Open issue RRC-21:

- [RRC-21] When reporting measurement results, for each measurement group the UE can include the absolute timestamp provided at the time of data collection configuration and a relative timestamp at the first sample of each measurement group..

Open issue RRC-25:

- [RRC-25] Dynamic activation/deactivation is NOT supported for data collection measurement configurations.

Open issue RRC-29:

- [RRC-29] When the UE is reporting a low power state, the UE just reports the low power state, and the UE is not required to report available data if the data has not reached the configured buffer threshold.

Open issue RRC-30:

- [RRC-30] No need for semi-persistent resources configuration for data collection.

Open issue RRC-31:

- [RRC-31] The logged data for AIML in the UE buffer will be released on the following conditions:

- Power-off or deregistration;

- 48 hrs after the release of logged measurement configuration;

- No explicit indication is needed from the serving gNB.

Open issue RRC-34:

- [RRC-34] A common approach should be considered for enhancements for NW-side data collection, and the following enhancements are common for all use cases:

- AS layer memory,

- logged data availability,

- low-power state indication.

Open issue RRC-36:

- [RRC-36] Legacy On how data is forwarded to OAM or the source gNB, the following is proposed:

- How data is forwarded to OAM is out of RAN2 scope and should be referred to SA5;

- Xn signaling procedure can be adopted on how data is forwarded to source gNB, and details should be discussed in RAN3;

- RAN2 to LS to RAN3/SA5 to let them consider how data is forwarded to OAM or source gNB.

- Reference

- R2-2505778 RRC open issues for AIML for NR air interface