3GPP TSG-RAN WG2 Meeting #131	R2-2505205

# Bengaluru, India, Aug 25th – 29th , 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | draft | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rx BSF optimization for NR RRM Ph5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson, Apple, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR is to introduce Rx BSF optimization for NR RRM Ph5 as requested by RAN4 in their LS R2-2503326. In RAN2#120 meeting, the following agreements were further made: In RRC draft CR, we don’t implement “TimeToTrigger and Hysteresis in activation condition” for now. Separate RSRP threshold and RSRQ threshold are introduced. RAN2 assumes If both are configured, it means the UE activates multi-Rx L3 measurement when both of the thresholds are satisfied. The new RSRP/RSRQ threshold parameters are added in MeasConfig IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Criterion for L3 fast beam sweeping operation is added. - Parameters for L3 fast beam sweeping are added in MeasConfig IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rx BSF optimization for NR RRM Ph5 is not supported in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.x, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

5.5	Measurements

<Unnecessary part omitted>

5.5.x	Criterion for L3 fast beam sweeping operation

The criterion to activate L3 fast beam sweeping operation is fulfilled when:

-	Serv-RSRP < FBS-ThresholdP, if FBS-ThresholdP is configured, and,

-	Serv-RSRQ < FBS-ThresholdQ, if FBS-ThresholdQ is configured,

Where:

-	Serv-RSRP = current L3 RSRP measurement of the PCell based on SSB (dBm).

-	Serv-RSRQ = current L3 RSRQ measurement of the PCell based on SSB (dB).

NEXT CHANGE

6.3.2	Radio resource control information elements

< Unnecessary part omitted >

–	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEASCONFIG-START

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                                              OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                                              OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList                                            OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList                                            OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList                                                  OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList                                                  OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                                       OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                                                      OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                                                       OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                                                OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                                                   OPTIONAL    -- Need R

]],

[[

effectiveMeasWindowConfig-r18       SetupRelease {MeasWindowConfig-r18}                                 OPTIONAL    -- Need M

]],

[[

fbs-Config-r19                      SetupRelease {FBS-Config-r19}                                       OPTIONAL    -- Need M

]]

}

MeasObjectToRemoveList ::=              SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=                  SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=            SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

FBS-Config-r19 ::=                      SEQUENCE {

fbs-ThresholdP-r19                      RSRP-Range                                            OPTIONAL,    -- Need R

fbs-ThresholdQ-r19                      RSRQ-Range                                            OPTIONAL     -- Need R

}

-- TAG-MEASCONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

effectiveMeasWindowConfig Used to setup and release effective measurement window in NR for E-UTRA measurements.

fbs-Config Indicates the configuration for fast beam sweeping operation for L3 measurements as specified in clause 5.5.X.

interFrequencyConfig-NoGap-r16 If the field is set to true, UE is configured to perform SSB based inter-frequency measurement without measurement gaps when the inter-frequency SSB is completely contained in the active DL BWP of the UE, as specified in TS 38.133 [14], clause 9.3. Otherwise, the SSB based inter-frequency measurement is performed within measurement gaps. In NR-DC, the field can only be configured in the measConfig associated with MCG, and when configured, it applies to all the inter-frequency measurements configured by MN and SN.

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS. This field is also applicable to L1 measurements configured with ltm-CSI-ResourceConfigToAddModList-r18. This field is not configured to a UE configured with event(s) related to the L2 U2N Relay UE.

measGapSharingConfig Specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

FBS-Config field descriptions

fbs-ThresholdP Parameter "FBS-ThresholdP" in clause 5.5.x, which is used to evaluate the criterion to activate fast beam sweeping operation for L3 measurements as specified in TS 38.133 [14].

fbs- ThresholdQ Parameter "FBS-ThresholdQ" in clause 5.5.x, which is used to evaluate the criterion to activate fast beam sweeping operation for L3 measurements as specified in TS 38.133 [14].

END OF CHANGES