3GPP TSG-RAN WG2 Meeting #131	                                                                    R2-2505216

Bengaluru, India, Aug 25th – 29th , 2025

Source:	CATT, Turkcell

Title:	Functionality management for RRM measurement prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

- Introduction

In previous RAN2 meetings, preliminary agreements were made for spec analysis of AI/ML mobility. In this contribution, we analyze the spec impact of RRM measurement prediction and measurement event prediction from the following aspects for UE-sided model:

- Inference;

- Performance monitoring.

- Discussion

  - Inference

Regarding inference configuration, it was agreed in RAN2#130 meeting that[1]:

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

In the following, we continue to discuss the inference report content and the conditions when to perform inference for AI/ML mobility.

Inference report content

It was agreed in RAN2#129bis meeting that [2]:

- For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

It can be seen that both periodic and event triggered reporting of predicated and/or actual RRM measurements are adopted for RRM prediction. Regarding the inference report content, the following cases are discussed.

2.1.1.1	Intra-frequency temporal domain case A

For intra-frequency temporal domain case A, the UE uses the historical measurement results to predict the future measurement results. There may be multiple predicted measurement instances within PW when PW length is larger than the measurement period. In this case, RAN2 needs to discuss which results needs to be reported in the measurement results. Generally, there are 3 options:

- Option 1: The measurement results for each time instance in PW;

- Option 2: The average measurement result in PW;

- Option 3: The last predicted measurement result in PW.

Option 1 can be used to predict whether measurement event will be triggered or not within the PW. On the other hand it also brings higher signaling burden on measurement report compared with the other 2 options. Regarding Option 2 or 3, the signaling overhead is the same level as the legacy measurement report but it doesn’t reflect the signaling strength trend precisely which results in the AI function doesn’t demonstrate its effectiveness sufficiently. Hence, it is proposed that:

Proposal 1: For intra-frequency temporal domain case A, the measurement results for each time instance within PW are reported in one measurementReport message.

2.1.1.2	Intra-frequency temporal domain case B

For intra-frequency temporal domain case B, it was agreed that the potential “inference parameters” which may contain Skipping pattern (e.g. SSB config that indicates SSBs transmitted) can be considered for inference/measurement configuration to provide the necessary information for UE to examine the applicability.

It can be seen that the network may configure skipping pattern as inference/measurement configuration. The UE can perform RRM prediction based on the skipping pattern. Therefore, the measurement results along the timeline can be actual measurement result or predicted measurement result. When measurement report is required to be report no matter periodically or based on measurement event, the measurement result can be actual or predicted. Then, from the network’s perspective, it may not identify whether the measurement result is actual or predicted considering the transmission delay.

Proposal 2: For intra-frequency temporal domain case B, whether measurement result is actual or predicated should be indicated in the measurement results.

2.1.1.3	Inter-frequency (frequency domain)

For inter-frequency prediction, the measurement result of one frequency is predicted based on the measurement results of another frequency. It was agreed that measured and predicted frequency (For inter-freq domain prediction) can be considered as the potential “inference parameters” for inference/measurement configuration. And it was agreed that the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework, where the report configuration is linked to measurement object which includes frequency information. Therefore, legacy RRM framework without new frequency information can be reused for inference measurement report.

Figure 1 MeasConfig Configuration

Proposal 3: For inter-frequency prediction, legacy measurement report is used as baseline.

Condition when to perform inference

In RAN2#129bis [2], the following agreements were achieved:

The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference

According to the agreements, it can be seen that it was FFS on when the UE can perform inference. And this issue was discussed in RAN2#130 meeting without conclusion. In the following, we analyze the report procedure of predicted measurement report for FR2-FR2 Intra-frequency temporal domain case A, FR1-FR1 Intra-frequency temporal domain case B, inter-frequency domain and spatial domain respectively.

2.1.2.1	FR2-FR2 Intra-frequency temporal domain case A

In intra-frequency temporal domain case A, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW and the measurement results in the future can be predicted to improve the handover performance.

Regarding AI/ML mobility, there are 4 options on when to trigger inference:

- Option 1: the inference is performed periodically as the real measurement result is obtained as shown in Figure 2;

Figure 2: Inference is performed periodically

- Option 2: the inference is performed when actual measurement event is triggered. For example, when the A3 entry condition is satisfied by actual measurement result, the inference is performed. And the prediction on the future result is performed which is shown in Figure 3.

Figure 3: Inference is performed when measurement event is triggered by actual measurement

- Option 3: the inference is performed when the moves to the cells edge, e.g. the serving cell RSRP is below the one threshold considering the UE performs handover at the cell edge.

Figure 5: Inference is performed when the UE moves to the cell edge

- Option 4: the inference is performed up to the UE implementation.

In BM, AI/ML functionalities may be activated autonomously when periodic CSI report configuration is configured for the UE or AI/ML functionality activation follows the existing CSI measurement and reporting, i.e., semi-persistent reporting can be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI. When the AI/ML functionality is activated, it is already enabled for performing inference. Hence, there is no other extra condition for the UE to decide when to perform inference in BM.

For FR2-FR2 Intra-frequency temporal domain case A in AI/ML mobility, it is studied to to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption). Regarding handover, it is typically happens at the cell edge, e.g., the channel quality of the neighbour cell is higher than that of the serving cell or the channel quality of the serving cell is below one threshold. Hence, inference performed under some conditions decreases the inference frequency which is beneficial on reducing of AI computation, storage and power consumption as well. Therefore, it is proposed that:

Proposal 4: For FR2-FR2 Intra-frequency temporal domain case A, inference is performed with conditions, e.g., at the cell edge.

2.1.2.2	FR1-FR1 intra-frequency temporal domain case B/inter-frequency domain/spatial domain

Regarding FR1-FR1 Intra-frequency temporal domain case B/inter-frequency domain/spatial domain, they are studied to reduce measurement efforts in temporal, frequency domain, spatial by using predicted measurements.

For FR1-FR1 intra-frequency temporal domain case B, the measurement is skipped to reduce the measurement overhead and measurement results in PW are predicted by historical measurement result(s) in OW. The UE needs to predict the measurement result for the skipped time instance based on the network configuration.

For inter-frequency, the measurement result of one frequency are predicted by the actual measurement result of another frequency to reduce the measurement overhead of different frequencies.

For Spatial domain, AI/ML is applied to predict the skipped beams. As a consequence, the measurement overhead in the spatial domain is reduced. The skipped beams can be predicted continuously.

Based on the analysis above, considering no dynamic signaling (e.g. L1/2) is required for inference configuration.  It is proposed that UE performs inference when inference configuration is configured for the UE and no other conditions are needed for FR1-FR1 Intra-frequency temporal domain case B/inter-frequency domain/spatial domain.

Proposal 5: For FR1-FR1 intra-frequency temporal domain case B/inter-frequency domain/spatial domain, UE can perform inference when inference configuration is configured for the UE and no other conditions are needed.

  - Performance monitoring

General

In RAN1#121, the following agreements were achieved for monitoring of BM-Case 1 [3].

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring. - Predefines a threshold X = 64 for the minimal slot offset, which is configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report. - Note: CSI reference resource corresponding to the CSI report for monitoring and inference is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP, considering discussing associated timeline separately. The associated working assumption made in RAN1#120b will not be confirmed.

According to RAN1 agreements, to obtain performance monitoring results, UE first determines the transmission occasion(s) of monitoring resource, and then identities the inference report linked with each measurement result. For AI/ML mobility, the same principle can also be applied. Hence, the configuration for performance monitoring for AI/ML mobility is linked with an inference report.

Proposal 6: The performance monitoring configuration for AI/ML mobility is linked with an inference report.

Regarding to the performance monitoring configuration, it can take the measurement object framework as baseline to configure the following parameters:

- RS resources;

- SMTC;

- Frequency.

Proposal 7: For monitoring configuration, taking the measurement object framework as baseline for AI/ML mobility.

UE-sided monitoring

For UE-sided model, UE-sided monitoring, the following agreements were achieved in RAN2#130 meeting [1]:

Agreements 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases.

Based on the agreements, it is still FFS on whether UE makes the management decision on performance monitoring and sends the decision to network for RRM prediction. For BM, UE-decision on the management is not supported which is because the low accuracy of AI/ML model brings negative impacts on network side performance which is concerned by the network, e.g., throughput, and LCM procedure should be under the network control. For AI/ML mobility, the case is a bit of complex.

For intra-frequency temporal domain case A, it is used to predict the measurement results in the future to decide whether specific measurement event can be triggered. If the UE detects the performance has deteriorated significantly, using the predicted results may lead to wrong handover decisions which greatly aggravate the handover performance.

For intra-frequency temporal domain case B, the measurement results are predicted based on actual measurement results to reduce the RS overhead. And for inter-frequency, the measurement results of one frequency are predicted based on the other frequency. If the accuracy of the predicted measurement results deteriorates below one threshold and the reported measurement results are used by the network for next step, it may be results in improper decision, e.g. adding unsuitable SCells. This also brings deterioration on network performance, e.g. throughput decline.

It can be seen that for AI/ML mobility use cases, the low accuracy of the AI/ML brings deterioration on network performance including handover performance and throughput which is more concerned by the network. Even though, UE-decision enables early deactivation of the AI/ML with low accuracy, it breaks the principle that LCM procedure is under the network control. Hence, like BM, we prefer not to support UE-decision for UE sided monitoring.

Proposal 8: UE-decision for UE sided monitoring of UE-sided model in AI/ML mobility is not supported.

In the last RAN1 120bis meeting [4], the type of resource for the set for monitoring and the report type of inference and performance monitoring in BM was discussed, and it was agreed that:

Agreement of RAN1 For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type: Monitoring report type Inference report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support.

Based on the above agreement, for Type 1 with Option 2 where UE calculates performance metric(s) and reports it to NW or reports an event to NW based on the performance metric(s), P/SP/AP reporting for performance monitoring is supported for BM. For performance monitoring of RRM measurement prediction, the following options can be considered:

- Option 1:Periodical reporting;

- Option 2: Reporting after being triggered by NW;

- Option 3: event triggered reporting.

In Option 1, the UE reports the monitoring results periodically based on the sampling period or configured by the network. In this Option, signaling overhead is relatively high. In Option 2, the UE reports the monitoring results when it is triggered by network indication which requires sensitive observation of the network on performance. In Option 3, the UE reports the monitoring results when the monitoring metric exceeds on threshold configured by the network. In this option, proper threshold configuration is needed but low signaling overhead can be achieved.

Proposal 9: For monitoring report, RAN2 can study the following options:

- Option 1: Periodical reporting;

- Option 2: Reporting triggered by NW;

- Option 3: event triggered reporting.

- Conclusion

In this contribution, we provide our views on functionality management for RRM measurement prediction, and the proposals are summarized as follows:

Inference

Proposal 1: For intra-frequency temporal domain case A, the measurement results for each time instance within PW are reported in one measurementReport message.

Proposal 2: For intra-frequency temporal domain case B, whether measurement result is actual or predicated should be indicated in the measurement results.

Proposal 3: For inter-frequency prediction, legacy measurement report is used as baseline.

Proposal 4: For FR2-FR2 Intra-frequency temporal domain case A, inference is performed with conditions, e.g., at the cell edge.

Proposal 5: For FR1-FR1 intra-frequency temporal domain case B/inter-frequency domain/spatial domain, UE can perform inference when inference configuration is configured for the UE and no other conditions are needed.

Performance monitoring

Proposal 6: The performance monitoring configuration for AI/ML mobility is linked with an inference report.

Proposal 7: For monitoring configuration, taking the measurement object framework as baseline for AI/ML mobility.

Proposal 8: UE-decision for UE sided monitoring of UE-sided model in AI/ML mobility is not supported.

Proposal 9: For monitoring report, RAN2 can study the following options:

- Option 1: Periodical reporting;

- Option 2: Reporting triggered by NW;

- Option 3: event triggered reporting.

- References

- R2-2505002, RAN2#130 Meeting Report, MCC;

- R2-2503302, RAN2#129bis Meeting Report, MCC;

- R1-2505102, RAN1#121 Meeting Report, MCC;

- R1-2503201, RAN1#120bis Meeting Report, MCC.