3GPP TSG-RAN WG2 Meeting #131                                                                    R2-2505218

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CATT, Turkcell

Title:	Discussion on UE-sided data collection

Agenda Item:	8.3.2.3

Document for:	Discussion and Decision

- Introduction

In RAN2#130 meeting, LCM and spec impact for AI/ML mobility was discussed, and only the following agreement about UE-sided data collection was achieved [1]:

Agreements For the collection of data for the training of a network-sided model for RRM measurement prediction: - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed) - The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS. - The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity. - The UE sends availability indication of collected logged data via UAI or RRCReconfigurationComplete message for HO case. - The availability indication can be triggered due to: - full buffer being reached (if configured), (FFS if buffer is per use case) - buffer threshold being reached (if configured), or (FFS if buffer is per use case) - low power (if configured) - Upon sending the availability indication, UE indicates: - Data is available - Reason for the triggering of the availability indication (full buffer. Threshold) - Low power indication - The UE sends the collected data upon explicit/on-demand request from the network (using UEInformationRequest/Response signaling) - The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO). - The UE releases the collected data upon transitioning to IDLE/INACTIVE - For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial. It can be up to WI phase if this is done depending on whether a simple solution can be found For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

In this contribution, we mainly focus on the detailed discussion about UE-sided data collection for RRM measurement prediction and measurement event prediction use cases, some of the agreements for network-sided data collection could be used for reference.

- Discussion

  - RRM measurement prediction

It is agreed to use AI/ML PHY request/configuration framework as baseline.

Data collection configuration

In AI/ML PHY, for UE-sided data collection, the following agreements were achieved for data collection configuration in RAN2.

Agreements on data collection configuration - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication). - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

The agreements above about data collection configuration are also appropriate for UE-sided data collection for RRM measurement prediction use case, thus, we propose,

Proposal 1: The following agreements on data collection configuration for UE-sided data collection in AI/ML PHY are also applied to RRM measurement prediction:

- The network can provide or release the data collection configuration (at any point in time), with or without UE request;

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration;

- The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

- Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

For network-sided data collection, the configuration framework and logging mechanism have been agreed in last meeting:

Agreements - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed) - The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS. - The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity.

Similarly, legacy RRM measurement framework can also be used for UE-sided data collection configuration without measurement reporting for AI mobility use cases. Whether we reuse existing Measconfig structure or we need a separate data logging configuration is for WI phase.   Required enhancements are FFS.

Proposal 2: For RRM measurement prediction, legacy L3 RRM measurement framework is used to provide UE-sided data collection configuration. Whether we reuse existing Measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS.

And for logging mechanism and content configuration, the agreements achieved for network-sided data collection could be directly reused.

Proposal 3: The agreements about logging mechanism and content configuration achieved for network-sided data collection could be directly reused for UE-sided data collection:

- UE can be configured to log, at a certain logging periodicity, one or more of the following:

- L3 cell level measurements;

- L3 beam level measurements;

- L1-filtered beam level measurements (if sub-case 1 and 3 is supported);

- Cell ID (FFS CGI of serving cell.  If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback);

- Time info (if as agreed by AI/ML and/or if needed).

- The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity.

UE requests data collection

In AI/ML PHY, for UE-sided data collection, the following agreements were achieved for UE requests data collection in RAN2.

Agreements on UE requests data collection - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

The agreements above about candidate data collection configurations and UE requests data collection are also appropriate for UE-sided data collection for RRM measurement prediction use case. Thus, we propose,

Proposal 4: The following agreements on candidate data collection configurations and UE requests data collection for UE-sided data collection in AI/ML PHY are also applied to RRM measurement prediction:

- The UE can request measurement configuration for data collection. The request can contain one or more of the following:

- An indication on start/stop of data collection;

- Preferred configuration from a list of candidate configurations provided by NW. It is up to network what it configures at the end.

- Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

Handling during mobility/state transition

For network-sided data collection, some UE behavior upon mobility and state transition have been agreed in last meeting:

Agreements - The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO). - The UE releases the collected data upon transitioning to IDLE/INACTIVE - For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial. It can be up to WI phase if this is done depending on whether a simple solution can be found

For RLF case, it is different agreement from the BM use case that for AI mobility we should retain the data relate to coverage. Therefore for UE-sided data collection, we should also follow this agreement.

Proposal 5: The agreements about UE behavior upon mobility and state transition achieved for network-sided data collection could be directly reused for UE-sided data collection:

- The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO);

- The UE releases the collected data upon transitioning to IDLE/INACTIVE;

- For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial. It can be up to WI phase if this is done depending on whether a simple solution can be found.

  - Measurement event prediction

In this section, we discuss UE-sided data collection for measurement event prediction involving both indirect measurement event prediction and direct measurement event prediction.

Indirect measurement event prediction

For indirect measurement event prediction, considering that the RRM measurement prediction (temporal, frequency, and spatial domain RRM prediction) model is used, the UE-sided data collection for training including e.g., data collection configuration (configuration mechanism, measurement framework, and logging content), UE requests data collection and handling during mobility/state transition etc. for indirect measurement event prediction are same as RRM measurement prediction.

Proposal 6: For indirect measurement event prediction, the data collection configuration (configuration mechanism, measurement framework, and logging content), UE requests data collection and handling during mobility/state transition are the same as that for RRM measurement prediction.

Direct measurement event prediction

Similar as UE-sided data collection in RRM measurement prediction, data collection configurations, UE requests data collection and handling during mobility/state transition agreed previously are also applied to direct measurement event prediction.

Proposal 7: The data collection configuration, UE requests data collection and handling during mobility/state transition agreed previously are also applied to direct measurement event prediction.

- Conclusion

According to the discussion in section 2, we propose:

RRM measurement prediction

Proposal 1: The following agreements on data collection configuration for UE-sided data collection in AI/ML PHY are also applied to RRM measurement prediction:

- The network can provide or release the data collection configuration (at any point in time), with or without UE request;

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration;

- The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

- Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

Proposal 2: For RRM measurement prediction, legacy L3 RRM measurement framework is used to provide UE-sided data collection configuration. Whether we reuse existing Measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS.

Proposal 3: The agreements about logging mechanism and content configuration achieved for network-sided data collection could be directly reused for UE-sided data collection:

- UE can be configured to log, at a certain logging periodicity, one or more of the following:

- L3 cell level measurements;

- L3 beam level measurements;

- L1-filtered beam level measurements (if sub-case 1 and 3 is supported);

- Cell ID (FFS CGI of serving cell.  If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback);

- Time info (if as agreed by AI/ML and/or if needed).

- The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity.

Proposal 4: The following agreements on candidate data collection configurations and UE requests data collection for UE-sided data collection in AI/ML PHY are also applied to RRM measurement prediction:

- The UE can request measurement configuration for data collection. The request can contain one or more of the following:

- An indication on start/stop of data collection;

- Preferred configuration from a list of candidate configurations provided by NW. It is up to network what it configures at the end.

- Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

Proposal 5: The agreements about UE behavior upon mobility and state transition achieved for network-sided data collection could be directly reused for UE-sided data collection:

- The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO);

- The UE releases the collected data upon transitioning to IDLE/INACTIVE;

- For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial. It can be up to WI phase if this is done depending on whether a simple solution can be found.

Measurement event prediction

Indirect measurement event prediction

Proposal 6: For indirect measurement event prediction, the data collection configuration (configuration mechanism, measurement framework, and logging content), UE requests data collection and handling during mobility/state transition are the same as that for RRM measurement prediction.

Direct measurement event prediction

Proposal 7: The data collection configuration, UE requests data collection and handling during mobility/state transition agreed previously are also applied to direct measurement event prediction.

- References

- R2-2505002  RAN2#130 Meeting Report    MCC