3GPP TSG-RAN WG2 Meeting #131	R2-2505226

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.8.2

Source: 	CATT

Title: 	Discussion on potential SMTC enhancements

Document for: 	Discussion and Decision

Introduction

This contribution further discusses potential SMTC enhancements based on the reply LS from RAN4 [1] and last meetings progresses [2].

Discussion

2.1	On the reply LS from RAN4

In RAN2#130, RAN2 sent an LS to RAN4 asking whether it is feasible to support two SMTC periodicities and more than 4 SMTCs per frequency layer within Rel-19 timeframe [3]. RAN4 responded in [1] with the following reply.

RAN4 thanks RAN2 for the information in LS R2-2503068. RAN4 discussed the RRM requirements impact of the SMTC enhancement, and would like to provide following conclusions achieved in RAN4#115: - For the SMTC enhancement on configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements: From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but that requires some clarifications/modifications to the existing RRM requirements. - RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer. Note: RAN4 has not estimated the full impact during RAN4#115 meeting. There is a concern on the workload on to support those agreements in Rel-19 timeframe from RAN4 requirements perspective. - For the SMTC enhancement on configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs: From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. However, the current requirements in TS 38.133 for the scenarios when the number of configured SMTCs exceed the parallelSMTC-r17 for idle/inactive UEs are captured below for reference: For Measurements of intra-frequency NR cells with NTN carrier: The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected. For Measurements of inter-frequency NR cells with NTN carrier: The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the value indicated by parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in table 4.2C.2.4-1 and table 4.2C.2.4-2 is expected..

RAN4 indicates they have not estimated the full impact and there is a concern on the workload on to support 2 SMTC periodicities in Rel-19 timeframe from RAN4 requirements perspective. Although RAN4 declares that the progress of RRM requirement definition for 2 SMTC periodicities does not block the completion of this WI, there is no explicit agreement to support that also.

Since supporting 2 SMTC periodicities will impose a significant workload on RAN4 and the cases under which this mechanism is indispensable remain unclear, it is proposed to support for only 6 SMTCs per frequency layer and exclude support for two different SMTC periodicities in Rel-19.

Observation 1: RAN4 has concerns on the workload on to support two different SMTC periodicities (with different offsets) in one frequency layer in Rel-19 timeframe. Supporting 2 SMTC periodicities will impose a significant workload on RAN4.

Proposal 1: RAN2 confirms support for 6 SMTCs per frequency layer for idle/inactive UEs, but exclude support for two different SMTC periodicities (with different offsets) in one frequency layer in Rel-19.

2.2	SMTC enhancements in RRC_IDLE/RRC_INACTIVE

In RAN2#130, on SMTC selection enhancements in RRC_IDLE/RRC_INACTIVE, location-based SMTC selection was agreed while SSB-index based SMTC remains under discussion.

3.	We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

We see benefits in supporting the SSB-index based SMTC mechanism alongside the location-based SMTC. The SSB-index based solution brings lower signalling overhead, because it does not require reference location information in SIB. Some companies argue that this mechanism is unsuitable for the single SSB beam cell. Nonetheless, it gives the NW greater flexibility, e.g. using location-based SMTC for single SSB beam cells and SSB-index based SMTC for multiple SSB beams cells.

Proposal 2: RAN2 considers support of SSB-index based SMTC selection mechanism. SMTC list is per SSB configured in SIB2 and SIB4.

Conclusion

In this contribution, DL coverage enhancement is further discussed with the corresponding observations and proposals listed below:

Observation 1: RAN4 has concerns on the workload on to support two different SMTC periodicities (with different offsets) in one frequency layer in Rel-19 timeframe. Supporting 2 SMTC periodicities will impose a significant workload on RAN4.

Proposal 1: RAN2 confirms support for 6 SMTCs per frequency layer for idle/inactive UEs, but exclude support for two different SMTC periodicities (with different offsets) in one frequency layer in Rel-19.

Proposal 2: RAN2 considers support of SSB-index based SMTC selection mechanism. SMTC list is per SSB configured in SIB2 and SIB4.

Reference

- R4-2508433	Reply LS on SMTC enhancements.

- RAN2_130_Meeting_Report.

- R2-2503068	LS on SMTC enhancements.

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".