3GPP TSG-RAN WG2 Meeting #131	R2-2505231

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.9.3

Source: 	CATT

Title: 	Discussion on open issues for CB-Msg3 EDT

Document for: 	Discussion and Decision

Introduction

This contribution further discusses the open issues for CB-Msg3 EDT listed in [1], based on the agreements of RAN2#130 [2].

Discussions on open issues

- Open issue MAC-2: CB-RNTI calculation

For CB-RNTI calculation, the following WA and agreements were given in last meeting:

Working Assumption:

The formula for RNTI for mMsg4 monitoring is:

RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id.

•	X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT,

•	Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19,

•	Y is ceil (Msg4_WS/Msg3_WP),

•	CE_level is the CE level, 0 <= CE_level < 3

•	carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0,

0 <= carrier_id < 16

Can come back to check if the NW can also simply configure RNTI = X

Agreements – part 2:

1.	The value of X is 4097 for NB-IoT and 2401 for eMTC

2.	The value of Msg4_WS is the maximum Msg4 window size

3.	The value of Msg3_WP is the minimum Msg3 window periodicity

We are fine to confirm the WA above, as it seems can be agreed by all the companies during the post email discussion [1].

Regarding the option of the network configure RNTI = X, we think it is the simplest approach. Since anyway UE will rely on the UE ID included in the Msg4 as the response to each UE sending CB-Msg3 EDT, using one specified RNTI is workable. And in the cast that the two adjacent Msg4 monitoring windows during which the CB-RNTI is used is not overlap with each other, there is no false alarm issue. However, if the RNTI-formula is endorsed, additionally support the RNTI = X approach requires further discussion on how these two approaches work together in the same system, e.g., on the conditions under which each applies (i.e., when to use the RNTI-formula and when to use the fixed value X). To ensure the WI can be completed on schedule, we are fine to follow the majority and not additional support the RNTI = X approach.

Proposal 1 (MAC-2): Confirm the WA on formula for CB-RNTI for Msg4 monitoring.

Additionally, during the post email discussion, several issues are proposed by companies.

Proposal 2: For the agreed CB-RNTI formula from P1, RAN2 to discuss - Whether to define Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity]. - Whether to define Y = 16. - Whether starting value X is optionally configurable.

We agree to replace Msg3_W_index with floor(start SFN_id of the selected CB-Msg3 Transmission window / minimum Tx window periodicity). This formulation is simpler and clearer. Because the resulting value is not the actual index of the selected CB-Msg3 transmission window, we propose dropping the Msg3_W_index term and incorporating the expression directly into the CB-RNTI formula.

As for whether to define Y as a fixed value rather than ceil (Msg4_WS/Msg3_WP), we think that defining Y as ceil(Msg4_WS / Msg3_WP) may make CB-RNTI values beyond the valid RNTI range and leave many middle values unused in actual deployment. Thus, we prefer to use a fixed value to replace the ceil(Msg4_WS/Msg3_WP) part. The exact value can be determined based on the possible lengths of CB-Msg3 response timer length and CB-Msg3 transmission window periodicity.

Regarding whether the starting value X is configurable, we see no compelling use case for such flexibility. Since anyway the total RNTIs value range is fixed (e.g., 1001-FFF3 for NB-IoT) and should cover the whole CB-RNTI range, the start point for CB-RNTI is irrelevant. For the sake of protocol simplicity, we therefore prefer to use fixed X value, i.e., 2401 for eMTC or 4097 for NB-IoT.

Proposal 2 (MAC-2): For the agreed CB-RNTI formula:

- Replace the Msg3_W_index with floor(SFN_id / [minimum Tx window periodicity]), where SFN_id is the index of the first radio frame of the selected CB-Msg3 transmission window.

- Replace Y (i.e., ceil(Msg4_WS/Msg3_WP)) as a fixed value; the exact value can be determined when the candidate lengths of CB-Msg3 response timer length and CB-Msg3 transmission window periodicity are finalized.

- Stick to fixed X value in the CB-RNTI formula, i.e., 2401 for eMTC or 4097 for NB-IoT.

- Open issue MAC-6:  The power ramping parameters and how the power ramping is done.

For the power ramping issue, the following agreements were given in RAN2#129bis

- RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply.

And RAN1 has given feedback in [3]:

Reply to Q1: RAN1 has not evaluated the potential performance of power ramping for CB-msg3-EDT, and it is likely that there will not be sufficient time to evaluate this topic within the R19 timeframe - For open loop power control the following UL power control parameters can be reused for CB-msg3-EDT - p0-UE-NPUSCH-r16 and alpha-r16 for NB-IoT NTN - There is no consensus in RAN1 on the need to define the set of narrowbands as a set. - mpdcch-FreqHopping-r16 is not needed

Because the resources for CB-Msg3-EDT are not UE specific, and a collision may happen, that power ramping for the CB-Msg3-EDT transmission may be useful. However, since RAN1 has no sufficient time to evaluate this topic within the R19 timeframe, we are also ok no power ramping is used in this release.

Proposal 3 (MAC-6): Power ramping is not supported for CB-msg3-EDT in Rel-19.

- Open issue MAC-9: Whether NW/UE processing time is needed when determine the Msg4 monitoring starts.

Open issue MAC-9: NW/UE processing time is needed or not. Issue description: In RAN2#129, it has been agreed that the Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT. FFS NW/UE processing time is needed or not. RAN2#129bis agreement: - The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not). It has been captured as editor’s note.

Until now, we do not see strong need from companies on support such "processing time". Most of the replicas are transmitted very early before the end of the Msg3 transmission window. It is not fair for these UEs to continue waiting a “processing time” for Msg4 monitoring, after the Msg3 transmission window. So for simplicity and progress of this feature, we propose to not consider any processing time.

Proposal 4 (MAC-9): No need to introduce NW/UE processing time when determine the Msg4 monitoring starts.

- Open issue MAC-15:  FFS on the details of the failure behaviour

Open issue MAC-15: Whether the UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails. Issue description: Whether the UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails was discussed in the RAN2#129bis. Most companies agreed. But when the CB-Msg3 procedure failes, which upper layer should be notified is still unknown. This issue was identified as an open MAC issue, but it was not discussed at RAN2#130. Proposed resolution: When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, MAC notifies the RRC layer. It is up to UE implementation to initial legacy connection establishment, EDT, or PUR.

We agree that the MAC layer should indicate to upper layers (RRC) when the CB-Msg3 EDT procedure fails, so that the RRC layer can continue trying to access the network even the CB-Msg3 procedure fails, because the data or signalling transmission of the UE has not been finished. The RRC may initiate legacy connection establishment procedure or EDT procedure based on the available network configuration, with the CB-Msg3 buffer flushed. However, for PUR, we wonder whether the UE will trigger CB-Msg3-EDT procedure if the UE has valid PUR resources. So maybe the PUR approach maybe not a reasonable option. Anyway, this can be left to RRC implementation, no specification impact is needed.

Proposal 5 (MAC-15): When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, MAC indicates CB-Msg3 transmission failure to the RRC layer.

Proposal 5a (MAC-15): Upon receives a CB-Msg3 EDT failure indication from MAC, it is up to RRC implementation to initiate legacy connection establishment or EDT for further RRC establishment attempt. No spec impact is needed.

- Open issue MAC-20: Should the CQI report in Msg3 in NB-IoT be supported in CB-Msg3-EDT?

During the [Post130][307] email discussion, the CQI report in CB-Msg3 EDT is discussed. In current release, the CB-Msg3 procedure is mainly used for EDT, which means that the UE will go to IDLE after a successful CB-Msg3 procedure, for the most of the time. And if supporting CQI report in CB-Msg3 EDT, additional RAN4 work needs to be introduced, it may be not a good option at this stage.

Proposal 6 (MAC-20): CQI reporting in anchor carrier and non-anchor is not supported in CB-Msg3-EDT.

Conclusion

In this contribution, we discussed remaining open issues for CB-Msg3 EDT procedure. Our proposals are as follows:

Proposal 1 (MAC-2): Confirm the WA on formula for CB-RNTI for Msg4 monitoring.

Proposal 2 (MAC-2): For the agreed CB-RNTI formula:

- Replace the Msg3_W_index with floor(SFN_id / [minimum Tx window periodicity]), where SFN_id is the index of the first radio frame of the selected CB-Msg3 transmission window.

- Replace Y (i.e., ceil(Msg4_WS/Msg3_WP)) as a fixed value; the exact value can be determined when the candidate lengths of CB-Msg3 response timer length and CB-Msg3 transmission window periodicity are finalized.

- Stick to fixed X value in the CB-RNTI formula, i.e., 2401 for eMTC or 4097 for NB-IoT.

Proposal 3 (MAC-6): Power ramping is not supported for CB-msg3-EDT in Rel-19.

Proposal 4 (MAC-9): No need to introduce NW/UE processing time when determine the Msg4 monitoring starts.

Proposal 5 (MAC-15): When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, MAC indicates CB-Msg3 transmission failure to the RRC layer.

Proposal 5a (MAC-15): Upon receives a CB-Msg3 EDT failure indication from MAC, it is up to RRC implementation to initiate legacy connection establishment or EDT for further RRC establishment attempt. No spec impact is needed.

Proposal 6 (MAC-20): CQI reporting in anchor carrier and non-anchor is not supported in CB-Msg3-EDT.

Reference

- [Post130][307][R19 IoT NTN] MAC CR (Mediatek).

- Draft_RAN2_130_Meeting_Report_v3.

- R1-2504905 Reply LS on CB Msg3 EDT for IoT NTN Ph3.