3GPP TSG-RAN WG2 Meeting #131	R2-2505241

Bengaluru, India, Aug. 25th– 29th, 2025

Agenda Item:	8.12.3

Source:	OPPO

Title:	Discussion on the remaining issues of UE-initiated beam report

Document for:	Discussion, Decision

# Introduction

According to the previous RAN2 discussion on the UE-initiated beam report, RAN2 made the following agreements on how to handle the UEI report at TAT expiry:

RAN2#130 meeting agreements [1]: When the TAT of the pTAG expires, UE releases PUCCH resource for mode-A/B UEI report and clears type-1 CG for mode-B UEI report. FFS for the case when the TAT expires on the sTAG.

In this contribution, we discuss the remaining issues related to the UE-initiated beam report.

# Discussion

## UEI report at TAT expiry

Email discussion summary for the MAC running CR: Proposal for discussion: Proposal 2: Regarding whether to initiate RACH when UEI report is triggered but TAT is expired, select one of the following options: - Option 1: UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts. - Option 3: UE initiates RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired, and UE indicates the cause of RACH. Discuss how to indicate the cause of RACH (e.g., MAC CE in Msg3). Proposal 3 (6/10): For mode-A UEI report, if TAT is expired after transmitting PUCCH and before PUSCH for a triggered report, no enhancement is needed.

According to the email discussion summary for the MAC running CR of Rel-19 MIMO, two Options are to be selected on how to handle the UEI report when TAT is expired. From our understanding, the gNB by implementation is able to know the running period of TAT, so that the uplink resource of a scheduled/configured uplink transmission will not be wasted as the MAC stops the uplink transmission and release the PUCCH resource when TAT is expired. If the network wants the UEI report, the network should provide the available resource of PUCCH and/or type-1 CG for the UEI report after providing the valid TA to the UE.

Observation 1: The gNB by implementation knows whether the TAT associated with PUCCH or type1-CG of UEI report is expired, and should provide the available resource of PUCCH and/or type-1 CG for the UEI report after providing the valid TA to the UE.

TS 38.214: 5.2.1.5.4.1 UE Initiated CSI reporting … For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, with eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE - starts a timer for such reference signal from the initial value equal to the time window length provided by eventDetectionTimeWindowLength and sets the counter to 1, if the timer for such reference signal is not running; or - increments the counter for such reference signal by 1, if the timer for such reference signal is running. If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 … After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CSI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH.

According to the current RAN1 specification of TS 38.214 as quoted above, when the UE fulfils the event for the UEI report, the PHY transmits the UEIRI on PUCCH, and the CSI report via DG (i.e. mode-A) or CG (i.e. mode-B) PUSCH associated to the PUCCH of UEIRI. There is no pending status of the UEI report, including both UEIRI on PUCCH and the CSI on PUSCH. If the transmission of the UEIRI on PUCCH and the CSI on PUSCH is stopped (i.e. by MAC) due to the TAT expiry, the UEIRI on PUCCH and the CSI on PUSCH is dropped. This is to say that the UEIRI on PUCCH and the CSI on PUSCH is one-shot transmission, unlike SR. Thus, indicating the transmission attempt of UEI report via RACH (e.g. Option 3) has no use, as the UEIRI on PUCCH and the CSI on PUSCH has already been dropped. For the legacy CSI report, the UCI triggered by PHY is also dropped due to the TAT expiry. As such, the gNB by implementation needs to reconfigure the PUCCH/PUSCH resource (which is released/cleared due to TAT expiry) to get the latest CSI report after providing a valid TA, according to the legacy CSI report procedure.

On the other hand, if the UE is able to fulfil the report event, the UE will keep triggering the UEI report for each evaluation window, and the UEI can be reported after the gNB provides a new configuration of PUCCH for the UEI report.

Observation 2: There is no pending UEI report (i.e. only one-shot UEI report) in PHY. The UCI on PUCCH and the UEI report on PUSCH which are not transmitted due to the TAT expiry (i.e. release PUCCH and clear type-1 CG) is lost, as legacy.

TS 38.321: The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when the timeAlignmentTimer(s) associated with all TAG(s) to which this Serving Cell belongs is not running, CG-SDT procedure is not ongoing and Positioning SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing. Furthermore, when the timeAlignmentTimer(s) associated with all PTAG(s) is not running, CG-SDT procedure is not ongoing and Positioning SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27 and the inactivePosSRS-TimeAlignmentTimer or inactivePosSRS-ValidityAreaTAT is not running. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission on a Serving Cell using TCI state(s) associated with a TAG for which the timeAlignmentTimer is not running.

According to the email discussion for the MAC running CR, some companies mentioned that the MAC needs to indicate the TAT expiry to PHY, so that the PHY can stop the event evaluation and the transmission of UEI report. Regarding the event evaluation, we think that there is no need to request the UE to autonomously stop the UEI event evaluation, as the gNB may soon provide a valid TA if the gNB wants to get the latest CSI. If the gNB wants to stop the UEI event evaluation, the gNB can send a new RRC message to remove the UEI report configuration or send a MAC CE to deactivate the CSI-RS resource for the event evaluation. Regarding whether to request PHY to stop the UEI report, we think that when the MAC releases the PUCCH resource and clear the type-1 CG for UEI report, the UEI in PHY is stopped. For the DG PUSCH of mode-A, the uplink transmission is also stopped by MAC as quoted above, based on the UE internal coordination between PHY and MAC (same as legacy).

Observation 3: At the TAT expiry, the current MAC specification already blocks the transmission of UCI (e.g. aperiodic CSI) triggered by PHY, i.e. by the UE internal interaction between MAC and PHY.

Thus, according to the analysis provided above, we consider that when the TAT corresponding to the UEI report is expired, the UE does not trigger RACH for the UEI report, and the MAC does not need to indicate the TAT expiry status to the PHY.

Proposal 1: When TAT is expired, the UE does not initiate RACH for the triggered UEI report, i.e. Option 1, same as the legacy (P/SP) CSI report.

Proposal 2: The TAT expiry is not indicated to PHY, same as the legacy CSI report.

# Conclusion

According to the analysis give above, we have the following observations and proposals:

Observation 1: The gNB by implementation knows whether the TAT associated with PUCCH or type1-CG of UEI report is expired, and should provide the available resource of PUCCH and/or type-1 CG for the UEI report after providing the valid TA to the UE.

Observation 2: There is no pending UEI report (i.e. only one-shot UEI report) in PHY. The UCI on PUCCH and the UEI report on PUSCH which are not transmitted due to the TAT expiry (i.e. release PUCCH and clear type-1 CG) is lost, as legacy.

Observation 3: At the TAT expiry, the current MAC specification already blocks the transmission of UCI (e.g. aperiodic CSI) triggered by PHY, i.e. by the UE internal interaction between MAC and PHY.

Proposal 1: When TAT is expired, the UE does not initiate RACH for the triggered UEI report, i.e. Option 1, same as the legacy (P/SP) CSI report.

Proposal 2: The TAT expiry is not indicated to PHY, same as the legacy CSI report.

# Reference

[1] RAN2#130 meeting minutes

[2] 3GPP TS38.321, “Technical Specification Group Radio Access Network; NR; Medium Access Control (MAC) protocol specification”.