3GPP TSG-RAN WG2 Meeting #131                                    R2-2505257

Bangalore, India Aug 25th – 29th, 2025

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues for S&F operation

Agenda Item:	8.9.2

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed. The WID is further refined and the latest version is in [1]. The main objectives related to support of Store&Forward (S&F) satellite operation are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In previous meetings, many agreements have been reached. In this paper, we will discuss the remaining issues for supporting S&F operation, especially the ones from post RAN2#130 meeting email discussion in “[Post130][306][R19 IoT NTN] RRC CR” and “[Post130][308][R19 IoT NTN] 36.304 CR”, and give our proposals.

Discussion

  - RRC open issues

2.1.1 AS-NAS interaction for operation mode transition time information

According to previous meeting agreements, RRC spec has captured the following two new parameters sf-OperationMode and t-ModeSwitching. Also the operation mode transition time indication is indicated from AS layer to NAS layer for NAS use, for example, to delay NAS procedure upon operation mode transits from normal mode to S&F mode.

TS 36.331 running CR 5.2.2.39 Actions upon reception of SystemInformationBlockType31 Upon receiving SystemInformationBlockType31 (SystemInformationBlockType31-NB), the UE shall: 1> start or restart timer T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime. 1> forward the t-ModeSwitching to upper layers, if present. …………….. SystemInformationBlockType1 field descriptions. …………………. sf-OperationMode Indicates that the cell is operating in Store and Forward mode. If this field is present, UEs supporting the Store and Forward operation ignores cellBarred-NTN and cellBarred. Value 'barred' means the cell is barred for NTN connectivity with the Store and Forward operation, as defined in TS 36.304 [4]. Value 'notBarred' means the cell allows UEs supporting the Store and Forward operation to access. If this field is absent, the NTN cell is operating in normal mode, i.e., not in the Store and Forward mode and UEs supporting the Store and Forward operation follow cellBarred-NTN.. SystemInformationBlockType31 field descriptions. …………………... t-ModeSwitching If sf-OperationMode is present in SIB1, this field indicates the time information on when an NTN cell is going to switch from the Store and Forward Satellite operation mode to the normal mode; otherwise, this field indicates the time information on when an NTN cell is going to switch from the normal mode to the Store and Forward Satellite operation mode.. ---

It can be seen that the current parameter t-ModeSwitching is bidirectional, e.g., a single parameter is used to represent the transition time of two directions, from S&F mode to normal mode or from normal mode to S&F mode. Therefore, without other information, after acquiring the t-ModeSwitching information, the NAS layer can only know a time point but cannot identify what’s the real meaning of this information. So it’s needed to also forward current operation mode information, e.g., the parameter sf-OperationMode received via SIB1, to the NAS layer simultaneously.

A straightforward way is to forward sf-OperationMode to upper layers upon reception of SIB1. However, since these two parameters are generally not received simultaneously, forwarding them to NAS layer immediately upon individual reception may lead to incorrect association between them. Therefore, it is suggested that when t-ModeSwitching is received and forwarded to upper layers, the stored sf-OperationMode information should also be forwarded to upper layers together.

Proposal 1: It’s suggested that when t-ModeSwitching is received and forwarded to upper layers, the stored sf-OperationMode information should also be forwarded to upper layers together.

  - Idle mode (36.304) open issues

2.2.1 S&F mode related information for neighbor satellites

During post meeting TS 36.304 running CR review, more companies show interests on introducing S&F mode related information for neighbor satellites. The following working assumption is given for next meeting discussion:

SF Mode parameter in neighour cell and UE Procedure changes Proposal 2: Working assumption on introducing SF mode for neighbour cell is confirmed. No additional specification changes needed. Usage of this parameter is left to UE implementation.

Some companies are fine to introduce such an indication and think it can assist prioritizing non S&F neighbor cell over S&F neighbor cell, e.g., when cell reselection.

However, traditional priority-based cell selection or reselection typically involves configuring priority parameters for different cells, so that the UE's criteria for (re)selecting a cell are clear (Note: NB-IoT does not currently support priority-based cell reselection, and mainly relies on radio quality-based cell ranking for cell reselection). Now, however, relying solely on the S&F mode parameter of neighbor cells without any additional information makes it difficult to standardize specific UE behavior.

Moreover, for UEs capable of recognizing this S&F mode parameter of neighbor cells, cells with S&F mode indication are still usable, not completely inaccessible. This further introduces uncertainty in UE behavior, for example, if the radio quality of an S&F neighbor cell is better than that of a non S&F neighbor cell, or even significantly better, should S&F neighbor cell still be prioritized or preferred? What is the combination of conditions corresponding to the turning point between "prioritized" and "not prioritized"? These things are still unclear. Therefore, we think it’s not easy to capture relevant UE behavior for this neighbor cell S&F mode information.

So, RAN2 might can only agree this working assumption, then only introduce S&F mode indication per neighbor cell in SIB33(-NB), but do not specify anything in TS 36.304 specification on how the UE would use this information.

Proposal 2: RAN2 can agree that only S&F mode indication is introduced per neighbor cell in SIB33(-NB), but we do not specify anything in TS 36.304 specification on how the UE would use this information.

2.2.2 Paging enhancements

During post meeting TS 36.304 running CR review, some companies show interests on paging monitoring relaxation for SF mode. Companies assumed that when satellite is in S&F mode and there is no stored DL data on it, UE in idle sate can have paging relaxation or not monitor at all. So the following proposal is given for next meeting discussion:

Proposal 4: Paging monitoring relaxation for cell operating in SF mode without any stored DL data can be considered for Rel-19 based on assessment of required spec changes. Required spec changes are evaluated based on text proposal from interested companies on the above issue.

We can understand the intention of such paging optimization. However, after further thinking, we believe this paging relaxation is unreasonable with following aspects:

- Basically, an eNB operating in S&F mode is indeed possible to have stored DL data to transmit and therefore still needs to page the UE. If the UE stops monitoring paging throughout the entire S&F operation period, even though it may achieve some power saving gain, the cost is being completely unable to receive DL traffic. We think this is unacceptable.

- If the UE only intermittently stops monitoring paging based on UE implementation, as eNB has no any idea on the paging monitoring stop and resumption, such behavior is also unacceptable from the paging mechanism perspective, unless the UE explicitly notifies the eNB when it stops and resumes monitoring paging. Again, explicit notification from UE also seems unacceptable as it cause unnecessary UE power consumption and signaling overhead.

Proposal 3: As an eNB operating in S&F mode is indeed possible to have stored DL data and therefore still needs to page the UE, UE is not allowed to relax or even stop paging monitoring within the cell operating in S&F mode.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1: It’s suggested that when t-ModeSwitching is received and forwarded to upper layers, the stored sf-OperationMode information should also be forwarded to upper layers together.

Proposal 2: RAN2 can agree that only S&F mode indication is introduced per neighbor cell in SIB33(-NB), but we do not specify anything in TS 36.304 specification on how the UE would use this information.

Proposal 3: As an eNB operating in S&F mode is indeed possible to have stored DL data and therefore still needs to page the UE, UE is not allowed to relax or even stop paging monitoring within the cell operating in S&F mode.

Reference

[1] RP-250472, Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. March, 2025

[2] TS 23.401, Evolved Universal Terrestrial Radio Access Network, (E-UTRAN) access (Release 19) V19.1.0, December 2024