3GPP TSG-RAN WG2 Meeting #131                                        R2-2505258

Bangalore, India, Aug 25th – 29th, 2025

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues for CB-msg3-EDT

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed. The WID is further refined and the latest one is approved in [1]. The main objectives related to support of capacity enhancements for uplink are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In previous meetings, RAN2 has made much progress on the CB-msg3-EDT procedure. The related agreements can be found in the Annex. In this document, we will discuss a few remaining issues for CB-msg3-EDT procedure. Based on the discussion, some proposals are given.

Discussion

MAC open issues

2.1.1 CB-RNTI calculation

In last meeting, RAN2 has made working assumption on the CB-RNTI formula as below:

RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id.

- X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT,

- Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19,

- Y is ceil (Msg4_WS/Msg3_WP),

- CE_level is the CE level, 0 <= CE_level < 3

- carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0,

- 0 <= carrier_id < 16

During the post meeting email discussion “[Post130][307][R19 IoT NTN] MAC CR”, majority companies agree to confirm the general format of above formula, while there are still two discussion points as below.

- Whether to use Msg3_W_index

The current factor of Msg3_W_index is descriptive in nature and just counted by UE. However, with reference to legacy, e.g., RA-RNTI calculation, it seldom directly introduces parameters such as the window index, instead, the formula typically consists of exact time-domain or frequency-domain resource factors or functions that include these factors. Based on such observation, it’s suggested to leverage the factor “floor(SFN_id/4)” in one of legacy RA-RNTI formula, e.g., the below RA-RNTI for NB-IoT case. By this way, the exact position information of the CB-Msg3 transmission window can also be indirectly obtained, and this way is better as it’s based on a clear mathematical calculation.

TS 36.321: For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0. For NB-IoT UEs operating in TDD mode, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2) where SFN_id is the index of the first radio frame of the specified PRACH and H-SFN is the index of the first hyper frame of the specified PRACH. The PDCCH transmission and the PRACH resource are on the same carrier.

In the factor [floor(SFN_id/4)], 4 corresponds to the minimum PRACH resource periodicity, so similarly, for CB-RNTI, the factor can be [floor(start SFN_id of Tx window/minimum Tx window periodicity)]. The only difference is that descriptive Msg3_W_index seems to be derived from the actual configuration (i.e., Msg3_W_index seems to be equivalent to [floor(start SFN_id of Tx window /configured Tx window periodicity). Meanwhile, in [floor(start SFN_id of Tx window / minimum Tx window periodicity)], since the minimum Tx window periodicity is used, the calculated Tx window index value may be larger than the actual index. Nevertheless, this does not affect the desired outcome where UEs selecting different Tx windows can have distinct Tx window information for the CB-RNTI calculation. Moreover, the factor [floor(start SFN_id of Tx window / minimum Tx window periodicity)] is more general that can adapt to different configurations.

Moreover, due to that two separate parameters, i.e., window size and window periodicity, have been introduced for defining CB-Msg3 transmission window, when the two parameters are different, for example, window size is shorted than window periodicity, the windows, or the boundaries between windows are not adjacent, but the window periods are contiguous (adjacent), this can be referred to the Figure 1 below. Therefore, it is incorrect to divide start SFN_id by the window size of Tx window, instead, one should divide by the periodicity of Tx window.

Figure 1

Proposal 1a: It’s suggested to use “floor (start SFN_id of Tx window/minimum Tx window periodicity)” to replace Msg3_W_index in the baseline CB-RNTI calculation formula.

Proposal 1b: After determination on the value range of RRC parameter windowPeriodicity-r19->cb-Msg3-TxWindow-r19, the “minimum Tx window size” can be replaced with the minimum value in this value range (generally with unit of 10ms).

- How to define Y

Per our understanding, the “modulo (Y) and Y = ceil (maximum Msg4 window size/minimum Msg3 window periodicity)” operation is designed to further compress the Tx window index values selected by the UEs, meanwhile, it also needs to guarantee that there is no same CB-RNTI when the CB-Msg4 response windows overlap.

During the RRC running CR review, we give some analysis for the case of small Y, that is, if two UEs select different Tx windows (for example, UE1 selects Tx window index 0 and UE2 selects Tx window index 2), and their Msg4 reception windows will not overlap (e.g., when Msg4 window size shorter than Msg3 window periodicity, or when Msg4 window size is longer than Msg3 window periodicity but shorter than or equal to two times of Msg3 window periodicity), it’s allowed to use the modulo (Y) operation to compress their indices into the same value. For example, if ceil (maximum Msg4 window size / minimum Msg3 window periodicity) = 2, UEs selecting Tx window index 0 and index 2 can both be adjusted to be with Tx window index 0. Even if other parameters (e.g., CE-level, carrier-id) are identical, the isolation of their Msg4 reception windows ensures they will not falsely detect each other's CB-RNTI.

However, from a mathematical formula perspective, in order to provide a deterministic coefficient for subsequent factors, it cannot use configured values of the relevant parameters to calculate Y—only the maximum or minimum values of the parameters can be used. Although the CB-Msg3 Tx window periodicity is currently still FFS, considering that for the case where configured number of replicas is 1, the CB-Msg3 Tx window might be allowed to equal to or close to the PUSCH resources periodicity, it can be assumed that the minimum value of Msg3 Tx window periodicity can be very small, e.g., about a few or tens of milliseconds. Meanwhile, the value range for Msg4 window (e.g., Msg3 response window) has already been captured and the maximum value is kind of big, e.g., about a few or tens of seconds (in practical deployment, such a large value may not be necessary to configure, as the Msg4 reception window starts relatively late). Therefore, if using the maximum Msg4 window size and minimum Msg3 window periodicity to calculate Y, Y may be very large. As a result, within the entire part “[floor (start SFN_id of Tx window/minimum Tx window periodicity)] modulo (Y)”, the modulo (Y) operation becomes useless, and what actually matters is the calculation related to SFN_id, which also determines the maximum value of this part. Hence, we think to introduce such part of “modulo (Y) and Y = ceil (maximum Msg4 window size/minimum Msg3 window periodicity)” is not so reasonable and not needed.

Observation 1: From formula perspective, in order to provide a deterministic coefficient for subsequent factors, it cannot use configured values of the relevant parameters to calculate Y—only the maximum or minimum values of the relevant parameters can be used. Then as a result, within the entire part “[floor (start SFN_id of Tx window/minimum Tx window periodicity)] modulo (Y)”, the modulo (Y) operation becomes useless.

Some companies have proposed introducing a fixed Y value instead. We think its usefulness is also doubtful. For example, if in the real deployment, the Msg3 Tx window is small while the Msg4 window is large, their ratio may exceed 16, in such cases, the modulo (Y) operation may result in compressed CB-RNTIs being identical, and they may even appear within overlapping Msg4 windows, causing collision. So the safer way is to keep the factor of SFN-id as it is and no need to further compressed, that is, no need to introduce any factor to adjust the result of [floor (start SFN_id of Tx window/minimum Tx window periodicity)].

Therefore,

Proposal 1c: It’s no need to introduce any factor (e.g., modulo (Y)) to adjust the result of [floor (start SFN_id of Tx window/minimum Tx window periodicity)]. So the final formula can be like:

RNTI=X + [floor (start SFN_id of Tx window/minimum Tx window periodicity)] + Y*CE_level + 3*Y*carrier_id

2.1.2 Details of Backoff/Fallback schemes for CB-Msg3-EDT

RAN2 has agreed that UEs that select the same Msg3 Tx window will monitor the same CB-RNTI. In some cases, if many UEs select a same Msg3 Tx window, some may transmit Msg3 successfully, while some may fail. Both successful and unsuccessful UEs will monitor the same CB-RNTI and may see the same Backoff Indicator in Msg4. Then the UEs which have successfully transmitted Msg3 may also think it fails and needs to backoff to another time to re-access, e.g., no longer waiting for Msg4. This is also a typical issue with using a Single Msg4 monitoring window and Single RNTI that only takes into account the start transmit resource of the transmission window - the original filtering function by RNTI is faded away (taking legacy RA-RNTI as example, it generally includes as many resource location information as possible, thereby enabling the differentiation of UEs that trigger access using different resources. In a summary, merely including waiting time information in the Backoff Indicator isn’t sufficient.

So in order to only trigger the unsuccessful UE to backoff a certain time period and access again by using CB-Msg3-EDT procedure, the Backoff Indicator in Msg4 also needs to include the information for targeting those unsuccessful UEs. A straightforward way is to include the resource information of the transmission location in the Backoff Indicator, if a collision is detected at that transmission position by the eNB. In other word, such resource information of the transmission location that identifies the time occasion, frequency occasion and OCC sequence can be used to distinguish UEs that had conflicts or failed CB-msg3 transmissions. Hence, Backoff Indicator also needs to include the information of time occasion, frequency occasion and/or OCC sequence where collisions are detected.

Proposal 2a: For CB-msg3 EDT procedure, Backoff Indicator also needs to include the information of time occasion, frequency occasion where collisions are detected.

In last meeting, RAN2 has agreed that when max re-attempt number for current CE level has been reached, the UE does not move to the next CE level. That means UE can only try maximum numbers of CB-Msg3-EDT procedure on current CE level. If maximum number is reached, UE can consider failure of CB-Msg3-EDT procedure.

Furthermore, considering the decoding requirements of CB-msg3 are more stringent than PRACH, the reception condition of PRACH may still be satisfied even when CB-Msg3-EDT procedure finally fails. So switching to RACH based procedure could increase the successful access probability. Hence, fallback to RACH based EDT procedure is feasible and also needs to be supported.

Proposal 2b: When UE considers failure of CB-Msg3-EDT procedure, e.g., after maximum attempt number in a certain CE level, UE can fallback to legacy RACH based EDT procedure.

RRC open issues

2.2.1 Sets for frequency-domain resources configuration

In RRC running CR, it’s still FFS for the frequency-domain resources configuration as below.

- – CB-Msg3-ConfigSIB The IE CB-Msg3-ConfigSIB is used to specify the CB-Msg3-EDT configuration. CB-Msg3-ConfigSIB information element -- ASN1START <skip unrelated parts> CB-Msg3-PUSCH-Config-r19 ::= SEQUENCE { numRUs-r19 BIT STRING (SIZE(2)), prb-AllocationInfoSet-r19 SEQUENCE (SIZE(1..FFS)) OF BIT STRING (SIZE(10)), mcs-r19 BIT STRING (SIZE(4)), numRepetitions-r19 BIT STRING (SIZE(3)), p0-UE-PUSCH-r19 INTEGER (-8..7), alpha-r19 Alpha-r12 } -- ASN1STOP - – CB-Msg3-ConfigSIB-NB The IE CB-Msg3-ConfigSIB-NB is used to specify the CB-Msg3-EDT configuration. CB-Msg3-ConfigSIB-NB information element -- ASN1START CB-Msg3-ConfigSIB-NB-r19 ::= SEQUENCE { <skip unrelated parts> cb-Msg3-PhysicalConfig-r19 ::= SEQUENCE { npusch-NumRUsIndex-r19 INTEGER (0..7), npusch-NumRepetitionsIndex-r19 INTEGER (0..7), npusch-SubCarrierSetList-r19 SEQUENCE (SIZE(1..FFS)) OF NPUSCH-SubCarrierSet-r19, npusch-MCS-r19 CHOICE { singleTone INTEGER (0..10), multiTone INTEGER (0..13) }, ack-NumRepetitions-NB-r19 ACK-NACK-NumRepetitions-NB-r13 OPTIONAL, --Need OP p0-UE-NPUSCH-r19 INTEGER (-8..7), alpha-NB-r19 ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}, npdcch-CarrierIndex-r19 INTEGER (1..maxNonAnchorCarriers-NB-r14) OPTIONAL, -- Need OP npdcch-NumRepetitions-r19 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1}, npdcch-StartSF-CSS-r19 ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}, npdcch-Offset-CSS-r19 ENUMERATED {zero, oneEighth, oneFourth, threeEighth} }, <skip unrelated parts> ... } -- ASN1STOP ------------

As the details of the frequency resources are defined by RAN1, we think RAN2 can simply define a number for the set, e.g., 3 with main consideration on certain level UE distribution. RAN2 can further inform RAN1 about the value to let RAN1 confirm.

Furthermore, we think it needs to further consider for NB-IoT that, whether two separate lists of sets of subcarriers need to be configured for single-tone and multi-tone transmission.

Proposal 3: It’s suggested to use value 3 as the number of frequency-domain resources set for both eMTC and NB-IoT.

2.2.2 The configuration for CB-msg3 transmission window

In RRC running CR as below, it’s still FFS for the detailed configuration of CB-msg3 transmission window:

- – CB-Msg3-ConfigSIB The IE CB-Msg3-ConfigSIB is used to specify the CB-Msg3-EDT configuration. CB-Msg3-ConfigSIB information element -- ASN1START <skip unrelated parts> CB-Msg3-Config-r19 ::= SEQUENCE { cb-Msg3-TBS-r19 ENUMERATED {b328, b408, b504, b600, b712, b808, b936, b1000}, cb-Msg3-NumOfReplicas-r19 INTEGER(1..4), cb-Msg3-TimeResource-r19 SEQUENCE { pusch-Periodicity-r19 ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, sf256}, pusch-StartSFN-r19 INTEGER (0..1023), pusch-StartSubframe-r19 INTEGER (0..9) }, cb-Msg3-MPDCCH-Config-r19 CB-Msg3-MPDCCH-Config-r19, cb-Msg3-PUCCH-Config-r19 CB-Msg3-PUCCH-Config-r19, cb-Msg3-PUSCH-Config-r19 CB-Msg3-PUSCH-Config-r19, cb-Msg3-TxWindow-r19 SEQUENCE { windowSize-r19 ENUMERATED {FFS}, windowPeriodicity-r19 ENUMERATED {FFS} } OPTIONAL, --Need OP cb-Msg3-ResponseWindow-r19 ENUMERATED {sf240, sf480, sf960, sf1920, sf3840, sf5760, sf7680, sf10240}, cb-Msg3-MaxAttemptNum-r19 ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n10} OPTIONAL, --Need OP, ... } <skip unrelated parts> -- ASN1STOP - – CB-Msg3-ConfigSIB-NB The IE CB-Msg3-ConfigSIB-NB is used to specify the CB-Msg3-EDT configuration. CB-Msg3-ConfigSIB-NB information element -- ASN1START <skip unrelated parts> CB-Msg3-Config-NB-r19 ::= SEQUENCE { <skip unrelated parts> cb-Msg3-TxWindow-NB-r19 SEQUENCE { windowSize-NB-r19 ENUMERATED {FFS}, windowPeriodicity-NB-r19 ENUMERATED {FFS} } OPTIONAL, --Need OP cb-Msg3-ResponseWindow-NB-r19 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}, cb-Msg3-MaxAttemptNum-NB-r19 ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n10} OPTIONAL, --Need OP ... } <skip unrelated parts> -- ASN1STOP ------------

For supporting the transmission of multiple replicas of one Msg3 packet, a CB-msg3 transmission window has been agreed. According to the captured signaling in the RRC running CR, there is no explicit configuration of start time of Msg3 Tx window, it means that the start time of Msg3 Tx window aligns with (is same as) the start of time domain (N)PUSCH resource.

In previous meeting, there was some discussion on how to define the Msg3 Tx window size but no agreement. Considering the following two aspects:

- The purpose of introducing this window is to provide a suitably sized resource pool with sufficient transmission occasions from which UEs can randomly select the required number of resources for transmitting multiple Msg3 replicas, trying to minimize collisions between different UEs' selections;

- If an absolute time duration is configured for the length of the transmission window, the time duration couldn’t guarantee to include multiple complete (N)PUSCH transmission occasions;

We think the most suitable way is to define the length of CB-msg3 transmission window with the unit of (N)PUSCH resources periodicity.

Proposal 4a: It’s suggested to define the length of CB-msg3 transmission window in units of (N)PUSCH resources periodicity.

Furthermore, typically, in order to provide sufficient randomness or dispersion, the number of resources in the resource pool can be several times greater than the required number of resources (e.g., the number of replicas), so we think a multiplication factor for parameter of replicas, e.g., K can be introduced, and the final value of CB-msg3 transmission window size in units of periodicity of (N)PUSCH resource can be the result of [(the number of replicas) multiplied by the factor of K]. Considering that the resource pool also cannot be too large that would cause resource waste, it’s suggested to define the maximum of K to be 4.

Proposal 4b: It’s suggested to introduce a multiplication factor for parameter of replicas, e.g., K as the configurable parameter for the length of CB-msg3 transmission window. The final value of the length of CB-msg3 transmission window is the result of [(the number of replicas) multiplied by the factor of K], that is:

Length of CB-msg3 transmission window = [(the number of replicas) * K] * (N)PUSCH resources periodicity

Proposal 4c: The number of K can be configured by the NW with the maximum value of 4.

In last meeting, RAN2 also agree to introduce another parameter, e.g. the periodicity of CB-msg3 transmission window, which can be same as or different from the length of CB-msg3 transmission window. When the periodicity of CB-msg3 transmission window is different from the window length, it means there is “empty” space between two consecutive windows, which can be used to schedule other services.

For simplicity, we think the periodicity of CB-msg3 transmission window only needs to be one or only a few periodicities of (N)PUSCH resource longer than the length of CB-msg3 transmission window. Therefore, it is also suggested to configure the periodicity of CB-msg3 transmission window as a value in units of (N)PUSCH resources periodicity.

Proposal 4d: It’s suggested to also define the periodicity of CB-msg3 transmission window as a value in units of (N)PUSCH resources periodicity.

Observation: Compared with the way to define the length and the periodicity of CB-msg3 transmission window directly in time units, e.g., ms, the proposed way in above Proposal 2a~2d of configuring the length and the periodicity of CB-msg3 transmission window, e.g., in units of (N)PUSCH resources periodicity, can offer the benefit of reducing signaling overhead.

The following Figure 1 gives an example to explain the way for configuring the length and the periodicity of CB-msg3 transmission window:

Figure 2

2.2.3 MO signaling via CB-msg3-EDT

During RRC running CR discussion, one company suggested to discuss whether MO signaling is allowed to transmission by using CB-Msg3-EDT procedure. One example use case is performing tracking area update where UE does not have to transition to connected mode.

From our memory, at the very early stage of R15 EDT discussion, this issue has been touched but finally companies agreed that EDT procedure is not applicable for UE that needs to perform NAS procedures. The main reason seems to be that NAS procedures generally require more than one packet to transmit in UL/DL.

We think this is still the case for CB-msg3-EDT, so it’s no need to support NAS signaling transmission by using CB-Msg3-EDT procedure.

Proposal 5: Considering NAS signaling procedure may involve multiple UL/DL data packets exchange, it’s no need to support NAS signaling transmission by using CB-Msg3-EDT procedure.

Other issue

2.3.1 Resources configuration and selection for the case of replica = 1

When configured number of replica equals to 1, since only one replica of Msg3 packet needs to be transmitted, there is no need to configure the CB-msg3 EDT transmission window. Similar as the preamble transmission, the UE can select the nearest (N)PUSCH resource for CB-msg3 transmission. Furthermore, as the UE has no other replica to be transmitted, it’s feasible for the UE to start the Msg4 monitoring window as early as possible, e.g., after the end of last repetition of CB-msg3 transmission (e.g., the end of (N)PUSCH transmission occasion) plus UE-eNB RTT. We see this option (Option 1) is the most suitable way for CB-msg3 transmission when the configured replica =1.

There may be some thoughts that similar CB-msg3 EDT transmission window as that for the case of replica larger than 1 can be configured. However, we see both of the following possible ways have problems, so neither of them is preferred:

- (Option 2) to configure a CB-msg3 EDT transmission window with only one periodicity of (N)PUSCH resource or only one (N)PUSCH resource: this option has same resource selection result as Option 1 (e.g., to select the only and also the nearest (N)PUSCH resource), but has a disadvantage of additional delay if the Msg4 monitoring window starts after the end of the CB-msg3 transmission window plus UE-eNB RTT.

- (Option 3) to configure a CB-msg3 EDT transmission window with several periodicities of (N)PUSCH resource: That means UE can randomly select a (N)PUSCH resource from multiple (N)PUSCH resources for the only one Msg3 replica transmission. At first glance, this option seems to reduce the probability of collisions among multiple UEs initiating Msg3 transmission at the same time, as they may select different (N)PUSCH resources within a certain range. However, this is not actually the case — these UEs can still collide with other UEs that trigger services at different times. Moreover, another issue arises that, UEs cannot always choose the nearest (N)PUSCH resource to initiate the Msg3 transmission, which introduces unnecessary latency.

Based on the above analysis, Option 1 is preferred.

Proposal 6: When configured number of replica equals to 1, there is no need to configure the CB-msg3 EDT transmission window. The UE can select the nearest (N)PUSCH resource for CB-msg3 transmission. Furthermore, UE starts the Msg4 monitoring window after the end of last repetition of CB-msg3 transmission (e.g., the end of (N)PUSCH transmission occasion) plus UE-eNB RTT.

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1a: It’s suggested to use “floor (start SFN_id of Tx window/minimum Tx window periodicity)” to replace Msg3_W_index in the baseline CB-RNTI calculation formula.

Proposal 1b: After determination on the value range of RRC parameter windowPeriodicity-r19->cb-Msg3-TxWindow-r19, the “minimum Tx window size” can be replaced with the minimum value in this value range (generally with unit of 10ms).

Proposal 1c: It’s no need to introduce any factor (e.g., modulo (Y)) to adjust the result of [floor (start SFN_id of Tx window/minimum Tx window periodicity)]. So the final formula can be like:

RNTI=X + [floor (start SFN_id of Tx window/minimum Tx window periodicity)] + Y*CE_level + 3*Y*carrier_id

Proposal 2a: For CB-msg3 EDT procedure, Backoff Indicator also needs to include the information of time occasion, frequency occasion where collisions are detected.

Proposal 2b: When UE considers failure of CB-Msg3-EDT procedure, e.g., after maximum attempt number in a certain CE level, UE can fallback to legacy RACH based EDT procedure.

Proposal 3: It’s suggested to use value 3 as the number of frequency-domain resources set for both eMTC and NB-IoT.

Proposal 4a: It’s suggested to define the length of CB-msg3 transmission window in units of (N)PUSCH resources periodicity.

Proposal 4b: It’s suggested to introduce a multiplication factor for parameter of replicas, e.g., K as the configurable parameter for the length of CB-msg3 transmission window. The final value of the length of CB-msg3 transmission window is the result of [(the number of replicas) multiplied by the factor of K], that is:

Length of CB-msg3 transmission window = [(the number of replicas) * K] * (N)PUSCH resources periodicity

Proposal 4c: The number of K can be configured by the NW with the maximum value of 4.

Proposal 4d: It’s suggested to also define the periodicity of CB-msg3 transmission window as a value in units of (N)PUSCH resources periodicity.

Proposal 5: Considering NAS signaling procedure may involve multiple UL/DL data packets exchange, it’s no need to support NAS signaling transmission by using CB-Msg3-EDT procedure.

Proposal 6: When configured number of replica equals to 1, there is no need to configure the CB-msg3 EDT transmission window. The UE can select the nearest (N)PUSCH resource for CB-msg3 transmission. Furthermore, UE starts the Msg4 monitoring window after the end of last repetition of CB-msg3 transmission (e.g., the end of (N)PUSCH transmission occasion) plus UE-eNB RTT.

Reference

- RP-250472 Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#107, March 2025

Annex

For reference, the agreements in previous RAN2#125bis~RAN2#130 meetings are listed as below:

RAN2#125bis agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements RAN2#126 agreements: - RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). - RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) - If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3. RAN2#127 agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism. - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition RAN2#127bis agreements: - RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA) - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA) - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA) - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN. RAN2#128 agreements: - Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs) - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource. - RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started) RAN2#129 agreements: - It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used - RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC - RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects) - Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN) - Frequency domain resources for (N)PUSCH occasions - repetition number - (N)PDCCH resource - MCS - For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels. - The UE shall at most have one ongoing CB EDT procedure at any time. - The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT. - The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels. - CB EDT Config has three RSRP thresholds for eMTC for the four CE levels. - As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config. - RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature Working assumption: - One CB-MSG4 can target multiple UEs simultaneously (FFS on the details) - For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today The UE first selects the next DSA transmission window and then randomly select K replicasinside the window. RAN2 assumes that a pointer solution is not needed in Rel-19 RAN2#129bis agreements: Agreements (part 1) - Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT - We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure - For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). - We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case) - We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation. - If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission. - The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource. - The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same. - RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply (CB-Msg3-EDT configuration for eMTC) - For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2. - For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed). - We will not support TDD related parameters. - For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources). - For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration - For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (CB-Msg3 configuration for NB-IoT) - For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier. - For NB-IoT, introduce below physical layer parameters in shared resources configuration as below: - Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16) - Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16) - Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set. - MCS configuration for NPUSCH (as in npusch-MCS-r16). - PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13) - The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used. NOTE: confirm with RAN1 is needed - For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration Agreements (part 2) - For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). - The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure) - The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not). - FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring Agreements (part 3) - The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific. - The Msg4 monitoring window configuration (e.g. length) is CE level specific - RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized. - RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT - For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject - For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject. - RAN2 also intends to support CB-msg3-EDT for MT cases - The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI) - Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1 - The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4. - Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached. - Backoff information could be included in CB-Msg4. - L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported. - HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.). RAN2#130 agreements: Agreements-part1: - The maximum TBS could be different for different CE levels. - Due to only CE mode A is supported for eMTC NTN, only 1 separate RSRP thresholds and 2 CE levels are supported (revised agreement) - Multiple contention resolution IDs could be included in CB-MSG4, the information related to multiple UEs can be multiplexed in the MAC PDU. - The number of Msg3 replies in one Msg4 can be left to eNB implementation. Expect no SPEC impact. - The HARQ feedback resource information can be included in the CB-Msg4 together with contention resolution ID which identity the specific UE. RAN2 could revisit this proposal if RAN1 has some concern. - Whether to send the HARQ feedback for CB-Msg4 can be controlled by NW. UE does not send HARQ NACK. - For NB-IoT, the SubCarrierSpacing of the HARQ feedback for CB-Msg4 is same as the CB-Msg3. - Reuse the existing format of HARQ ACK allocation signalling in the DCI. There is 2-bit HARQ ACK resource for eMTC and 4-bit HARQ ACK resource for NB-IoT. Reuse the meaning of DCI field in R1 SPEC. Send LS to RAN1 for information on all RAN2 decisions related to HARQ feedback - Introduce a new MAC PDU for CB-Msg4 including new types of MAC sub-header and a new type of MAC payload - The MAC PDU for CB-Msg4 consists of sub-header(s) followed by MAC payload and optional padding if needed. - Introduce a new CB BI MAC sub-header in CB-MSg4 for backoff parameter. There is 4 bits BI for backoff indication. - Introduce a new CB-Msg3 Response (CBR) MAC sub-header in CB-Msg4. It has 1bit E for sub-header/payload indication, 2 bits T for sub-header type, 1bit T2 for HARQ ACK resource present, 1 bit T3 for TAC present, 1 bit T4 for C-RNTI present and 2bit R for reservation (the field names and sub-header names in above agreements could be further changed during MAC running CR review) Agreements-part2: - The CB-Msg3-EDT configuration is configured per carrier (including anchor and non-anchor carrier). Within each carrier, the CB-Msg3-EDT could be configured per CE level - Regarding the mapping of NPUSCH resource to enhanced coverage levels, enhanced coverage levels are numbered from 0 and the mapping of NPUSCH resources to enhanced coverage levels are done in increasing [number of repetition] order (as legacy RACH) - For NB-IoT, when multiple carriers provide CB-Msg3-EDT resources for the same enhanced coverage level, the NB-IoT UE selects the carrier based on the probabilities of each carrier. A new probability parameter for anchor carrier is introduced in SIB22-NB. The remaining probability is evenly split among the non-anchor carriers. - When max re-attempt number for current CE level has been reached, the UE does not move to the next CE level (FFS on the details of the failure behaviour) - The TAC is optionally used in the CB-Msg3 response. - RAN2 assumes that NTA=0 for initial CB-msg3 transmission. Include this in the LS to RAN1 and RAN4 - RAN2 assumes the length of the TAC field is 6 bits (we can revisit this if there is major R1 impact on TA calculation) Agreements-part3: - Introduce a new CB Data MAC sub-header in CB-MSg4 for MAC SDU for logical channel data. It has 1 bit E for subhead/payload indication, 2 bits T for subhead type, 5 bits LCID, 7 bits or 15 bits L for MAC SDU length, 1 bit F for 15 bits L indication. There is one L field per CB Data sub-header except for the last sub-header. - Introduce a new CB-Msg3 Response (CBR) with variable length. It has 48-bit contention resolution ID, optional HARQ ACK, optional TAC, optional 16-bit C-RNTI. Agreements: - HARQ process 0 is used to transmit all the CB-Msg3 replicas in the transmission window (RV0 is used to transmit the first repetition of each CB-Msg3 replica in the transmission window) - We do not specify another way of starting Msg4 monitoring window, i.e. it is confirmed that the Msg4 monitoring window always starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not) - A CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution. Working Assumption: The formula for RNTI for mMsg4 monitoring is: RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id. - X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT, - Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19, - Y is ceil (Msg4_WS/Msg3_WP), - CE_level is the CE level, 0 <= CE_level < 3 - carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0, - 0 <= carrier_id < 16 Can come back to check if the NW can also simply configure RNTI = X Agreements – part 2: - The value of X is 4097 for NB-IoT and 2401 for eMTC - The value of Msg4_WS is the maximum Msg4 window size - The value of Msg3_WP is the minimum Msg3 window periodicity