3GPP TSG RAN WG2 Meeting #131                                                            R2-2505265

# Bangalore, India, August 25th - 29th, 2025

Agenda item:	8.2.4

Source:	Ofinno

Title:	Remaining issues on A-IoT Data Transmission

Document for:	Discussion and decision

# Introduction

This document discusses how the UE’s monitoring behaviour for the R2D Upper Layer Data Transfer messages (MAC running CR captures) and the success/failure of D2R Upper Layer Data Transfer message (which was a new issue raised during email discussion [POST129bis][017]).

# Discussion

## Monitoring of R2D Upper Layer Data Transfer messages

In our understanding, there is no clear description of the device’s operation on when it should monitor for R2D Upper Layer Data Transfer messages. This could easily be addressed by clarifying the device’s behaviour for the transmission or reception of upper layer data. For example, as shown in below suggested TP highlighting:

- [TP#1] Purpose of the A-IoT upper layer data procedures (i.e., transmission and reception of upper layer and segmentation support only for D2R Upper Layer Data Transfer message) [TP #2 highlighted in yellow].

- [TP#2] Monitoring conditions for R2D Upper Layer Data Transfer messages (i.e., after successful completion of access procedure in accordance to clause 5.3, and while the AS ID is stored (i.e. not released) or for the reception of the first R2D message after successfully initiating Contention-Free Access procedure) [TP#2 highlighted in green]. In our companion RACH TDoc [4], we explain the motivation and propose adding in the procedural text within clause 5.3 when the Random procedure is considered as successful.

## 5.4 A-IoT upper layer data procedures transmission The purpose of this procedure is for a device to transmit or receive upper layer data. The segmentation is only supported for D2R Upper Layer Data Transfer message. After the access procedure is considered successful in accordance to clause 5.3, the device monitors for R2D Upper Layer Data Transfer messages while the AS ID is stored (i.e. not released) or for the reception of the first R2D Upper Layer Data Transfer message after successfully initiating Contention-Free Access procedure. ### 5.4.1 D2R message transmission Upon initiation of the procedure, the A-IoT MAC entity shall: …. ### 5.4.2 R2D message reception Once a R2D Upper Layer Data Transfer message is received, the A-IoT MAC entity shall: …

- The A-IoT upper layer data procedures (MAC running CR §5.4) is updated to describe the purpose of this procedure and device’s monitoring behaviour for R2D Upper Layer Data Transfer messages (with example TPs shown within this document), highlighting the following:

  - [TP#1] The Purpose of the A-IoT upper layer data procedures (i.e., transmission and reception of upper layer and segmentation support only for D2R Upper Layer Data Transfer message).

  - [TP#2] The monitoring conditions for the R2D Upper Layer Data Transfer messages (i.e., after the access procedure is considered successful in accordance to clause 5.3, and while the AS ID is stored (i.e. not released) or for the reception of the first R2D Upper Layer Data Transfer message after successfully initiating Contention-Free Access procedure).

## [New issue] Success/failure of D2R message

In our understanding, the UE behaviour considering the success and failure scenarios upon transmission of any D2R message (even after Msg3) requires some discussion to avoid monitoring for R2D message unnecessarily and reduce device’s power consumption. The following RAN2 agreements are taken for the NACK based mechanism (for MSG3) in relation to CBRA and CFRA:

[Agreements (RAN2#128)] 1. RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID 2. If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported). [Agreements (RAN2 #129)] 1. NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback. [Agreements (RAN2 #129)] 1 For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). 2 For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging 3 For CFRA, NACK feedback and re-access is not supported. FFS how to achieve 4 FFS on end of procedure. [Agreements (RAN2 #130)] NACK feedback is defined as an explicit message (i.e. new message type). AS ID(s) is/are included to indicate the failure for given device(s). Multiplexing of NACK feedback is supported in one message After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access. FFS how to specify.

RAN2 has still not concluded on whether a device needs (or not) any mechanism or timer to be used for the device to confirm the success or failure after transmitting its D2R message. According to RAN2 agreements of previous meetings, there is no way to know whether a D2R transmission is successful from the device point of view (PoV). The first option that we can have in Rel-19 A-IoT may be that the device should always consider a D2R transmission successful immediately after the D2R transmission. The second option may be that the device should wait until a next R2D message to implicitly or explicitly identify a success or failure of the previously transmitted D2R message. The first option might be simple from the device PoV but might not be very optimum if the reader needs to request the device to retransmit the previously transmitted D2R message at some point in the future. The second option might be simple from the network PoV but might not be ideal from the device PoV as the device might require keeping the previously transmitted D2R message in limbo for long time, e.g. until next/applicable R2D message is received or the ongoing A-IoT procedure is terminated (as success or failure). If so, this would not be ideal from power consumption PoV.

After sending a D2R message (after MSG3), the A-IoT device is not aware whether or when to consider this message successful. This may impact negatively to the A-IoT device (e.g., if it keeps the previously transmitted D2R message until next applicable R2D message is received or the A-IoT procedure is terminated), or to the network (e.g., if the A-IoT device always considers a transmission of a D2R message as successful upon its transmission).

In addition, it is important to clarify that RAN2 agreed not to define a timer in relation to the failure or re-access scenario. However there has no explicit discussion on whether a timer may be used by a device to trigger an implicit success of a previously transmitted D2R message.

It is important to keep also in mind, that RFID also specifies the usage of similar timer/behaviour between consecutive transmissions and receptions defining, when applicable, the minimum, nominal and maximum values. For example, corresponding RFID table/figure [3] which provides linking timing relation and parameters are included in Annex B for reference. It can be observed how RFID technology already defines multiple/different timers for the diverse scenarios and pairs of transmission/reception.

RFID technology already defines multiple/different timers for the diverse scenarios and pairs of transmission/reception (as captured in §6.3.1.6.5 of RFID specification).

Summarizing, in our understanding, the usage of a short time or an end condition can be a simple approach to solve the concerns explained above. Figure 1 provides an overview of this simple mechanism that could be used by the A-IoT device to determine when the transmission of the D2R message is or can be considered successful.

Figure 1. Mechanism to determine if a D2R message is or can be considered successful

The points to highlight of the mechanism shown in Figure 1 are as follows:

- Point (A) Upon transmission of the D2R message (steps [C], [E] and [H] in Figure 1), a condition is defined for the device to consider the transmission of D2R message successful or not.

    - [RAN2 already agreed] After transmission of Msg 1 (steps [E] in Figure 1), upon reception of R2D addressed to the device, the device considers this as successful transmission of Msg1 before next [1st or Kth] Access Trigger message or paging message (which considers it as failure).

    - [RAN2 already agreed] After transmission of Msg 3 (steps [C] in Figure 1), upon reception of R2D Upper Layer Data Transfer addressed to the device, the device considers this as successful transmission of Msg3 before reception of NACK feedback message addressed to the device or next paging message (which considers it as failure or abrupt termination).

    - [Open case] After transmission of D2R Upper Layer Data Transfer (steps [H] in Figure 1), there is no mechanism for the device to consider this transmission as successful until next paging message is received (which triggers the failure of the procedure). A termination condition should be defined by new short timer TD2R_ACK is started immediately or after certain time of transmitting the D2R Upper Layer Data Transfer, referred as TOffset (steps [H] or [H’] in Figure 1).

  - FFS if TOffset is needed (which represents a delay to start the new short timer TD2R_ACK after sending D2R message) and if so, FFS whether this TOffset is same or different to TD→Rmin (defined in TS 38.291).

  - FFS how this new short timer TD2R_ACK is defined as an absolute time or a relative time to one of the R2D messages (e.g. Xth reception of subsequent Access Trigger or A-IoT Paging messages)

- Point (B) While the new timer TD2R_ACK is running, device’s behavior upon reception of the applicable R2D messages (e.g., step [J] in Figure 1) is already defined case by case on A-IoT MAC specification [4]

    - Successful transmission of previous D2R message upon reception of R2D message addressed to the device.

    - Abrupt termination of the A-IoT procedure upon reception of next paging message with a different transaction ID.

- Point (C) Expiry of the new timer TD2R_ACK (e.g., step [I] in Figure 1) means that the device considers implicitly the success (or failure, if preferred) of the previously transmitted D2R message.

- [New issue] To define a mechanism for the device to determine when a transmission of the D2R Upper Layer Data Transfer message is or not considered successful (or failure) via a new “short timer (TD2R_ACK)” which works as follows:

  - To discuss whether this new “short timer (TD2R_ACK)” is defined as an absolute value or relative to one R2D message.

    - If absolute value is preferred, this new “short timer (TD2R_ACK)” starts immediately after the transmission of the D2R message, or after certain time (TOffset which could be same or related to TD→Rmin defined in TS 38.291).

    - If relative value is preferred, this new “short timer (TD2R_ACK)” is defined in terms of the reception of Xth subsequent Access Trigger messages and/or A-IoT Paging messages.

  - Upon expiry of TD2R_ACK (or meeting the condition), the previously transmitted D2R message is considered as successful (or if preferred, failed) by the device.

# Conclusion

The observations captured are the following:

Observation 1.	After sending a D2R message (after MSG3), the A-IoT device is not aware whether or when to consider this message successful. This may impact negatively to the A-IoT device (e.g., if it keeps the previously transmitted D2R message until next applicable R2D message is received or the A-IoT procedure is terminated), or to the network (e.g., if the A-IoT device always considers a transmission of a D2R message as successful upon its transmission).

Observation 2.	RFID technology already defines multiple/different timers for the diverse scenarios and pairs of transmission/reception (as captured in §6.3.1.6.5 of RFID specification).

The proposals captured are the following:

Proposal 1.	The A-IoT upper layer data procedures (MAC running CR §5.4) is updated to describe the purpose of this procedure and device’s monitoring behaviour for R2D Upper Layer Data Transfer messages (with example TPs shown within this document), highlighting the following:

Proposal 1.1.	[TP#1] The Purpose of the A-IoT upper layer data procedures (i.e., transmission and reception of upper layer and segmentation support only for D2R Upper Layer Data Transfer message).

Proposal 1.2.	[TP#2] The monitoring conditions for the R2D Upper Layer Data Transfer messages (i.e., after the access procedure is considered successful in accordance to clause 5.3, and while the AS ID is stored (i.e. not released) or for the reception of the first R2D Upper Layer Data Transfer message after successfully initiating Contention-Free Access procedure).

Proposal 2.	[New issue] To define a mechanism for the device to determine when a transmission of the D2R Upper Layer Data Transfer message is or not considered successful (or failure) via a new “short timer (TD2R_ACK)” which works as follows:

Proposal 2.1.	To discuss whether this new “short timer (TD2R_ACK)” is defined as an absolute value or relative to one R2D message.

Proposal 2.1.1.	If absolute value is preferred, this new “short timer (TD2R_ACK)” starts immediately after the transmission of the D2R message, or after certain time (TOffset which could be same or related to TD→Rmin defined in TS 38.291).

Proposal 2.1.2.	If relative value is preferred, this new “short timer (TD2R_ACK)” is defined in terms of the reception of Xth subsequent Access Trigger messages and/or A-IoT Paging messages.

Proposal 2.2.	Upon expiry of TD2R_ACK (or meeting the condition), the previously transmitted D2R message is considered as successful (or if preferred, failed) by the device.

# Reference

- [POST130][027][AIoT], Remaining A-IoT MAC open issues.

- [POST130][027][AIoT], MAC running CR, TS 38.391 (2025-08).

- Specification for RFID Air Interface Protocol for Communications at 860 MHz – 930 MHz, EPC Radio-Frequency Identity Generation-2 UHF RFID Standard, Release 3.0.

- R2-2505264, Remaining issues on A-IoT random access, Ofinno.

# Annex B

Link timing scenarios and parameters captured by RFID specification, §6.3.1.6.5 [3], are copied below for reference.