3GPP TSG-RAN WG2 Meeting #131											R2-2505273

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.7.4

Source: 	Sharp, Ericsson

Title:  	Removing Non-delay-reporting RLC SDU from RLC specification

Document for: 	Discussion

Introduction

In this document, we discussed the Open issue [RLC-13] and proposed to not distinguish Delay-reporting RLC SDU and Non-delay-reporting RLC SDU in RLC but rely on the indication from PDCP as we did for Delay-reporting RLC SDU.

(RLC-13) Whether the delay-reporting or non-delay-reporting information is visible or not to RLC

In current RLC running CR [1], the definition of delay-reporting RLC SDU relies on the indication from PDCP. But the non-delay reporting RLC SDU is defined independently in RLC.

- 3.1 Definitions For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1]. Data field element: An RLC SDU or an RLC SDU segment that is mapped to the Data field. Delay-critical RLC SDU: RLC SDU corresponding to a PDCP PDU indicated as delay-critical by PDCP (see TS 38.323 [4]). Delay-reporting RLC SDU: a delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold is an RLC SDU corresponding to a PDCP PDU indicated by PDCP as delay-reporting associated with the i:th dsr-ReportingThreshold (see TS 38.323 [4]). Editor’s Note: The terminology is to be aligned with other specifications. Non-delay-reporting RLC SDU: a non-delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold is an RLC SDU that will be transmitted prior to any of the delay-reporting RLC SDU(s) associated with the i:th dsr-ReportingThreshold. Editor’s Note: The terminology is to be aligned with other specifications.

Based on current definition, all the descriptions related to non-delay reporting PDCP SDU in PDCP are duplicately captured for non-delay reporting RLC SDU in RLC, which makes the specification looks very complicated.

From RLC perspective, there is no need/benefit to distinguish delay-reporting RLC SDU and non-delay-reporting RLC SDU. RLC can simply rely on current delay reporting indication from PDCP to calculate RLC data volume for DSR. For example, if dsr-ReportNonDelayCriticalData is not configured, PDCP only indicates the delay-reporting PDCP SDUs associated with the i-th dsr-ReportingThreshold as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold. If dsr-ReportNonDelayCriticalData is configured, PDCP indicates both delay-reporting PDCP SDUs and non-delay reporting PDCP PDUs associated with the i-th dsr-ReportingThreshold as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold. Based on the existing delay-reporting indication from PDCP, the non-delay-reporting information can be invisible to RLC and all the description related to non-delay reporting RLC SDU in RLC spec can be removed.

(RLC-13) If dsr-ReportNonDelayCriticalData is configured, PDCP indicates non-delay reporting PDCP PDUs associated with the i:th dsr-ReportingThreshold as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold.

(RLC-13) Remove all the description related to non-delay reporting RLC SDU from RLC specification.

The corresponding TPs f are provided in the Annex.

Conclusions

Based on the discussion above, RAN2 is requested to agree the following proposals:

- (RLC-13) If dsr-ReportNonDelayCriticalData is configured, PDCP indicates non-delay reporting PDCP PDUs associated with the i:th dsr-ReportingThreshold as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold.

(RLC-13) Remove all the description related to non-delay reporting RLC SDU from RLC specification.

Reference

[1] R2-25xxxxx	RLC running CR for R19 XR, vivo

Annex

TP for TS 38.323

--------------------------------------------Start Change of TS38.323---------------------------------------

- 5.15	Data volume calculation for delay status reporting

For the purpose of single entry MAC delay status reporting, the transmitting PDCP entity shall consider the following as delay-critical PDCP data volume:

-	the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed;

-	the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers;

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

The transmitting PDCP entity provides a delay-critical indication for the PDCP Data PDU to lower layers when:

-	the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-critical PDCP SDU; or

-	the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-critical PDCP SDU.

For the purpose of multiple entry MAC delay status reporting, the transmitting PDCP entity shall evaluate the delay-reporting PDCP data volume in ascending order of dsr-ReportingThreshold, and consider the following as delay-reporting PDCP data volume associated with the i:th dsr-ReportingThreshold:

-	the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i;

-	the PDCP Data PDUs that contain the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i;

-	if i = 1, the PDCP Control PDUs;

-	if i = 1, for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	if i = 1, for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

If dsr-ReportNonDelayCriticalData is configured, the transmitting PDCP entity shall further consider the following as delay-reporting PDCP data volume associated with the i:th dsr-ReportingThreshold:

-	the non-delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i;

-	the PDCP Data PDUs that contain the non-delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i.

The transmitting PDCP entity provides a delay-reporting indication associated with the i:th dsr-ReportingThreshold for the PDCP Data PDU to lower layers when:

-	the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold; or

-	the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold.

-	if dsr-ReportNonDelayCriticalData is configured, the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold; or

-	if dsr-ReportNonDelayCriticalData is configured, the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold.

If the transmitting PDCP entity is associated with at least two RLC entities, when indicating the delay-critical PDCP data volume or delay-reporting PDCP data volume to a MAC entity for DSR triggering and Buffer Size calculation (as specified in TS 38.321 [4]), the transmitting PDCP entity shall:

-	if the PDCP duplication is activated for the RB:

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume to the MAC entity associated with the primary RLC entity;

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume excluding the PDCP Control PDU to the MAC entity associated with the RLC entity other than the primary RLC entity activated for PDCP duplication;

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume as 0 to the MAC entity associated with RLC entity deactivated for PDCP duplication;

-	else (i.e. the PDCP duplication is deactivated for the RB):

-	if the split secondary RLC entity is configured; and

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the primary RLC entity and the split secondary RLC entity is equal to or larger than ul-DataSplitThreshold:

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume to both the MAC entity associated with the primary RLC entity and the MAC entity associated with the split secondary RLC entity;

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume as 0 to the MAC entity associated with RLC entity other than the primary RLC entity and the split secondary RLC entity;

-	else:

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume to the MAC entity associated with the primary RLC entity;

-	indicate the delay-critical PDCP data volume or delay-reporting PDCP data volume as 0 to the MAC entity associated with the RLC entity other than the primary RLC entity.

--------------------------------------------End Change of TS38.323---------------------------------------

TP for TS 38.322

---------------------------------------------Start Change of TS38.322---------------------------------------

- 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Data field element: An RLC SDU or an RLC SDU segment that is mapped to the Data field.

Delay-critical RLC SDU: RLC SDU corresponding to a PDCP PDU indicated as delay-critical by PDCP (see TS 38.323 [4]).

Delay-reporting RLC SDU: a delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold is an RLC SDU corresponding to a PDCP PDU indicated by PDCP as delay-reporting associated with the i:th dsr-ReportingThreshold (see TS 38.323 [4]).

Editor’s Note: The terminology is to be aligned with other specifications.

[not related part are omitted]

- 5.5	Data volume calculation

For the purpose of MAC buffer status reporting, the UE shall consider the following as RLC data volume:

-	RLC SDUs and RLC SDU segments that have not yet been included in an RLC data PDU;

-	RLC data PDUs that are pending for initial transmission;

-	RLC data PDUs that are pending for retransmission (RLC AM).

For the purpose of MAC single entry delay status reporting, the UE shall consider the following as delay-critical RLC data volume:

-	delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU;

-	RLC data PDUs pending for initial transmission, and containing a del ay-critical RLC SDU or a delay-critical RLC SDU segment;

-	RLC data PDUs that are pending for retransmission (RLC AM).

For the purpose of MAC multiple entry delay status reporting, the UE shall evaluate the delay-reporting RLC data volume in ascending order of dsr-ReportingThreshold, and consider the following as delay-reporting RLC data volume associated with the i:th dsr-ReportingThreshold in dsr-ReportingThresList, where i starts from 1:

-	delay-reporting RLC SDUs and delay-reporting RLC SDU segments that are associated with the i:th dsr-ReportingThreshold and have not yet been included in an RLC data PDU, and are not considered as delay-reporting RLC data volume associated with any of the k:th dsr-ReportingThreshold where k < i;

-	RLC data PDUs pending for initial transmission, and containing a delay-reporting RLC SDU or a delay-reporting RLC SDU segment associated with the i:th dsr-ReportingThreshold, and that are not considered as delay-reporting RLC data volume associated with any of the k:th dsr-ReportingThreshold where k < i;

-	if i=1, RLC data PDUs that are pending for retransmission (RLC AM).

In addition, if a STATUS PDU has been triggered and t-StatusProhibit is not running or has expired, the UE shall estimate the size of the STATUS PDU that will be transmitted in the next transmission opportunity, and consider this as part of RLC data volume for MAC buffer status reporting, as part of delay-critical RLC data volume for MAC single entry delay status reporting, and as part of the delay-reporting RLC data volume associated with the first (i.e. i=1) dsr-ReportingThreshold for MAC multiple entry delay status reporting.

--------------------------------------------End Change of TS38.322---------------------------------------