3GPP TSG-RAN WG2 Meeting #131	R2-2505311

Bangalore, India Aug 25th – 29th, 2025

# Agenda item:	8.6.2

# Source:	ETRI

# Title:	Remaining issues on inter-CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN#130 meeting, agreements on inter-CU LTM design have been made as the followings [1]:

Agreements on inter-CU LTM - RAN2 agree to not support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell. - If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency. - RAN2 not support CSI-RS based CFRA resources indicated via LTM cell switch command MAC CE. - The Enhanced LTM CSC MAC CE always contains an NCC value field. The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

In this paper, we discuss remaining issues regarding inter -CU LTM.

- Discussion

  - Enhanced LTM CSC MAC CE format

The Enhanced LTM CSC MAC CE includes the NCC field as a mandatory field, as agreed by RAN2 in the last meeting. The Enhanced LTM CSC MAC CE is used when the R19 set ID of the source cell and the target cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

Figure 1 Enhanced LTM CSC MAC CE format

As shown in Figure 1, the NCC is grouped with CFRA-related fields (S/U, Repetition number.) in Octet 5. In RACH-less LTM and security change scenarios, the bits reserved for CFRA are transmitted with the NCC, even though the UE ignores them. This creates unnecessary overhead, reducing bit efficiency. The NCC requires only 3 bits, but a full byte is transmitted due to the extra bits, causing inefficiency. This issue was raised by Ericsson during an email discussion and it was suggested to add the NCC to a new octet and use a spare bit in Octet 3 to indicate if the NCC is present. However, this suggestion conflicts with the RAN2 decision to make the NCC field mandatory.

To address this issue, we propose placing the TCI state ID (7 bits), UL TCI state ID (6 bits), and NCC (3 bits) in a direct sequence as shown in Figure 2. When unifiedTCI-StateType = joint, the UL TCI state ID is set to zero and ignored by the UE, but the NCC is always included. This keeps the transmitted size the same for both joint and separate cases. By separating the NCC from CFRA parameters, we avoid transmitting unnecessary bits in RACH-less LTM and security change scenarios, simplifying the structure and removing field dependencies.

Figure 2 Proposed Enhanced LTM CSC MAC CE format

Proposal 1: Modify the Enhanced LTM CSC MAC CE format as shown in Figure 2.

- Conclusions

In this paper, we discuss Enhanced LTM CSC MAC CE format. Following is a proposal made in this paper:

Proposal 1: Modify the Enhanced LTM CSC MAC CE format as shown in Figure 2.

- References

- Chair notes RAN2#130

- Discussion summary and list of MAC open issues for Mob Ph4

- Running MAC CR for enhanced mobility Ph4