3GPP TSG-RAN WG2 Meeting #131	R2- 2505338

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.5.2

Source:	vivo

Title:	Remaining issues of on-demand SSB for SCell operation

Document for:	Discussion and Decision

Introduction

According to RAN2#130 meeting, the following agreements were made [1]:

Agreement 1. (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst. 2. The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index. TP in R2-2504280 will be baseline for the running CR. Detailed wordings can be further discussed. 3. When UE receives MAC CE with Cj bit set to 0 for a SCell for which OD-SSB is currently activated: • OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped. 4. For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the OD-SSB Activation/Deactivation MAC CE. …… 9. RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported. 10. When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell. 11. RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

In this contribution, we will discuss the remaining issues of on-demand SSB for SCell.

Discussion

OD-SSB transmission activation by RRC signalling

It was agreed at RAN1#121 that the following OD-SSB activation/deactivation combinations are supported [2]:

Agreement For a cell supporting on-demand SSB SCell operation, the following combinations are supported. - For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA; - Subject to UE capability - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA; - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD; - Subject to UE capability - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD; - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD. - Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication.

According to the RRC running CR [3], it is now allowed from signalling perspective to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. However, RAN1 did not reach a consensus whether to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. Therefore, RAN2 needs to discuss this open issue.

From our point of view, there is no actual technique concern to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. Nevertheless, if RAN2 agrees to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst, RAN2 needs to send LS to RAN1 to trigger the discussion of OD-SSB transmission timeline upon reception of the RRC activation signalling. Considering that RAN1 has no TU left for this discussion, we propose RAN2 to confirm not to support this function.

Observation 1: If RAN2 agrees to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst, we need to send LS to RAN1 to trigger further discussion of OD-SSB transmission timeline upon reception of the RRC activation signalling, which is not preferred at this late stage.

Proposal 1: Not support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst.

L3 measurement considering OD-SSB for case 2b

According to the latest running CR [3], the new servingCellMO field is placed under the OD-SSBConfig IE as a per OD-SSB configuration.

RAN2#130 agreement - RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement. - RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported. Running CR in the 38.331 OD-SSB-Config information element -- ASN1START -- TAG-OD-SSB-CONFIG-START OD-SSB-Config-r19 ::= SEQUENCE { od-ssb-ConfigId-19 INTEGER (0.. maxNrofOD-SSB-1-r19) OPTIONAL, -- Need R od-ssb-ActivationStatus-r19 ENUMERATED {true} OPTIONAL, -- Need R od-ssb-Periodicity-r19 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } OPTIONAL, -- Need S od-ssb-sfn-Offset-19 INTEGER (0..15) OPTIONAL, -- Need S od-ssb-nrofBurst-r19 INTEGER (0..8) OPTIONAL, -- Need R od-ssb-halfFrameIndex-r19 ENUMERATED {zero, one} OPTIONAL, -- Need S od-ssb-absoluteFrequency-r19 ARFCN-ValueNR OPTIONAL, -- Need S ... } OPTIONAL, -- Need S servingCellMO-r19 MeasObjectId OPTIONAL, -- Cond InterFreq od-smtc-r19 SSB-MTC OPTIONAL, -- Cond IntraFreq ... }

In comparison, legacy servingCellMO is placed under the ServingCellConfig, which is related to a certain SSB frequency. During the last meeting, it was agreed that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, and not support multiple OD-SSBs with different frequencies [4].

Based on the agreements, our understanding is that for case 2b, at most one OD-SSB frequency that is different from the AO-SSB frequency can be configured. In other words, even multiple candidate OD-SSB configurations with different periodicity can be configured for a serving cell, and servingCellMO is configured per OD-SSB configuration, these configurations still share the same servingCellMO. Thus, to save signalling overhead, we propose that the new servingCellMO is per cell configured, i.e. the new servingCellMO is placed under servingCellConfig.

Proposal 2: When AO-SSB and OD-SSB have different center frequency, the new servingCellMO is per cell configured, i.e. servingCellMO-r19 is placed under servingCellConfig.

Field description for od-ssb-absoluteFrequency

During the RAN1 #120 & #120bis meeting [5][6], agreements regarding od-ssb-absoluteFrequency have been clarified, and the description of od-ssb-absoluteFrequency is defined as follows:

Agreement for RAN1 #120 - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) Agreement for RAN1 #120bis For a cell supporting on-demand SSB SCell operation, - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) can be absent for Case #2 - If absent, the center frequency of on-demand SSB is the same as that of always-on SSB. the ‘Description’ of od-ssb-absoluteFrequency in RAN1 parameters list Indicate frequency of OD-SSB with additional restrictions as described in subclause 4.4. of TS38.213.

From the above agreements, we understand od-ssb-absoluteFrequency can be absent only when absoluteFrequencySSB configured in IE FrequencyInfoDL for this serving cell is configured (case#2a and case#2b) and od-ssb-absoluteFrequency is the same as absoluteFrequencySSB, so that the NW does not need to configure od-ssb-absoluteFrequency repetitively. However, the description of od-ssb-absoluteFrequency in RRC running CR is now captured as below [3]:

od-ssb-absoluteFrequency Indicates the frequency of the OD-SSB when the frequency is different from absoluteFrequencySSB configured in IE FrequencyInfoDL for this serving cell. Additional restrictions as described in subclause 4.4. of TS 38.213.

From our perspective, the description of od-ssb-absoluteFrequency in RRC running CR is not accurate to reflect the RAN1 agreement. Considering absoluteFrequencySSB in IE FrequencyInfoDL for this serving cell is absent in OD-SSB case#1, od-ssb-absoluteFrequency should be configured in the OD-SSB configuration for case#1. This point can also be reflected by the following text in [7]:

<irrelevant text omitted> the frequency location of the second SS/PBCH blocks is indicated by od-absoluteFrequencySSB, if provided; otherwise, by absoluteFrequencySSB <irrelevant text omitted> When the UE is not provided absoluteFrequencySSB for the SCell, the UE does not expect the transmission of the second SS/PBCH blocks provided by od-ssb-config to be deactivated while the SCell is activated.

In sum, we propose to capture the field description of od-ssb-absoluteFrequency as below: ‘Indicates the frequency of the OD-SSB. If the field is absent, the UE applies absoluteFrequencySSB indicated in FrequencyInfoDL for this serving cell, if configured. Additional restrictions as described in subclause 4.4. of TS 38.213.’

Proposal 3: Capture the field description of od-ssb-absoluteFrequency as below: ‘Indicates the frequency of the OD-SSB. If the field is absent, the UE applies absoluteFrequencySSB indicated in FrequencyInfoDL for this serving cell, if configured. Additional restrictions as described in subclause 4.4. of TS 38.213.’

Conclusion

Based on the discussion, we have the following observation and proposals:

2.1.	OD-SSB transmission activation by RRC signalling

Observation 1: If RAN2 agrees to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst, we need to send LS to RAN1 to trigger further discussion of OD-SSB transmission timeline upon reception of the RRC activation signalling, which is not preferred at this late stage.

Proposal 1: Not support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst.

2.2.	L3 measurement considering OD-SSB for case 2b

Proposal 2: When AO-SSB and OD-SSB have different center frequency, the new servingCellMO is per cell configured, i.e. servingCellMO-r19 is placed under servingCellConfig.

2.3.	Field description for od-ssb-absoluteFrequency

Proposal 3: Capture the field description of od-ssb-absoluteFrequency as below: ‘Indicates the frequency of the OD-SSB. If the field is absent, the UE applies absoluteFrequencySSB indicated in FrequencyInfoDL for this serving cell, if configured.’

References

- RAN2#130 meeting minutes

- RAN1#121 meeting minutes

- [POST130][107][NES] Running RRC CR (Ericsson)

- R2-2504671	Report from session on R18 SL, R18/19 MOB, and R19 NES	Vice Chairman (Samsung)

- R1#120 meeting minutes.

- R1#120bis meeting minutes.

- TS 38.213, V19.0.0