3GPP TSG-RAN WG2 Meeting #131			           		R2-2505345

Bangalore, India, August 25 - 29, 2025

Agenda item:	8.1.2.2

Source:	Samsung

Title:	Remaining issues in LCM for BM and CSI prediction

Document for:	Discussion and Decision

# Introduction

RAN2 identified remaining open RRC issues during [POST130][026][AI PHY] 38.331 CR (Ericsson) which the moderator suggests to bring a contribution for further discussion. Therefore, in this contribution we provide our view on those open issues.

# Open issues

## RRC-3: UE data collection request

###### Open issue RRC-3: UE data collection request Issue description: For UE-side data collection, the details of UAI signaling for the UE start/stop request and especially the following aspects are not yet clarified: - how to refer in UAI to a preferred radio resource candidate configuration from a list of candidate configurations provided by NW - where/what the NW provides as candidate configurations - what the content of otherConfig for enabling UE data collection requests in UAI should be (e.g. just a flag, the list of candidate UE data collection configurations, etc.). This issue refers to the RAN2#129bis agreements: The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request. The issue is captured as an editor’s notes in endorsed RRC running CR [R2-2504349], clause 5.7.4.3, 6.2.2, and 6.3.4. RAN2#130 agreed: 3 The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. 4 For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling). Proposed resolution: This open issue is treated in the email discussion [POST130][037][AI PHY] UE candidate data collection (Xiaomi/Ericsson). If companies think there are other relevant aspects not covered by the email discussion, it is suggested that they address them in the contributions.

During [POST130][037][AI PHY] UE candidate data collection (Xiaomi/Ericsson), we made a proposal i.e., UE should be able to provide not only configuration ID lists it prefers to start, but also configuration ID lists it prefers to stop. Otherwise, when UE wants to stop a specific configuration (while keeping the other configurations), there is no way for UE to request to stop selectively. Then NW would release all configurations.

Besides, we would like to clarify UE should be able to include the two lists i.e., 1) configuration ID lists it prefers to start and 2) configuration ID lists it prefers to stop together in a common UAI message. Given UE’s capability is limited, UE may not be able to measure all preferred configurations at the same time. So, it is practical scenario to request its preference to stop one configuration and start another configuration simultaneously (i.e., in a single UAI message).

Proposal 1: (RRC-03) Introduce configuration ID lists UE prefers to stop for UE-side data collection in UAI message. UE can include both 1) configuration ID lists it prefers to start and 2) configuration ID lists it prefers to stop together in a common UAI message.

## RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability

###### Open Issue RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability Issue description: The UE needs to synchronize the time duration for an AI functionality to become available for inference with the network when the UE reports applicability for either a full inference configuration or a set of inference parameters. Unlike conventional non-AI features, where the algorithm size is fixed, AI algorithms can vary significantly in size depending on their generalization performance and specific use cases. These sizes can range from several kilobytes to tens of megabytes. Additionally, UE implementations may use various types of memory to balance cost and performance, and these different memory types and constraints can affect the access speed of stored models. The UE should indicate the time duration for the AI functionality to become available when reporting applicability to the network. This time duration refers to how long it takes for the UE to load the AI models into RAM or another accessible memory for inference. By providing this information, the network can better understand the timeframe within which the configured radio resources and AI model can be utilized, enabling more efficient and effective resource management, especially for periodic CSI. Another option is to let RAN4 address this time duration in terms of requirements. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

It seems correct that processing time is required to load a model to use for inference. However, the required time may be different depending on UE implementation.

Regarding potential solutions, from signaling point of view, UE is already allowed to send a change of applicability status. That is, if UE cannot enable inference before RRCReconfiguraitonComplete message (i.e. within 16ms considering RRC processing delay), the UE could indicate “inapplicability” in RRCReconfiguraitonComplete message and send UAI to update the change of applicability after loading is completed.

Alternatively, we might be also ok to add time information to indicate when a model can be actually applicable. However, given that it is not yet known the range or level of the required time, it is not clear whether we need additional signaling.

Proposal 2: (RRC-15) RAN2 asks RAN1 on the required time to enable a model after applicability reporting.

## RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations

###### Open issue RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations Issue description: Based on the RAN2 agreements below, it is not clear whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations stored at the UE, or whether it can contain only updates for the inference configurations for which the UE already reported the applicability status in the past. This open issue refers to the RAN2#129 agreements: 3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause. In the rapporteur’s understanding, there are two interpretations of the agreements above: - The RRCReconfigurationComplete contains the applicable/inapplicable status for all inference configurations. Only UAI contains updates. - The RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message and only applicability changes for those inference configurations received in earlier RRCReconfiguration messages, for which the UE already reported applicable/inapplicable at least once. UAI contains updates. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

From signaling point of view, both interpretations are possible to support. On the other hand, interpretation #2 is more flexible to include previous configured CSI-ReportConfig. We envision that the UE may need to perform model switching considering that the number of models that UE can enable would be limited. It would result in that UE changes model applicability status of existing CSI-ReportConfig when new CSI-ReportConfigs are added.

Observation 1: (RRC-44) applicability status of existing CSI-ReportConfig may be affected with newly added CSI-ReportConfig.

Proposal 3: (RRC-44) RAN2 agree interpretation #2 i.e. the RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message and only applicability changes for those inference configurations received in earlier RRCReconfiguration messages, for which the UE already reported applicable/inapplicable previously.

## RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications

###### Open issue RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications Issue description: RAN2 agreed that the UE should activate a periodic inference configuration in CSI-ReportConfig only if it reports that it is applicable. There are several possibilities to capture this in the specifications, as commented by several companies, and it should be discussed how to best do this. Furthermore, it should be clarified if the solution can be captured only in RRC specifications or whether RAN1 should also capture (part of) the solution. This open issue refers to the RAN2#129 agreement: - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).. Some solutions suggested by companies for several aspects are: - Where exactly in RRC to capture the solution: - In Section 5.3.5.3 when setting the content of RRCReconfigurationComplete, or - In Section 5.3.5.3 when sending RRCReconfigurationComplete to the lower layers - What to write in RRC specifications: - Upon including the applicable status in RRCReconfigurationComplete, indicate to lower layers to activate the configuration, or - Submit/do not submit the configuration to lower layers, which would be the equivalent of activation/no activation. Furthermore, there is an incoming LS [R4-2508085], which is discussing this issue, so it is suggested that according to this LS the companies bring contributions about whether: 1) the applicable functionalities are already activated before reporting applicable functionalities via RRCReconfigurationComplete; or 2) UE starts to activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete. In particular, related to the periodic reporting, according to Rapporteur point of view, it should be taken into account that the gNB cannot know whether the UE is sending L1 prediction reports or not, until the NW receives successfully the applicability report from the UE. Until then, if the UE is allowed to multiplex UCI (with CSI inference predictions) in PUCCH or PUSCH, the NW will try to decode the multiplexed information, which will turn into decoding issues if the inference predictions are not sent by the UE and this will impact also other decoded information from the UE. So a standardized behaviour is needed in order to avoid the gNB incurring in such ambiguity, e.g: a) the UE always sends a L1 report in UCI for periodic CSI-ReportConfig, even if the configuration is not active/applicable; b) the UE does not multiplex UCI containing inference predictions and specify that the configuration is activated only after finishing sending RRCReconfigurationComplete. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

Regarding where exactly to capture, firstly, it should be clarified that there is no intention in RAN1 to change existing RRC procedure to autonomously activate periodic CSI-ReportConfig. The existing RRC procedure means that RRC applies configuration while it proceeds RRC reconfiguration message and generates the content of RRCRconfigurationComplete instead of applying configuration while sending RRCReconfigurationComplete message.

Observation 2: (RRC-45) existing RRC behavior is aligned with when setting the content of RRCReconfigurationComplete.

In addition, technically, we also need to consider what is difference/benefit of introducing new behavior of activating periodic CSI-ReportConfig upon transmission of RRCReconfigurationComplete message. Given that gNB RRC knows after successful completion of RRCReconfigurationComplete message, there is no difference in gNB behavior i.e. gNB should assume that all configured periodic CSI-ReportConfig are ready to activated before successful reception of RRCReconfigurationComplete.

Observation 3: (RRC-45) there is no difference in gNB behavior depending on whether UE activates periodic CSI-ReportConfig setting the content of RRCReconfigurationComplete and when sending RRCReconfigurationComplete to the lower layers.

Proposal 4: (RRC-45) Handling of periodic CSI-ReportConfig is described in Section 5.3.5.3 when setting the content of RRCReconfigurationComplete.

Regarding what to write in RRC spec, as we commented in RRC running CR, RAN1 spec doesn’t describe any procedure regarding activation and deactivation for periodic CSI-ReportConfig.

Observation 4: (RRC-45) RAN1 spec doesn’t specify any procedure regarding activation and deactivation for periodic CSI-ReportConfig.

Unless RAN2 requests RAN1 to newly define or specify activation/deactivation of periodic CSI-ReportConfig, only way would be for RRC layer to handle it. That is, if it is activated, RRC layer provides the corresponding CSI-ReportConfig to the lower layer. Otherwise, RRC layer should not provide the corresponding CSI-ReportConfig.

Proposal 5: (RRC-45) RRC layer submit/do not submit periodic CSI-ReportConfig configuration to lower layers, which would be the equivalent of activation/no activation. That is, RRC layer configures lower layers to perform CSI-ReportConfig if the applicabilityStatus is set to applicable.

If proposal 4 is acceptable, we propose the following change to RRC spec.

6> if reportConfigType in CSI-ReportConfig is periodic: 7> if the applicabilityStatus is set to applicable: 8> configure lower layers to perform CSI-ReportConfig;

## RRC-47: FFS how to implement RAN1 parameters in CSI-ReportConfig

###### Open issue RRC-47: FFS how to implement RAN1 parameters in CSI-ReportConfig Issue description: RAN1 LS (R2-2505000) needs implementation and adoption to ASN.1 principles and RAN2 agreements. There are several ways to implement these parameters in CSI-ReportConfig in ASN.1. There are two main solution directions: - List all parameters (for inference, performance monitoring, UE data collection) one after another without any hierarchy; or - Group some of the parameters with a CHOICE structure, to clarify which parameters are allowed to be configured together. For instance, the latest version of the RRC running CR proposes a structure with a CHOICE branch for inference (and UE data collection) and a second one for performance monitoring. Proposed resolution: It is suggested that companies provide contributions to resolve the issue, if they do not agree to the current implementation in the running CR.

The following is RAN1 parameter structure from the updated running CR.

nrofReportedRS-v19xy ENUMERATED {n6, n8} OPTIONAL, -- Need R reportQuantity-r19 ReportQuantity-r19 OPTIONAL, -- Need R predictionConfiguration-r19 CHOICE { configurationForChannelPrediction-r19 SEQUENCE { csi-InferencePrediction-r19 ENUMERATED {true} OPTIONAL, -- Need R resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R associatedIdForChannelPrediction-r19 AssociatedId-r19 OPTIONAL, -- Need R associatedIdForChannelMeasurement-r19 AssociatedId-r19 OPTIONAL, -- Need R nrofReportedPredicted-RS-r19 ENUMERATED {n1, n2, n3, n4} OPTIONAL, -- Need R nrofTimeInstance-r19 ENUMERATED {FFS} OPTIONAL, -- Need R timeGap-r19 ENUMERATED {FFS} OPTIONAL, -- Need R ... } configurationForChannelMonitoring-r19 SEQUENCE { refToPredictionConfig-r19 CSI-ReportConfigId, nrofBestBeamForMonitoring-r19 ENUMERATED {n1, n2} OPTIONAL, -- Need R nrofTransmissionOccasion-r19 ENUMERATED {n1, n3, n7, n15} OPTIONAL, -- Need R timeInstanceFor-RS-PAI-r19 ENUMERATED {FFS} OPTIONAL, -- Need R mappingToResourcesForChannelPrediction-r19 BIT STRING (SIZE (1..17)) OPTIONAL, -- Need R timeInstanceFor-SGCS-r19 ENUMERATED {FFS} OPTIONAL, -- Need R ... } } OPTIONAL -- Need R ]]

It is our understanding that BM use case is supported when p-cri-r19, p-ssb-index-r19 p-cri-RSRP-r19 or p-ssb-index-RSRP-r19 is configured. Therefore, it might be clear if we can link configurationForChannelPrediction-r19 to these reportQuantity.

There are not many parameters to share between BM and CSI-prediction. For example, associated IDs, resourcesForChannelPrediction-r19, nrofReportedPredictedRS-r19, nrofTimeInstance-r19 and timeGap-r19 are used only for BM use case.  It is also same for performance monitoring. For CSI prediction, only refToPredictionConfig-r19 and            timeInstanceForSGCS-r19 are required. Other parameters are for BM use case.

Proposal 6: (RRC-47) a separate structure should be defined for BM and CSI prediction.

In case of CSI prediction, RAN1 introduced a new RRC parameter to distinguish AI-CSI prediction and non-AI CSI prediction (legacy type II doppler) although details is left to RAN2. In our understanding, this new RRC parameter is only parameter for AI-CSI prediction and remaining RRC parameters in CSI-ReportConfig will be provided according to RAN1 specification.

Therefore, it is reasonable to pull out csi-InferencePrediction-r19 from configurationForChannelPrediction-r19 and define it as an independent parameter.

Proposal 7: (RRC-47) csi-InferencePrediction-r19 is defined out of configurationForChannelPrediction-r19.

The below is our proposed structure based on the above proposals.

Alterantive #1

nrofReportedRS-v19xy                ENUMERATED {n6, n8}                                                     OPTIONAL,   -- Need R

reportQuantity-r19                          ReportQuantity-r19                                              OPTIONAL,   -- Need R

configurationForChannelPredictionBM-r19   SEQUENCE {

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                        OPTIONAL,   -- Need R

nrofReportedPredicted-RS-r19                ENUMERATED {n1, n2, n3, n4}                             OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {FFS}                                        OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {FFS}                                        OPTIONAL,   -- Need R

...

}

configurationForChannelMonitoringBM-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,

nrofBestBeamForMonitoring-r19               ENUMERATED {n1, n2}                                     OPTIONAL,   -- Need R

nrofTransmissionOccasion-r19                ENUMERATED {n1, n3, n7, n15}                            OPTIONAL,   -- Need R

timeInstanceFor-RS-PAI-r19                  ENUMERATED {FFS}                                        OPTIONAL,   -- Need R

mappingToResourcesForChannelPrediction-r19  BIT STRING (SIZE (1..17))                               OPTIONAL,   -- Need R

...

}

csi-InferencePrediction-r19                 ENUMERATED {true}                                       OPTIONAL,   -- Need R

csi-configurationForChannelMonitoring-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,            OPTIONAL    -- Need R

timeInstanceForSGCS-r19                     ENUMERATED {FFS}    	   	OPTIONAL    -- Need R

}

}                                                                                                           OPTIONAL    -- Need R

]]

Alterantive #2

nrofReportedRS-v19xy                ENUMERATED {n6, n8}                                                     OPTIONAL,   -- Need R

reportQuantity-r19         CHOICE {

p-cri-r19                   configurationForChannelPrediction-r19,

p-ssb-index-r19             configurationForChannelPrediction-r19,

p-cri-RSRP-r19              configurationForChannelPrediction-r19,

p-ssb-index-RSRP-r19        configurationForChannelPrediction-r19,

none-BM-r19                 configurationForDataCollection-r19,

bm-rspai-r19                configurationForChannelMonitoring-r19,

none-CSI-r19			 NULL,

csi-SGCS-r19                SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,            OPTIONAL    -- Need R

timeInstanceForSGCS-r19                     ENUMERATED {FFS}    	   	OPTIONAL    -- Need R

}

}  OPTIONAL    -- Need R

csi-InferencePrediction-r19                 ENUMERATED {true}                                       OPTIONAL,   -- Need R

configurationForChannelPrediction-r19   SEQUENCE {

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                        OPTIONAL,   -- Need R

nrofReportedPredicted-RS-r19                ENUMERATED {n1, n2, n3, n4}                             OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {FFS}                                        OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {FFS}                                        OPTIONAL,   -- Need R

...

}

configurationForDataCollection-r19 SEQUENCE {

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                        OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {FFS}                                        OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {FFS}                                        OPTIONAL,   -- Need R

...

}

configurationForChannelMonitoring-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,

nrofBestBeamForMonitoring-r19               ENUMERATED {n1, n2}                                     OPTIONAL,   -- Need R

nrofTransmissionOccasion-r19                ENUMERATED {n1, n3, n7, n15}                            OPTIONAL,   -- Need R

timeInstanceFor-RS-PAI-r19                  ENUMERATED {FFS}                                        OPTIONAL,   -- Need R

mappingToResourcesForChannelPrediction-r19  BIT STRING (SIZE (1..17))                               OPTIONAL,   -- Need R

...

}

## RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided

###### Open issue RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided Issue description: Based on current agreements, the UE reports the applicability of both full inference configurations and corresponding inference related parameter sets, even if some of them may correspond to each other. The issue was raised that once the network provides full inference configuration for applicable inference related parameter set, it would be highly inefficient if UE keeps reporting both full inference configuration ID and inference related parameter set config ID upon change in applicability. One solution is to allow the UE to either provide the applicability of inference related parameter set or inference configuration once the inference configuration is provided based on the applicability of inference related parameter set. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

It is definitely reasonable not to send duplicated applicability reporting. Given that a set of inference related parameters are also configured in CSI-ReportConfig as a part of prediction configuration, UE can know the association between CSI-ReportConfig and related to set of inference related parameters. Therefore, UE can skip applicability reporting of CSI-ReportConfig if there is inference related parameter set configuration having the same parameters.

Proposal 8: (RRC-48) UE skips to send applicability reporting of CSI-ReportConfig if there is inference related parameter set configuration having the same parameters.

# Conclusion

In this contribution, we discussed remaining open issues. Based on the discussion, we propose the following proposals and observations.

Observation 1: (RRC-44) applicability status of existing CSI-ReportConfig may be affected with newly added CSI-ReportConfig.

Observation 2: (RRC-45) existing RRC behavior is aligned with when setting the content of RRCReconfigurationComplete.

Observation 3: (RRC-45) there is no difference in gNB behavior depending on whether UE activates periodic CSI-ReportConfig setting the content of RRCReconfigurationComplete and when sending RRCReconfigurationComplete to the lower layers.

Observation 4: (RRC-45) RAN1 spec doesn’t specify any procedure regarding activation and deactivation for periodic CSI-ReportConfig.

Proposal 1: (RRC-03) Introduce configuration ID lists UE prefers to stop for UE-side data collection in UAI message. UE can include both 1) configuration ID lists it prefers to start and 2) configuration ID lists it prefers to stop together in a common UAI message.

Proposal 2: (RRC-15) RAN2 asks RAN1 on the required time to enable a model after applicability reporting.

Proposal 3: (RRC-44) RAN2 agree interpretation #2 i.e. the RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message and only applicability changes for those inference configurations received in earlier RRCReconfiguration messages, for which the UE already reported applicable/inapplicable previously.

Proposal 4: (RRC-45) Handling of periodic CSI-ReportConfig is described in Section 5.3.5.3 when setting the content of RRCReconfigurationComplete.

Proposal 5: (RRC-45) RRC layer submit/do not submit periodic CSI-ReportConfig configuration to lower layers, which would be the equivalent of activation/no activation. That is, RRC layer configures lower layers to perform CSI-ReportConfig if the applicabilityStatus is set to applicable.

Proposal 6: (RRC-47) a separate structure should be defined for BM and CSI prediction.

Proposal 7: (RRC-47) csi-InferencePrediction-r19 is defined out of configurationForChannelPrediction-r19.

Proposal 8: (RRC-48) UE skips to send applicability reporting of CSI-ReportConfig if there is inference related parameter set configuration having the same parameters.