3GPP TSG-RAN WG2 Meeting #131                                  R2-2505357

Bangalore, India,  Aug 25th -29th, 2025

Agenda item:		8.3.2.1

Source:				Samsung

Title:					Discussion on functionality management for RRM measurement prediction

Document for:		Discussion & Decision

1	Introduction

In this paper, we would like to discuss the functionality management for RRM measurement prediction (i.e., applicability/capability reporting, inference configuration, inference report) for the use cases agreed to study the spec. impact.

2	Discussion

At the last RAN2#129bis, the following agreement was made for the general LCM framework for AI mobility.

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

According to the agreement above, we can discuss the potential spec. impact of AI mobility based on the baseline procedure agreed for beam management (BM) use case and the discussion here can focus on the aspect that can be different from BM use case for AI mobility. Also, considering the agenda given in the chair note, this paper discusses some aspects (i.e., capability/applicability reporting, inference configuration and reporting, performance monitoring) of LCM framework for UE-side model case.

2.1 Capability/Applicability reporting

Definition:

In Rel-19 AI/ML for air WI session, RAN2 agreed the definition of “supported functionalities”, “applicable functionalities”, and “Activated functionalities” as below.

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

To facilitate discussion by ensuring we have the same understanding on the definitions in AI/ML for mobility SI, we would like to propose the following.

Proposal. 1: To facilitate discussion in AI/ML for mobility SI/WI, RAN2 adopt the definition of supported/applicable/activated functionalities agreed in Rel-19 AI/ML for air WI.

UE Capability Reporting:

Regarding UE capability reporting, RAN2 made the following agreement in Rel-19 AI/ML for air WI.

Agreements

1.	Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

2.	Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Since we do not see any issue specific to AI/ML for mobility in UE capability reporting procedure, we would like to propose the following.

Proposal. 2: For AI/ML for mobility, UE can report the supported AI/ML-enabled Features/FGs and supported functionalities to NW via UECapabilityInformation message.

Applicability Reporting:

Regarding applicability reporting, the following agreement was made at RAN2#130.

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced.

Regarding use of Option A and Option B, we believe both should be adopted for AI/ML mobility as well. Option B offers the advantage of checking the applicability of a candidate AI configuration while minimizing the signalling overhead, also in AI/ML mobility use cases. For instance, NW can provide UE with a set of candidate/partial configurations with different inference parameters (e.g., PW length). Then, the UE can feedback the applicability of these configurations before receiving full configuration from NW. Without Option B, NW should always provide the full configuration and assess its applicability through "try-and-error" approach. This would result in multiple iterations of configuration and applicability reporting, which we consider inefficient.

Observation. 1: Without option B, NW would need to verify the applicability of each configuration through "try-and-error" approach, making the applicability reporting procedure inefficient.

Proposal. 3: For AI/ML mobility use cases, support both option A and Option B like scheme.

2.2 Inference & reporting

For UE-side model inference, NW can provide some configuration for inference at UE-side. Then, the UE can perform the inference with the input data that is internally available at UE and report the outcome of the inference to NW. For the spec. impact, we can focus on the inference configuration from NW to UE and the report from UE to NW.

2.2.1 Inference configuration

At the last RAN2#129bis, RAN2 agreed the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. In our view, the existing RRM measurement configuration consisting of MeasObject/MeasId/ReportConfig can be easily extended/enhanced to support the use cases of AI for mobility. Therefore, in this part, we propose some potential enhancement based on the existing RRM measurement configuration.

Common part:

For the common part across the use cases (i.e., frequency/temporal domain RRM measurement prediction, measurement event prediction, L3 beam-level prediction, Spatial domain prediction), NW can indicate the target MO for which the UE predicts RRM measurement.

Proposal. 4: For AI/ML mobility use cases, NW can indicate the target MO (Measurement object) for which the UE performs the prediction (e.g., RRM measurement prediction) as part of inference configuration.

Also, as in BM use case, for the consistency across training and inference, NW can provide the NW-side additional condition (e.g., associated ID) which can be used for applicability check.

Proposal. 5: For AI/ML mobility use cases, NW can indicate NW-side additional condition (e.g., associated ID) for the applicability check as part of inference configuration if needed.

Temporal domain Case A:

For temporal domain Case A, NW can indicate how far the UE should predict the measurement results in time domain (i.e., PW length). Then, UE can predict the L3 Cell measurement results of multiple time instances within the prediction window according to the NW configuration. The PW length was agreed as part of inference configuration for temporal domain Case A at the last RAN2#130.

Meanwhile, we think it is inefficient for UE to predict the future measurement results for all the cells detected during the measurement for the target MO. Therefore, NW can indicate the list of cells for which the UE should perform the prediction. For example, NW can configure the UE to perform the prediction only for the cells that can be the candidate HO target cells.

Proposal. 6: For temporal domain Case A, NW can indicate the list of target cells for which the UE performs the prediction as part of inference configuration.

Temporal domain Case B:

For temporal domain Case B, the skipping pattern was agreed as part of inference configuration as below.

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI

Based on the offline discussion during the last meeting, there are two possible interpretations of the “skipping pattern”:

- Option 1: SSB timing configuration (similar to the existing IE SSB-MTC), which indicates the timing of NW's SSB transmission.

- Option 2: Skipping configuration, which indicates the timing of UE's SSB measurement/skipping.

From our understanding, in legacy RRM operations, as long as the UE meets the RAN4 requirement on the measurement period (at least one measurement within the period), the exact timing of SSB measurement within that period is left to UE implementation. Additionally, it is impractical for NW to indicate the precise timing of UE's measurement. In this context, the option 1 makes more sense than the option 2.

Observation. 2: According to the legacy RAN4 requirement, the exact timing of SSB measurement within the measurement period is determined by UE implementation.

Proposal. 7: For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

If the skipping pattern is agreed as SSB configuration (as proposed in Proposal 7), we can consider reusing the existing IE SSB-MTC for its configuration. However, if the periodicity in IE SSB-MTC is doubled (e.g., 40 ms -> 80 ms) for measurement reduction in Case B, the length of measurement period would be also doubled (e.g., N*40ms -> N*80 ms). This effectively relaxes the RAN4 requirement, potentially leading to the HO performance degradation. Note that the measurement period increases proportionally with the SMTC period (the periodicity in IE SSB-MTC), as it is calculated as multiples of the SMTC period according to RAN4 Specification (TS 38.133).

To address this, we can introduce a separate/new IE for the skipping pattern, which would indicate the increased SSB interval while maintaining the measurement period same as in legacy (based on the legacy IE SSB-MTC).

Observation. 3: Increasing the periodicity in IE SSB-MTC for measurement reduction in Case B would also increase the measurement period, effectively relaxing the RAN4 requirement and potentially degrading HO KPI.

Proposal. 8: For skipping pattern configuration in Case B, we can introduce a separate/new IE to indicate the increased SSB interval while maintaining the measurement period as in legacy (based on the legacy IE SSB-MTC).

For temporal domain Case B, UE should be able to reduce the measurement effort/overhead (i.e., skip the measurement) while maintaining the HO KPI (i.e., adhering to the same RAN4 requirement as before). In other words, as long as the UE satisfies the RAN4 requirement on the measurement period and accuracy, the details of how often and when to skip the measurement (i.e., predict the measurement results) can be left to UE implementation. Here, the requirement on the measurement period can be understood as UE performs measurement and/or predication for the applicable cells at least once within the period. The requirement on the accuracy ensures that the measurement/prediction error remains within the bound defined by RAN4.

Proposal. 9: For temporal domain Case B, as long as the UE meets the RAN4 requirement on measurement period and accuracy, the details of how often and when to skip the measurement (i.e., predict measurement results) can be left to UE implementation.

While the discussion above highlights that the actual measurement skipping pattern can vary based on the UE implementation, NW may not be aware of it. However, we believe that NW’s awareness on the skipping pattern is necessary to maximize energy savings at UE-side (i.e., to align the C-DRX cycle, measurement gap timing with the skipping pattern). To achieve this, NW can configure UE to provide its actual measurement skipping pattern as an assistance data for NW.

Proposal. 10: For temporal domain Case B, NW can configure UE to report its measurement skipping pattern as an assistance data for NW.

Spatial domain prediction:

For spatial domain prediction, the following agreement was made at the last RAN2#130.

Agreements

1	The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts.     Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s).

2	The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

Agreements The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI

Based on the agreement above, we can study the spec. impact for the spatial domain prediction in beam dimension. From our understanding, the expected UE operation for spatial domain prediction can be summarized as below.

- Legacy: UE performs the measurement for the beams indicated by ssb-ToMeasure and calculate the cell measurement results with consolidation (e.g., as average RSRP of Top-K beams) based on the beam measurement results.

- Spatial domain (Sub-use case 1): UE performs the measurement for the beams indicated by ssb-ToMeasure and predict the measurement results for the remaining beams (or the target beams indicated by NW). The cell measurement results can be calculated with consolidation (e.g., average RSRP of Top-K beams) based on the beam measurement/prediction results.

- Spatial domain (Sub-use case 3): UE perform the measurement for the beams indicated by ssb-ToMeasure and directly predict the cell measurement results based on the beam measurement results without consolidation.

Based on the summary, it appears that the measured beam pattern is required for both Sub-use case 1 and Sub-use case 3. However, the predicted beam pattern is applicable only to Sub-use case 1. Therefore, if the predicted beam pattern is not configured, UE can choose between Sub-use case 1 or Sub-use case 3 based on its implementation. If the predicted beam pattern is configured, the UE will perform prediction using Sub-use case 1.

Observation. 4: The measured beam pattern is applicable to both Sub-use case 1 and 3, but the predicted beam pattern is applicable only to Sub-use case 1.

Proposal. 11: For spatial domain prediction (Sub-use case 1 and Sub-use case 3), the measured beam pattern can be configured by NW via the existing ssb-ToMeasure field.

Proposal. 12: For spatial domain prediction (Sub-use case 1), the predicted beam pattern can be configured by NW via a new field.

L3 beam-level prediction:

For L3 beam-level prediction, NW can configure UE to predict the beam-level measurement results for the best candidate cell (e.g., the cell with highest RSRP) for HO. Also, considering that beam-level prediction is to configure a dedicated RACH resource for some potential best beam of the target cell, UE does not need to report the prediction results for all beams. Thus, NW can configure UE to report only the Top-K beam indexes.

Proposal. 13: For L3 beam-level prediction, NW can indicate the maximum number of beams to report (i.e., K) as part of inference configuration (to configure UE to report Top-K beam indexes).

2.2.2 Inference report

At the last RAN2#129bis, RAN2 agreed the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. In our view, the existing measurement reporting mechanism can be simply extended/enhanced for UE to report the inference results of AI for mobility. Therefore, in this part, we propose some potential enhancement with the assumption of reusing the legacy measurement reporting mechanism.

Regarding further details on the potential spec. impact for the measurement (inference) reporting, RAN2 made the following agreements.

- Agreements

For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

With the above as baseline, we can further discuss the potential spec. impact for each use case as below.

Temporal domain Case A:

For temporal domain Case A, UE can initiate the periodical or event-triggered reporting as in legacy, but it can report the predicted results for the multiple time instances within PW for the target cells. Here, the target cells can be determined by the inference configuration from NW. For example, NW can explicitly configure the target cells, or the target cell can be the serving cells and Top-K neighbor cells where the maximum cells numbers to report (i.e., K) is given by NW.

Proposal. 14: For temporal domain Case A, UE can report the predicted results for the multiple time instances within PW for the target cells. The target cells can be determined based on the inference configuration from NW.

Temporal domain Case B:

For temporal domain Case B, we understand that the UE can skip the measurement at some time instances and replace the measurement results for the skipped time instances with the predicted values. And the UE can trigger the measurement reporting and report the measurement results as in the legacy. The only difference is that some of the measurement results used for measurement reporting are not the actual measurement results but the predicted ones. More specifically, for event-triggered reporting, the UE can initiate the measurement reporting procedure when the entry condition applicable for the event is fulfilled by all actual and predicted measurement results during timeToTrigger. And the UE can report the latest (real/predicted) measurement results for the applicable cell. However, from NW’s perspective, there is no need to know whether the reported measurement result is real or predicted if we assume that the UE performs the prediction with good accuracy under RAN4 requirement. Therefore, we do not see any potential enhancement on the measurement report for temporal domain Case B.

Observation. 5: For temporal domain Case B, UE can trigger the measurement reporting and report the latest (predicted) measurement results as in the legacy. The only difference compared to the legacy is that some of the measurement results used for measurement reporting are not the actual measurement results but the predicted ones.

Proposal. 15: For temporal domain Case B, UE can trigger the measurement reporting and report the latest (real/predicted) measurement results as in the legacy. No further enhancement is needed for measurement (inference) reporting.

Frequency domain prediction:

For frequency domain prediction, UE can initiate the periodical or event-triggered reporting as in legacy and it can report the latest predicted measurement result for the target cells. Here, the target cells can be determined by the inference configuration from NW. For example, NW can explicitly configure the target cells or the target cell can be the Top-K neighbor cells for the target MO given by NW.

Proposal. 16: For frequency domain prediction, UE can report the latest predicted measurement result for the target cells. The target cells can be determined based on the inference configuration from NW.

Spatial domain prediction:

For spatial domain prediction, UE can initiate the periodical or event-triggered reporting as in legacy and it can report the latest predicted measurement result for the target cells. Here, the target cells can be determined by the inference configuration from NW. For example, NW can explicitly configure the target cells or the target cell can be the Top-K neighbor cells for the target MO given by NW. Also, the predicted measurement results can include cell-level and beam-level results for Sub-use case 1, and it can include only cell-level results for Sub-use case 3.

Proposal. 17: For spatial domain prediction, UE can report the latest predicted measurement result for the target cells. The target cells can be determined based on the inference configuration from NW.

Proposal. 18: For spatial domain prediction, the latest predicted measurement result for the target cells can include cell-level and/or beam-level results for Sub-use case 1, while include only cell-level results for Sub-use case 3.

L3 beam-level prediction:

For L3 beam-level prediction, UE can initiate the periodical or event-triggered reporting as in legacy and it can report the beam-level prediction results for the best candidate cell for the dedicated RACH resource configuration during HO. Here, the best candidate cell can be determined based on the actual or predicted measurement results. Also, the beam-level prediction results can include the predicted Top-K beam indexes and/or the predicted beam-level measurement results for the Top-K beams where the maximum beam number to report (i.e., K) is given by NW.

Proposal. 19: For L3 beam-level prediction, UE can report the beam-level prediction results for the best neighbor cell. The best neighbor cell can be determined based on the actual or predicted measurement results.

Proposal. 20: For L3 beam-level prediction, the beam-level prediction results for the best neighbor cell can include the predicted Top-K beam indexes and/or the predicted beam-level measurement results for the Top-K beams.

2.3 Performance monitoring

At the last RAN2#130, RAN2 made the following agreement for performance monitoring of UE-side model.

Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

For NW-sided monitoring, we understand UE should “periodically” report the measurements and inference output to NW, as NW performs the monitoring and the UE is not aware of the monitoring results (i.e., it cannot determine when reporting is necessary).

In contrast, for UE-sided monitoring, UE performs the monitoring and can get the monitoring results in real time (i.e., it can determine when reporting is required). In this case, UE can report monitoring result (for NW-decision) or management decision (for UE-decision) in an “event-triggered” manner based on network configuration (e.g., trigger reporting when the RSRP difference exceeds a NW-configured threshold).

Proposal. 21: For UE-sided model, NW-sided monitoring, “periodical” reporting can serve as the baseline. I.e., UE periodically report the measurement and inference output to NW.

Proposal. 22: For UE-sided model, UE-sided monitoring, “event-triggered” reporting can be the baseline. I.e., UE report monitoring result (for NW-decision) or management decision (for UE-decision) in event-triggered manner based on the NW configuration.

Since RAN2 agreed RSRP difference is used as the performance monitoring metric, the definition of “RSRP difference” should be clear without any ambiguity. In temporal domain prediction case A, UE can predict an RSRP value multiple times for one real value at the same time instance as the observation/prediction windows move forward (i.e., as long as the time instance of the real value belongs to prediction window). Also, it is obvious that the prediction performance tends to get better as the observation window gets close to the time instance of the real RSRP value. Similar to case A, in temporal domain prediction case B, if multiple RSRP values need be predicted with the same input sequence (e.g., when MRRT = 2/3, 4/5, …), the prediction accuracy will depend on the temporal gap between the measurement timing of the input and that of the real RSRP value (For measurement reduction actual RSRP values are not measured normally, but here it is assumed that actual RSRP values are acquired as commanded by NW for monitoring purpose). Therefore, in calculating L3 RSRP difference, the temporal gap between the measurement time of the input used for RSRP prediction and that of real RSRP value must be carefully considered.

Proposal. 23: For temporal domain prediction cases A and B, RAN2 need to clarify the definition of RSRP difference in WI phase (taking into account the time gap between the prediction time and the future time instance to predict.)

In frequency domain prediction, even though NW configures monitoring-related procedures to UE for performance monitoring purposes, UE may not be able to calculate L3 RSRP difference because UE does not have an ability to measure signals on multiple frequency bands simultaneously. How to define L3 RSRP difference in the above-mentioned scenario should be discussed in WI phase.

Proposal 24: For frequency domain prediction, RAN2 need to discuss how to calculate RSRP difference in WI phase (considering the case UE is not able to measure signals on multiple frequency bands simultaneously).

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Capability/Applicability reporting:

Proposal. 1: To facilitate discussion in AI/ML for mobility SI/WI, RAN2 adopt the definition of supported/applicable/activated functionalities agreed in Rel-19 AI/ML for air WI.

Proposal. 2: For AI/ML for mobility, UE can report the supported AI/ML-enabled Features/FGs and supported functionalities to NW via UECapabilityInformation message.

Observation. 1: Without option B, NW would need to verify the applicability of each configuration through "try-and-error" approach, making the applicability reporting procedure inefficient.

Proposal. 3: For AI/ML mobility use cases, support both option A and Option B like scheme.

Inference configuration:

Common part:

Proposal. 4: For AI/ML mobility use cases, NW can indicate the target MO (Measurement object) for which the UE performs the prediction (e.g., RRM measurement prediction) as part of inference configuration.

Proposal. 5: For AI/ML mobility use cases, NW can indicate NW-side additional condition (e.g., associated ID) for the applicability check as part of inference configuration if needed.

Temporal domain Case A:

Proposal. 6: For temporal domain Case A, NW can indicate the list of target cells for which the UE performs the prediction as part of inference configuration.

Temporal domain Case B:

Observation. 2: According to the legacy RAN4 requirement, the exact timing of SSB measurement within the measurement period is determined by UE implementation.

Proposal. 7: For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

Observation. 3: Increasing the periodicity in IE SSB-MTC for measurement reduction in Case B would also increase the measurement period, effectively relaxing the RAN4 requirement and potentially degrading HO KPI.

Proposal. 8: For skipping pattern configuration in Case B, we can introduce a separate/new IE to indicate the increased SSB interval while maintaining the measurement period as in legacy (based on the legacy IE SSB-MTC).

Proposal. 9: For temporal domain Case B, as long as the UE meets the RAN4 requirement on measurement period and accuracy, the details of how often and when to skip the measurement (i.e., predict measurement results) can be left to UE implementation.

Proposal. 10: For temporal domain Case B, NW can configure UE to report its measurement skipping pattern as an assistance data for NW.

Spatial domain prediction:

Observation. 4: The measured beam pattern is applicable to both Sub-use case 1 and 3, but the predicted beam pattern is applicable only to Sub-use case 1.

Proposal. 11: For spatial domain prediction (Sub-use case 1 and Sub-use case 3), the measured beam pattern can be configured by NW via the existing ssb-ToMeasure field.

Proposal. 12: For spatial domain prediction (Sub-use case 1), the predicted beam pattern can be configured by NW via a new field.

L3 beam-level prediction:

Proposal. 13: For L3 beam-level prediction, NW can indicate the maximum number of beams to report (i.e., K) as part of inference configuration (to configure UE to report Top-K beam indexes).

Inference report:

Temporal domain Case A:

Proposal. 14: For temporal domain Case A, UE can report the predicted results for the multiple time instances within PW for the target cells. The target cells can be determined based on the inference configuration from NW.

Temporal domain Case B:

Observation. 5: For temporal domain Case B, UE can trigger the measurement reporting and report the latest (predicted) measurement results as in the legacy. The only difference compared to the legacy is that some of the measurement results used for measurement reporting are not the actual measurement results but the predicted ones.

Proposal. 15: For temporal domain Case B, UE can trigger the measurement reporting and report the latest (real/predicted) measurement results as in the legacy. No further enhancement is needed for measurement (inference) reporting.

Frequency domain prediction:

Proposal. 16: For frequency domain prediction, UE can report the latest predicted measurement result for the target cells. The target cells can be determined based on the inference configuration from NW.

Spatial domain prediction:

Proposal. 17: For spatial domain prediction, UE can report the latest predicted measurement result for the target cells. The target cells can be determined based on the inference configuration from NW.

Proposal. 18: For spatial domain prediction, the latest predicted measurement result for the target cells can include cell-level and/or beam-level results for Sub-use case 1, while include only cell-level results for Sub-use case 3.

L3 beam-level prediction:

Proposal. 19: For L3 beam-level prediction, UE can report the beam-level prediction results for the best neighbor cell. The best neighbor cell can be determined based on the actual or predicted measurement results.

Proposal. 20: For L3 beam-level prediction, the beam-level prediction results for the best neighbor cell can include the predicted Top-K beam indexes and/or the predicted beam-level measurement results for the Top-K beams.

Performance monitoring:

Proposal. 21: For UE-sided model, NW-sided monitoring, “periodical” reporting can serve as the baseline. I.e., UE periodically report the measurement and inference output to NW.

Proposal. 22: For UE-sided model, UE-sided monitoring, “event-triggered” reporting can be the baseline. I.e., UE report monitoring result (for NW-decision) or management decision (for UE-decision) in event-triggered manner based on the NW configuration.

Proposal. 23: For temporal domain prediction cases A and B, RAN2 need to clarify the definition of RSRP difference in WI phase (taking into account the time gap between the prediction time and the future time instance to predict.)

Proposal 24: For frequency domain prediction, RAN2 need to discuss how to calculate RSRP difference in WI phase (considering the case UE is not able to measure signals on multiple frequency bands simultaneously).