3GPP TSG-RAN WG2 Meeting #131                                   R2-2505358

Bangalore, India,  Aug 25th -29th, 2025

Agenda item:		8.3.2.2

Source:				Samsung

Title:					Discussion on functionality management for RRM measurement event prediction

Document for:		Discussion & Decision

1	Introduction

In this paper, we would like to discuss the functionality management (i.e., applicability/capability reporting, inference configuration, inference report, monitoring) for RRM measurement event prediction.

2	Discussion

2.1 Clarification on measurement event prediction

Based on the discussion so far, the word "Measurement event prediction" has been used with two different understandings/concepts as below.

- Measurement event prediction for early report: UE predicts a future measurement event and initiates event-triggered reporting for the predicted event in advance before the actual event.

- Measurement event detection with measurement reduction (prediction): UE can detect a measurement event and initiates event-triggered reporting (for the current event) as in legacy. The only difference is that some of the measurement results used for the measurement reporting are not the actual measurement results but the predicted ones.

Measurement event prediction for early report can be regarded as temporal domain future measurement event prediction and this is aligned with the understanding during our evaluation phase. In our view, the early measurement event prediction/reporting is a new reporting procedure for which we need to study the spec. impact separately from RRM measurement prediction.

Measurement event detection with measurement reduction (prediction) can be regarded as the combination of legacy event-triggered reporting and measurement reduction (prediction). From our understanding, this can be naturally done by configuring the legacy event-triggered reporting together with temporal (Case B)/frequency/spatial domain prediction for measurement reduction. Therefore, we don’t need to study the spec. impact for this separately from RRM measurement prediction.

Observation. 1: The word "Measurement event prediction" has been used with two different understandings/concepts as below.

- Measurement event prediction for early report: UE predicts a future measurement event and initiates event-triggered reporting for the predicted event in advance before the actual event.

- Measurement event detection with measurement reduction (prediction): UE can detects a measurement event and initiates event-triggered reporting (for the current event) as in legacy. The only difference is that some of the measurement results used for the measurement reporting are not the actual measurement results but the predicted ones.

Observation. 2: Measurement event prediction for early report requires a new reporting procedure for which we need to study the spec. impact separately from RRM measurement prediction.

Observation. 3: Measurement event detection with measurement reduction (prediction) can be naturally done by configuring the legacy event-triggered reporting together with temporal (Case B)/frequency/spatial domain prediction for measurement reduction.

Proposal. 1: For spec. impact study on measurement event prediction, RAN2 can focus on measurement event prediction for early report (i.e., temporal domain event prediction).

2.2 Inference & reporting

For UE-side model inference, NW can provide some configuration for inference at UE-side. Then, the UE can perform the inference with the input data that is internally available at UE and report the outcome of the inference to NW. For the spec. impact, we can focus on the inference configuration from NW to UE and the report from UE to NW.

2.2.1 Inference configuration

At the last RAN2#129bis, RAN2 agreed the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. In our view, the existing RRM measurement configuration consisting of MeasObject/MeasId/ReportConfig can be easily extended/enhanced to support the use cases of AI for mobility. Therefore, in this part, we propose some potential enhancement based on the existing RRM measurement configuration.

Measurement event prediction:

For measurement event prediction, NW can configure UE to initiate the reporting procedure when the entry condition for the target event is predicted to be fulfilled for all measurements during timeToTrigger within PW. For that, NW can include the indication to configure UE to trigger the measurement reporting based on the prediction, target event (e.g., eventId for eventA3), and PW length in the inference configuration.

Proposal. 2: For measurement event prediction for early report, NW can include the indication to configure UE to initiate the measurement reporting procedure when it predicts the target event within PW as part of inference configuration.

Proposal. 3: For measurement event prediction for early report, NW can indicate the prediction window (PW) length (i.e., how far the UE should predict the measurement event in time domain) as part of inference configuration. (For indirect prediction, PW length of RRM measurement prediction can be reused for this.)

Proposal. 4: For measurement event prediction for early report, NW can indicate the target event (e.g., eventId for eventAx) for which the UE performs the measurement event prediction as part of inference configuration.

2.2.2 Inference report

At the last RAN2#129bis, RAN2 agreed the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. In our view, the existing measurement reporting mechanism can be simply extended/enhanced for UE to report the inference results of AI for mobility. Therefore, in this part, we propose some potential enhancement with the assumption of reusing the legacy measurement reporting mechanism.

Measurement event prediction:

For measurement event prediction, UE can trigger the event-triggered reporting based on the prediction. Specifically, the UE can initiate the measurement (inference) reporting procedure when the entry condition applicable for the event is predicted to be fulfilled by all measurement results for one or more applicable cells during timeToTrigger. And the UE can report the latest measurement result and/or the predicted results for the multiple time instances for the cells for which the entry condition is predicted to be fulfilled during timeToTrigger.

Proposal. 5: For measurement event prediction for early report, the UE can report the latest measurement result and/or the predicted results for the multiple time instances for the applicable cells. The applicable cells can be the cells for which the event is predicted.

When the UE predicts the event, it can report some additional information for the predicted event. For the additional information, the UE can report the expected occurrence time (for indirect method) or the probability of event occurrence within PW (for direct method).

Proposal. 6: For measurement event prediction for early report, the UE can report some additional information for the predicted event. The additional information can be the expected occurrence time (for indirect method) or the probability of event occurrence within PW (for direct method).

Meanwhile, the UE can continue the measurement event prediction even after the first event prediction report and the results of event prediction can be updated/changed from “predicted” to “not predicted”. In this case, the UE should be able to cancel the previous inference report to avoid wrong HO decision by NW based on the old inference report. Therefore, we see the need of supporting the inference report cancellation.

Proposal. 7: For measurement event prediction for early report, UE can continue the measurement event prediction after sending the first event prediction report. If the event prediction result is updated compared to the last report, the UE can report the updated result to NW.

2.3	Performance Monitoring

During evaluation phase, “F1 score” was used as a metric for prediction accuracy of measurement event prediction. Thus, we can consider to use it as a performance monitoring metric for measurement event prediction. However, if we think about it more, we can realize that it’s impractical for UE to get the label data (i.e., true/false/missed detection) for F1 score calculation because NW will hand over UE to a new target cell before the time of real event arrives. Therefore, in our view, “F1 score” can not be used as a metric for real time performance monitoring procedure.

Observation. 4: In real field, it’s impractical for UE to get the label data (i.e., true/false/missed detection) for F1 score calculation. “F1 score” cannot be used as a performance monitoring metric for measurement event prediction.

For indirect measurement event prediction, the output of RRM measurement prediction is reused for measurement event prediction. Thus, we can consider to reuse the monitoring metric of measurement prediction (i.e., RSRP difference) also for measurement event prediction.

Observation. 5: For indirect measurement event prediction, the performance monitoring metric agreed for RRM measurement prediction (i.e., RSRP difference) can be reused.

For direct measurement event prediction, it operates independently from RRM measurement prediction. Thus, the monitoring metric of RRM measurement prediction (i.e., RSRP difference) cannot be reused. To the best of our knowledge, there is no feasible performance monitoring metric for direct measurement event prediction.

Observation. 6: For direct measurement event prediction, there is no feasible performance monitoring metric.

Proposal. 8: For measurement event prediction, RAN2 support only indirect prediction in WI phase.

Proposal. 9: For measurement event prediction, RSRP differences can be used as the performance metric for monitoring.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Clarification on measurement event prediction:

Observation. 1: The word "Measurement event prediction" has been used with two different understandings/concepts as below.

- Measurement event prediction for early report: UE predicts a future measurement event and initiates event-triggered reporting for the predicted event in advance before the actual event.

- Measurement event detection with measurement reduction (prediction): UE can detects a measurement event and initiates event-triggered reporting (for the current event) as in legacy. The only difference is that some of the measurement results used for the measurement reporting are not the actual measurement results but the predicted ones.

Observation. 2: Measurement event prediction for early report requires a new reporting procedure for which we need to study the spec. impact separately from RRM measurement prediction.

Observation. 3: Measurement event detection with measurement reduction (prediction) can be naturally done by configuring the legacy event-triggered reporting together with temporal (Case B)/frequency/spatial domain prediction for measurement reduction.

Proposal. 1: For spec. impact study on measurement event prediction, RAN2 can focus on measurement event prediction for early report (i.e., temporal domain event prediction).

Inference configuration:

Proposal. 2: For measurement event prediction for early report, NW can include the indication to configure UE to initiate the measurement reporting procedure when it predicts the target event within PW as part of inference configuration.

Proposal. 3: For measurement event prediction for early report, NW can indicate the prediction window (PW) length (i.e., how far the UE should predict the measurement event in time domain) as part of inference configuration. (For indirect prediction, PW length of RRM measurement prediction can be reused for this.)

Proposal. 4: For measurement event prediction for early report, NW can indicate the target event (e.g., eventId for eventAx) for which the UE performs the measurement event prediction as part of inference configuration.

Inference report:

Proposal. 5: For measurement event prediction for early report, the UE can report the latest measurement result and/or the predicted results for the multiple time instances for the applicable cells. The applicable cells can be the cells for which the event is predicted.

Proposal. 6: For measurement event prediction for early report, the UE can report some additional information for the predicted event. The additional information can be the expected occurrence time (for indirect method) or the probability of event occurrence within PW (for direct method).

Proposal. 7: For measurement event prediction for early report, UE can continue the measurement event prediction after sending the first event prediction report. If the event prediction result is updated compared to the last report, the UE can report the updated result to NW.

Performance monitoring:

Observation. 4: In real field, it’s impractical for UE to get the label data (i.e., true/false/missed detection) for F1 score calculation. “F1 score” cannot be used as a performance monitoring metric for measurement event prediction.

Observation. 5: For indirect measurement event prediction, the performance monitoring metric agreed for RRM measurement prediction (i.e., RSRP difference) can be reused.

Observation. 6: For direct measurement event prediction, there is no feasible performance monitoring metric.

Proposal. 8: For measurement event prediction, RAN2 support only indirect prediction in WI phase.

Proposal. 9: For measurement event prediction, RSRP differences can be used as the performance metric for monitoring.