3GPP TSG-RAN WG2 Meeting#131                               	R2-2505369

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:	8.9.3

Source: 	Google

Title: 	Leftover issues on CB-Msg3-EDT

Document for:	Discussion and Decision

# Introduction

In Rel-19, the introduction of the Msg3 based EDT procedure without Msg1/RAR transmissions is considered as one of the enhancements for boosting uplink capacity and the following scopes are included in the revised WID of Rel-19 IoT-NTN [1].

- Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

Significant progress was made in the past few RAN2 meetings, on the CB-RNTI calculation, CB-Msg3 transmission window, CB-Msg3 response timer, and the format of CB-Msg4 [2][3][4]. There are still some leftover issues with regard to how a UE ends/terminates a CB-Msg3-EDT procedure upon receiving a matching contention resolution identity, which are discussed in this paper with specific proposals being proposed.

# Discussion

## End a CB-Msg3-EDT procedure without an RRC message

In the legacy RA preamble-based MO-EDT, after receiving an RRCEarlyDataRequest from a UE, the eNB can first send a contention resolution identity to the UE to terminate the RA procedure triggered by the MO-EDT. Subsequently, it can determine whether to: 1) transmit a standalone RRCEarlyDataComplete to the UE, 2) transmit an RRCEarlyDataComplete concatenated with available DL data, or 3) transmit an RRCConnectionSetup to the UE, depending on at least the availability of DL data and traffic characteristic. This approach provides the eNB additional time to await potential DL data before terminating the MO-EDT procedure.

For RA preamble based MO-EDT, eNB can send the Contention Resolution Identity to the UE at the earliest timing to terminate the RA procedure first, and determine at a later time whether to concatenate the DL data (if available) with the RRCEarlyDataComplete message. This allows the eNB to wait for potential DL data before terminating the MO-EDT procedure.

In RAN2#130 meeting, it was agreeed that a CB-Msg4 without an RRC message (but with contention resolution identity) is allowed as a complete response to the CB-Msg3 in the CP solution. The intention is to save the RRC message overhead in the situation when the network intends to end a CB-Msg-EDT procedure and keep the UE in RRC_IDLE if no further data are expected.

Agreement in RAN2#130: - A CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution

Such an enhancement works well when there is no further DL data, but presents challenges when DL data arrives after the CB-Msg3ResponseTimer for monitoring the CB-Msg4 expires. Since now sending a Contention Resolution Identity to a UE would terminate the EDT procedure, an eNB cannot hold the RRCEarlyDataComplete (and wait for possible DL data) for too long, otherwise the UE may reattempt the CB-Msg3-EDT procedure upon the expiry of the CB-Msg3ResponseTimer. Consequently, the eNB's waiting period for DL data is limited by the CB-Msg3ResponseTimer's duration, thereby reducing the likelihood of completing a bidirectional transaction within a single CB-Msg3-EDT procedure.

For CB-Msg3-EDT, as sending a Contention Resolution Identity to a UE would terminate the EDT procedure, an eNB cannot hold the RRCEarlyDataComplete (and wait for possible DL data) for too long, otherwise the UE may reattempt the CB-Msg3-EDT procedure upon the expiry of the CB-Msg3ResponseTimer.

To address this issue, including UE’s C-RNTI in the CB-Msg4 (without an RRC message) can be considered as an indication that the eNB intends to await potential DL data. In such a scenario, the UE continues monitoring PDCCH for receiving an RRC message upon receiving the CB-Msg4, provided it includes a matching Contention Resolution Identity and a C-RNTI. On the other hand, upon receiving the CB-Msg4 including only the matching Contention Resolution Identity (without including a C-RNTI or an RRC message), the UE considers the CB-Msg3-EDT procedure ends and remains in RRC_IDLE.

Upon receiving a matching Contention Resolution Identity and a C-RNTI in a CB-Msg4, the UE does not consider the CB-Msg3-EDT procedure ends and continues monitoring PDCCH for receiving an RRC message responding to the RRCEarlyDataRequest.

Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without including an RRC message or a C-RNTI, the UE terminates the CB-Msg3-EDT procedure and keeps in RRC_IDLE.

## UE assisted CB-Msg3-EDT procedure

Assuming P1 and P2 are agreed upon, a Rel-19 UE should be able to differentiate between three cases after receiving CB-Msg4: 1) CB-Msg4 includes a matching Contention Resolution Identity and an RRCEarlyDataComplete, 2) CB-Msg4 only includes a matching Contention Resolution Identity, and 3) CB-Msg4 includes a matching Contention Resolution Identity and a C-RNTI. The UE shall complete the CB-Msg3-EDT procedure in case 1) and case 2), and shall continue with the CB-Msg3-EDT procedure in case 3). Since legacy UEs always rely on an RRC message to finish a MO-EDT procedure, the ability to distinguish between these cases and to terminate an EDT procedure without an RRC message should be communicated to the eNB as part of the UE's capabilities.

The UE indicates to the network, whether it is able to terminate a CB-Msg3-EDT procedure upon receiving a CB-Msg4 containing no RRC message, as one of its capabilities.

If the eNB knows that there will be no DL data following the CB-Msg3 and would like to terminate the CB-Msg3-EDT procedure immediately with CB-Msg4, the eNB would use either case 1) or case 2) to terminate the CB-Msg3-EDT procedure, depending on UE’s capability reported in P3. If the eNB has no idea about whether there could be DL data following the CB-Msg3, it can use case 3) to extend UE’s active time if needed, regardless of UE’s capability reported in P3. Since in some situations the UE knows much better (than the eNB does) on the likelihood of having DL data following the CB-Msg3, it is beneficial if the UE can indicate in CB-Msg3, whether DL data following the UL data in CB-Msg3 is expected or not.

The UE can indicate in CB-Msg3, whether DL data following the UL data in CB-Msg3 is expected or not.

# Conclusion

In this paper, we discuss the leftover issues on the UE handling upon receiving a CB-Msg4 including no RRC message. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Observation 1	For RA preamble based MO-EDT, eNB can send the Contention Resolution Identity to the UE at the earliest timing to terminate the RA procedure first, and determine at a later time whether to concatenate the DL data (if available) with the RRCEarlyDataComplete message. This allows the eNB to wait for potential DL data before terminating the MO-EDT procedure.

Observation 2	For CB-Msg3-EDT, as sending a Contention Resolution Identity to a UE would terminate the EDT procedure, an eNB cannot hold the RRCEarlyDataComplete (and wait for possible DL data) for too long, otherwise the UE may reattempt the CB-Msg3-EDT procedure upon the expiry of the CB-Msg3ResponseTimer.

Proposal 1	Upon receiving a matching Contention Resolution Identity and a C-RNTI in a CB-Msg4, the UE does not consider the CB-Msg3-EDT procedure ends and continues monitoring PDCCH for receiving an RRC message responding to the RRCEarlyDataRequest.

Proposal 2	Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without including an RRC message or a C-RNTI, the UE terminates the CB-Msg3-EDT procedure and keeps in RRC_IDLE.

Proposal 3	The UE indicates to the network, whether it is able to terminate a CB-Msg3-EDT procedure upon receiving a CB-Msg4 containing no RRC message, as one of its capabilities.

Proposal 4	The UE can indicate in CB-Msg3, whether DL data following the UL data in CB-Msg3 is expected or not.

# Reference

RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

R2-2501702, Report of 3GPP TSG RAN WG2 meeting #129, ETSI MCC.

R2-2503302, Report of 3GPP TSG RAN WG2 meeting #129bis, ETSI MCC.

Draft Report of 3GPP TSG RAN WG2 meeting #130, ETSI MCC.