3GPP TSG-RAN WG2 Meeting #131										R2-2505376

Bengaluru, India, Aug 25th – 29th, 2025

Source: 		ZTE Corporation, Sanechips

Title:                 Open issues for random access

Agenda item:   8.2.3

Document for: Discussion and Decision

# Introduction

In this contribution we take a look at the remaining open issues for RACH procedure.

# CBRA failure detection

At RAN2#130 meeting it was agreed that the MSG2 monitoring window length will be limited and will not be extended until the subsequent paging message. Further, it was agreed that a device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further down-selected between two options below:

The device does not process MSG2 (re)transmission received after the boundary.

- Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message

- Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS)

Advantage of option B is that it is simple and has a well bounded end for the CBRA failure detection. Option C on the other hand has the following issues:

- It requires the device to keep count of a number of trigger messages

- It increases the overall collision probability as any device transmitting with “k” trigger messages may collide and hence increases the overall failure rate

Given the above and the lack of a clear use case for k>1, it is proposed to adopt option B.

Proposal 1 (Issue 2-4): For MSG2 retransmission window, adopt option B i.e. the device does not process MSG2 (re)transmission received after the subsequent R2D trigger message or the paging message whichever is earlier

# Frequency index indication in MSG2

One of the open issues for CBRA is whether to include frequency index along with the RN16 in MSG2 to reduce collisions of MSG1 between devices. As noted in the previous section, the collision probability is a function of the MSG2 window size and the total number of contending devices on a given slot. Since slotted aloha algorithm will anyway distribute the devices uniformly with the entire paging frame, the collision probability is very minimal if k = 1 is adopted for MSG2 window size. Thus, the usefulness of having a separate frequency index indication is really not that critical in this case. Furthermore, requiring the device to check the frequency index along with the RN16 echoed in MSG2 will increase device complexity. Thus, it is proposed to avoid this.

Proposal 2: (Issue 2-5): There is no need to include the frequency index along with RN16 in MSG2

# Conclusion

Proposal 1 (Issue 2-4): For MSG2 retransmission window, adopt option B i.e. the device does not process MSG2 (re)transmission received after the subsequent R2D trigger message or the paging message whichever is earlier

Proposal 2: (Issue 2-5): There is no need to include the frequency index along with RN16 in MSG2

- References