3GPP TSG-RAN WG2 Meeting #131		R2-2505400

Bengaluru, India, 25th – 29th Aug. 2025

Source:	vivo

Title:		Discussion on LTM measurement event evaluation and reporting

Agenda Item:		8.6.3

Document for:	Discussion and Decision

- Introduction

For the agenda item 8.6.3 (L1 event triggered measurement reporting), most open issues are closed. This contribution will discuss the remaining issues that need to be clarified, to help to finalize the specifications.

- Discussion

In [1], the open issues for the agenda 8.6.3 (L1 event triggered measurement reporting) are summarized:

| MAC-8 | FFS the coexistence between event triggered report and mTRP, e.g. how UE determine the current beam or candidate beam in this case? |
| --- | --- |
| MAC-24 | It is still working assumption on the details for truncated L1 MR MAC CE Editor’s NOTE: It is working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE. |
| MAC-25 | Handling of the activated SP CSI-RS for target cell after LTM cell switch Editor’s NOTE: After reconfiguration with sync that is triggered by LTM, it is FFS how to handle the configured Semi-persistent CSI-RS/CSI-IM resource sets for target cell. Based on the following agreement: UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell |

As further comments from companies, the MAC-8 will be discussed in another summary report of [POST130][117], MAC-24 may be resolved by proposing to confirm the working assumption straightforwardly, and MAC-25 will be handled in agenda 8.6.2, according to chairman guidelines. Therefore, it seems no contribution for these three issues are needed.

Observation 1: MAC-8/24/25 does not need further contribution input in agenda item 8.6.3 and will be handled in other agenda/ email thread.

On the other hand, there are other issues which may need discussion/ clarification:

| Samsung | We may need an open issue for reporting of beam measurements for LTM2, to be discussed in next meeting. Here are some aspects which are not clear in current CRs. 1.Whether the UE reports the current beam for LTM2 irrespective of the reportCurrentBeam or whether the network will alway configure the reportCurrentBeam while LTM2 is configured. There is no conditional presence defined in RRC spec for LTM2, and it is not clearly mentioned in the MAC spec that UE always reports current beam. 2. allowReportAnyBeam indicates whether the UE can report the measurement results for the beams not satisfying the conditions of the events. However it is not clear whether other beams of the serving cell can be reported, as those beams are not evaluated (and hence we can not say they are “not satisfying the LTM2 condition”) 3. In running CR,the Type field is specified to indicates the type of the RS i of LTM candidate cell. LTM2 is associated with the current beam and not any candidate cell. So it is not clear what needs to be set. |
| --- | --- |

In current running CR[2], the reportCurrentBeam is defined as follows:

-	reportCurrentBeam: whether the UE is required to report the measurement result of the current beam.

In our understanding, when we reached the agreements (as follows [3]) related to reportCurrentBeam, we mainly focused on the LTM events other than LTM2.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

As reportCurrentBeam is optional field in TS 38.331, it is fine that we clarify the correct UE/Network behaviour for LTM2, e.g.

- Option 1: The UE should always report current beam for LTM2 even reportCurrentBeam is not configured by network;

- Option 2: The UE assumes reportCurrentBeam is always configured by network for report associated to LTM2.

We understand both can work, for option-1, as the current beam is the beam used for evaluation for LTM2, it is straightforward that current beam should be reported anyway, and the following NOTE3[2] also clearly states where to put it for LTM2:

- NOTE 3: For the measurement report triggered by LTM2, the RS with Type of 00 is the current beam, which is always included in the last octet.

On the other hand, it may be clearer to go for option-2 in the spec and avoid further ambiguity. Considering there is not much spec impact, we can try to go for option-2.

Proposal 1: For LTM2, UE assumes reportCurrentBeam is always configured by network for report associated to LTM2.

The next parameter that may need clarification is allowReportAnyBeam:

-	allowReportAnyBeam: whether the UE can report the measurement results for the beams not satisfying the conditions of the events;

For LTM2, we agreed that current beam should be reported, as well as the LTM candidate beam associated with the report ID for LTM2[4]. If allowReportAnyBeam is configured for LTM2, then we understand the beams of serving cell other than current beam, should not be reported, because:

- They are not evaluated for LTM2 of this report configuration;

- We use one octet to put current beam at the last of L1 MR MAC CE and there is no room for beams of serving cell other than current beam, and if we put them in the octets in front, we have to set RSRI for them and also this is not aligned with our previous agreement to only include RS(s) associated with the report ID for LTM2.

Observation 2: When allowReportAnyBeam is configured for LTM2, still that only current beam and candidate beam(s) associated with the report ID for LTM2, will be included in L1 MR MAC CE.

- Conclusion

In this contribution, we discussed some remaining issues that needs further clarification for LTM2 in L1 LTM measurement event evaluation and reporting.

We have the following Observations and Proposals:

Observation 1: MAC-8/24/25 does not need further contribution input in agenda item 8.6.3 and will be handled in other agenda/ email thread.

Proposal 1: For LTM2, UE assumes reportCurrentBeam is always configured by network for report associated to LTM2.

Observation 2: When allowReportAnyBeam is configured for LTM2, still that only current beam and candidate beam(s) associated with the report ID for LTM2, will be included in L1 MR MAC CE.

- References

- R2-2505398, Discussion summary and list of MAC open issue for Mob Ph4, vivo

- R2-2505397, Introduction of NR mobility enhancements Phase 4 in MAC, vivo

- RAN2 #127bis meeting chair notes;

- RAN2 #130 meeting chair notes;