3GPP TSG-RAN WG2 Meeting#131       			                                                                   R2-2505429

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.2.2

Source:	ETRI

Title:	Remaining open issues on paging

Document for:	Discussion and Decision

# Introduction

In last RAN2#130 meeting, followings are agreed for parallel service requests and A-IoT paging[1].

Agreements on parallel service request - Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in stage 2 specification. - The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in stage 3 specification. - Send LS to RAN3 to notify them of agreements 1 and 2 - Parallel service request for overlapping reader scenario can be addressed in Rel-20

Agreements on paging - For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent. FFS on the need for the transaction ID for command case. - For CFRA, the device always responds to paging regardless of transaction ID (if we put a transaction ID) (i.e. as long as it is addressed to the corresponding device). - To ensure forward compatibility for paging with multiple identifiers, introduce at least one R field. FFS if more than one R bit is required. - Rel-19 devices would ignore the content of future release instead of ignoring the whole paging message. - Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS

In this paper, we discuss remaining issues for A-IoT paging and propose our views.

# Discussions

## Issue 1-2, 1-5: Transaction ID

In email discussions about the A-IoT MAC open issues, two key questions have been raised regarding the Transaction ID. The first is how the Transaction ID should be generated, and the second is its appropriate length. The Transaction ID is used to identify a specific service. It can be designed to have a one-to-one mapping with the Correlation ID from the Core Network (CN). This implies that the Transaction ID is unique between a device and a reader for a specific time. Given this uniqueness, the specification does not need to dictate how the Transaction ID is generated; instead, that is left to the NW implementation.

In addition to service identification, the Transaction ID is also used to distinguish between individual services requested by the CN. Consequently, its length should consider both the maximum number of simultaneous services and the number of readers in a multi-reader environment. This becomes particularly important in a Rel-20 UE-based multi-reader scenario, where UEs may face difficulties in coordinating to avoid multiple paging requests for the same service from the CN. Therefore, we believe that a minimum of 4 bits should be allocated for the Transaction ID to ensure it can effectively distinguish between services and provide sufficient scalability for future releases.

Proposal 1. The transaction ID is generated by NW implementation and its length is at least 4 bits.

Another issue under discussion is the necessity of including a Transaction ID in CFA-based paging. The primary role of the Transaction ID in a paging message is to prevent redundant responses to the same service. In CBRA, the Transaction ID also helps determine whether a paging message is intended for re-access.

However, in CFA, NACK-based re-access is not supported, and devices are expected to respond to every newly received paging message. Even in cases where the reader fails to receive a response due to transmission errors, it can simply retransmit the paging message. As a result, the Transaction ID does not provide additional benefits in this context. Therefore, we do not see a strong justification for including a Transaction ID in paging messages used for CFA.

Proposal 2. A Transaction ID is not required in paging message for CFA-based access.

## Issue 1-3: Paging ID length field

The Paging ID in an A-IoT paging message can be either an A-IoT device permanent identifier or filtering information, both provided by CN.

The A-IoT device permanent identifier is used to uniquely identify an A-IoT device and consists of multiple components, such as the identifier type, domain information, and identification information. Filtering information, by contrast, is a subset of this permanent identifier and is used to identify or filter multiple A-IoT devices. It may consist of one or more components of the permanent identifier or be represented as a bitstring covering selected components.

Since certain components—such as the PLMN ID and the identification information—can have variable lengths, both the device permanent identifier and filtering information can also vary in size. As a result, the Paging ID in the A-IoT paging message should support variable length.

To address this, RAN2 sent a LS to CT4 and SA2 to determine the appropriate length requirement for the Paging ID. In their response, CT4 and SA2 indicated that the maximum size of filtering information is 256 bits. Accordingly, an 8-bit length field is required to represent all possible Paging ID sizes.

Proposal 3. An 8-bit field should be included to indicate the length of the Paging ID in the A-IoT paging message.

## Issue 1-4: Number of AO field

The number of access occasions (AO) indicates the total number of access opportunities between two paging messages. In the last RAN2 meeting, it was decided that this field would be 4 bits long, with its value interpreted using an exponential mapping.

However, a 4-bit exponential scheme may not perfectly represent all possible totals number of access occasions. This is because the total number of AOs is determined by the product of three factors: the number of time-domain resources (X), the number of frequency-domain resources (Y), and the number of access trigger messages. Since this product may not be a power of two, some AO values may not exactly align with the exponential representation and could go unused.

For example, consider a paging round with 12 AOs, as illustrated in the Figure 1. If the field is set to a value of 3 (corresponding to 8 AOs), then AOs numbered 8 through 11 would never be used for CFRA, resulting in resource waste. Conversely, if the field is set to 4 (corresponding to 16 AOs), a device could select one of the non-existent AOs in the rage of 12 to 15, leading to re-access due to failed random access.

Despite this, if the number of AOs within a paging round is configured to the nearest power of two, the inefficiency introduced by the small number of unused AOs is marginal. The simplicity and resource efficiency offered by a 4-bit exponential encoding outweigh this drawback.

Proposal 4. The number of AO field has a 4-bit length and its value is interpreted using an exponential mapping.

Figure 1. An example of resource configuration for CBRA

## Issue 4-4: MAC spec implementation (R field)

In the last RAN2 meeting, it was agreed to introduce a reserved (R) field to ensure forward compatibility for paging messages containing multiple identifiers. The purpose of the R field is to support the coexistence of multiple releases, allowing newer versions of the specification to add fields while maintaining interoperability with Rel-19 devices.

Our understanding is that when the R field is configured, Rel-19 devices will ignore any unknown fields—i.e., content introduced in future releases—thereby enabling backward compatibility. To enable this differentiation, a single bit is sufficient to distinguish between paging messages intended for Rel-19 and those extended in later releases.

Proposal 5. A 1-bit R field is sufficient to distinguish between paging messages from Rel-19 and those from future releases.

## Issue 4-5: Forward compatibility

Another issue related to forward compatibility is whether similar considerations apply beyond the A-IoT paging message. We believe that the same mechanism—such as the reserved (R) field—could potentially be extended to other R2D messages, if a need for forward compatibility arises in future releases (e.g., due to new traffic types or deployment scenarios in Rel-20 and beyond).

However, based on the current set of requirements, we have not identified any other R2D messages that would necessitate the inclusion of a specific forward compatibility mechanism at this stage.

Proposal 6. No other R2D message requires a specific mechanism for forward compatibility.

# Conclusions

Based on the discussion in the contribution, we have the following proposals:

Proposal 1. The transaction ID is generated by NW implementation and its length is at least 4 bits.

Proposal 2. A Transaction ID is not required in paging message for CFA-based access.

Proposal 3. An 8-bit field should be included to indicate the length of the Paging ID in the A-IoT paging message.

Proposal 4. The number of AO field has a 4-bit length and its value is interpreted using an exponential mapping.

Proposal 5. A 1-bit R field is sufficient to distinguish between paging messages from Rel-19 and those from future releases.

Proposal 6. No other R2D message requires a specific mechanism for forward compatibility.

# References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #130”, May. 2025

- 3GPP TS 23.369, Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Rel-19), June, 2025.