3GPP TSG-RAN WG2 Meeting #131	R2-2505435

Bengaluru, India, 25-29 August, 2025

Agenda Item:	8.13.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on service continuity for Multi-hop Relay

Document for:	Discussion and Decision

# Introduction

In the WID for NR sidelink multi-hop relay in [1], we have the following description for service continuity scenarios for multi-hop U2N relay.

Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this paper, we discuss additional aspects of the two scenarios, namely Scenarios C and D, and provide the proposals to update the description of signalling flows for these two scenarios in the running stage 2 CR.

# Discussion

## 2.1 Candidate Target Path determination by the remote UE for Scenario C and D

As indicated in the WID,

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

In RAN2#130 meeting, it was agreed to introduce indication information in the discovery message to indicate whether the relay UE is in RRC_CONNECTED state.

Working assumption: The Relay UE includes an indication of whether it is RRC_CONNECTED in the discovery message RRC container. As a baseline, remote UE relay (re)selection behaviour based on this information is left to implementation, and it can be discussed in spec implementation/maintenance if some guidance is needed. No SA2 spec impact is expected.

Therefore, if the first relay UE provides its RRC state information in the discovery message and is in the RRC_CONNECTED state, the remote UE can infer that all upstream relays in the multi-hop chain are also in the RRC_CONNECTED state. This enables the remote UE to select an appropriate path where all relay UEs are in the RRC_CONNECTED state, ensuring service continuity for scenarios C and D

Proposal 1: The remote UE shall obtain the RRC state information of the first relay UE from the discovery message, enabling selection of an appropriate candidate path in which all relay UEs are in the RRC_CONNECTED state for Scenarios C and D.

## 2.2 Open Issue RRC-14: Clarification on Reporting of Target Relay UEs in RRC_CONNECTED State for Scenarios C and D

Some clarification is needed in the specification to explicitly state that Remote UEs shall only report target relay UEs that are in RRC_CONNECTED state for Scenario C and Scenario D. This ensures consistent behavior across implementations and avoids ambiguity regarding the state of candidate relay UEs during reporting procedures.

Since the Relay UE includes an indication of its RRC_CONNECTED status in the discovery message RRC container. The Remote UE can utilize this indication to determine the connection state of each candidate Relay UE and subsequently report only those candidate First /Intermediate Relay UEs that are in the RRC_CONNECTED state. Hence we have the following proposal

Proposal 2: Clarify in TS 38.300 that, for Scenarios C and D, the Remote UE shall report only those Relay UEs in the RRC_CONNECTED state for path switching, based on the Relay State indication received in the discovery message RRC container.

## 2.3 Multi-hop target relaying path selection

Unlike the conventional single hop direct-to-indirect path switching, the gNB needs to select a multi-hop relaying path based on the measurement reporting, not just a target relay UE, for the remote UE to do path switching. The following information can help gNB to choose a better target path.

- Relay UE’s serving cell ID, and the sidelink measurement quantity between the remote UE and the first relay UE.

- The sidelink measurement quantity between the adjacent relay UEs on the candidate multi-hop relaying path.

- The hop number of the relaying path.

To capture this, we have the following proposal

Proposal 3: The gNB can determine the target multi-hop relaying path based on the following information:

- First Relay UE’s serving cell ID, and the sidelink measurement quantity between the remote UE and the first relay UE.

- The sidelink measurement quantity between the adjacent relay UEs on the candidate multi-hop relaying path.

- The hop number of the relaying path

For the above proposal, the information in the first bullet could already be contained in the measurement reporting by reusing the Rel-17 mechanism.

The information in the second bullet  is important, since the sidelink measurement quantity on each hop can impact the data forwarding of the remote UE. Based on legacy mechanism, remote UE can only measure and report the sidelink measurement quantity of the first relay UE. Therefore, enhancements are needed to take the sidelink measurement quantity between adjacent relay UEs along the candidate relaying path into consideration when selecting the target relaying path.

One possible solution is each relay UEs can indicates the sidelink measurement quantity of its adjacent relay UE (in upstream direction) to the its adjacent relay UE in the downstream direction.  With this method, the first relay UE can get all the sidelink measurement quantity of each hop of the Relay UEs along its relaying path, then it can forward the sidelink measurement quantity to the remote UE in the discovery message. Remote UE can use the individual sidelink measurement quantity to determine candidate relaying path, or just report this information to gNB for target path determination.

Proposal 4: The remote UE can obtain the sidelink measurement quantity information between each adjacent relay UEs along the candidate multi-hop relaying path to help determining a better target path. This information can be provided by the first relay UE in the discovery message.

## 2.4 How to restrict inter-gNB direct/single-hop to multi-hop indirect path switching

In Rel-18, inter-gNB indirect-to-indirect path switching is supported, whereas inter-gNB direct-to-multi-hop path switching and single-hop-to-multi-hop path switching are not supported in this release.. When performing measurements for Service Continuity Scenarios C and D, a Remote UE may discover one or more candidate Relay UEs in the RRC_CONNECTED state belonging to different gNBs (see Figure 1) and report them to the serving gNB. Mechanisms are needed to restrict the gNB from selecting a target Relay UE that connects to another gNB via an indirect path (i.e., a target multi-hop relay path for the Remote UE in another gNB), as this may lead to path switching failure.

.

Figure 1. Remote UE discovers candidate Relay UEs

Possible restriction mechanisms:

- UE capability–based restriction

One simple solution to restrict inter gNB path switch for Scenario C and D is that upon receiving a measurement report containing a candidate relay UE that does not belong to the serving gNB cells, the serving gNB evaluates the remote UE's capabilities to determine whether the Remote UE is a Rel-18 or Rel-19 Remote UE before initiating an inter-gNB handover or path switch and then takes the decision as follows.

- If the Remote UE is a Rel-18 Remote UE, the serving gNB may initiate an inter-gNB handover by sending a Handover Request to the target gNB, thereby allowing an indirect-to-indirect single-hop path switch.

- If the Remote UE is a Rel-19 Remote UE, the serving gNB does not initiate an inter-gNB path switch toward the target gNB. This is because it cannot be guaranteed that the inter-gNB candidate relay UE reported by the Remote UE on the target path has a single-hop.

This restriction is required because, in the current release, inter-gNB path switches from an indirect single-hop to an indirect multi-hop configuration are not supported.

- Indication based restriction

If the serving gNB cannot identify the candidate Relay UE reported by the Remote UE, it may determine that the candidate Relay UE is in a neighbouring gNB cell. However, the serving gNB cannot know whether that candidate Relay UE is performing a single-hop or multi-hop relay operation.

In this case, the serving gNB may send a Handover Request to the neighbouring gNB, which could respond with a Handover Failure if the candidate Relay UE is performing multi-hop operation (since inter-gNB multi-hop path switch is not supported in this release).

An alternative to avoid such failures is to include an additional indication in the measurement report to specify whether the candidate Relay UE is on a single-hop or multi-hop target path, or to directly provide the hop count information.

Proposal 5: Either the serving gNB shall only allow inter-gNB path switching from indirect (single-hop) to indirect (single-hop) paths for Rel-18 UEs, or RAN2 shall consider adding an indication in the measurement report to specify whether the candidate Relay UE is on a single-hop or multi-hop target path, or to provide the hop count information.

# Conclusion

In this paper, we discuss the some aspects for scenarios C and D, and the enhancements for the path information reporting and path selection.  We have the following observation and proposals.

Candidate Target Path determination by the remote UE for Scenario C and D.

Proposal 1: The remote UE shall obtain the RRC state information of the first relay UE from the discovery message, enabling selection of an appropriate candidate path in which all relay UEs are in the RRC_CONNECTED state for Scenarios C and D.

Open Issue RRC-14: Clarification on Reporting of Target Relay UEs in RRC_CONNECTED State for Scenarios C and D.

Proposal 2: Clarify in TS 38.300 that, for Scenarios C and D, the Remote UE shall report only those Relay UEs in the RRC_CONNECTED state for path switching, based on the Relay State indication received in the discovery message RRC container.

Multi-hop target relaying path selection

Proposal 3: the gNB can determine the target multi-hop relaying path based on the following information:

- First Relay UE’s serving cell ID, and the sidelink measurement quantity between the remote UE and the first relay UE.

- The sidelink measurement quantity between the adjacent UEs on the candidate multi-hop relaying path.

- The hop number of the relaying path

Proposal 4: The remote UE can obtain the sidelink measurement quantity information between each adjacent relay UEs along the candidate multi-hop relaying path to help determining a better target path. This information can be provided by the first relay UE in the discovery message.

How to restrict inter gNB path switch Scenarios C and D

Proposal 5: Either the serving gNB shall only allow inter-gNB path switching from indirect (single-hop) to indirect (single-hop) paths for Rel-18 UEs, or RAN2 shall consider adding an indication in the measurement report to specify whether the candidate Relay UE is on a single-hop or multi-hop target path, or to provide the hop count information.

# References

[1] RP-242349 Revised WID SL_relay_multihop

[2] 3GPP TS 38.300, V18.3.0, NR; NR; NR and NG-RAN Overall Description

[3] RAN 2 # 130 - Report of session on positioning and sidelink relay